3GPP TSG-SA5 Meeting #151 	S5-236618

Xiamen, China, 9 - 13 October 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.255 | CR | 0450 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add 5G MBS charging procedure for multicast and broadcast communication creation and deletion |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS_CH |  |  |  |  |  | Date: |  |  | 2023-09-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To support 5G MBS charging, the MB-SMF shall support MBS session charging. Thus, the charging procedures should be added based on MBS procedures defined in TS 23.247, in order to clarify the interactions between MB-SMF and CHF for charging data related to MBS sessions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Charging procedures are added based on the procedures for broadcast and multicast communication defined in TS 23.247, including: - MBS session creation for both broadcast and multicast - MBS session deletion for both broadcast and multicast |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The charging procedures for 5G MBS are not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

5.2.2.X		5G MBS charging for multicast and broadcast communciation

5.2.2.X.1		General

This clause specifies MBS session charging for multicast and broadcast communication as defined in TS 23.247 [204].

5.2.2.X.2		MBS session creation

The following figure X.3.2.1.2-1 describes the charging procedure for MBS session creation, based on TS 23.247 [204] figure 7.1.1.2-1 and 7.1.1.3-1.

Figure 5.2.2.X.2: MBS session creation

Steps 1 to 11 per 3GPP TS 23.247 [204] Figure 7.1.1.2-1: MBS Session Creation without PCC,

13. If the dynamic PCC is deployed, refer to steps 21-27 in TS 23.247 [204] Figure 7.1.1.3-1: MBS Session Creation with PCC. If not, refer to step 13 in TS 23.247 [204] Figure 7.1.1.2-1: MBS Session Creation without PCC.

13ch-a.	The MB-SMF creates a MBS Charging Identifier for the MBS session and sends Charging Data Request [Initial] to the CHF for authorization for the subscriber to start the MBS session which is triggered by the start of MBS session. The charging information shall include MBS session ID referring to clause 6.5.1 of TS 23.247 [204], MBS service type (broadcast or multicast) and QoS information. It may also include MBS service area information, MBS start time and end time.

13ch-b. The CHF opens the CDR for this MBS session.

13ch-c. The CHF acknowledges by sending Charging Data Response [Initial] to the MB-SMF.

Steps 14 to 22 per 3GPP TS 23.247 [204] Figure 7.1.1.2-1: MBS Session Creation without PCC.

5.2.2.X.3	 MBS session deletion

The following figure 5.2.2.X.3 describes charging procedures for MBS session deletion, based on TS 23.247 [204] figure 7.1.1.4-1 and 7.1.1.5-1.

Figure 5.2.2.X.3: MBS Session Deletion

Steps 1 to 7 per 3GPP TS 23.247 [204] Figure 7.1.1.4-1: MBS Session Deletion without PCC.

7ch-a.	The MB-SMF sends Charging Data Request [Termination] to the CHF, for terminating the charging associated with MBS session, with the trigger “End of MBS session”, and charging information of MBS data volumes and MBS session duration.

7ch-b. The CHF closes the CDR for MBS session.

7ch-c. The CHF acknowledges by sending Charging Data Response [Termination] to the MB-SMF.

Steps 8 to 14 per 3GPP TS 23.247 [204] Figure 7.1.1.4-1: MBS Session Deletion without PCC.

End of Change