3GPP TSG-SA5 Meeting #151 	S5-236723

Xiamen, China, 9 - 13 October 2023

# Source:	Ericsson

# Title:	Update of charging data request

# Document for:	Approval

Agenda Item:	7.4.1

# 1	Decision/action requested

Include the proposed changes in TS 28.203.

# 2	References

[1]		3GPP TS 28.203: "Network slice admission control charging in the 5G System (5GS)"

# 3	Rationale

Moving the NSAC specific triggers and used units close to the current used units and triggers, for easier handling.

# 4	Detailed proposal

First change

#### 6.1.1.2	Charging Data Request message

Table 6.1.1.2.1 illustrates the basic structure of a Charging Data Request message from the NSACF as used for network slice admission control.

Table 6.1.1.2.1: Charging Data Request message contents

| Information Element | Converged Charging Category | Description |
| --- | --- | --- |
| Session Identifier | OC | Described in 3GPP TS 32.290 [5] |
| Subscriber Identifier | - | This field is not applicable. |
| NF Consumer Identification | M | Described in 3GPP TS 32.290 [5] and holds the identifier of the NSACF |
| NF Functionality | M | Described in 3GPP TS 32.290 [5]. |
| NF Name | OC | Described in 3GPP TS 32.290 [5]. |
| NF Address | OC | Described in 3GPP TS 32.290 [5]. |
| NF PLMN ID | OC | Described in 3GPP TS 32.290 [5]. |
| Charging Identifier | OM | Described in 3GPP TS 32.290 [5]. |
| Invocation Timestamp | M | Described in 3GPP TS 32.290 [5]. |
| Invocation Sequence Number | M | Described in 3GPP TS 32.290 [5]. |
| Retransmission Indicator | OC | Described in 3GPP TS 32.290 [5]. |
| One-time Event | OC | Described in 3GPP TS 32.290 [5]. |
| One-time Event Type | OC | Described in 3GPP TS 32.290 [5]. |
| Notify URI | OC | Described in 3GPP TS 32.290 [5]. |
| Supported Features | OC | Described in 3GPP TS 32.290 [5]. |
| Service Specification Information | OC | Described in 3GPP TS 32.290 [5]. |
| Triggers | - | This field is not applicable. |
| NSAC Triggers | OC | This field holds the network slice admission control specific triggers described in clause 5.2.1. It is based on triggers described in 3GPP TS 32.290 [5]. |
| Multiple Unit Usage | OM | Described in 3GPP TS 32.290 [5]. |
| Rating Group | OM | Described in 3GPP TS 32.290 [5]. |
| Requested Unit | OC | Described in 3GPP TS 32.290 [5]. |
| Time | - | This field is not applicable. |
| Total Volume | - | This field is not applicable. |
| Uplink Volume | - | This field is not applicable. |
| Downlink Volume | - | This field is not applicable. |
| Service Specific Units | - | This field is not applicable. |
| NSAC Units | OC | This field holds the network slice admission control specific requested units described in clause 6.2.1.x. |
| Used Unit Container | OC | Described in 3GPP TS 32.290 [5]. |
| Service Identifier | OC | This field is not applicable. |
| Quota management Indicator | OC | Described in 3GPP TS 32.290 [5]. |
| Triggers | - | This field is not applicable. |
| NSAC Triggers | OC | This field holds the network slice admission control specific triggers described in clause 5.2.1. It is based on triggers described in 3GPP TS 32.290 [5]. |
| Trigger Timestamp | - | This field is not applicable. |
| Time | - | This field is not applicable. |
| Total Volume | - | This field is not applicable. |
| Uplink Volume | - | This field is not applicable. |
| Downlink Volume | - | This field is not applicable. |
| Service Specific Unit | - | This field is not applicable. |
| Event Time Stamps | - | This field is not applicable. |
| NSAC Units | OC | This field holds the network slice admission control specific used units described in clause 6.x.1.x. |
| Local Sequence Number | OM | Described in 3GPP TS 32.290 [5]. |
| NSAC Container Information | OC | This field holds the network slice admission control container specific information described in clause 6.x.1.3 |
| S NSSAI | M | This field holds the Single Network Slice Selection Assistance Information identifying the network slice. |
| NSAC Charging Information | OM | This field holds NSAC specific information described in clause 6.x.1.2 |

Editor's Note: the content and IE names of the table is ffs

Second change

#### 6.2.1.3	Definition of NSAC Container Information

Specific charging information used for Network slice admission control charging is provided within the NSAC Container Information.

The detailed structure of the NSAC Container Information can be found in table 6.2.1.3.1.

Table 6.2.1.3-1: Structure of NSAC Container Information

| Information Element | Category | Description |
| --- | --- | --- |

Editor's Note: the table structure is ffs

Third change

#### 6.2.1.x	Definition of NSAC Unit

Service specific units used for network slice admission control charging is provided.

Table 6.2.1.x-1: Structure of NSAC Units

| Information Element | Category | Description |
| --- | --- | --- |
| Nb of UEs | OC | This field holds the current number of registered UEs for an S-NSSAI requested or reported by an NSACF |
| Nb of PDUs | OC | This field holds the current number of established PDU sessions for an S-NSSAI requested or reported by an NSACF |

Editor's Note: the tables structure is ffs

End of changes