# 3GPP TSG-SA5 Meeting #154	S5-241782

# 15 - 19 April 2024, Changsha, Hunan, China

# Source:	Nokia, Nokia Shanghai Bell

# Title:	pCR 28.866 MDA management data correlation analytics

# Document for:	Approval

# Agenda Item:	6.19.2

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1] 	3GPP TR 28.866-000 “Study on Management Data Analytics (MDA) – Phase 3”.

# 3	Rationale

The data from different aspects of the network are correlated, that in many cases real analytics value comes from leveraging the correlation among the data. This pCR is to add an MDA use cases on correlation of data for different uses and from different sources.

# 4	Detailed proposal

1st Change

# Y	Use cases and requirements for MDA capabilities and services

# Y.2.x	Data correlation analytics

#### Y.2.x.1.1	Description

This MDA capability is for data correlation analytics on network, computing and slice resources across different aspects consisting of network domains(RAN/ core/ transport), resource domain  (network and cloud) and slice / slice subnet aspects.

#### Y.2.x.1.2			Use cases

##### Y.2.x.1.2.a	Measurement data correlation analytics for ML training

For a ML model training, a large amount of measurement data instances may be collected and does not necessarily add value to training performance. The collected measurement data for ML training can be highly correlated (linear or non-linear), hence there may be high redundancy among the collected measurement data. Hence using all measurement data for a model training (and inference) can be a waste of computing resources and energy. Therefore, it’s necessary to optimise the training data preparation based on the correlation analytics and redundancy information. A correlation analytics may help to detect the redundancy pattern among  the measurement data for ML training, such as:

-	For a given task (e.g., an analytics, model training), analyses the correlation among the given set of measurement data, the output can be a (much) smaller set of measurement data, with which ML model (re-)training could be much more efficient with limited (or managed) impact to model training performance (comparing to use full set of data). The output may include a recommendation, for example recommendation on reducing redundancy to optimize measurement data collection for the model training.

-	Regularly renew the correlation analytics as time progresses, since the correlation relationship might change; this is especially useful when there is a need to regularly re-train the ML model.

##### Y.2.x.1.2.b			Multi-aspect resource optimization (MARO)

MDA may provide analytics on different domains and aspects of the network, including radio, core and transport domains and on different functions in these domains including physical and virtual network functions in different architectures, e.g the split RAN architecture (CU-CP/ CU-UP/ DU), Network slicing and slice subnets as well as edge cloud architectures, etc. The behaviour of these network domains and functions (all together called aspects) is correlated, requiring that any optimisation actions need to be correlated. The MDA should support a Multi-aspect resource optimization (MARO) capability that provides information on degree of correlation observed in the data from these network aspects.

Besides observing the correlations, MARO could evaluate the related impact of one domain or aspect on the other based on which it derives recommendations for how to optimize resources among these aspects in a correlated way. For example, to optimize the allocation of load among cells, MARO may provide a report that contains recommendations on changes to be concurrently made to the edge cloud resources hosting the cells functions (CU-CP/ CU-UP/ DU) as well as on the handover parameters of the cells to allow a set of users to be moved among those cells.

#### Y.2.x.1.3	Potential Requirements

Table Y.2.x.1.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-MDA_CORE-01 | MDA capability for Measurement data correlation analytics for ML training shall include the capability to provide the measurement data redundancy analysis including which measurement data correlate to which measurement data, the rate of redundancy, and recommendation to reduce redundancy to optimize measurement data collection for the model training. | correlation analytics (clause Y.2.x.1.2.a) |
| REQ-MDA_CORE-03 | The MDA MnS producer should have the capability enabling an authorized MnS consumer to configure scheduling of the analysis of the correlation of management data. | correlation analytics (clause Y.2.x.1.2.a) |
| REQ-MDA_CORE-04 | The MDA MnS producer should have the capability enabling an authorized MnS consumer to configure context of analytics of the correlation of management data. | correlation analytics (clause Y.2.x.1.2.a) |
| REQ-MDA_MARO_CORE-01 | The MnS for MDA should include the capability to provide a report on the correlation/concurrent optimization actions to be undertaken in multiple network domains including physical and virtual infrastructure. | correlation analytics (clause Y.2.x.1.2.b) |

#### Y.2.x.1.4			Potential Solutions

##### Y.2.x.1.4.1 Possible solution for measurement data correlation analytics for ML training

- Introduce a data type for measurement data correlation analytics recommendation, called measurementDataCorrelationRecommendation. The data taype can be the contents of the analytics report representing the recommendations from MDA for the measurement data correlation analytics for ML training.

  - The contents of this data type may be a set of 3GPP and non-3GPP recommendations.

- An optional attribute may configure the analytics context in MDARequest. The context may include attributes

  - An optional attribute may indicate the training performance requirement.

  - An attribute may configure the scenario type of correlation as an enumeration.

  - An optional attribute may configure the scheduling of regular MDA analytics.

##### Y.2.x.1.4.2		Potential Solutions Multi-aspect resource optimization (MARO)

- Introduce a data type for a multi-aspect resource optimization recommendation, called mARORecommendation. The data taype can be the contents of the analytics report representing the recommendations from MDA for the set of concurrent changes to be made to different aspects of the network.

  - The contents of this data type may be a set of 3GPP and non-3gpp recommendations.

End Change