3GPP TSG-SA5 Meeting #154	S5-242186

# Changsha, CN,15th – 19th April 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.702 | CR | 0009 | rev | 1 | Current version: | 13.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-13 CR TS 28.702 correction of attribute definition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson-LG Co., LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI11 |  |  |  |  |  | Date: |  |  | 2024-03-18 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to TS 28.300 a Distinguished Name (DN) is used to uniquely identify a MO within a name space. In TS 28.702 this rule is not followed. According to TS 32.156 If the property is present for attributes with a multiplicity of greater than “1”, isOrdered shall be set to either “True” or “False”. It shall not be set to “N/A”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct Attribute property "isUnique" and “isOrdered” in the document when they attribute Type is DN and multiplicity>1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect attribute property remains in the document and leads to wrong implementation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

### 4.4.1	Attribute properties

The following table defines the properties of attributes that are specified in the present document.

Table 4.4.1: Attributes

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| defaultMsc | Whether this MSC Server is default CN node in MscPool or not (Ref. 3GPP TS 23.236 [18]). A value of 0 represents that this MSC Server is not default CN node and a value of 1 represents that it is default CN node. allowedValues: 0..1 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| gcaList | List of Group Call Area (Ref. 3GPP TS 23.003 [12]). allowedValues: N/A | type: String multiplicity: 1..* isOrdered: False isUnique: N/A defaultValue: None isNullable: True |
| lacList | List of Location Area Codes (Ref. 3GPP TS 23.003 [12]). allowedValues: N/A | type: String multiplicity: 1..* isOrdered: False isUnique: N/A defaultValue: None isNullable: True |
| mccList | List of Mobile Country Codes, MCC (part of the PLMN Id, Ref. 3GPP TS 23.003 [12]). allowedValues: N/A | type: String multiplicity: 1..* isOrdered: False isUnique: N/A defaultValue: None isNullable: True |
| mncList | List of Mobile Network Codes, MNC (part of the PLMN Id, Ref. 3GPP TS 23.003 [12]). allowedValues: N/A | type: String multiplicity: 1..* isOrdered: False isUnique: N/A defaultValue: None isNullable: True |
| mscId | Unique MSC ID (Ref. 3GPP TS 23.002 [8]). allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| nriList | NRI shall be part of the TMSI. The NRI has a configurable length of 0 to 10 bits. (Ref. 3GPP TS 23.003 [12]). allowedValues: N/A | type: String multiplicity: 1..* isOrdered: False isUnique: N/A defaultValue: None isNullable: True |
| pLMNIdList | It is a list of PLMN-Id, PLMN-Id= Mobile Country Codes (MCC)¦¦ Mobile Network Codes(MNC) (Ref. 3GPP TS 23.003[12]) The MscPoolArea.pLMNIdList purpose is to identify the PLMNs (related to MscFunction) the Msc Pool is serving. The MscEunction.pLMNIdList purpose is as following. One operator may have several PLMN Ids and accordingly RAN broadcasts these Ids to enable UEs of different PLMN (i.e, UEs with different MNC in their IMSIs) to access its network. If CN node does not know this PLMN list, UEs of different PLMN than the one combined in Msc might be treated as UEs from other operators. This will affect Location Update and Inter-Msc handover procedures, and also the changing rate. allowedValues: A list of at most six entries of PLMN Identifiers. The PLMN Identifier is composed of a Mobile Country Code (MCC) and a Mobile Network Code (MNC). | type: Integer multiplicity: 1..* isOrdered: False isUnique: N/A defaultValue: None allowedValues: N/A isNullable: True |
| proceduralStatus | See definition in 3GPP TS 28.625 [13] (State Management Data Definition IRP IS). There are two cases resulting in a status change to be reported: - Case 1: A notification may be generated to indicate that restart procedure is about to begin or has just begun but has not finished. - the value for this attribute indicates original state == "Not Initialized" and new state == "Initializing". - Case 2: A notification shall be generated to indicate that a restart procedure has completed successfully - the value for this attribute indicates original state == "Initializing" to new state == NULL (see [13]). | See 3GPP TS 28.625 [13]. |
| racList | List of Routeing Area Codes covered by MSC (Ref. 3GPP TS 23.003 [12]). allowedValues: N/A | type: String multiplicity: 1..* isOrdered: False isUnique: N/A defaultValue: None isNullable: True |
| sacList | List of Service Area Codes covered by MSC (Ref. 3GPP TS 23.003 [12]). allowedValues: N/A | type: String multiplicity: 1..* isOrdered: False isUnique: N/A defaultValue: None allowedValues: N/A isNullable: True |
| sgsnId | Unique SGSN ID (Ref. 3GPP TS 23.002 [8]). allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| Attribute related to role |  |  |
| mscServerFunction-GsmCell | This holds a set of DNs of GSMCell . allowedValues: N/A | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| gsmCell-MscServerFunction | This holds the DN of an MscServerFunction. allowedValues: N/A | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mscServerFunction-ExternalGsmCell | This holds a set of DNs of ExternalGsmCell. allowedValues: N/A | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| externalGsmCell-MscServerFunction | This holds the DN of an MscServerFunction. allowedValues: N/A | type: DN multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| mscServerFunction-CsMgwFunction | This holds a set of DNs of CsMgwFunction. allowedValues: N/A | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| csMgwFunction-MscServerFunction | This holds the DN of an MscServerFunction. allowedValues: N/A | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| sgsnFunction-GsmCell | This holds a set of DNs of GSMCell . allowedValues: N/A | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| gsmCell-SgsnFunction | This holds the DN of an SgsnFunction. allowedValues: N/A | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| sgsnFunction-ExternalGsmCell | This holds a set of DNs of ExternalGsmCell. allowedValues: N/A | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| externalGsmCell-SgsnFunction | This holds the DN of an SgsnFunction. allowedValues: N/A | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| csMgwFunction-IucsLink | This holds a set of DNs of IucsLink. allowedValues: N/A | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| connectedRnc | This holds the DN of an RncFunction or an ExternalRncFunction. allowedValues: N/A | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| csMgwFunction-ALink | This holds a set of DNs of ALink. allowedValues: N/A | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| connectedBss | This holds the DN of an BssFunction or an ExternalBssFunction. allowedValues: N/A | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mscPool-MscServerFunction | This holds a set of DNs of MscServerFunction. allowedValues: N/A | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| mscServerFunction-MscPool | This holds a set of DNs of MscPool. allowedValues: N/A | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| mscPool-MscPoolArea | This holds the DN of an MscPoolArea. allowedValues: N/A | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mscPoolArea-MscPool | This holds the DN of an MscPool. allowedValues: N/A | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| sgsnPool-SgsnFunction | This holds the DN of an SgsnFunction. allowedValues: N/A | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| sgsnFunction-SgsnPool | This holds the DN of an SgsnPool. allowedValues: N/A | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| sgsnPool-sgsnPoolArea | This holds the DN of an SgsnPoolArea. allowedValues: N/A | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| sgsnPoolArea-SgsnPool | This holds the DN of an SgsnPool. allowedValues: N/A | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

End of changes