3GPP TSG-SA5 Meeting #155 	S5-242531

# Jeju, South Korea, 27 - 31 May 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.105 | CR | Input to draftCR | rev | <Rev#> | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 Input to DraftCR TS 28.105 corrections on the terminology for ML emulation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_MGT |  |  |  |  |  | Date: |  |  | 2024-04-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | During 3GPP SA5#154 meeting, S5-241931 was agreed to change the term “ML entity” to “ML Model” and not using the word “phase”. This contribution propose to align the terminology for emulation part. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the ML entity to ML model. Remove word “phase”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent description |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3, 6.3.1, 6.3.2, 6.3.3, 7.3a.2, 7.3a.2.2.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

6.3	AI/ML emulation

6.3.1	Description

Before the ML model is applied in the production network, the MnS inference consumer may want to receive results of inference in one or more environments that emulate (to different extents) the expected inference characteristics, in a process that may be termed as Inference emulation. The Inference emulation enables this.

6.3.2	Use cases

6.3.2.1	AI/ML Inference emulation

After the ML model is validated and tested during development, the MnS consumer may wish to receive information from an inference emulation process that indicates if the ML model or the associated ML inference function is working correctly under certain runtime context.

The management system should have the capabilities enabling an MnS consumer:

-	request an inference emulation function to provide emulation reports; and

-	to receive the results from running inference through an AI/ML inference emulation environment available at the emulation MnS producer.

6.3.3	Requirements for Managing AI/ML Inference emulation

Table 6.3.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-AI/ML_EMUL-1: | The MnS producer for AI/ML inference emulation should have a capability enabling an authorized MnS consumer to receive reporting about the ML inference emulation. | AI/ML Inference emulation (clause 6.3.2.1) |
| REQ-AI/ML_EMUL-2: | The MnS producer for AI/ML inference emulation should have a capability enabling an authorized MnS consumer to request an inference emulation function to provide inference emulation reports on an ML model or inference Function. | AI/ML Inference emulation (clause 6.3.2.1) |

Next change

7.3a.2	Information model definitions for ML emulation

7.3a.2.1	Class diagram

7.3a.2.1.1	Relationships

Figure 7.3a.2.1.1-1: NRM fragment for AI/ML inference emulation Control

7.3a.2.1.2	Inheritance

Figure 7.3a.2.1.2-1: AI/ML inference emulation Inheritance Relations

7.3a.2.2	Class definitions

7.3a.2.2.1	AIMLInferenceEmulationFunction

7.3a.2.2.1.1	Definition

This IOC represents the properties of a function that undertakes AI/ML Inference Emulation.

This AIMLInferenceEmulationFunction instance can be created by the system (AI/ML inference emulation MnS producer) or pre-installed.

An AIMLInferenceEmulationFunction may be associated with one or more MLModel(s). AIMLInferenceEmulationFunction is name contained with AIMLInferenceEmulationReport(s) that delivers the outcomes of the emulation processes.

NOTE:	The way of triggering of an AI/ML inference emulation and the instantiation of the related AI/ML inference emulation process is not in the scope of the present document.

7.3a.2.2.1.2	Attributes

The AIMLInferenceEmulationFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| aIMLInferenceEmulationFunctionId | M | T | F | F | F |
| Attributes related to Role |  |  |  |  |  |
| aIMLInferenceEmulationReportRefList | M | T | F | F | F |

7.3a.2.2.1.3	Attribute constraints

None.

7.3a.2.2.1.4	Notifications

The common notifications defined in clause 7.6 are valid for this IOC, without exceptions or additions.

End of change