3GPP TSG-SA5 Meeting #155 	S5-242700

# Jeju, South Korea, 27 - 31 May 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.271 | CR | 0022 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add message flows of converged charging for UE positioning assisted by Sidelink Positioning and involving 5GC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | DUMMY |  |  |  |  |  | Date: |  |  | 2024-05-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Message flows of converged charging for UE positioning assisted by Sidelink Positioning and involving 5GC are missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add message flows of converged charging for UE positioning assisted by Sidelink Positioning and involving 5GC |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Converged charging for UE positioning assisted by Sidelink Positioning and involving 5GC will not be supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.x.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

2	References

The following documents contain provisions, which through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 32.240: "Telecommunication management; Charging management; Charging architecture and principles".

[2] - [9]	Void.

[10]	3GPP TS 32.250: "Telecommunication management; Charging management; Circuit Switched (CS) domain charging".

[11] - [19]	Void.

[20]	3GPP TS 32.260: "Telecommunication management; Charging management; IP Multimedia Subsystem (IMS) charging".

[21] - [29]	Void.

[30]	3GPP TS 32.270: "Telecommunication management; Charging management; Multimedia Messaging Service (MMS) charging".

[31] - [49]	Void

[50]	3GPP TS 32.299: "Telecommunication management; Charging management; Diameter charging application".

[51]	3GPP TS 32.298: "Telecommunication management; Charging management; Charging Data Record (CDR) encoding rules description".

[52]	3GPP TS 32.297: "Telecommunication management; Charging management; Charging Data Record (CDR) file format and transfer".

[53]	3GPP TS 32.296: "Telecommunication management; Charging management; Online Charging System (OCS) applications and interfaces".

[54]	3GPP TS 32.295: "Telecommunication management; Charging management; Charging Data Record (CDR) transfer".

[55] - [99]	Void.

[100]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[101] 				3GPP TS 22.115: "Service aspects; Charging and billing".

[102] - [199]		Void.

[200]	Void.

[201]	3GPP TS 23.271: "Functional stage 2 description of Location Services (LCS)".

[202]	Void.

[203]	3GPP TS 25.305: "Stage 2 functional specification of User Equipment (UE) positioning in UTRAN".

[204]	3GPP TS 43.059: "Functional stage 2 description of Location Services (LCS) in GERAN".

[205] 	3GPP TS 24.002: "GSM - UMTS Public Land Mobile Network (PLMN) Access Reference Configuration".

[xx] 	3GPP TS 23.586: “Architectural Enhancements to support Ranging based services and Sidelink Positioning”.

[206] - [299]	Void.

[301] - [399]	Void.

[400]	Void.

[401]	Void.

[402]	IETF RFC 4006 (2005): "Diameter Credit-Control Application".

Next change

5.x.2	Message flows

5.x.2.1	Introduction

The different scenarios below focus on the different messages from/to the GMLC and corresponding interaction with the CHF, based on scenarios specified in TS 23.586 [xx].

5.x.2.2	UE positioning assisted by Sidelink Positioning and involving 5GC

5.x.2.2.1	Triggers for converged charging for UE positioning assisted by Sidelink Positioning and involving 5GC

For converged charging, the following tables summarize the set of trigger conditions for UE positioning assisted by Sidelink Positioning and involving 5GC with GMLC.

Table 5.x.2.2.1-1: Triggers for charging events for UE positioning assisted by Sidelink Positioning and involving 5GC with GMLC

| Trigger Conditions | Trigger level | Default category | CHF allowed to change category | CHF allowed to enable and disable | Message when "immediate reporting" category |
| --- | --- | --- | --- | --- | --- |
| Received Ngmlc_Location_LocationUpdate service operation from AMF | - | Immediate | Not Applicable | Not Applicable | PEC: Charging Data Request [Event] |
| Received Namf_Location_ProvidePositioningInfo Response from AMF | - | Immediate | Not Applicable | Not Applicable | PEC: Charging Data Request [Event] |

5.x.2.2.2	Message flow for mobile originated UE positioning assisted by Sidelink Positioning and involving 5GC charging – PEC

The figure 5.x.2.2.2-1 describes the high-level charging procedure for GMLC (CTF) Converged charging for mobile originated UE positioning assisted by Sidelink Positioning and involving 5GC.

Figure 5.x.2.2.2-1: Message flow for mobile originated UE positioning assisted by Sidelink Positioning and involving 5GC charging – PEC

0.	mobile originated Sidelink positioning/Ranging procedure and the location calculation procedure as described in TS 23.586 [xx].

1.	The AMF invokes the Ngmlc_Location_LocationUpdate service operation towards the GMLC carrying the location result and the additional information regarding the location estimate.

2ch-a. The GMLC sends Charging Data Request [Event] to the CHF, including the charging information.

2ch-b. The CHF generates a CDR.

2ch-c. The CHF returns Charging Data Response [Event] to GMLC.

3.	GMLC transfers the location request response to UE and optionally AF/LCS client as described in TS 23.586 [xx].

5.x.2.2.3	Message flow for mobile terminated UE positioning assisted by Sidelink Positioning and involving 5GC – PEC

The figure 5.x.2.2.3-1 describes the high-level charging procedure for GMLC (CTF) Converged charging for mobile terminated UE positioning assisted by Sidelink Positioning and involving 5GC.

Figure 5.x.2.2.3-1: Message flow for mobile terminated UE positioning assisted by Sidelink Positioning and involving 5GC charging - PEC

0.	Mobile terminated Sidelink positioning/Ranging procedure and the location result for the Target UE is calculated as described in TS 23.586 [xx].

1.	The AMF invokes the Namf_Location_ProvidePositioningInfo service operation towards the GMLC carrying the location result and the additional information regarding the location estimate.

2ch-a. The GMLC sends Charging Data Request [Event] to the CHF, including the charging information.

2ch-b. The CHF generates a CDR.

2ch-c. The CHF returns Charging Data Response [Event].

3.	GMLC transfers the location request response to AF/LCS client as described in TS 23.586 [xx].

End of changes