3GPP TSG-SA5 Meeting #159	S5-250363

Sophia-Antipolis, France, 17 - 21 February 2025

# Source:	Huawei

# Title:	Use case exposure of management services through the CAPIF framework

# Document for:	Approval

Agenda Item:	6.19.21.1

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1]		3GPP TS 28.579 v0.0.0: Management services exposure to external consumers through CAPIF

# 3	Rationale

This contribution proposes to add use case exposure of management services through the CAPIF framework into the draft TS 28.579 [1].

# 4	Detailed proposal

This document proposes the following changes in draft TS 28.579 [1].

1st Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[Y]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

Next Change

# 5	Specification level requirements

## 5.1	Use cases

### 5.1.1	Exposure of management services through the CAPIF framework

#### 5.1.1.1	Description

To expose management services through the CAPIF framework, the 3GPP management system should define the functional entity to provide the API provider domain functions (i.e., API Exposing Function (AEF), API management function (AMF), API publishing function (APF)) as defined by the CAPIF framework in clause 6.3 of TS 23.222 [Y].

For the present document, the functional entity is called the management services exposure domain function (MSEDF). The MSEDF can consume CCF APIs (i.e., CAPIF-3, CAPIF-4 and CAPIF-5 interfaces) and allow API invokers to access management services through service APIs (using CAPIF-2/2e interface).

Figure 5.1.1.1-1: MSEDF providing the API provider domain functions as defined by the CAPIF framework.

The MSEDF provides the CAPIF API provider domain functions, and the following can apply for this functional entity:

-	The MSEDF requires providing CAPIF interfaces, which are not defined in the 3GPP management system.

-	The MSEDF publishes the management service APIs for discovery through the CAPIF core function (CCF) and exposes the management services for invocation by external MnS consumers through the AEF of the MSEDF.

-	The possible deployment options of the MSEDF is FFS.

-	Whether the MSEDF needs to provide the required mappings between SA5 and SA6 data types is FFS.

Traceability: REQ-MEXPO-FUN-01.

Next Change

## 5.2	Requirements

### 5.2.1	Requirements for exposure of management services through the CAPIF framework

REQ-MEXPO-FUN-01: The management service producer responsible for MSDEF should support the capability to provide the API provider domain functions defined for CAPIF.

End of change