3GPP TSG-SA5 Meeting #160	S5-251249

Goteborg, Sweden, 07 - 11 April 2025

Source:	Nokia

Title:	Pseudo-CR on CCL General NRM

Document for:	Approval

Agenda item:	6.19.4.1

Spec:	3GPP TS28.567

Version:	0.1.0

Work Item:	Closed Control Loop Management

Comments

This pCR is to add NRM for the CCL as was agreed in the CCL study in TR28. 867

Proposed Changes

* * * First Change * * * *

# 6	Model

## 6.1	Imported and associated information entities

TBD

### 6.1.1	Imported information entities and local labels

TBD

### 6.1.2	Associated information entities and local labels

TBD

## 6.2	Class diagram

#### 6.2.1	Relationships

Figure 6.2.1.1-1: Relations for common information models for AI/ML management

#### 6.2.2	Inheritance

Figure 6.2.1.2-1: Inheritance Hierarchy for Closed Control Loops

### 6.3	Class definitions

#### 6.3.1	ClosedControlLoop

6.3.1.1	Definition

This IOC represents the closed control loop. It represents the information for controlling and monitoring a CCL associated with a stated scope.

The ClosedControlLoop is name-contained by SubNetwork or ManagedElement and is associated with a CCLreport that contans reportedinformation about the CCL. Accordingly, the report about a CCL can existi even when the CCL is deleted.

The capabilities of the CCL are contained in one or more CCLPurposes that describe what the CCL is capable of doing or can be configured to do - including information the network resources for which the CCL can execute decisions and actions. So, the ClosedControlLoop is associated with one or more CCLPurpose(s) that indicate(s) a list of characteristics that describe what a CCL can/is expected to be able do. The purpose describes the type of functionality that can be executed including problem recovery and fault management .

The operational information about the CCL is contained in the CCLScope(s) and CCLGoal(s), so the ClosedControlLoop is associated with one or more CCLScope(s) and one or more CCLGoal(s). The CCLScope defines what the CCL has been configured to read, evaluate, control, etc; while the CCLGoal defines what the CCL is required to achieve for the defined scope.

For temporary deactivation of the  CCL, the MnS consumer can modify the value of the administrative state attribute to “LOCKED”.  The MnS producer may disable the  CCL, for example in conflict situations, by setting the operational state attribute to “disabled”. When a CCL is enabled by the MnS producer, the operational state is set again to “enabled”. For the activation of an  CCL, the MnS consumer can modify the value of the administrative state attribute to “UNLOCKED”.

6.3.1.2	Attributes

The CCLControlLoop IOC includes attributes inherited from Top IOC (defined TS 28.622[5]) and the following attributes:

Table 6.3.1.2-1

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| operationalState | M | T | F | F | T |
| administrativeState | M | T | T | F | T |
| controlLoopLifeCyclePhase | M | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| cCLPurposeRefList | M | T | T | T | T |

6.3.1.3	Attribute constraints

None

6.3.1.4	Notifications

The common notifications defined in clauses 6.1 are valid for this IOC, without exceptions.

##### 6.3.2	CCLGoal

6.3.2.1	Definition

This IOC represents goal of a CCL.

To 	express a new goal of a CCL, the MnS consumer needs to request the MnS producer to create an instance of CCLGoal. MnS producer can also trigger the creation of an instance of CCLGoal.

The attribute “cCLTargets” defines a list of targets that should be achieved by the ClosedControlLoop.

6.3.2.2	Attributes

The CCLGoal IOC includes attributes inherited from Top IOC (defined TS 28.622[5]) and the following attributes:

Table 6.3.2.2-x

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| Attributes related to role |  |  |  |  |  |

6.3.2.3	Attribute constraints

None.

6.3.2.4	Notifications

The common notifications defined in clauses 6.1 are valid for this IOC, without exceptions.

##### 6.3.3	CCLScope

6.3.3.1	Definition

It indicates the target for a goal in terms of location. A particular ACCL can target for a particular location. The goal status is ascertained based on the appropriately collected performance measurements as per the target location.

The CCLScope includes the attribute scopeType that indicates the type of scope that represented by the particular scope instance.

6.3.3.2	Attributes

The CCLScope IOC includes attributes inherited from Top IOC (defined TS 28.622[5]) and the following attributes:

Table 6.3.3.2-x

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| scopeType | O | T | F | F | T |

6.3.3.3	Attribute constraints

None.

6.3.3.4	Notifications

The common notifications defined in clauses 6.1 are valid for this IOC, without exceptions.

##### 6.3.4	CCLReport

6.3.4.1	Definition

This class represents the reported outcomes on a CCL instance, e.g., the information about the outcomes on one or multiple CCLGoals and one or multiple CCLTargets. An CCLReport is contained by the entity containing the CCL, since the CCLreport can exist beyond the life of the CCL on which it is reporting.

There is one CCLReport per CCL for an observation time. The content of the CCLReport may be different for different observation time.

6.3.4.2	Attributes

The CCLReport IOC includes attributes inherited from Top IOC (defined TS 28.622[5]) and the following attributes:

Table 6.3.4.2-x

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| Attributes related to role |  |  |  |  |  |

6.3.4.3	Attribute constraints

None.

6.3.4.4	Notifications

The common notifications defined in clauses 6.1 are valid for this IOC, without exceptions.

##### 6.3.5	CCLPurpose <<dataType>>

6.3.5.1	Definition

This data type represents a single purpose that describes what a CCL can do. The purpose is alist of characteristics that describe the capabilities of the CCL.

6.3.5.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| Attributes related to role |  |  |  |  |  |

6.3.5.3	Attribute constraints

None.

6.3.5.4	Notifications

The common notifications defined in clauses 6.1 are valid for this IOC, without exceptions..

6.4	Attribute definitions

6.4.1	Attribute properties

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| scopeType | It indicates the type of scope that represented by the particular scope instance. allowedValues: CCL_MEASUREMENT_SCOPE, CCL_TARGET_SCOPE, CCL_CONTROL_SCOPE, CCL_IMPACT_SCOPE | type: Enum multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| operationalState | It indicates the operational state of the ClosedControlLoop instance. It describes whether the resource is installed and partially or fully operable (ENABLED) or the resource is not installed or not operable (DISABLED). allowedValues: "ENABLED", "DISABLED". The meaning of these values is as defined in 3GPP TS 28.625 [14] and ITU-T X.731 [15]. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: DISABLED isNullable: False |
| administrativeState | It indicates the administrative state of the ClosedControlLoop instance. It describes the permission to use or the prohibition against using the ClosedControlLoop instance. The administrative state is set by the MnS consumer. allowedValues: "LOCKED", "UNLOCKED". The meaning of these values is as defined in 3GPP TS 28.625 [14] and ITU-T X.731 [15]. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: LOCKED isNullable: False |

* * * Next Change * * * *

Annex A (informative):
UML code for model diagrams

# A.X	UML code for CCL management model diagrams

This annex contains the PlantUML source code for the NRM diagrams defined in clause 6.2 of the present document.

# A.X.1	CCL NRM fragment (Figure 6.2.1.1-1)

@startuml

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide members

hide circle

class ManagedEntity <<ProxyClass>>

class ClosedControlLoop <<InformationObjectClass>>

class CCLPurpose <<InformationObjectClass>>

class CCLGoal << InformationObjectClass >>

class CCLScope << InformationObjectClass >>

class CCLReport <<InformationObjectClass>>

ManagedEntity "1" *-- "*" ClosedControlLoop: <<names>>

ClosedControlLoop "1" *--> "*" CCLPurpose

ClosedControlLoop "1" *-- "*" CCLGoal: <<names>>

ClosedControlLoop "1" *-- "*" CCLScope: <<names>>

ClosedControlLoop "1" *-- "*" CCLReport: <<names>>

note left of ManagedEntity

Represents the following IOCs:

SubNetwork or

ManagedElement

end note

note top of CCLPurpose

Can be any of these CCL purposes:

NetworkProblemRecovery

FaultManagement

...

end note

@enduml

# A.X.2	CCL inheritance relationships (Figure 6.2.1.1-2)

@startuml

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide members

hide circle

class Top << InformationObjectClass >>

class ClosedControlLoop <<InformationObjectClass>>

class CCLGoal <<InformationObjectClass>>

class CCLReport <<InformationObjectClass>>

class CCLScope <<InformationObjectClass>>

Top <|-- ClosedControlLoop

Top <|-- CCLGoal

Top <|-- CCLScope

Top <|-- CCLReport

@enduml

* * * End of Changes * * * *