# 3GPP TSG SA WG5 Meeting #160	S5-251464

# Goteborg, SWEDEN 7 - 11 April 2025

# Source:	Huawei

# Title:	Discussion on the charging domain for NRF registration and discovery

# Document for:	Discussion

Agenda Item:	7.4.1

1	Decision/action requested

To discuss the charging domain based CHF discovery.

2	References

[1]	3GPP TS 29.500 5G System; Technical Realization of Service Based Architecture; Stage 3

[2]	3GPP TS 32.291 Telecommunication management; Charging management; 5G system, charging service; Stage 3

[3]	3GPP TS 29.510 5G System; Network function repository services; Stage 3

3	Rationale

According to the Feature Negotiation mechanism defined in TS 29.500 [1] clause 6.6.2, the list of supported features can serve two purposes:

-	Support the negotiation of applicable features between NF consumer and producer for a specific resource. The features are defined separately for each API.

“For any API that defines resources, suitable resources associated to or representing the NF Service Consumer (e.g. a top-level resource or a sub-resource representing the NF Service Consumer) shall be identified in each API to support the negotiation of the applicable features between the NF Service Consumer and NF Service Producer for this resource. Each such resource for a 5G API shall contain an attribute (e.g. "supportedFeatures") of the SupportedFeatures data type defined in 3GPP TS 29.571 [13] containing a bitmask to indicate supported features. The features and their positions in that bitmask are defined separately for each API.”  - TS 29.500 [1] clause 6.6.2

-	Support the registration of NF’s supported features in the NRF to allow NRF based discovery.

“Additionally, a NF instance should register all the features it supports to the NRF, to enable NF Service Consumers to discover NF Service Producers supporting specific features. A NF instance should register all the features it supports as NF Service Consumer in the corresponding default notification subscription in its NF profile to the NRF, to enable NF Service Producer to select NF Service Consumer supporting specific features.”   - TS 29.500 [1] clause 6.6.2

According to the supportedFeatures IE definition in TS 32.291 [2] clause 6.1.6, it is used for negotiating optional charging features (e.g. SMF support of inter-CHF communication) between CHF and NF(CTF). In other words, the default charging service/domain (e.g. SMF support of data connectivity charging) is not deemed to be negotiated using this IE, because CHF will anyway terminate the session in response to the initial request from a NF(CTF) if the CHF does not support the default charging service/domain for this NF(CTF).

“This IE shall be present if at least one optional feature defined in clause 6.1.8 is supported.”   - TS 32.291 [2] clause 6.1.6

Having multiple methods for CHF selection and discovery as defined in the service specific TS, e.g. locally provisioned CHF address, NRF based discovery, the purpose of supported feature differs depends on the way CHF is being selected:

-	When NF(CTF) select a CHF using, e.g. locally provisioned CHF addresses, the NF(CTF) starts to send charging data request to the selected CHF and CHF decides whether this NF(CTF) can be served. If the session continues, i.e. CHF could support the charging request from the NF(CTF), they may continue to negotiate any additional features using the supported Feature IE in charging data request/response message, as per defined in TS 32.291 [2].

-	When NF(CTF) select a CHF using NRF based discovery, as per defined in TS 29.510 [3], the supported feature may be used as a filter to match the appropriate CHF instances. In this case, it is helpful for CHF to register the supported features to the NRF, to support the NRF discovery procedure. Once the NF(CTF) select an appropriate CHF that matched the requested features, they may also continue to negotiate any additional features using the supported Feature IE in charging data request/response message, as per defined in TS 32.291 [2].

In this case, the list of supported features would have two applicable use cases in the SA5 charging specification:

-	Feature negotiation via the CHF Service API: NF (CTF) and CHF negotiate the list of optional supported features using the Nchf_ConvergedCharging Service API, by indicating the list of features in the SupportedFeature IE in the Charging Data Request/Response.

-	NRF registration/discovery via the NRF Service API: CHF register the list of supported features to NRF using the Nnrf_NFManagement Service API, which allows the NF consumer (CTF) to discover the target CHF using the Nnrf_NFDiscovery Service API.

4	Detailed proposal

Extend the supported feature table to include the charging domains, along with a note for each to indicate their applicability, i.e. only applicable for NRF registration/discovery. This solution could avoid the backward compatibility issue for the use of SupportedFeature IE in the Charging Data Request/Response.

Therefore, this solution is proposed for approval (S5-251465/CR0617).