3GPP TSG-SA5 Meeting #160	S5-251743

# Stor-Göteborg, Sweden, 7th Apr 2025 - 11th Apr 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.532 | CR | 0383 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 28.532 Update notifications with common header |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson India Private Limited |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | SBMA_Ph3 |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Notifications are updated to follow the definitions of the new common header in 28.532, including the addition of 2 new parameters to support reliable notification transfer. Unless the notification parameter specifies more detailed information, the definition of the common header parameters shall be just a reference to the common header definition. The proposals are based on TR 28.871 V19.0.0 (2024-12) clauses 5.7.3 and 5.13.3.1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update notification headers |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing functionality, inconsistent header definitions |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1.1.7.2, 11.1.1.8.2, 11.1.1.9.2, 11.1.1.10.2, 11.1.1.11.2, 11.3.1.3.2, 11.4.1.1.2, 11.6.1.1.2, 11.6.1.2.2, 12.1.1.2.2, 12.1.1.2.3 , 12.1.1.2.4, 12.1.1.2.5, 12.1.1.2.6, 12.1.1.4.1a.4, 12.1.1.4.1a.5, 12.1.1.4.1a.6, 12.1.1.4.1a.7, 12.1.1.4.1a.8, 12.1.1.4.1a.10, 12.3.1.2.2.2, 12.3.1.2.4.2.1, 12.4.1.2.2, 12.6.1.2.2, 12.6.1.2.3, 12.6.1.4.2.2, 12.6.1.4.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

##### 11.1.1.7.2	Input parameters

| Parameter Name | S | Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| objectClass | M | ManagedEntity.objectClass | It specifies the class name of the IOC. A network event has occurred in an instance of this class. |
| objectInstance | M | ManagedEntity.objectInstance | It specifies a new instance of the above IOC in which the network event related to by carrying the Distinguished Name (DN) for the instance. |
| notificationId | M | This is an identifier for the notification, which may be used to correlate notifications. | The identifier of the notification shall be chosen to be unique across all notifications of a particular managed object instance throughout the time that correlation is significant, it uniquely identifies the notification from other notifications generated by the subject MOI. |
| notificationType | M | It specifies the type of provisioning management services related notifications. The value “notifyMOICreation” shall be carried. | It specifies the type of notification. |
| eventTime | M | It indicates the MOICreation event time. | See RFC 3339 [52] section 5.6 for details. |
| systemDN | M | See clause 11.0.2 | - |
| sequenceNo | CM | See clause 11.0.2 |  |
| subscriptionId | CM | See clause 11.0.2 |  |
| correlatedNotifications | CM | It specifies a set of notifications that are correlated to the subject notification. | The condition is that the MnS producer support the correlation of notifications |
| additionalText | O | It can contain further information in text on the event of the ManagedEntity(s). | - |
| sourceIndicator | O | ENUM( Resource_operation, Management_operation, SON_operation,Unknown) | This parameter, when present, indicates the source of the operation that led to the generation of this notification. It can have one of the following values: 1. resource operation: The notification was generated in response to an internal operation of the resource; 2. management operation: The notification was generated in response to a management operation applied across the managed object boundary external to the managed object; 3. SON operation: The notification was generated as result of a SON (Self Organising Network) process like self-configuration, self-optimization, self-healing etc. . 4. unknown: It is not possible to determine the source of the operation. Remark: A provisioning MnS provider may not in any case be aware that SON operation lead to the generation of this generation. In this case another value than SON_operation for sourceIndicator might be sent. |
| attributeList | O | LIST OF SEQUENCE <AttributeName, AttributeValue> | The attributes (name/value pairs) of the created MOI. |

Next change

##### 11.1.1.8.2	Input parameters

| Parameter Name | S | Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| objectClass | M | ManagedEntity.objectClass | It specifies the class name of the IOC. A network event has occurred in an instance of this class. |
| objectInstance | M | ManagedEntity.objectInstance | It specifies an existing instance of the above IOC in which the network event related to by carrying the Distinguished Name (DN) for the instance. |
| notificationId | M | This is an identifier for the notification, which may be used to correlate notifications. | The identifier of the notification shall be chosen to be unique across all notifications of a particular managed object throughout the time that correlation is significant, it uniquely identifies the notification from other notifications generated by the subject MOI. |
| notificationType | M | It specifies the type of provisioning management services related notifications. The value “notifyMOIDeletion” shall be carried. | It specifies the type of notification. |
| eventTime | M | It indicates the MOIDeletion event time. | See RFC 3339 [52] section 5.6 for details. |
| systemDN | M | See clause 11.0.2 | - |
| sequenceNo | CM | See clause 11.0.2 |  |
| subscriptionId | CM | See clause 11.0.2 |  |
| correlatedNotifications | CM | It specifies a set of notifications that are correlated to the subject notification. | The condition is that the MnS producer support the correlation of notifications |
| additionalText | O | It can contain further information in text on the event of the ManagedEntity(s). | - |
| sourceIndicator | O | ENUM( Resource_operation, Management_operation, SON_operation,Unknown) | This parameter, when present, indicates the source of the operation that led to the generation of this notification. It can have one of the following values: 1. resource operation: The notification was generated in response to an internal operation of the resource; 2. management operation: The notification was generated in response to a management operation applied across the managed object boundary external to the managed object; 3. SON operation: The notification was generated as result of a SON (Self Organising Network) process like self-configuration, self-optimization, self-healing etc. . 4. unknown: It is not possible to determine the source of the operation. Remark: A provisioning MnS provider may not in any case be aware that SON operation lead to the generation of this generation. In this case another value than SON_operation for sourceIndicator might be sent. |
| attributeList | O | LIST OF SEQUENCE <AttributeName, AttributeValue> | The attributes (name/value pairs) of the deleted MOI. |

Next change

##### 11.1.1.9.2	Input parameters

| Parameter Name | S | Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| objectClass | M | ManagedEntity.objectClass | It specifies the class name of the IOC. A network event has occurred in an instance of this class. |
| objectInstance | M | ManagedEntity.objectInstance | It specifies the existing instance of the above IOC in which the network event related to by carrying the Distinguished Name (DN) for the instance. |
| notificationId | M | This is an identifier for the notification, which may be used to correlate notifications. | The identifier of the notification shall be chosen to be unique across all notifications of a particular managed object throughout the time that correlation is significant, it uniquely identifies the notification from other notifications generated by the subject Information Object. |
| notificationType | M | It specifies the type of provisioning management services related notifications. The value “notifyMOIAttributeValueChange” shall be carried. | It specifies the type of notification. |
| eventTime | M | It indicates the MOIAttributeValueChange event time. | See RFC 3339 [52] section 5.6 for details. |
| systemDN | M | See clause 11.0.2 | - |
| sequenceNo | CM | See clause 11.0.2 |  |
| subscriptionId | CM | See clause 11.0.2 |  |
| correlatedNotifications | CM | It specifies a set of notifications that are correlated to the subject notification. | The condition is that the MnS producer support the correlation of notifications |
| additionalText | O | It can contain further information in text on the event of the ManagedEntity(s). | - |
| sourceIndicator | O | ENUM( Resource_operation, Management_operation, SON_operation,Unknown) | This parameter, when present, indicates the source of the operation that led to the generation of this notification. It can have one of the following values: 1. resource operation: The notification was generated in response to an internal operation of the resource; 2. management operation: The notification was generated in response to a management operation applied across the managed object boundary external to the managed object; 3. SON operation: The notification was generated as result of a SON (Self Organising Network) process like self-configuration, self-optimization, self-healing etc. . 4. unknown: It is not possible to determine the source of the operation. Remark: A provisioning MnS provider may not in any case be aware that SON operation lead to the generation of this generation. In this case another value than SON_operation for sourceIndicator might be sent. |
| attributeValueChange | M | LIST OF SEQUENCE <AttributeName, NewAttributeValue, CHOICE [NULL, OldAttributeValue]> | The changed attributes (name/value pairs) of the MOI (with both new and, optionally, old values). |

Next change

##### 11.1.1.10.2	Input parameters

| Parameter Name | S | Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| objectClass | M | ManagedEntity.objectClass | -- |
| objectInstance | M | ManagedEntity.objectInstance | -- |
| notificationId | M | It carries the identifier for the subject notification. | See Note 1. |
| eventTime | M | It indicates the time of the event. | See RFC 3339 [52] section 5.6 for details. |
| systemDN | M | See clause 11.0.2 | -- |
| notificationType | M | "notifyEvent" | -- |
| sequenceNo | CM | See clause 11.0.2 |  |
| subscriptionId | CM | See clause 11.0.2 |  |
| specificProblem | M | It indicates a problem detected. | -- |
| additionalText | O | It carries additional information. | -- |
| additionalInformation | O | It carries additional information. | -- |

Next change

##### 11.1.1.11.2	Input parameters

| Parameter Name | S | Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| objectClass | M | See clause 11.0.2 | Identifies the classe name of a common ancestor object of the objects for which changes are reported. A MnS producer may set this parameter always to the class name of the parent of the local root object in the MIB. |
| objectInstance | M | See clause 11.0.2 | Identifies the instance of a common ancestor object of the objects for which changes are reported. A MnS producer may set this parameter always to the instance of the parent of the local root object in the MIB. |
| notificationId | M | See clause 11.0.2 |  |
| notificationType | M | “notifyMOIChanges” |  |
| eventTime | M | See clause 11.0.2 |  |
| systemDN | M | See clause 11.0.2 |  |
| sequenceNo | CM | See clause 11.0.2 |  |
| subscriptionId | CM | See clause 11.0.2 |  |
| moiChanges | M | SEQUENCE OF SET { notificationId (M), correlatedNotifications (O), additionalText (O), sourceIndicator (O), op (M), path (M), value (M) , oldValue (O) } | This parameter describes the reported NRM updates. It is a list of items; each item reports a single NRM update. The "notificationId" identifies an item. The NRM update itself is described by the parameters "op", "path", "value" and "oldValue". The parameters "correlatedNotifications", "additionalText" and "sourceIndicator " provide context information. The parameter "op" specifies the type of operation reporting the NRM update. Valid values are "add", "remove" and "replace". The operation describes what has conceptually happened to the NRM on the MnS producer. The operation applied to the NRM by the MnS producer and causing the reported NRM update can be different. "add" shall be used for reporting the creation of an object, attribute, attribute field or multi-value attribute element. "remove" shall be used for reporting the deletion of an object, attribute, attribute field or multi-value attribute element. "replace" shall be used for reporting the replacement of an existing attribute value, attribute field value or multi-value attribute element. The "path" and "objectInstance" identify the object, attribute, attribute field or multi-value attribute element, that was created, deleted or replaced. If an object creation is reported with "add", the "value" shall carry a complete representation of the created object. If an object deletion is reported with "remove", the "value" shall be absent. It may optionally carry a complete representation of the deleted object. If an attribute, attribute field or multi-value attribute element creation is reported with "add", the "value" shall carry the value of the created attribute, attribute field or multi-value attribute element. If an attribute, attribute field or multi-value attribute element deletion is reported with "remove", the "value" shall be absent. It may optionally carry the old value of the deleted attribute, attribute field or multi-value attribute element. If the replacement of an attribute, attribute field or multi-value attribute element value is reported with "replace", the "value" shall carry the new value of the attribute, attribute field or multi-value attribute element. The "oldValue" may optionally carry the old value of the attribute, attribute field or multi-value attribute element before the replacement. If multiple objects are created, the creation of parent objects shall be reported before the creation of the child objects. Vice versa, when the deletion of multiple objects is reported, the deletion of child objects shall be reported before the deletion of the parent objects. |

Next change

##### 11.3.1.3.2	Notification information

| Parameter Name | S | Information Type | Comment |
| --- | --- | --- | --- |
| objectClass | M | ManagedEntity.objectClass | Class of the managed object, where the threshold crossing occurred. |
| objectInstance | M | ManagedEntity.objectInstance | Instance of the managed object, where the threshold crossing occurred. |
| notificationId | M | See clause 11.0.2 |  |
| notificationType | M | "notifyThresholdCrossing" |  |
| eventTime | M | See clause 11.0.2 | Time when the threshold crossing occurred. |
| systemDN | M | See clause 11.0.2 |  |
| sequenceNo | CM | See clause 11.0.2 |  |
| subscriptionId | CM | See clause 11.0.2 |  |
| observedPerfMetricName | M | ThresholdMonitor.thresholdInfoList.\ performanceMetrics (see TS 28.622 [11]) | Name of the performance metric that has crossed the threshold. |
| observedPerfMetricValue | M | -- | Value of the performance metric, that has crossed the threshold, when the threshold crossing was observed |
| observedPerfMetricDirection | M | -- | Direction ("UP" or "DOWN") of the performance metric, when the threshold crossing was observed |
| thresholdValue | M | ThresholdMonitor.thresholdInfoList.\ thresholdvalue (see TS 28.622 [11]) | Threshold value of the triggered threshold |
| hysteresis | O | ThresholdMonitor.thresholdInfoList.\ hysteresis (see TS 28.622 [11]) | Hysteresis of the triggered threshold |
| monitorGranularityPeriod | M | ThresholdMonitor.monitorGranularityPeriod | Granularity period of the threshold monitor |
| additionalText | O | -- | Vendor specific information |

Next change

##### 11.4.1.1.2	Input parameters

| Parameter Name | S | Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| objectClass | M | HeartbeatControl.objectClass |  |
| objectInstance | M | HeartbeatControl.objectInstance | Instance controlling the emission of this notifyHeartbeat notification. |
| notificationId | M | See clause 11.0.2 |  |
| notificationType | M | "notifyHeartbeat" |  |
| eventTime | M | See clause 11.0.2 |  |
| systemDN | M | See clause 11.0.2 |  |
| sequenceNo | CM | See clause 11.0.2 |  |
| subscriptionId | CM | See clause 11.0.2 |  |
| heartbeatNtfPeriod | M | HeartbeatControl.heartbeatNtfPeriod |  |

Next change

##### 11.6.1.1.2	Input parameters

| Parameter Name | S | Information Type | Comment |
| --- | --- | --- | --- |
| objectClass | M | Entity.objectClass | See clause 11.6.1.1.1 for the definition of Entity |
| objectInstance | M | Entity.objectInstance | See clause 11.6.1.1.1 for the definition of Entity |
| notificationId | M | See clause 11.0.2 |  |
| notificationType | M | "notifyFileReady" |  |
| eventTime | M | See clause 11.0.2 | Time when the file, that triggered this notification, was ready for upload. |
| systemDN | M | See clause 11.0.2 |  |
| sequenceNo | CM | See clause 11.0.2 |  |
| subscriptionId | CM | See clause 11.0.2 |  |
| fileInfoList | M | List of struct < fileLocation (M), fileCompression (M), fileSize (O), fileDataType (M), fileFormat (M), fileReadyTime (O), fileExpirationTime (O), …jobId (CO) > Each element is defined as following: - "fileLocation": Location of the file. The location may be a directory path or a URL, for example "\\202.112.101.1\D:\user\Files\<xxx>", or "ftp://nms.telecom_org.com/datastore/<xxx>, where <xxx> is the filename. - "fileCompression": Name of the algorithm used for compressing the file. An empty or absent "fileCompression" parameter indicates the file is not compressed. The MnS producer selects the compression algorithm. It is encouraged to use popular algorithms such as GZIP. - "fileSize": Size of the file. Its value is a non negative integer. The unit is byte. - "fileDataType": Type of the management data stored in the file. Allowed values are: - "PERFORMANCE" - "TRACE" - "ANALYTICS" - "PROPRIETARY" The value "PERFORMANCE" refers to measurements and KPIs. - "fileFormat": Identifier of the XML or ASN.1 schema (incl. its version) used to produce the file content. - "fileReadyTime": Date and time when the file was closed (the last time) and made available on the MnS producer. The file content will not be changed anymore. - "fileExpirationTime": Date and time after which the file may be deleted. It shall not be empty and shall be later than "fileReadyTime". - "jobId": Job identifier of the "PerfMetricJob" (TS 28.622 [11]) or "TraceJob" (TS 28.622 [11]) that produced the file. This parameter should be present, when the file is related to a job and that job is represented by a "PerfMetricJob" or "TraceJob". Multiple jobs may share the same job identifier. This may for example be the case for jobs collecting measurements to compuate a KPI or for jobs related to a specific task in some analytics application. Note that a specific job is identified by the objectClass/objectInstance parameters of the notification header. | Information (meta data) about the new file, that became ready for upload and triggered this notification, and information about files, which became ready for upload earlier and are still available for upload when the notification is sent. |
| additionalText | O | -- | Allows a free form text description to be reported as defined in ITU-T Rec. X. 733 [4] |

Next change

##### 11.6.1.2.2	Input parameters

| Parameter Name | S | Information Type | Comment |
| --- | --- | --- | --- |
| objectClass | M | Entity.objectClass. | See clause 11.6.1.1.1 for the definition of Entity |
| objectInstance | M | Entity.objectInstance | See clause 11.6.1.1.1 for the definition of Entity. |
| notificationId | M | See clause 11.0.2 | See Table 11.6.1.1.2-1. |
| notificationType | M | "notifyFilePreparationError" |  |
| eventTime | M | See clause 11.0.2 | Time when the file preparation error occured |
| systemDN | M | See clause 11.0.2 |  |
| sequenceNo | CM | See clause 11.0.2 |  |
| subscriptionId | CM | See clause 11.0.2 |  |
| fileInfoList | M | See Table 11.6.1.1.2-1. | Each list item contains information about a file where a file preparation error occurred and that is kept on the MnS producer. Files, that are deleting or not created at all, have no list item. |
| reason | M | -- | Detailed error reason, including - errorInPreparation - hardDiskFull - hardDiskFailure - tooManyFiles - collectionTimeOut - incompleteTruncatedFile - corruptedFile - lowMemory - dataNotAvailable |
| additionalText | O | -- | Allows a free form text description to be reported as defined in ITU-T Rec. X. 733 [4] |

Next change

##### 12.1.1.2.2	Notification notifyMOICreation

The IS notification parameters are mapped to SS equivalents according to table 12.1.1.2.2-1.

Table 12.1.1.2.2-1: Mapping of IS notification input parameters to SS equivalents (HTTP POST)

| IS parameter name | SS parameter location | SS parameter name | SS parameter type | S |
| --- | --- | --- | --- | --- |
| objectClass | request body | href | Uri | M |
| objectInstance |  |  |  |  |
| notificationId | request body | notificationId | NotificationId | M |
| notificationType | request body | notificationType | NotificationType | M |
| eventTime | request body | eventTime | DateTime | M |
| systemDN | request body | systemDN | SystemDN | M |
| sequenceNo | request body | sequenceNo | integer | CM |
| subscriptionId | request body | subscriptionId | DN | CM |
| correlatedNotifications | request body | correlatedNotifications | array(CorrelatedNotification) | O |
| additionalText | request body | additionalText | AdditionalText | O |
| sourceIndicator | request body | sourceIndicator | SourceIndicator | O |
| attributeList | request body | attributeList | AttributeNameValuePairSet | O |

##### 12.1.1.2.3	Notification notifyMOIDeletion

The IS notification parameters are mapped to SS equivalents according to table 12.1.1.2.3-1.

Table 12.1.1.2.3-1: Mapping of IS notification input parameters to SS equivalents (HTTP POST)

| IS parameter name | SS parameter location | SS parameter name | SS parameter type | S |
| --- | --- | --- | --- | --- |
| objectClass | request body | href | Uri | M |
| objectInstance |  |  |  |  |
| notificationId | request body | notificationId | NotificationId | M |
| notificationType | request body | notificationType | NotificationType | M |
| eventTime | request body | eventTime | DateTime | M |
| systemDN | request body | systemDN | SystemDN | M |
| sequenceNo | request body | sequenceNo | integer | CM |
| subscriptionId | request body | subscriptionId | DN | CM |
| correlatedNotifications | request body | correlatedNotifications | array(CorrelatedNotification) | O |
| additionalText | request body | additionalText | AdditionalText | O |
| sourceIndicator | request body | sourceIndicator | SourceIndicator | O |
| attributeList | request body | attributeList | AttributeNameValuePairSet | O |

##### 12.1.1.2.4	Notification notifyMOIAttributeValueChanges

The IS notification parameters are mapped to SS equivalents according to table 12.1.1.2.4-1.

Table 12.1.1.2.4-1: Mapping of IS notification input parameters to SS equivalents (HTTP POST)

| IS parameter name | SS parameter location | SS parameter name | SS parameter type | S |
| --- | --- | --- | --- | --- |
| objectClass | request body | href | Uri | M |
| objectInstance |  |  |  |  |
| notificationId | request body | notificationId | NotificationId | M |
| notificationType | request body | notificationType | NotificationType | M |
| eventTime | request body | eventTime | DateTime | M |
| systemDN | request body | systemDN | SystemDN | M |
| sequenceNo | request body | sequenceNo | integer | CM |
| subscriptionId | request body | subscriptionId | DN | CM |
| correlatedNotifications | request body | correlatedNotifications | array(CorrelatedNotification) | O |
| additionalText | request body | additionalText | AdditionalText | O |
| sourceIndicator | request body | sourceIndicator | SourceIndicator | O |
| attributeListValueChanges | request body | attributeListValueChange | AttributeValueChangeSet | M |

##### 12.1.1.2.5	Notification notifyMOIChanges

The IS notification parameters are mapped to SS equivalents according to table 12.1.1.2.5-1.

Table 12.1.1.2.5-1: Mapping of IS notification input parameters to SS equivalents (HTTP POST)

| IS parameter name | SS parameter location | SS parameter name | SS parameter type | S |
| --- | --- | --- | --- | --- |
| objectClass | request body | href | Uri | M |
| objectInstance |  |  |  |  |
| notificationId | request body | notificationId | NotificationId | M |
| notificationType | request body | notificationType | NotificationType | M |
| eventTime | request body | eventTime | DateTime | M |
| systemDN | request body | systemDN | SystemDN | M |
| sequenceNo | request body | sequenceNo | integer | CM |
| subscriptionId | request body | subscriptionId | DN | CM |
| moiChanges | request body | mOIChanges | array(MoiChange) | M |

##### 12.1.1.2.6	Notification notifyEvent

The IS notification parameters are mapped to SS equivalents according to table 12.1.1.2.6-1.

Table 12.1.1.2.6-1: Mapping of IS notification input parameters to SS equivalents (HTTP POST)

| IS parameter name | SS parameter location | SS parameter name | SS parameter type | S |
| --- | --- | --- | --- | --- |
| objectClass | request body | href | Uri | M |
| objectInstance |  |  |  |  |
| notificationId | request body | notificationId | NotificationId | M |
| notificationType | request body | notificationType | NotificationType | M |
| eventTime | request body | eventTime | DateTime | M |
| systemDN | request body | systemDN | SystemDN | M |
| sequenceNo | request body | sequenceNo | integer | CM |
| subscriptionId | request body | subscriptionId | DN | CM |
| specificProblem | request body | specificProblem | SpecificProblem | M |
| additionalText | request body | additionalText | string | O |
| additionalInformation | request body | additionalInformation | AttributeNameValuePairSet | O |

Next change

###### 12.1.1.4.1a.4	Type MoiChange

Table 12.1.1.4.1a.4 -1: Definition of type MoiChange

| Attribute name | Data type | Description | S |
| --- | --- | --- | --- |
| notificationId | NotificationId | Notification identifier as defined in ITU-T Rec. X. 733 [4] | M |
| correlatedNotifications | array(CorrelatedNotification) | Set of all notifications to which this notification is considered to be correlated as defined in ITU-T Rec. X. 733 [4] | O |
| additionalText | string | Allows a free form text description to be reported as defined in ITU-T Rec. X. 733 [4] | O |
| sourceIndicator | SourceIndicator | Indicates the source of the operation that led to the generation of this notification. | O |
| op | Operation | Operation associated to the reported change ("add", "remove , "replace"). | M |
| path | string | URI path component segments specifying when appended to "href" the created, deleted or updated resource or secondary resource | M |
| insert | Insert | Indicates whether the new attribute element was added before or after the attribute element specified by "path", only valid for attributes with the property isOrdered=True. It can take the values "before" and "after". If missing, it defaults to "before". The "insert" attribute shall be supported only when changes from YANG defined NRMs are reported. For JSON defined NRMs the attribute shall not be supported. | CM |
| value | any type | New value of the created or updated resource or secondary resource. Optional old value of the deleted resource or secondary resource | M |
| oldValue | any type | Old value of the updated secondary resource | O |

The properties "op", "path" and "value" shall use the 3GPP JSON Patch format (TS 32.158 [15]) for reporting NRM changes. The "merge" operation specified by 3GPP JSON Patch is not supported in "notifyMOIChanges". The "move", "copy" and "test" operations specified by JSON Patch are not supported either.

The "oldValue" is an optional extension for "notifyMOIChanges" allowing to report also the value that the attribute had before replacing the value with the new value, that is contained in "value".

The following example notification (where JSON is expressed in YAML notation) reports an object creation

href: https://example.com/3gpp

...

moiChanges

- notificationId: 123456789

op: add

path: /ClassA=1

value:

id: 1,

objectClass: ClassA,

attributes:

attrA: 123

attrB:

subAttrB1: ABC

subAttrB2: 56

The following example reports the deletion of that object.

href: https://example.com/3gpp

...

moiChanges

- notificationId: 123456789

op: remove

path: /ClassA=1

The following example reports the addition of a new attribute "attrC".

href: https://example.com/3gpp

...

moiChanges

- notificationId: 123456789

op: add

path: /ClassA=1#/attributes/attrC

value: xyz

The following example reports the deletion of the attribute "attrC".

href: https://example.com/3gpp

...

moiChanges

- notificationId: 123456789

op: remove

path: /ClassA=1#/attributes/attrC

The following example reports a value change for the simple attribute "attrA".

href: https://example.com/3gpp

...

moiChanges

- notificationId: 123456789

op: replace

path: /ClassA=1#/attributes/attrA

value: 456

When the old value is reported as well, the notification looks like.

href: https://example.com/3gpp

...

moiChanges

- notificationId: 123456789

op: replace

path: /ClassA=1#/attributes/attrA

value: 456

oldValue: 123

The following example reports a value change for the complex attribute "attrB".

href: https://example.com/3gpp

...

moiChanges

- notificationId: 123456789

op: replace

path: /ClassA=1#/attributes/attrB

value:

subAttrB1: abc

subAttrB2: 78

The previous two notifications can be combined into a single notification as follows.

href: https://example.com/3gpp

...

moiChanges

- notificationId: 123456789

op: replace

path: /ClassA=1#/attributes/attrA

value: 456

- notificationId: 123456789

op: replace

path: /ClassA=1#/attributes/attrB

value:

subAttrB1: abc

subAttrB2: 78

Note the operation "replace" has replace semantics and not merge semantics. The following notification reports the value change of the attribute field "attrB:subAttrB1" to "def" and the deletion of the attribute field "attrB:subAttrB2".

href: https://example.com/3gpp

...

moiChanges

- notificationId: 123456789

op: replace

path: /ClassA=1#/attributes/attrB

value:

subAttrB1: def

The value change of the attribute field "attrA:subAttrB1" is reported as follows.

href: https://example.com/3gpp

...

moiChanges

- notificationId: 123456789

op: replace

path: /ClassA=1#/attributes/attrA/subAttrB1

value: def

Assume "attrD" is a JSON array with simple elements, then the creation of this multi-valued attribute is reported as follows.

href: https://example.com/3gpp

...

moiChanges

- notificationId: 123456789

op: add

path: /ClassA=1#/attributes/attrD

value:

- 1

- 2

- 3

Its deletion is reported by the following notification.

href: https://example.com/3gpp

...

moiChanges

- notificationId: 123456789

op: remove

path: /ClassA=1#/attributes/attrD

The complete replacement of the array is reported by the following notification.

href: https://example.com/3gpp

...

moiChanges

- notificationId: 123456789

op: add

path: /ClassA=1#/attributes/attrD

value:

- 11

- 21

- 31

The following example reports the second item in the array changed to "22".

href: https://example.com/3gpp

...

moiChanges

- notificationId: 123456789

op: replace

path: /ClassA=1#/attributes/attrD/1

value: 22

Note the array index of the second item is "1".

Assume now "attrE" is a JSON array with complex array items, for example.

[{subItemE1: 11, subItemD2: abc}, {subItemE1: 21, subItemE2: def}, {subItemE1: 31, subItemE2": ghi}.

A value change to

[{subItemE1: 11, subItemE2: abc}, {subItemE1: 21, subItemE2: xyz}, {subItemE1: 31, subItemE2": ghi}.

is reported by

href: https://example.com/3gpp

...

moiChanges

- notificationId: 123456789

op: replace

path: /ClassA=1#/attributes/attrE/1/subItemE2

value: xyz

When "subItemE2" is defined as array item key at stage 2, then "attrE" should contain a JSON map.

attrE:

11:

subItemE2: abc

21:

subItemE2: def

31:

subItemE2: ghi

The same change as above is now reported by the notification.

href: https://example.com/3gpp

...

moiChanges

- notificationId: 123456789

op: replace

path: /ClassA=1#/attributes/attrE/21/subItemD2

value: xyz

When all attributes of an object have been updated with a new value, the MnS producer may use a compact format reporting that the "attributes" container was updated completely.

POST /3gpp-management/cm-notification-sink HTTP/1.1 Host: example.com Content-Type: application/json { "href": "http://example.com/3gpp", "notificationId": 123456789, "notificationType": "notifyMOIChanges", "eventTime": "2019-08-06T16:50:26-08:00", "systemDN":"DC=example.com,ManagedElement=ME1,MnsAgent=MA1", "sequenceNo": "123", "subscriptionId": "DC=example.com,ManagedElement=ME1,NtfSubscriptionControl=CmMirror", "moiChanges": [ { "notificationId": 123, "op": "replace", "path": "/ClassA=1#/attributes", "value": { "attrA": "newValueAttrA", "attrB": "newValueAttrB" } } ] }

Note that clause 4.3 of IETF RFC 6902 [36] does not consider it as an error if an attribute value is replaced with exactly the same value. For that reason, it would not be an error if in the example above an attribute value is included in the "value" property that did not change value. A MnS producer may consider this compact format hence also for the case that not all attributes of an object have been updated.

POST /3gpp-management/cm-notification-sink HTTP/1.1 Host: example.com Content-Type: application/json { "href": "http://example.com/3gpp", "notificationId": 123456789, "notificationType": "notifyMOIChanges", "eventTime": "2019-08-06T16:50:26-08:00", "systemDN":"DC=example.com,ManagedElement=ME1,MnsAgent=MA1", "sequenceNo": "123", "subscriptionId": "DC=example.com,ManagedElement=ME1,NtfSubscriptionControl=CmMirror", "moiChanges": [ { "notificationId": 123, "op": "replace", "path": "/ClassA=1#/attributes", "value": { "attrA": "newValueAttrA", "attrB": "oldValueAttrB" } } ] }

To allow the MnS consumer to understand which attributes have been updated, the MnS producer may decide to send the following notification.

POST /3gpp-management/cm-notification-sink HTTP/1.1 Host: example.com Content-Type: application/json { "href": "http://example.com/3gpp", "notificationId": 123456789, "notificationType": "notifyMOIChanges", "eventTime": "2019-08-06T16:50:26-08:00", "systemDN":"DC=example.com,ManagedElement=ME1,MnsAgent=MA1", "sequenceNo": "123", "subscriptionId": "DC=example.com,ManagedElement=ME1,NtfSubscriptionControl=CmMirror", "moiChanges": [ { "notificationId": 123, "op": "replace", "path": "/ClassA=1#/attributes", "value": { "attrA": "newValueAttrA", "attrB": "oldValueAttrB" }, "oldValue": { "attrA": "oldValueAttrA", "attrB": "oldValueAttrB" } ] }

###### 12.1.1.4.1a.5	Type NotifyMoiCreation

Table 12.1.1.4.1a.5 -1: Definition of type NotifyMoiCreation

| Attribute name | Data type | Description | S |
| --- | --- | --- | --- |
| href | Uri | URI of the resource where the event (alarm) occurred | M |
| notificationId | NotificationId | Notification identifier as defined in ITU-T Rec. X. 733 [4] | M |
| notificationType | NotificationType | Notification type ("notifyMOICreation") | M |
| eventTime | DateTime | Event (MOI creation) occurrence time | M |
| systemDN | SystemDN | System DN | M |
| sequenceNo | integer | See clause 11.0.2 | CM |
| subscriptionId | DN | See clause 11.0.2 | CM |
| correlatedNotifications | array(CorrelatedNotification) | Set of all notifications to which this notification is considered to be correlated as defined in ITU-T Rec. X. 733 [4] | O |
| additionalText | string | Allows a free form text description to be reported as defined in ITU-T Rec. X. 733 [4] | O |
| sourceIndicator | SourceIndicator | Indicates the source of the operation that led to the generation of this notification. | O |
| attributeList | AttributeNameValuePairSet | The attributes (name/value pairs) of the created MOI. | O |

The following example shows a notification reporting the creation of an object with two attributes "attrA" and "attrB". Note that the notification includes the name/value pairs representing the attributes of the created object only and not the complete object representation.

POST /3gpp-management/cm-notification-sink HTTP/1.1 Host: example.com Content-Type: application/json { "href": "http://example.com/ManagedElement=ME1/ClassA=CA1", "notificationId": 123456789, "notificationType": "notifyMOICreation", "eventTime": "2019-08-06T16:50:26-08:00", "systemDN":"DC=example.com,ManagedElement=ME1,MnsAgent=MA1", "sequenceNo": "123", "subscriptionId": "DC=example.com,ManagedElement=ME1,NtfSubscriptionControl=CmMirror", "attributeList": { "attrA": "valueAttrA", "attrB": "valueAttrB" } }

The creation of an empty object not containing any attribute values is reported as follows.

POST /3gpp-management/cm-notification-sink HTTP/1.1 Host: example.com Content-Type: application/json { "href": "http://example.com/ManagedElement=ME1/ClassA=CA1", "notificationId": 123456789, "notificationType": "notifyMOICreation", "eventTime": "2019-08-06T16:50:26-08:00", "systemDN":"DC=example.com,ManagedElement=ME1,MnsAgent=MA1", "sequenceNo": "123", "subscriptionId": "DC=example.com,ManagedElement=ME1,NtfSubscriptionControl=CmMirror", "attributeList": { } }

###### 12.1.1.4.1a.6	Type NotifyMoiDeletion

Table 12.1.1.4.1a.6 -1: Definition of type NotifyMoiDeletion

| Attribute name | Data type | Description | S |
| --- | --- | --- | --- |
| href | Uri | URI of the resource where the event (alarm) occurred | M |
| notificationId | NotificationId | Notification identifier as defined in ITU-T Rec. X. 733 [4] | M |
| notificationType | NotificationType | Notification type ("notifyMOIDeletion") | M |
| eventTime | DateTime | Event (MOI deletion) occurrence time | M |
| systemDN | SystemDN | System DN | M |
| sequenceNo | integer | See clause 11.0.2 | CM |
| subscriptionId | DN | See clause 11.0.2 | CM |
| correlatedNotifications | array(CorrelatedNotification) | Set of all notifications to which this notification is considered to be correlated as defined in ITU-T Rec. X. 733 [4] | O |
| additionalText | string | Allows a free form text description to be reported as defined in ITU-T Rec. X. 733 [4] | O |
| sourceIndicator | SourceIndicator | Indicates the source of the operation that led to the generation of this notification. | O |
| attributeList | AttributeNameValuePairSet | Attributes (name/value pairs) of the deleted MOI. | O |

The following example demonstrates the deletion of an object. The message body includes the name/value pairs representing the attributes of the deleted object.

POST /3gpp-management/cm-notification-sink HTTP/1.1 Host: example.com Content-Type: application/json { "href": "http://example.com/ManagedElement=ME1/ClassA=CA1", "notificationId": 123456789, "notificationType": "notifyMOIDeletion", "eventTime": "2019-08-06T16:50:26-08:00", "systemDN":"DC=example.com,ManagedElement=ME1,MnsAgent=MA1" "sequenceNo": "123", "subscriptionId": "DC=example.com,ManagedElement=ME1,NtfSubscriptionControl=CmMirror", }

The message body may include the name/value pairs representing the attributes of the deleted object.

POST /3gpp-management/cm-notification-sink HTTP/1.1 Host: example.com Content-Type: application/json { "href": "http://example.com/ManagedElement=ME1/ClassA=CA1", "notificationId": 123456789, "notificationType": "notifyMOIDeletion", "eventTime": "2019-08-06T16:50:26-08:00", "systemDN":"DC=example.com,ManagedElement=ME1,MnsAgent=MA1", "sequenceNo": "123", "subscriptionId": "DC=example.com,ManagedElement=ME1,NtfSubscriptionControl=CmMirror", "attributeList": { "attrA": "valueAttrA", "attrB": "valueAttrB" } }

###### 12.1.1.4.1a.7	Type NotifyMoiAttributeValueChanges

Table 12.1.1.4.1a.7 -1: Definition of type NotifyMoiAttributeValueChanges

| Attribute name | Data type | Description | S |
| --- | --- | --- | --- |
| href | Uri | URI of the resource where the event (alarm) occurred | M |
| notificationId | NotificationId | Notification identifier as defined in ITU-T Rec. X. 733 [4] | M |
| notificationType | NotificationType | Notification type ("notifyMOIAttributeValueChanges") | M |
| eventTime | DateTime | Event (MOI attribute value changes) occurrence time | M |
| systemDN | SystemDN | System DN | M |
| sequenceNo | integer | See clause 11.0.2 | CM |
| subscriptionId | DN | See clause 11.0.2 | CM |
| correlatedNotifications | array(CorrelatedNotification) | Set of all notifications to which this notification is considered to be correlated as defined in ITU-T Rec. X. 733 [4] | O |
| additionalText | string | Allows a free form text description to be reported as defined in ITU-T Rec. X. 733 [4] | O |
| sourceIndicator | SourceIndicator | Indicates the source of the operation that led to the generation of this notification. | O |
| attributeListValueChanges | AttributeValueChangeSet | List with names of changed attributes, together with new value and optionally old value | M |

The following example notification reports the modification of the attribute values for "attrA" and "attrB".

POST /3gpp-management/cm-notification-sink HTTP/1.1 Host: example.org Content-Type: application/json { "href": "http://example.com/ManagedElement=ME1/ClassA=CA1", "notificationId": 123456789, "notificationType": "notifyMOIAttributeValueChanges", "eventTime": "2019-08-06T16:50:26-08:00", "systemDN":"DC=example.com,ManagedElement=ME1,MnsAgent=MA1", "sequenceNo": "123", "subscriptionId": "DC=example.com,ManagedElement=ME1,NtfSubscriptionControl=CmMirror", "attributeListValueChanges": [ { "attrA": "newValueAttrA", "attrB": "newValueAttrB" } ] }

In addition to the new values, the old values may be included in the notification.

POST /3gpp-management/cm-notification-sink HTTP/1.1 Host: example.org Content-Type: application/json { "href": "http://example.com/ManagedElement=ME1/ClassA=CA1", "notificationId": 123456789, "notificationType": "notifyMOIAttributeValueChanges", "eventTime": "2019-08-06T16:50:26-08:00", "systemDN":"DC=example.com,ManagedElement=ME1,MnsAgent=MA1", "sequenceNo": "123", "subscriptionId": "DC=example.com,ManagedElement=ME1,NtfSubscriptionControl=CmMirror", "attributeListValueChanges": [ { "attrA": "newValueAttrA", "attrB": "newValueAttrB" }, { "attrA": "oldValueAttrA", "attrB": "oldValueAttrB" } ] }

In the example above the attribute values are of simple type. For attributes of structured type, the question is if all attribute fields of an attribute, where at least one attribute field changed its value, need to be reported or only those attribute fields that changed value. Assume an attribute field that did not change value is not included in the notification. In this case the MnS consumer receiving the notification cannot tell if the attribute field was deleted or if the attribute field did not change value. It is not possible to distinguish these two cases based on the information in the notification. For that reason, always all attribute fields of an attribute need to be included in the notification. For structured attributes with many attribute fields this may not be very efficient.

In the next example "attrA" is a structured attribute with the attribute fields "attrFieldAA" and "attrFieldAB". The attribute field "attrFieldAA" changed value, the attribute field "attrFieldAB" did not change value. The attribute value change notification reporting this change may look as follows.

POST /3gpp-management/cm-notification-sink HTTP/1.1 Host: example.org Content-Type: application/json { "href": "http://example.com/ManagedElement=ME1/ClassA=CA1", "notificationId": 123456789, "notificationType": "notifyMOIAttributeValueChanges", "eventTime": "2019-08-06T16:50:26-08:00", "systemDN":"DC=example.com,ManagedElement=ME1,MnsAgent=MA1", "sequenceNo": "123", "subscriptionId": "DC=example.com,ManagedElement=ME1,NtfSubscriptionControl=CmMirror", "attributeListValueChanges": [ { "attrA": { "attrFieldAA": "newValueAttrFieldAA", "attrFieldAB": "oldValueAttrFieldAB" } }, { "attrA": { "attrFieldAA": "oldValueAttrFieldAA", "attrFieldAB": "oldValueAttrFieldAB" } } ] }

Note also that for multi-valued attributes all attribute elements of the new value need to be reported. It is not possible to report only added, deleted, or modified attribute elements. Furthermore, the notification does not allow reporting of deleted attributes. When this is required the MnS producer needs to include always all attributes of the object in the notification. It is outside the present document how the MnS producer signals to the MnS consumer if all attributes or only the changed ones are included in the attribute value change notification. The notification itself does not include this information.

###### 12.1.1.4.1a.8	Type NotifyMoiChanges

Table 12.1.1.4.1a.8 -1: Definition of type NotifyMoiChanges

| Attribute name | Data type | Description | S |
| --- | --- | --- | --- |
| href | Uri | URI of a common ancestor resource (object) of the resources for which changes are reported. A MnS producer may set this attribute always to the parent of the root resource in the MIB. | M |
| notificationId | NotificationId | Notification identifier as defined in ITU-T Rec. X. 733 [4]. | M |
| notificationType | NotificationType | Notification type (notifyMOIChanges) | M |
| eventTime | DateTime | Event (NRM updates) occurrence time | M |
| systemDN | SystemDN | System DN | M |
| sequenceNo | integer | See clause 11.0.2 | CM |
| subscriptionId | DN | See clause 11.0.2 | CM |
| moiChanges | array(MoiChange) | MOI changes to be reported | M |

###### 12.1.1.4.1a.9	Type PatchItem

Table 12.1.1.4.1a.9 -1: Definition of type PatchItem

| Attribute name | Data type | Description | S |
| --- | --- | --- | --- |
| op | PatchOperation | Patch operation. | M |
| from | string | Present only for "copy" and "move" operations, identifies the value to be copied or moved to the location specified by path. | M |
| path | string | Path specifying the patched value. | M |
| value | any type | New value for the resource identified by "path". | M |

###### 12.1.1.4.1a.10	Type NotifyMoiEvent

Table 12.1.1.4.1a.10 -1: Definition of type NotifyEvent

| Attribute name | Data type | Description | S |
| --- | --- | --- | --- |
| href | Uri | URI of the resource where the event (alarm) occurred | M |
| notificationId | NotificationId | Notification identifier as defined in ITU-T Rec. X. 733 [4] | M |
| notificationType | NotificationType | Notification type ("notifyEvent") | M |
| eventTime | DateTime | Date and time of the event | M |
| systemDN | SystemDN | It carries the DN of producer of the notification. | M |
| sequenceNo | integer | See clause 11.0.2 | CM |
| subscriptionId | DN | See clause 11.0.2 | CM |
| specificProblem | SpecificProblem | It indicates a problem detected | M |
| additionalText | string | It carries additional information. | O |
| additionalInformation | AttributeNameValuePairSet | It carries additional information. | O |

The following is an example of the notifyEvent notification.

POST /3gpp-management/cm-notification-sink HTTP/1.1 Host: myMns.mytelecom.com Content-Type: application/json { "href": "http://myNode.com/ManagedElement=ME1 ", "notificationId": 123456789, "notificationType": "notifyEvent", "eventTime": "2019-08-06T16:50:26-08:00", "systemDN":"DC=myNode.com,ManagedElement=ME1,MnsAgent=MA1", "sequenceNo": "123", "subscriptionId": "DC=example.com,ManagedElement=ME1,NtfSubscriptionControl=Cm2", "specificProblem": "Restart", "additionalText": "Restart due to overheating", "additionalInformation": { "temperature": "94.7", "trendIndication": "MORE_SEVERE" } }

Next change

###### 12.3.1.2.2.2	Notification notifyThresholdCrossing

The IS notification parameters are mapped to SS equivalents according to table 12.3.1.2.2.2-1.

Table 12.3.1.2.2.2-1: Mapping of IS notification input parameters to SS equivalents (HTTP POST)

| IS parameter name | SS parameter location | SS parameter name | SS parameter type | S |
| --- | --- | --- | --- | --- |
| objectClass | request body | href | Uri | M |
| objectInstance |  |  |  |  |
| notificationId | request body | notificationId | NotificationId | M |
| notificationType | request body | notificationType | NotificationType | M |
| eventTime | request body | eventTime | DateTime | M |
| systemDN | request body | systemDN | SystemDN | M |
| sequenceNo | request body | sequenceNo | integer | CM |
| subscriptionId | request body | subscriptionId | DN | CM |
| observedPerfMetricName | request body | observedPerfMetricName | string | M |
| observedPerfMetricValue | request body | observedPerfMetricValue | PerfMetricValue | M |
| observedPerfMetricDirection | request body | observedPerfMetricDirection | PerfMetricDirection | M |
| thresholdValue | request body | thresholdValue | PerfMetricValue | M |
| hysteresis | request body | hysteresis | PerfMetricValue) | M |
| monitorGranularityPeriod | request body | monitorGranularityPeriod | integer | M |
| additionalText | request body | additionalText | string | O |

Next change

12.3.1.2.4.2.1	Type NotifyThresholdCrossing

Table 12.3.1.2.4.2.1-1: Definition of type NotifyThresholdCrossing

| Attribute name | Data type | Description | S |
| --- | --- | --- | --- |
| href | Uri | URI of the resource where the event (threshold crossing) occurred | M |
| notificationId | NotificationId | Notification identifier as defined in ITU-T Rec. X. 733 [4] | M |
| notificationType | NotificationType | Notification type (notifyThresholdCrossing) | M |
| eventTime | DateTime | Event (threshold crossing) occurrence time | M |
| systemDN | SystemDN | System DN | M |
| sequenceNo | integer | See clause 11.0.2 | CM |
| subscriptionId | DN | See clause 11.0.2 | CM |
| observedPerfMetricName | string | Name of the performance metric that has crossed the threshold | M |
| observedPerfMetricValue | PerfMetricValue | Value of the performance metric, that has crossed the threshold, when the threshold crossing was observed | M |
| observedPerfMetricDirection | PerfMetricDirection | Direction ("UP" or "DOWN") of the performance metric, when the threshold crossing was observed | M |
| thresholdValue | PerfMetricValue | Threshold value of the triggered threshold | M |
| hysteresis | PerfMetricValue | Hysteresis of the triggered threshold | M |
| monitorGranularityPeriod | integer | Granularity period of the threshold monitor | M |
| additionalText | string | Vendor specific information | O |

Next change

##### 12.4.1.2.2	Notification "notifyHeartbeat"

The IS notification parameters are mapped to SS equivalents according to table 12.4.1.2.2-1.

Table 12.4.1.2.2-1: Mapping of IS notification input parameters to SS equivalents (HTTP POST)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | S |
| --- | --- | --- | --- | --- |
| objectClass | request body | href | Uri | M |
| objectInstance |  |  |  |  |
| notificationId | request body | notificationId | NotificationId | M |
| notificationType | request body | notificationType | NotificationType (notifyHeartbeat) | M |
| eventTime | request body | eventTime | DateTime | M |
| systemDN | request body | systemDN | systemDN | M |
| sequenceNo | request body | sequenceNo | integer | CM |
| subscriptionId | request body | subscriptionId | DN | CM |
| heartbeatNtfPeriod | request body | heartbeatNtfPeriod | integer | M |

Next change

##### 12.6.1.2.2	Notification notifyFileReady

The IS notification parameters are mapped to SS equivalents according to table 12.6.1.2.2-1.

Table 12.6.1.2.2-1: Mapping of IS notification input parameters to SS equivalents (HTTP POST)

| IS parameter name | SS parameter location | SS parameter name | SS parameter type | S |
| --- | --- | --- | --- | --- |
| objectClass | request body | href | Uri | M |
| objectInstance |  |  |  |  |
| notificationId | request body | notificationId | NotificationId | M |
| notificationType | request body | notificationType | NotificationType | M |
| eventTime | request body | eventTime | DateTime | M |
| systemDN | request body | systemDN | SystemDN | M |
| sequenceNo | request body | sequenceNo | integer | CM |
| subscriptionId | request body | subscriptionId | DN | CM |
| fileInfoList | request body | fileInfoList | array(FileInfo) | M |
| additionalText | request body | additionalText | string | O |

##### 12.6.1.2.3	Notification notifyFilePreparationError

The IS notification parameters are mapped to SS equivalents according to table 12.6.1.2.3-1.

Table 12.6.1.2.3-1: Mapping of IS notification input parameters to SS equivalents (HTTP POST)

| IS parameter name | SS parameter location | SS parameter name | SS parameter type | S |
| --- | --- | --- | --- | --- |
| objectClass | request body | href | Uri | M |
| objectInstance |  |  |  |  |
| notificationId | request body | notificationId | NotificationId | M |
| notificationType | request body | notificationType | NotificationType | M |
| eventTime | request body | eventTime | DateTime | M |
| systemDN | request body | systemDN | SystemDN | M |
| sequenceNo | request body | sequenceNo | integer | CM |
| subscriptionId | request body | subscriptionId | DN | CM |
| fileInfoList | request body | fileInfoList | array(FileInfo) | M |
| reason | request body | reason | string | O |
| additionalText | request body | additionalText | string | O |

Next change

12.6.1.4.2.2	Type NotifyFileReady

Table 12.6.1.4.2.2-1: Definition of type NotifyFileReady

| Attribute name | Data type | Description | S |
| --- | --- | --- | --- |
| href | Uri | URI of the object representing the process, managed element or management node, which made the file available | M |
| notificationId | NotificationId | Notification identifier as defined in ITU-T Rec. X. 733 [4] | M |
| notificationType | NotificationType | Notification type (notifyFileReady, etc.) | M |
| eventTime | DateTime | Event occurrence time (e.g., the file ready time) | M |
| systemDN | SystemDN | DN of the MnS Agent emitting the notification | M |
| sequenceNo | integer | See clause 11.0.2 | CM |
| subscriptionId | DN | See clause 11.0.2 | CM |
| fileInfoList | array(FileInfo) | Information describing the available files | M |
| additionalText | string | Allows a free form text description to be reported as defined in ITU-T Rec. X. 733 [4] | O |

12.6.1.4.2.3	Type NotifyFilePreparationError

Table 12.6.1.4.2.3-1: Definition of type NotifyFilePreparationError

| Attribute name | Data type | Description | S |
| --- | --- | --- | --- |
| href | Uri | URI of the object representing the process, managed element or management node, where the file preparation error occured | M |
| notificationId | NotificationId | Notification identifier as defined in ITU-T Rec. X. 733 [4] | M |
| notificationType | NotificationType | Notification type (notifyFileReady, etc.) | M |
| eventTime | DateTime | Event occurrence time (e.g., the file ready time) | M |
| systemDN | SystemDN | DN of the MnS Agent emitting the notification | M |
| sequenceNo | integer | See clause 11.0.2 | CM |
| subscriptionId | DN | See clause 11.0.2 | CM |
| fileInfoList | array(FileInfo) | Information about the files with a preparation error. | M |
| reason | string | Reason for the file preparation error | O |
| additionalText | string | Allows a free form text description to be reported as defined in ITU-T Rec. X. 733 [4] | O |

End of  changes