3GPP TSG-SA5 Meeting #160	S5-251954

# Stor-Göteborg, Sweden, 7th Apr 2025 - 11th Apr 2025

# Source:	Ericsson

# Title:	Rel-19 pCR TS 28.572 Allow recursive Planned configuration groups

# Document for:	Approval

Agenda Item:	6.19.9.1 - Management of planned configurations

# 1	Decision/action requested

The group is requested to discuss and approve the pCR below.

# 2	References

[1]		3GPP TS 28.572: "Management of planned configurations"

# 3	Rationale

Introduce the possibility of planed configuration groups being a member of other planed configuration groups.

# 4	Detailed proposal

The following changes are proposed for TS 28.572[1].

Begin of modifications

## 6.2	Planned configuration groups

### 6.2.1	Definition

A planned configuration group is a set of planned configurations or planned configuration groups that shall be processed (validated and activated) together.

For example, one plan may create a new cell, and another plan may start the collection of measurements for the new cell, or one plan may configure one base station and another plan may configure another base station, when both base stations need to be consistently configured to enable handovers between them.

### 6.2.2	Planned configuration group members

A planned configuration (or planned configuration group) belonging to a planned configuration group is also called a member of that planned configuration group. A planned configuration (or planned configuration group) can be a member of more than one planned configuration group.

The members of a planned configuration group shall be considered as atomic or non-atomic (best effort), in the same way as the operations of an operation set may form an atomic or a non-atomic (best effort) set.

Furthermore, the members of a planned configuration group can be ordered or not ordered. If ordered, the planned configuration group members are processed in the order as specified in the planned configuration group. If not ordered, the members can be processed in any order or in parallel.

For example, when the configuration for a new Network Function is in one plan and the creation of a performance metric collection job in another plan, then the plans must be ordered (assuming the job object is contained under a configuration object).

Not ordered are for example plans adding performance metric collection jobs to already configured Network Functions.

There shall not be any circular membership relationships between two or more planned configuration groups. E.g. group-A is a member of group-B then group-B is not allowed to be a member of group-A.

Figure 6.2.1-1: NOT allowed circular planed configuration group membership relation (with 3 groups)

A planned configuration group may not include any individual planned configuration more than once.The following is not allowed because Group_A cannot contain Plan2 twice, It is also invalid because Group_B contains Plan1 twice, once via Group_C and once via Group_A.

Figure 6.2.1-2: NOT allowed multiple containment of planed configuration

All member planned configurations shall have the same configuration content type.

### 6.2.3	Conflicts between planned configuration group members

Conflicting operations may be contained in planned configuration group members. For example, a planned configuration sets an attribute to some value and another planned configuration sets the same attribute to some other value.

If the configuration group members are ordered, the result is deterministic. The conflicting operations are processed in the order as they appear in the ordered plans. The last processed operation from the ordered set of operations manipulating the same node determines the result. If the configuration group members are not ordered, the result is not deterministic. Depending on which operation from the unordered set of operations manipulating the same node is processed last, the result may be different.

It is configurable if a detected conflict should invalidate the planned configuration group.

### 6.2.4	Meta data for planned configuration groups

Each planned configuration group is described by meta data. This includes the name, the version, and a human readable textual description.

Furthermore, it includes the information if the group members are atomic or non-atomic, if they are ordered or unordered, and if a conflict between planned configuration group members shall invalidate the planned configuration group and prevent its activation.

The date and time at which information in the planned configuration group was modified the last time is provided as well.

### 6.2.5	Planned configuration group descriptor

The planned configuration group members and its meta data are specified in a planned configuration group descriptor. Each descriptor has a unique identifier.

Next modification

### 6.5.4	Validation of planned configuration groups

The planned configuration group members are validated in the order specified. If no order is specified, they can be validated in any order. Each member is validated as specified in this and the previous clause. The validation of atomic members concludes with either validated or failed. The validation of best effort members concludes with either validated, partially validated or failed.

An atomic group is considered as valid, if atomic members are valid, and best effort members are valid or partially valid. If the validation of one (atomic or best effort) member fails, the validation of the complete group shall fail.

A best effort group is considered as valid, if atomic members are valid, and best effort members are valid or partially valid. If the validation of at least one (atomic or best effort) member fails, the group is considered as partially valid. If the validation of all members fails, the validation of the complete group shall fail.

Next modification

## 6.12	Managing planned configuration groups

Planned configuration group descriptors are created, updated and deleted by MnS consumers on MnS producers. The planned configurations and planned configuration groups referred to as members in a planned configuration group shall exist.

Multiple planned configuration group descriptors can be maintained on a MnS producer. Each planned configuration group is identified by an identifier. The identifier is assigned by the MnS producer.

Next modification

## 7.2	PlannedConfigurationGroupDescriptor

### 7.2.1	Definition

This definition represents a planned configuration group descriptor.

### 7.2.2	Information Elements

The following table specifies the information elements of a planned configuration group descriptor.

| Information element name | S | Documentation and Allowed Values | Properties |
| --- | --- | --- | --- |
| id | M | The identifier of the planned configuration group descriptor | type: String multiplicity: 1 isInvariant: True isWritable: False |
| name | M | The name of the planned configuration group. | type: String multiplicity: 0..1 isInvariant: False isWritable: True |
| version | M | The version of the planned configuration group. Its format is implementation specific. | type: String multiplicity: 0..1 isInvariant: False isWritable: True |
| description | M | The textual human-readable description of the planned configuration group. | type: String multiplicity: 0..1 isInvariant: False isWritable: True |
| customProperties | O | The container allowing to specify additional consumer defined properties (key value pairs) describing and qualifying the planned configuration. | type: String multiplicity: * isOrdered: False isUnique: True isInvariant: False isWritable: True |
| memberIds | M | The identifiers of planned configurations or the planned configuration groups that are members of this planned configuration group. | type: String multiplicity: 1..* isOrdered: True isUnique: True isInvariant: False isWritable: True |
| isOrdered | M | Specifies if the members of the planned configuration group are ordered. When ordered, the planned configuration group members shall be validated/activated in the specified order. When not ordered the planned configuration group members can be validated/activated in any order. | type: Boolean multiplicity: 1 isInvariant: False isWritable: True |
| applyMode | M | Specifies if the members of the planned configuration group are treated as atomic, best effort, or if processing shall stop on the occurrence of the first error. allowedValues: - ATOMIC - BEST_EFFORT - STOP_ON_ERROR | type: ENUM multiplicity: 1 isInvariant: False isWritable: True |
| isFailOnConflict | M | Specifies if the activation shall fail on detection of conflicts, or if the operations shall be processed as if there were no conflicts. | type: Boolean multiplicity: 1 isInvariant: False isWritable: True |
| lastModifiedAt | M | The date and time at which the planned configuration group descriptor was modified the last time by a MnS consumer. | type: DateTime multiplicity: 0..1 isInvariant: False isWritable: False |
| validationState | M | The outcome of the last validation, or "NOT_VALIDATED" if the planned configuration group has not been validated yet. What if a member is updated? allowedValues: - NOT_VALIDATED - VALID - PARTIALLY_VALID - INVALID | type: ENUM multiplicity: 1 isInvariant: False isWritable: False |
| lastValidatedAt | M | The date and time at which the planned configuration group was validated (the last time). The information element is absent, when the planned configuration has not been validated yet. | type: DateTime multiplicity: 0..1 isInvariant: False isWritable: False |

Next modification

Annex A (informative):
PlantUML for figures

Figure 6.2.1-1: NOT allowed circular planed configuration group membership relation (with 3 groups)

@startuml

left to right direction

object Group_A

object Group_B

object Group_C

Group_A ---o Group_B : is_member_of

Group_B ---o Group_C : is_member_of

Group_C ---o Group_A : is_member_of

@enduml

Figure 6.2.1-1: NOT allowed circular planed configuration group membership relation (with 3 groups)

@startuml

hide empty members

object Group_A

object Group_B

object Group_C

object Plan1

object "Plan1 "

Object Plan2

Object "Plan2 "

Group_A -up-o Group_B : is_member_of

Group_B -down-o Group_C : is_member_of

Plan1 -up-o Group_A : is_member_of

Plan2 -up-o Group_A : is_member_of

"Plan2 " -up-o Group_A : is_member_of

"Plan1 " -up-o Group_C : is_member_of

@enduml

Figure 6.2.1-2: NOT allowed multiple containment of planed configuration

End of modifications