3GPP TSG-SA5 Meeting #160	S5-251468

Goteborg, SWEDEN 7 - 11 April 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.254 | CR | 0059 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 32.254 Addition of the CAPIF Charging Principle and Scenarios |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | CH_CAPIF_EXT |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In order to support API charging for the industry, the Common API Framework and service operations are specifed by SA6 and CT3 to support any service APIs when used by northbound entities, as well as any interactions between the CAPIF and the service APIs themselves. The corresponding charging support should be enhanced. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the list of API procedures to be supported by CAPIF. Add new clause to specify the charging principle, message flows for CAPIF charging. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The CAPIF service is not supported from the charging aspect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.0, 5.1.1, 5.4.2.1, 5.X(New), |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

5.1.0	General

The following are high level charging requirements for northbound API for Exposure Function which are specified in TS 23.682 [243], TS 23.501 [200] and TS 23.222[244]:

For northbound API invocation/notification, the SCEF or NEF or CCF shall collect the following charging information:

-	invocations/notifications count of the northbound APIs.

-	identification of the SCS/AS or AF and the associated northbound API invocation/notification.

-	timestamp of the northbound API invocation/notification.

-	northbound API related information, e.g. location.

Next Change

5.1.1	Northbound API procedures

All procedures that operate across the T8 reference point, as specified in 3GPP TS 23.682 [243] and TS 29.122 [230], are covered, which are the following:

-	Monitoring

-	Resource management of Background Data Transfer

-	Changing the chargeable party at session set up or during the session

-	Non-IP Data Delivery

-	Device Triggering

-	Group Message Delivery

-	Reporting of Network Status

-	Communication Pattern Parameters Provisioning

-	PFD Management

-	Enhanced Coverage Restriction Control

-	Network Parameter Configuration

-	Setting up an AS session with required QoS

-	MSISDN-less Mobile Originated SMS

-	Request for time synchronization and time sensitive communication service

-	Request for Ranging and Sidelink Positioning service

The following clauses 5.2 and 5.3 describe the trigger conditions and simplified message flows for Event Based Charging (IEC/ECUR), with interfaces specified in 3GPP TS 32.299 [50].

The Northbound APIs supported by the NEF via the set of exposed services defined in 3GPP TS 23.502 [201] are covered for converged charging, with the trigger conditions and message flows defined in clause 5.4.

The Northbound APIs supported by the CAPIF via the set of exposed service APIs defined in 3GPP TS 23.222 [244] are covered, which are the following. For CAPIF converged charging, with the trigger conditions and message flows via CAPIF Core Function is defined in clause 5.5.

-	Service activation: Onboarding the API invoker to the CAPIF, as specified in TS 23.222 [244], clause 8.1;

-	Service in-activation: Offboarding the API invoker from the CAPIF, as specified in TS 23.222 [244], clause 8.2.

-	Service API publish, as specified in the TS 23.222 [244] clause 8.3

-	Service API unpublish, as specified in the TS 23.222 [244] clause 8.4

-	Service API retrieve, as specified in the TS 23.222 [244] clause 8.5

-	Service API update, as specified in the TS 23.222 [244] clause 8.6

-	Discover service APIs, as specified in TS 23.222 [244] Clause 8.7.

-	Subscription, un-subscription and notifications for the CAPIF events (e.g. API notification for the Monitoring service API invocation) as specified in the TS 23.222 [244] clause 8.8.

-	Revoking subscription of the CAPIF events, as specified in the TS 23.222 [244] clause 8.9.

-	API invocation, e.g. API invoker identifier, timestamp of API invocations

-	The result of the API invocation response, e.g. the success or failure of API invocation request.

Next Change

5.4.2.1	Introduction

The different scenarios below focus on the different messages from/to the NEF and corresponding interaction with the CHF, based on scenarios specified in TS 23.501 [200].

Next Change

5.X	Common API Framework converged charging scenarios

5.X.1	Basic principles

5.X.1.1	General

Converged charging may be performed by the CCF interacting with CHF using Nchf specified in TS 32.290 [57] and TS 32.291 [58]. In order to provide the data required for the management activities outlined in TS 32.240 [1] (Credit-Control, accounting, billing, statistics etc.), the CCF shall be able to perform converged charging for the northbound service API operations described in the clause 5.1.1.

The CCF shall be able to perform converged charging by interacting with CHF, for charging data related to service API operation. The Charging Data Request and Charging Data Response are exchanged between the CCF and the CHF, based on PEC, IEC or ECUR scenarios specified in TS 32.290 [57]. The Charging Data Request is issued by the CCF towards the CHF when certain conditions (chargeable events) are met.

Converged charging uses centralized or decentralized unit determination and centralized rating scenarios for event based convergent charging specified in TS 32.290 [57].

The contents and purpose of each charging event that triggers interaction with CHF, as well as the chargeable events that trigger them, are described in the following clauses.

A detailed formal description of the converged charging parameters defined in the present document is to be found in TS 32.291 [58].

A detailed formal description of the CDR parameters defined in the present document is to be found in TS 32.298 [51].

The selection of the CHF can be configured in the CCF but may also rely on NRF.

5.X.1.2	Applicable Triggers in the CAPIF Core Function

5.X.1.2.1	General

When a charging event is issued towards the CHF, it includes details such as API invoker/API provider domian (e.g. identifier of the AF).

Each trigger condition (i.e. chargeable event) defined for the CCF converged charging functionality, is specified with the associated behaviour when they are met.

The default trigger conditions in CCF is the same with the Default trigger condition in the NEF specified in the clause 5.4.1.2.1

5.X.2	Message flows

5.X.2.1	Introduction

The interaction between CCF and CHF for the API invoker/API provider domain service operation is the same with the interaction between NEF and CHF for the API invoker service operation. The message flows about CAPIF Core Function converged charging for IEC/PEC/ECUR charging scenarios is the same with message flows about NEF converged charging in clause 5.4.2.

Based on the message flows described in clase 5.4.2, the detailed PEC charging scenarios below focus on the different messages between API provider domain/API invoker and the CCF and corresponding interaction with the CHF for example, based on scenarios specified in TS 23.222 [244].

Figure 5.x.2.1-1: API Provider Invocation Request to CCF using PEC

1-2.	CCF receives a CAPIF API Request (e.g. CAPIF_Events_API, CAPIF_API_Provider_Management_API specified in the TS 29.222 [245]) from an API provider via the API publishing function, API exposing function or API management.

3.	CCF performs the actions needed to fulfil the CAPIF API invoked.

4.	If authorized, the CCF continues the CAPIF API invocation processing and sends the CAPIF API Response to API provider domain.

4ch-a. The CCF sends Charging Data Request [Event] to CHF for the received CAPIF API Invocation.

4ch-b. The CHF creates a CDR for this CAPIF API Invocation.

4ch-c. The CHF acknowledges by sending Charging Data Response [Event] to the CCF.

5. The API publishing function, API exposing function or API management forward the CAPIF API response to Serive provider.

Note: in the different deployment modes described in the TS 23.222 [244] clause Annex B (informative) CAPIF relationship with network exposure aspects of 3GPP systems, NEF may be implemented as the CAPIF core functioun and API provder domain, or API provider domain.

Figure 5.x.2.1-2: API Invocation Request to CCF using PEC

1.	CCF receives a CAPIF API Request (e.g. CAPIF_Events_API, CAPIF_API_Invoker_Management_API specified in the TS 29.222 [245]) from an API invoker.

2.	CCF performs the actions needed to fulfil the CAPIF API invoked.

3.	If authorized, the CCF continues the CAPIF API invocation processing and sends the CAPIF API Response to API invoker.

3ch-a. The CCF sends Charging Data Request [Event] to CHF for the received CAPIF API Invocation.

3ch-b. The CHF creates a CDR for this CAPIF API Invocation.

3ch-c. The CHF acknowledges by sending Charging Data Response [Event] to the CCF.

5.X.3	CDR generation

5.X.3.1	Introduction

The northbound API CHF CDRs for NEF Charging generated by the CHF are applicable for the CCF Charging, to collect charging information that they subsequently transfer to the Charging Gateway Function (CGF).

5.X.4	Ga record transfer flows

Details of the Ga protocol application are specified in TS 32.295 [6].

5.X.5	Bea CDR file transfer

Details of the Bea protocol application are specified in TS 32.297 [5].

End of Change