3GPP TSG-SA5 Meeting #160	S5-251895

Goteborg, Sweden, 07 - 11 April 2025

Source:	NTT DOCOMO, China Mobile

Title:	pCR TR 28.869 3GPP management architecture evolution based on PaaS

Document for:	Approval

Agenda item:	6.19.6

Spec:	3GPP TR 28.869

Version:	V1.3.0

Work Item:	FS_Cloud_OAM

Comments

The contribution proposes to add a use case and solution for 3GPP management architecture evolution based on PaaS.

Proposed Changes

* * * First Change * * * *

## 4.1	Background of VNF generic OAM functions

### 4.1.1	Overview

ETSI GR NFV-EVE 019 [3] defines and analyses the type of OAM functions for VNFs that can be generalized and be provided as a "generic function". It also describes potential solutions regarding the VNF generic OAM functions architectural framework and the interactions with the other entities in the NFV ecosystem.

3GPP TR 28.834"Study on management of cloud-native Virtualized Network Functions (VNF)" [4] in Release 18 studies potential use cases, requirements and solutions for the management of cloud-native virtualized network functions and the impacts on the 3GPP management system. It also includes some use cases related to VNF generic OAM functions, but the relevant specific solutions were not studied thoroughly due to the fact that the corresponding ETSI GS NFV-IFA 049 [2] was not published prior to the completion of the TR 28.834. As a result, the scope of Rel-18 "WID on management of cloud-native Virtualized Network Functions" [5] did not include enhancements related to VNF generic OAM functions.

ETSI GS NFV-IFA 049 [2] specifies the VNF generic OAM functions framework and also specifies the functional requirements and the interface requirements for VNF generic OAM functions. Considering that ETSI GS NFV-IFA 049 has been published, clause 5.1 of this Rel-19 TR studies the use of VNF generic OAM functions from the perspective of 3GPP management system.

As specified in ETSI GS NFV-IFA 049 [2] a close mapping between VNF generic OAM functions and Platform as a Service (PaaS) Services exists. According to ETSI GS NFV-IFA 049 [2], any VNF generic OAM function can be understood as a PaaS Service of a specific type, i.e. a PaaS Service that provides and handles generic OAM functionality for VNFs. See ETSI GR NFV 003 [11] for a definition of the term PaaS Service.

Telco PaaS services can be used to support advanced OAM capabilities for VNF non-application items. For example, Telco PaaS services can offer support for OAM related operations on top of different cloud IaaS platforms. This can also enhance portability of cloud native VNFs from one cloud environment to another.

* * * End of Changes * * * *