3GPP TSG-SA5 Meeting #161	S5-252278

# Fukuoka, Japan, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.111 | CR | 0043 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 28.111 Clarify alarmType |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson Hungary Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSBMA |  |  |  |  |  | Date: |  |  | 2025-05-03 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In annex B the term Event type is used while everywhere else the same item is called alarmType. While other specifications do use event type for the same item, in this document alarmType should be used consistently. In the first line of the first table in Annex B the following is found M.3100 Probable cause (string); (integer); Event Type. Indeterminate; 0; Unknown. However, "Unknown" is not a valid value for alarmType/eventType. Other specifications, e.g. RFC3877 (snmp alarm-mib) and RFC8632 (alarm YANG model) include one more value for event/alarmType: "other". It is proposed to introduce the value "other" as a new allowedValue for alarmType. ProbableCause=indeterminate shall be associated with alarmType "other". alarmType "other" will also be useful for vendor defined alarms that do not fit into the existing alarmTypes. |
| Summary of change: |  | Change Event type to alarmType. Add new value "other" to alarmTypes. Specify the alarmType value for probable cause Indeterminate |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Interoperability problems for probable cause Indeterminate. Misunderestanding due to the inconsistent alarmType / eventType terminology |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4, 7.4.1, Annex B, Forge |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | YANG Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1714 at commit a4be93d1d3bd14bd629d493dea5dffd1c672e68b YAML Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1717 at commit 58dce913d9940fe197a671be505e90625a4448c2 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

# 4	Concepts and overview

A (managed) system may experience faults such as malfunctions, a defect in system design, a defect in the software, or external interference. These faults may (or may not) lead to a system state that is different from the correct or desired system state. An incorrect system state is called error. Errors are hence caused by faults. Faults and errors are not always externally observable and may remain undetected.

Errors, in turn, may (or may not) cause failures. A failure is the inability to deliver the correct service as defined by the service specification. A failure is hence always externally observable.

In summary, a fault may case one or more errors, and an error may cause one or more failures.

An alarm is the management representation of a fault, a (detected) error or a failure that requires attention or reaction by an operator or some machine.

Fault Management is concerned with representing, managing, and reporting alarms. Fault Management is often also referred to as Alarm Management. The alarm model is independent from the underlying managed system. The same model can be used to represent alarms from any 3GPP generation or other networks and any resource. Specifics of the managed system manifest themselves only in the values of the information elements of the alarm model.

Alarms allow to report any kind of issue, from small faults without service impact to large scale failures of telecommunication services affecting many users.

A prerequisite for Fault Management as defined in the present document is that the managed system is represented by managed objects, that are organized in hierarchical object trees, in the management system.

The solution specified in the present document is based on ITU-T X.733 [8].

Fault Management is considered a generic management service. It shall be able to support fault indications about any generation of 3GPP or other networks and any resource that can be addressed by a distinguished name e.g. ManagedElements, ENBs or NetworkSlices or non-3GPP managed resources.

Fault management can handle alarms about any kind of fault in a 3GPP system from small hardware errors to service failures effecting many users.

Next change

### 7.4.1	Attribute properties

The following table defines the properties of attributes specified in the present document.

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| objectInstance | Managed object instance identified by its DN. | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| administrativeState | Administrative state of a managed object instance. The administrative state describes the permission to use or prohibition against using the object instance. The administrative state is set by the MnS consumer. allowedValues: LOCKED, UNLOCKED. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: LOCKED isNullable: False |
| operationalState | Operational state of manged object instance. The operational state describes if an object instance is operable ("ENABLED") or inoperable ("DISABLED"). This state is set by the object instance or the MnS producer and is hence READ-ONLY. allowedValues: ENABLED, DISABLED. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: DISABLED isNullable: False |
| alarmRecords | List of alarm records | type: AlarmRecord multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| numOfAlarmRecords | Number of alarm records in the AlarmList. allowedValues: Non-negative numbers. | type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| lastModification | Time an alarm record was modified the last time. | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| unreliableAlarmScope | Identifies, the part(s) of the alarm scope that may not be reliable. If this parameter is equal to the instance carried in systemDN, then all AlarmRecord instances in the AlarmList may not be reliable. If this parameter is equal to some instance represented by MonitoredEntity, then only AlarmRecord related to this instance and its descendants may not be reliable. | type: DN multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| alarmId | Identifies an AlarmRecord in the AlarmList. The value is unique within the AlarmList MOI. | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| notificationId | The Id of the last notification sent as a consequence of updating the AlarmRecord. | type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| alarmRaisedTime | Date and time the alarm was raised. | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| alarmChangedTime | It indicates the last date and time when the AlarmRecord is changed by the alarmed resource. Changes to AlarmRecord caused by invocations of the management service consumer would not change this date and time. | type: DateTime multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| alarmClearedTime | Date and time the alarm was cleared. | type: DateTime multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| alarmType | It indicates the type of alarm. Communications Alarm: An alarm of this type is associated with the procedure and/or process required conveying information from one point to another (ITU-T Recommendation X.733 [8]). Quality of Service Alarm: An alarm of this type is associated with degradation in the quality of a service (ITU T Recommendation X.733 [8]). Processing Error Alarm: An alarm of this type is associated with a software or processing fault (ITU T Recommendation X.733 [8]). Equipment Alarm: An alarm of this type is associated with an equipment fault (ITU-T Recommendation X.733 [8]). Environmental Alarm: An alarm of this type is associated with a condition related to an enclosure in which the equipment resides (ITU-T Recommendation X.733 [8]). Security related alarm types Integrity Violation: An indication that information may have been illegally modified, inserted or deleted. Operational Violation: An indication that the provision of the requested service was not possible due to the unavailability, malfunction or incorrect invocation of the service. Physical Violation: An indication that a physical resource has been violated in a way that suggests a security attack. Security Service or Mechanism Violation: An indication that a security attack has been detected by a security service or mechanism. Time Domain Violation: An indication that an event has occurred at an unexpected or prohibited time. Other: The type of the alarm does not fit into any of the other types or is not known. Allow values: COMMUNICATIONS_ALARM, QUALITY_OF_SERVICE_ALARM, PROCESSING_ERROR_ALARM, EQUIPMENT_ALARM, ENVIRONMENTAL_ALARM, INTEGRITY_VIOLATION, OPERATIONAL_VIOLATION, PHYSICAL_VIOLATION, SECURITY_SERVICE_OR_MECHANISM_VIOLATION, TIME_DOMAIN_VIOLATION OTHER | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| probableCause | It qualifies alarm and provides further information than alarmType. This attribute value shall be single-value and of simple type such as integer or string. See Annex B for a complete listing. The producer should choose the most specific probableCause applicable. | type: string or integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| specificProblem | It provides further refinement to the probableCause. This attribute value shall be single-valued and of simple type such as integer or string. See definition in ITU-T Recommendation X.733 [8] clause 8.1.2.2. | type: string or integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| perceivedSeverity | It indicates the relative level of urgency for operator attention. allowedValues: CRITICAL, MAJOR, MINOR, WARNING, INDETERMINATE, CLEARED | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| backedUpStatus | It indicates if an object (the MonitoredEntity) has a back up. See definition in ITU-T Recommendation X.733 [8] clause 8.1.2.4. | type: boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |
| backUpObject | Backup object of the alarmed object as defined in ITU-T Rec. X. 733 [8] | type: DN multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| trendIndication | It indicates if some observed condition is getting better, worse, or not changing. AllowedValues: MORE_SEVERE, NO_CHANGE, LESS_SEVERE | type: ENUM multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| thresholdInfo | It indicates the crossed threshold information such as: - The identifier of the monitored attribute whose value has crossed a threshold, - The threshold settings, - The observed value that have crossed a threshold, etc. See definition in ITU-T Recommendation X.733 [8] clause 8.1.2.7. See also for information in 1 32.401 [12] clause 5.6. | type: ThresholdInfo multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| stateChangeDefinition | It indicates attribute value changes associated with the alarm for state attributes of the monitored entity (state transitions). The change is reported with the name of the state attribute, the new value and an optional old value. See definition in ITU-T Recommendation X.733 [8] clause 8.1.2.11. The content of the attribute is a list of attributeNames and attributeValues. AttributeValues may be complex types. Beside the new value it may contain the old value as well. | type: AttributeValueChange multiplicity: 0..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| monitoredAttributes | It indicates attributes of the monitored entity and their values at the time the alarm occurred that are of interest for the alarm report. How these attributes are chosen is outside of the scope of the present document. See definition in ITU-T Recommendation X.733 [8] clause 8.1.2.11. The content of the attribute is a list of attributeName- attributeValue pairs. AttributeValues may be complex types. | type: NameValuePair multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| proposedRepairActions | Used if the cause is known and the system being managed can suggest one or more solutions to fix the problem causing the alarm as defined in ITU-T Recommendation X. 733 [8] | type: string multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| additionalText | Allows a free form text description to be reported as defined in ITU-T Recommendation X. 733 [8]. | type: string multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| additionalInformation | This attribute when present allows the inclusion of a set of vendor specific alarm information in the alarm. A specific condition for this optional population is when an alarm presented by the Management System (e.g. via the user interface) has different values of perceived severity, and / or alarm type, compared with the values presented to the Itf-N. Any other uses of additional information on the alarm and its semantics are outside the scope of the present document The content of the attribute is a list of attributeNames and string attributeValues. | type: NameValuePair multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| rootCauseIndicator | It indicates that this AlarmRecord is the root cause of the events captured by the notifications whose identifiers are in the related CorrelatedNotification instances. | type: boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ackTime | It identifies the time when the alarm has been acknowledged or unacknowledged the last time, i.e. it registers the time when ackState changes. | type: DateTime multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ackUserId | It identifies the last user who has changed the acknowledgement state. | type: string multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ackSystemId | It identifies the system that last changed the ackState of an alarm, i.e. acknowledged or unacknowledged the alarm. | type: string multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ackState | It identifies the acknowledgement state of an alarm. AllowedValues: ACKNOWLEDGED, UNACKNOWLEDGED | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| clearUserId | It carries the identity of the user who invokes the clearAlarms operation. | type: string multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| clearSystemId | Identifier of a system clearing an alarm | type: string multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| serviceUser | It identifies the service-user whose request for service provided by the serviceProvider led to the generation of the security alarm. | type: string multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| serviceProvider | It identifies the service-provider whose service is requested by the serviceUser and the service request provokes the generation of the security alarm. | type: string multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| securityAlarmDetector | It carries the identity of the detector of the security alarm. | type: string multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| comments | List of comments and data about the comments. | type: AlarmComment multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| correlatedNotifications | List of correlated notifications. | type: CorrelatedNotification multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| commentTime | Date and Time the comment was created. | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| commentUserId | It carries the identification of the user who made the comment. | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| commentSystemId | It carries the identification of the system (Management System) from which the comment is made. That system supports the user that made the comment. | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| commentText | It carries the textual comment. | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| CorrelatedNotification.sourceObjectInstance | It identifies one MonitoredEntity. It is unique within a multivalue attribute based on the CorrelatedNotification data type. | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| CorrelatedNotification.notificationIds | A list of correlated notificationIds. | type: integer multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| NOTEs: none. |  |  |

Next change

Annex B (informative):
Probable Causes

This annex lists probable causes.

Sources of these probable causes are ITU-T Recommendation M.3100 [7], ITU-T Recommendation X.733 [8], and ITU-T Recommendation X.736 [13]. In addition, probable causes for wireless systems are listed in ETSI TS 101 251 V6.3.0 (1999-07) [3].

The listed probable cause strings (or alternatively integers) should be used. If none of them represents the real probable cause appropriately, probable cause strings (or alternatively integers) not listed below may be used as well.

Table B.1: Probable Causes from ITU-T Recommendation M.3100 [7]

| M.3100 Probable cause (string) | (integer) | alarmType |
| --- | --- | --- |
| Indeterminate | 0 | Other |
| Alarm Indication Signal (AIS) | 1 | Communications |
| Call Setup Failure | 2 | Communications |
| Degraded Signal | 3 | Communications |
| Far End Receiver Failure (FERF) | 4 | Communications |
| Framing Error | 5 | Communications |
| Loss Of Frame (LOF) | 6 | Communications |
| Loss Of Pointer (LOP) | 7 | Communications |
| Loss Of Signal (LOS) | 8 | Communications |
| Payload Type Mismatch | 9 | Communications |
| Reserved | 10 |  |
| Remote Alarm Interface | 11 | Communications |
| Excessive Bit Error Rate (EBER) | 12 | Communications |
| Path Trace Mismatch | 13 | Communications |
| Unavailable | 14 | Communications |
| Signal Label Mismatch | 15 | Communications |
| Loss Of Multi Frame | 16 | Communications |
| Communications Receive Failure | 17 | Communications |
| Communications Transmit Failure | 18 | Communications |
| Modulation Failure | 19 | Communications |
| Demodulation Failure | 20 | Communications |
| Reserved | 21-26 |  |
| Reserved for M.3100 potential future extensions. | 27-50 |  |
| Back Plane Failure | 51 | Equipment |
| Data Set Problem | 52 | Equipment |
| Equipment Identifier Duplication | 53 | Equipment |
| External IF Device Problem | 54 | Equipment |
| Line Card Problem | 55 | Equipment |
| Multiplexer Problem | 56 | Equipment |
| NE Identifier Duplication | 57 | Equipment |
| Power Problem | 58 | Equipment |
| Processor Problem | 59 | Equipment |
| Protection Path Failure | 60 | Equipment |
| Receiver Failure | 61 | Equipment |
| Replaceable Unit Missing | 62 | Equipment |
| Replaceable Unit Type Mismatch | 63 | Equipment |
| Synchronization Source Mismatch | 64 | Equipment |
| Terminal Problem | 65 | Equipment |
| Timing Problem | 66 | Equipment |
| Transmitter Failure | 67 | Equipment |
| Trunk Card Problem | 68 | Equipment |
| Replaceable Unit Problem | 69 | Equipment |
| Real Time Clock Failure | 70 | Equipment |
| Reserved | 71-80 |  |
| Protection Mechanism Failure | 81 | Equipment |
| Protecting Resource Failure | 82 | Equipment |
| Reserved for M.3100 potential future extensions. | 83-100 |  |
| Air Compressor Failure | 101 | Environmental |
| Air Conditioning Failure | 102 | Environmental |
| Air Dryer Failure | 103 | Environmental |
| Battery Discharging | 104 | Environmental |
| Battery Failure | 105 | Environmental |
| Commercial Power Failure | 106 | Environmental |
| Cooling Fan Failure | 107 | Environmental |
| Engine Failure | 108 | Environmental |
| Fire Detector Failure | 109 | Environmental |
| Fuse Failure | 110 | Environmental |
| Generator Failure | 111 | Environmental |
| Low Battery Threshold | 112 | Environmental |
| Pump Failure | 113 | Environmental |
| Rectifier Failure | 114 | Environmental |
| Rectifier High Voltage | 115 | Environmental |
| Rectifier Low F Voltage | 116 | Environmental |
| Ventilation System Failure | 117 | Environmental |
| Enclosure Door Open | 118 | Environmental |
| Explosive Gas | 119 | Environmental |
| Fire | 120 | Environmental |
| Flood | 121 | Environmental |
| High Humidity | 122 | Environmental |
| High Temperature | 123 | Environmental |
| High Wind | 124 | Environmental |
| Ice Build Up | 125 | Environmental |
| Intrusion Detection | 126 | Environmental |
| Low Fuel | 127 | Environmental |
| Low Humidity | 128 | Environmental |
| Low Cable Pressure | 129 | Environmental |
| Low Temperature | 130 | Environmental |
| Low Water | 131 | Environmental |
| Smoke | 132 | Environmental |
| Toxic Gas | 133 | Environmental |
| Reserved for M.3100 potential future extensions. | 134-150 |  |
| Storage Capacity Problem | 151 | Processing Error |
| Memory Mismatch | 152 | Processing Error |
| Corrupt Data | 153 | Processing Error |
| Out Of CPU Cycles | 154 | Processing Error |
| Software Environment Problem | 155 | Processing Error |
| Software Download Failure | 156 | Processing Error |
| Loss of Real Time | 157 | Processing Error |
| Reinitialized | 158 | Processing Error |
| Reserved | 159-167 |  |
| Reserved for M.3100 potential future extensions. | 168-200 |  |
| Reserved | 201-202 |  |
| Excessive Error Rate | 203 | Quality of service |
| Reserved | 204-207 |  |
| Reserved for M.3100 potential future extensions. | 208-300 |  |

Table B.2: Probable Causes from ITU-T Recommendation X.733 [8]

| X.733 Probable Cause (string) | (integer) | alarmType |
| --- | --- | --- |
| Adapter Error | 301 | Equipment |
| Application Subsystem Failure | 302 | Processing error |
| Bandwidth Reduction | 303 | Security Service or Mechanism Violation |
| Reserved | 304 |  |
| Communication Protocol Error | 305 | Communications |
| Communication Subsystem Failure | 306 | Communications |
| Configuration or Customizing Error | 307 | Processing error |
| Congestion | 308 | Quality of service |
| Reserved | 309 |  |
| CPU Cycles Limit Exceeded | 310 | Processing error |
| Data Set or Modem Error | 311 | Equipment |
| Reserved | 312 |  |
| DTE-DCE Interface Error | 313 | Communications |
| Reserved | 314 |  |
| Equipment Malfunction | 315 | Communications |
| Excessive Vibration | 316 | Integrity Violation |
| File Error | 317 | Environmental |
| Reserved | 318-320 |  |
| Heating or Ventilation or Cooling System Problem | 321 | Environmental |
| Humidity Unacceptable | 322 | Environmental |
| Input/Output Device Error | 323 | Equipment |
| Input Device Error | 324 | Environmental |
| LAN Error | 325 | Processing error |
| Leak Detection | 326 | Environmental |
| Local Node Transmission Error | 327 | Communications |
| Reserved | 328-329 |  |
| Material Supply Exhausted | 330 | Environmental |
| Reserved | 331 |  |
| Out of Memory | 332 | Processing error |
| Output Device Error | 333 | Equipment |
| Performance Degraded | 334 | Quality of service |
| Reserved | 335 |  |
| Pressure Unacceptable | 336 | Operational Violation |
| Reserved | 337-338 |  |
| Queue Size Exceeded | 339 | Quality of service |
| Receive Failure | 340 | Equipment |
| Reserved | 341 |  |
| Remote Node Transmission Error | 342 | Communications |
| Resource at or Nearing Capacity | 343 | Quality of service |
| Response Time Excessive | 344 | Quality of service |
| Re-transmission Rate Excessive | 345 | Quality of service |
| Software Error | 346 | Processing error |
| Software Program Abnormally Terminated | 347 | Processing error |
| Software Program Error | 348 | Processing error |
| Reserved | 349 |  |
| Temperature Unacceptable | 350 | Environmental |
| Threshold Crossed | 351 | Quality of service |
| Reserved | 352 |  |
| Toxic Leak Detected | 353 | Environmental |
| Transmit Failure | 354 | Equipment |
| Reserved | 355 |  |
| Underlying Resource Unavailable | 356 | Processing error |
| Version Mismatch | 357 | Processing error |
| Reserved for potential future X.721/X.733 extensions | 358-500 |  |

Table B.3: Probable Causes for Wireless Systems from ETSI TS 101 251 V6.3.0 (1999-07) [3]

| Wireless Systems (string) | (integer) | alarmType |
| --- | --- | --- |
| A-bis to BTS interface failure | 501 | Equipment |
| A-bis to TRX interface failure | 502 | Equipment |
| Antenna problem | 503 | Equipment |
| Battery breakdown | 504 | Equipment |
| Battery charging fault | 505 | Equipment |
| Clock synchronization problem | 506 | Equipment |
| Combiner problem | 507 | Equipment |
| Disk problem | 508 | Equipment |
| Reserved | 509 |  |
| Excessive receiver temperature | 510 | Equipment |
| Excessive transmitter output power | 511 | Equipment |
| Excessive transmitter temperature | 512 | Equipment |
| Frequency hopping degraded | 513 | Equipment |
| Frequency hopping failure | 514 | Equipment |
| Frequency redefinition failed | 515 | Equipment |
| Line interface failure | 516 | Equipment |
| Link failure | 517 | Equipment |
| Loss of synchronization | 518 | Equipment |
| Lost redundancy | 519 | Equipment |
| Mains breakdown with battery back-up | 520 | Equipment |
| Mains breakdown without battery back-up | 521 | Equipment |
| Power supply failure | 522 | Equipment |
| Receiver antenna fault | 523 | Equipment |
| Reserved | 524 |  |
| Receiver multicoupler failure | 525 | Equipment |
| Reduced transmitter output power | 526 | Equipment |
| Signal quality evaluation fault | 527 | Equipment |
| Timeslot hardware failure | 528 | Equipment |
| Transceiver problem | 529 | Equipment |
| Transcoder problem | 530 | Equipment |
| Transcoder or rate adapter problem | 531 | Equipment |
| Transmitter antenna failure | 532 | Equipment |
| Transmitter antenna not adjusted | 533 | Equipment |
| Reserved | 534 |  |
| Transmitter low voltage or current | 535 | Equipment |
| Transmitter off frequency | 536 | Equipment |
| Database inconsistency | 537 | Processing error |
| File system call unsuccessful | 538 | Processing error |
| Input parameter out of range | 539 | Processing error |
| Invalid parameter | 540 | Processing error |
| Invalid pointer | 541 | Processing error |
| Message not expected | 542 | Processing error |
| Message not initialized | 543 | Processing error |
| Message out of sequence | 544 | Processing error |
| System call unsuccessful | 545 | Processing error |
| Timeout expired | 546 | Processing error |
| Variable out of range | 547 | Processing error |
| Watch dog timer expired | 548 | Processing error |
| Cooling system failure | 549 | Environmental |
| External equipment failure | 550 | Environmental |
| External power supply failure | 551 | Environmental |
| External transmission device failure | 552 | Environmental |
| Reserved | 553-560 |  |
| Reduced alarm reporting | 561 | Quality of service |
| Reduced event reporting | 562 | Quality of service |
| Reduced logging capability | 563 | Quality of service |
| System resources overload | 564 | Quality of service |
| Broadcast channel failure | 565 | Communications |
| Connection establishment error | 566 | Communications |
| Invalid message received | 567 | Communications |
| Invalid MSU received | 568 | Communications |
| LAPD link protocol failure | 569 | Communications |
| Local alarm indication | 570 | Communications |
| Remote alarm indication | 571 | Communications |
| Routing failure | 572 | Communications |
| SS7 protocol failure | 573 | Communications |
| Transmission error | 574 | Communications |
| Reserved | 575 |  |
| Reserved for potential future ETSI extensions | 576-700 |  |

Table B.4: Probable Causes for Security Alarm from X.736 [13]

| Wireless Systems (string) | (integer) | alarmType |
| --- | --- | --- |
| Authentication Failure | 701 | security service or mechanism violation |
| Breach of Confidentiality | 702 | security service or mechanism violation |
| Cable Tamper | 703 | physical violation |
| Delayed Information | 704 | time domain violation |
| Denial of Service | 705 | operational violation |
| Duplicate Information | 706 | integrity violation |
| Information Missing | 707 | integrity violation |
| Information Modification Detected | 708 | integrity violation |
| Information Out of Sequence | 709 | integrity violation |
| Intrusion Detection | 710 | physical violation |
| Key Expired | 711 | time domain violation |
| Non Repudiation Failure | 712 | security service or mechanism violation |
| Out of Hours Activity | 713 | time domain violation |
| Out of Service | 714 | operational violation |
| Procedural Error | 715 | operational violation |
| Unauthorised Access Attempt | 716 | security service or mechanism violation |
| Unexpected Information | 717 | integrity violation |
| Unspecified Reason | 718 | security service or mechanism violation |
| Reserved for X.736 potential future extensions. | 719-800 |  |

Next  change

*** START OF CHANGE 1 ***

*** OpenAPI/TS28111_FaultNrm.yaml ***

<CODE BEGINS>

openapi: 3.0.1

info:

title: Fault Management NRM

version: 19.1.0

description: >-

OAS 3.0.1 definition of the Fault Supervision MnS

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.111; Fault Management

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.111/

servers:

- url: '{MnSRoot}/FaultSupervisionMnS/{MnSversion}'

variables:

MnSRoot:

description: See subclause 4.4.3 of TS 32.158

default: http://example.com/3GPPManagement

MnSversion:

description: Version number of the OpenAPI definition

default: XXX

paths: {}

components:

schemas:

#---- Definition of AlarmRecord ----------------------------------------------------#

AlarmId:

type: string

AlarmType:

type: string

enum:

- COMMUNICATIONS_ALARM

- QUALITY_OF_SERVICE_ALARM

- PROCESSING_ERROR_ALARM

- EQUIPMENT_ALARM

- ENVIRONMENTAL_ALARM

- INTEGRITY_VIOLATION

- OPERATIONAL_VIOLATION

- PHYSICAL_VIOLATION

- SECURITY_SERVICE_OR_MECHANISM_VIOLATION

- TIME_DOMAIN_VIOLATION

- OTHER

readOnly: true

ProbableCause:

description: >-

The value of the probable cause may be a specific standardized string, or any

vendor provided string. Probable cause strings are not standardized in the

present document. They may be added in a future version. Up to then the

mapping of the generic probable cause strings "PROBABLE_CAUSE_001" to

"PROBABLE_CAUSE_005" is vendor specific.

The value of the probable cause may also be an integer. The mapping of integer

values to probable causes is vendor specific.

oneOf:

- anyOf:

- type: string

enum:

- PROBABLE_CAUSE_001

- PROBABLE_CAUSE_002

- PROBABLE_CAUSE_003

- PROBABLE_CAUSE_004

- PROBABLE_CAUSE_005

readOnly: true

- type: string

readOnly: true

- type: integer

readOnly: true

SpecificProblem:

oneOf:

- type: string

readOnly: true

- type: integer

readOnly: true

PerceivedSeverity:

type: string

enum:

- INDETERMINATE

- CRITICAL

- MAJOR

- MINOR

- WARNING

- CLEARED

TrendIndication:

type: string

readOnly: true

enum:

- MORE_SEVERE

- NO_CHANGE

- LESS_SEVERE

ThresholdHysteresis:

type: object

required:

- high

properties:

high:

oneOf:

- type: integer

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/FloatRo'

low:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/FloatRo'

ThresholdLevelInd:

oneOf:

- type: object

properties:

up:

$ref: '#/components/schemas/ThresholdHysteresis'

- type: object

properties:

down:

$ref: '#/components/schemas/ThresholdHysteresis'

ThresholdInfo:

type: object

properties:

observedMeasurement:

type: string

readOnly: true

observedValue:

type: number

readOnly: true

thresholdLevel:

$ref: '#/components/schemas/ThresholdLevelInd'

armTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTimeRo'

required:

- observedMeasurement

- observedValue

CorrelatedNotification:

type: object

properties:

sourceObjectInstance:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnRo'

notificationIds:

type: array

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationId'

required:

- sourceObjectInstance

- notificationIds

CorrelatedNotifications:

type: array

items:

$ref: '#/components/schemas/CorrelatedNotification'

AckState:

type: string

enum:

- ACKNOWLEDGED

- UNACKNOWLEDGED

AlarmRecord:

description: >-

The alarmId is not a property of an alarm record. It is used as key

in the map of alarm records instead.

type: object

properties:

# alarmId:

#  $ref: '#/components/schemas/AlarmId'

objectInstance:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnRo'

notificationId:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationId'

alarmRaisedTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTimeRo'

alarmChangedTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTimeRo'

alarmClearedTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTimeRo'

alarmType:

$ref: '#/components/schemas/AlarmType'

probableCause:

$ref: '#/components/schemas/ProbableCause'

specificProblem:

$ref: '#/components/schemas/SpecificProblem'

perceivedSeverity:

$ref: '#/components/schemas/PerceivedSeverity'

backedUpStatus:

type: boolean

backUpObject:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnRo'

trendIndication:

$ref: '#/components/schemas/TrendIndication'

thresholdinfo:

$ref: '#/components/schemas/ThresholdInfo'

correlatedNotifications:

$ref: '#/components/schemas/CorrelatedNotifications'

stateChangeDefinition:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeValueChangeSet'

monitoredAttributes:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeNameValuePairSet'

proposedRepairActions:

type: string

readOnly: true

additionalText:

type: string

readOnly: true

additionalInformation:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeNameValuePairSet'

rootCauseIndicator:

type: boolean

readOnly: true

ackTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTimeRo'

ackUserId:

type: string

ackSystemId:

type: string

ackState:

$ref: '#/components/schemas/AckState'

clearUserId:

type: string

clearSystemId:

type: string

serviceUser:

type: string

readOnly: true

serviceProvider:

type: string

readOnly: true

securityAlarmDetector:

type: string

readOnly: true

AlarmList-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

administrativeState:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AdministrativeState'

operationalState:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/OperationalState'

numOfAlarmRecords:

type: integer

readOnly: true

lastModification:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTimeRo'

alarmRecords:

description: >-

This resource represents a map of alarm records.

The alarmIds are used as keys in the map.

type: object

additionalProperties:

$ref: '#/components/schemas/AlarmRecord'

unreliableAlarmScope:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnRo'

#---- Definition of alarm notifications --------------------------------------------#

AlarmNotificationTypes:

type: string

enum:

- notifyNewAlarm

- notifyChangedAlarm

- notifyChangedAlarmGeneral

- notifyAckStateChanged

- notifyCorrelatedNotificationChanged

- notifyComments

- notifyClearedAlarm

- notifyAlarmListRebuilt

- notifyPotentialFaultyAlarmList

AlarmListAlignmentRequirement:

type: string

enum:

- ALIGNMENT_REQUIRED

- ALIGNMENT_NOT_REQUIRED

NotifyNewAlarm:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

required:

- alarmId

- alarmType

- probableCause

- perceivedSeverity

properties:

alarmId:

$ref: '#/components/schemas/AlarmId'

alarmType:

$ref: '#/components/schemas/AlarmType'

probableCause:

$ref: '#/components/schemas/ProbableCause'

specificProblem:

$ref: '#/components/schemas/SpecificProblem'

perceivedSeverity:

$ref: '#/components/schemas/PerceivedSeverity'

backedUpStatus:

type: boolean

backUpObject:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

trendIndication:

$ref: '#/components/schemas/TrendIndication'

thresholdInfo:

$ref: '#/components/schemas/ThresholdInfo'

correlatedNotifications:

$ref: '#/components/schemas/CorrelatedNotifications'

stateChangeDefinition:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeValueChangeSet'

monitoredAttributes:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeNameValuePairSet'

proposedRepairActions:

type: string

additionalText:

type: string

additionalInformation:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeNameValuePairSet'

rootCauseIndicator:

type: boolean

NotifyNewSecAlarm:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

required:

- alarmId

- alarmType

- probableCause

- perceivedSeverity

- serviceUser

- serviceProvider

- securityAlarmDetector

properties:

alarmId:

$ref: '#/components/schemas/AlarmId'

alarmType:

$ref: '#/components/schemas/AlarmType'

probableCause:

$ref: '#/components/schemas/ProbableCause'

specificProblem:

$ref: '#/components/schemas/SpecificProblem'

perceivedSeverity:

$ref: '#/components/schemas/PerceivedSeverity'

correlatedNotifications:

$ref: '#/components/schemas/CorrelatedNotifications'

additionalText:

type: string

additionalInformation:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeNameValuePairSet'

rootCauseIndicator:

type: boolean

serviceUser:

type: string

serviceProvider:

type: string

securityAlarmDetector:

type: string

NotifyClearedAlarm:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

required:

- alarmId

- alarmType

- probableCause

- perceivedSeverity

properties:

alarmId:

$ref: '#/components/schemas/AlarmId'

alarmType:

$ref: '#/components/schemas/AlarmType'

probableCause:

$ref: '#/components/schemas/ProbableCause'

specificProblem:

$ref: '#/components/schemas/SpecificProblem'

perceivedSeverity:

$ref: '#/components/schemas/PerceivedSeverity'

correlatedNotifications:

$ref: '#/components/schemas/CorrelatedNotifications'

clearUserId:

type: string

clearSystemId:

type: string

NotifyChangedAlarm:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

required:

- alarmId

- alarmType

- probableCause

- perceivedSeverity

properties:

alarmId:

$ref: '#/components/schemas/AlarmId'

alarmType:

$ref: '#/components/schemas/AlarmType'

probableCause:

$ref: '#/components/schemas/ProbableCause'

specificProblem:

$ref: '#/components/schemas/SpecificProblem'

perceivedSeverity:

$ref: '#/components/schemas/PerceivedSeverity'

NotifyChangedAlarmGeneral:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

required:

- alarmId

- alarmType

- probableCause

properties:

alarmId:

$ref: '#/components/schemas/AlarmId'

alarmType:

$ref: '#/components/schemas/AlarmType'

probableCause:

$ref: '#/components/schemas/ProbableCause'

specificProblem:

$ref: '#/components/schemas/SpecificProblem'

perceivedSeverity:

$ref: '#/components/schemas/PerceivedSeverity'

correlatedNotifications:

$ref: '#/components/schemas/CorrelatedNotifications'

backedUpStatus:

type: boolean

backUpObject:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

trendIndication:

$ref: '#/components/schemas/TrendIndication'

thresholdInfo:

$ref: '#/components/schemas/ThresholdInfo'

stateChangeDefinition:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeValueChangeSet'

monitoredAttributes:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeNameValuePairSet'

proposedRepairActions:

type: string

additionalText:

type: string

additionalInformation:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeNameValuePairSet'

rootCauseIndicator:

type: boolean

changedAlarmAttributes:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeNameValuePairSet'

NotifyChangedSecAlarmGeneral:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

required:

- alarmId

- alarmType

- probableCause

- serviceUser

- serviceProvider

- securityAlarmDetector

properties:

alarmId:

$ref: '#/components/schemas/AlarmId'

alarmType:

$ref: '#/components/schemas/AlarmType'

probableCause:

$ref: '#/components/schemas/ProbableCause'

specificProblem:

$ref: '#/components/schemas/SpecificProblem'

perceivedSeverity:

$ref: '#/components/schemas/PerceivedSeverity'

correlatedNotifications:

$ref: '#/components/schemas/CorrelatedNotifications'

additionalText:

type: string

additionalInformation:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeNameValuePairSet'

rootCauseIndicator:

type: boolean

serviceUser:

type: string

serviceProvider:

type: string

securityAlarmDetector:

type: string

changedAlarmAttributes:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeNameValuePairSet'

NotifyCorrelatedNotificationChanged:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

required:

- alarmId

- correlatedNotifications

- alarmType

- probableCause

properties:

alarmId:

$ref: '#/components/schemas/AlarmId'

alarmType:

$ref: '#/components/schemas/AlarmType'

probableCause:

$ref: '#/components/schemas/ProbableCause'

specificProblem:

$ref: '#/components/schemas/SpecificProblem'

correlatedNotifications:

$ref: '#/components/schemas/CorrelatedNotifications'

rootCauseIndicator:

type: boolean

NotifyAckStateChanged:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

required:

- alarmId

- alarmType

- probableCause

- perceivedSeverity

- ackState

- ackUserId

properties:

alarmId:

$ref: '#/components/schemas/AlarmId'

alarmType:

$ref: '#/components/schemas/AlarmType'

probableCause:

$ref: '#/components/schemas/ProbableCause'

specificProblem:

$ref: '#/components/schemas/SpecificProblem'

perceivedSeverity:

$ref: '#/components/schemas/PerceivedSeverity'

ackState:

$ref: '#/components/schemas/AckState'

ackUserId:

type: string

ackSystemId:

type: string

NotifyComments:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

required:

- alarmId

- alarmType

- probableCause

- perceivedSeverity

- comments

properties:

alarmId:

$ref: '#/components/schemas/AlarmId'

alarmType:

$ref: '#/components/schemas/AlarmType'

probableCause:

$ref: '#/components/schemas/ProbableCause'

specificProblem:

$ref: '#/components/schemas/SpecificProblem'

perceivedSeverity:

$ref: '#/components/schemas/PerceivedSeverity'

comments:

$ref: '#/components/schemas/Comments'

NotifyPotentialFaultyAlarmList:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

required:

- reason

properties:

reason:

type: string

NotifyAlarmListRebuilt:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

required:

- reason

properties:

reason:

type: string

alarmListAlignmentRequirement:

$ref: '#/components/schemas/AlarmListAlignmentRequirement'

#---- Definition of resources ------------------------------------------------------#

Comment:

type: object

properties:

commentTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTimeRo'

commentUserId:

type: string

commentSystemId:

type: string

commentText:

type: string

Comments:

description: >-

Collection of comments. The comment identifiers are allocated by the

MnS producer and used as key in the map.

type: object

additionalProperties:

$ref: '#/components/schemas/Comment'

#----- Definitions in TS 28.111 for TS 28.532 --------------------------#

resources-faultNrm:

oneOf:

- $ref: '#/components/schemas/AlarmList-Single'

#----- Definitions in TS 28.111 for TS 28.532 --------------------------#

<CODE ENDS>

*** END OF CHANGE 1 ***

*** START OF CHANGE 1 ***

*** yang-models/_3gpp-common-fm.yang ***

<CODE BEGINS>

module _3gpp-common-fm {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-common-fm";

prefix "fm3gpp";

import ietf-yang-types { prefix yang; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-yang-extensions { prefix yext3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Defines a Fault Management model

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.111";

revision 2025-05-01 { reference "CR-0042 CR-0043"; }  // common for R18, R19

revision 2025-03-25 { reference "CR-0025 CR-0026"; }

revision 2024-05-12 {

description "The definition of the module was from TS 28.623 to TS 28.111";

reference CR-0008 ;

}

revision 2024-03-06 { reference CR-0333 ; }

revision 2024-02-24 { reference CR-0346; }

revision 2024-01-18 {

description "The specification of the file is moved from 28.623 to 28.532";

reference "28.623 CR-0315";

}

revision 2023-09-18 { reference CR-0271; }

revision 2023-05-10 { reference CR-0250; }

revision 2022-10-24 { reference CR-0196;   }

revision 2021-08-08 { reference "CR-0132"; }

revision 2021-06-02 { reference "CR-0130"; }

revision 2020-06-03 { reference "CR-0091"; }

revision 2020-02-24 { reference "S5-201365"; }

feature AcknowledgeByConsumer {

description "Indicates whether alarm acknowledgement by the consumer is

supported.";

}

typedef eventType {

type enumeration {

enum OTHER {

value 1;

}

enum COMMUNICATIONS_ALARM {

value 2;

}

enum QUALITY_OF_SERVICE_ALARM {

value 3;

}

enum PROCESSING_ERROR_ALARM {

value 4;

}

enum EQUIPMENT_ALARM {

value 5;

}

enum ENVIRONMENTAL_ALARM {

value 6;

}

enum INTEGRITY_VIOLATION {

value 7;

}

enum OPERATIONAL_VIOLATION {

value 8;

}

enum PHYSICAL_VIOLATION {

value 9;

}

enum SECURITY_SERVICE_OR_MECHANISM_VIOLATION {

value 10;

}

enum TIME_DOMAIN_VIOLATION {

value 11;

}

}

description "General category for the alarm.";

}

typedef severity-level {

type enumeration {

enum CRITICAL { value 3; }

enum MAJOR { value 4; }

enum MINOR { value 5; }

enum WARNING { value 6; }

enum INDETERMINATE { value 7; }

enum CLEARED { value 8; }

}

description "The possible alarm severities";

}

grouping AlarmCommentGrp {

leaf commentTime {

type yang:date-and-time;

config false;

mandatory true;

yext3gpp:inVariant;

description "Date and Time the comment was created.";

}

leaf commentUserId {

type string;

mandatory true;

yext3gpp:inVariant;

description "It carries the identification of the user who made the

comment.";

}

leaf commentSystemId {

type string;

mandatory true;

yext3gpp:inVariant;

description "It carries the identification of the system (

Management System) from which the comment is made. That system

supports the user that made the comment.";

}

leaf commentText {

type string;

mandatory true;

yext3gpp:inVariant;

description "It carries the textual comment.";

}

}

grouping ThresholdHysteresisGrp {

description "The ThresholdHysteresis defines the threshold boundaries to

control the hysteresis mechanism.

The high attribute of ThresholdHysteresis identifies the higher value of

a threshold with hysteris, the integer type is used for counter

thresholds and the float type for gauge thresholds. The low attribute

of ThresholdHysteresis identifies the lower value of a threshold with

hysteresis, applicable only to gauge thresholds.";

leaf high {

type union {

type int64;

type decimal64 {

fraction-digits 7;

}

}

mandatory true;

description "Higher value of a threshold with hysteris, the integer type

is used for counter thresholds and the float type for gauge

thresholds.";

}

leaf low {

type decimal64 {

fraction-digits 7;

}

description "Lower value of a threshold with hysteresis, applicable

only to gauge thresholds.";

}

}

grouping ThresholdLevelIndGrp {

description "The up attribute indicates for counter and gauge thresholds

that the threshold crossing occurred when going up. The down attribute

only indicates for gauge thresholds that the threshold crossing occurred

when going down, applicable only to gauge thresholds.";

list up {

description "Indicates for counter and gauge thresholds that the

threshold crossing occurred when going up.";

max-elements 1;

key idx;

leaf idx { type int32; }

uses ThresholdHysteresisGrp;

}

list down {

description "Indicates for gauge thresholds that the threshold crossing

occurred when going down, applicable only to gauge thresholds.";

max-elements 1;

key idx;

leaf idx { type int32; }

uses ThresholdHysteresisGrp;

}

}

grouping ThresholdCrossingGrp {

description "The datatype indicates the crossed threshold

information regardless of the gauge threshold, which represents an

instantaneous value that changes over time, or the counter threshold,

which represents monotonically increasing cumulative quantity.

The observedMeasurement attribute of TheresholdInfo specifies the name

of the monitored measurement that crossed the threshold and that

caused the notification (Rec. ITU-T X. 733[8]). The observedValue

attribute indicates the value of the gauge or counter which crossed

the threshold. This may be different from the threshold value if, for

example, the gauge may only take on discrete values. Integer values

are used for counters and float values for gauges (Rec. ITU-T X. 733).

Note that a 'number' type property can contain both integers and

floating point numbers.

For the thresholdLevel attribute, in the case of a gauge, it specifies

a pair of threshold values, the first being the value of the crossed

threshold and the second, its corresponding hysteresis; in the case of

a counter, it specifies only the threshold value (Rec. ITU-T X. 733).

For the armTime attribute, for a gauge threshold, it specifies the

time at which the threshold was last re-armed, namely the time after

the previous threshold crossing at which the hysteresis value of the

threshold was exceeded thus again permitting generation of

notifications when the threshold is crossed; for a counter threshold,

the later of the time at which the threshold offset was last applied,

or the time at which the counter was last initialized (for resettable

counters) (Rec. ITU-T X. 733).";

leaf observedMeasurement {

type string;

mandatory true;

description "The name of the monitored measurement that crossed the

threshold and that caused the notification (Rec. ITU-T X. 733 ";

}

leaf observedValue {

type union {

type int64;

type decimal64 {

fraction-digits 7;

}

}

mandatory true;

description "The value of the gauge or counter which crossed the

threshold. This may be different from the threshold value if, for

example, the gauge may only take on discrete values.

Integer values are used for counters and float values for gauges

(Rec. ITU-T X. 733). Note that a 'number' type property can contain

both integers and floating point numbers.";

}

list thresholdLevel {

description "In the case of a gauge the threshold level specifies

a pair of threshold values, the first being the value of the crossed

threshold and the second, its corresponding hysteresis; in the case

of a counter the threshold level specifies only the threshold value

(Rec. ITU-T X. 733).";

max-elements 1;

key idx;

leaf idx { type int32; }

uses ThresholdLevelIndGrp;

}

leaf armTime {

type yang:date-and-time;

description "For a gauge threshold, the time at which the threshold

was last re-armed, namely the time after the previous threshold

crossing at which the hysteresis value of the threshold was

exceeded thus again permitting generation of notifications when the

threshold is crossed. For a counter threshold, the later of the time

at which the threshold offset was last applied, or the time at

which the counter was last initialized (for resettable counters)

(Rec. ITU-T X. 733)";

}

}

grouping AlarmRecordGrp {

description "Contains alarm information of an alarmed object instance.

A new record is created in the alarm list when an alarmed object

instance generates an alarm and no alarm record exists with the same

values for objectInstance, alarmType, probableCause and specificProblem.

When a new record is created the MnS producer creates an alarmId, that

unambiguously identifies an alarm record in the AlarmList.

Alarm records are maintained only for active alarms. Inactive alarms are

automatically deleted by the MnS producer from the AlarmList.

Active alarms are alarms whose

a)  perceivedSeverity is not CLEARED, or whose

b)  perceivedSeverity is CLEARED and its ackState is not ACKNOWLEDED.";

leaf alarmId {

type string;

mandatory true;

description "Identifies the alarmRecord";

yext3gpp:notNotifyable;

yext3gpp:inVariant;

}

leaf objectInstance {

type types3gpp:DistinguishedName;

config false ;

mandatory true;

yext3gpp:notNotifyable;

yext3gpp:inVariant;

}

leaf notificationId {

type int32;

config false ;

mandatory true;

description "The Id of the last notification updating the AlarmRecord.";

yext3gpp:notNotifyable;

}

leaf alarmRaisedTime {

type yang:date-and-time ;

mandatory true;

config false ;

yext3gpp:notNotifyable;

}

leaf alarmChangedTime {

type yang:date-and-time ;

config false ;

description "not applicable if related alarm has not changed";

yext3gpp:notNotifyable;

}

leaf alarmClearedTime {

type yang:date-and-time ;

config false ;

description "not applicable if related alarm was not cleared";

yext3gpp:notNotifyable;

}

leaf alarmType {

type eventType;

config false ;

mandatory true;

description "General category for the alarm.";

yext3gpp:notNotifyable;

yext3gpp:inVariant;

}

leaf probableCause {

type union {

type int32;

type string;

}

config false ;

mandatory true;

yext3gpp:notNotifyable;

yext3gpp:inVariant;

}

leaf specificProblem {

type union {

type int32;

type string;

}

config false ;

reference "ITU-T Recommendation X.733 clause 8.1.2.2.";

yext3gpp:notNotifyable;

yext3gpp:inVariant;

}

leaf perceivedSeverity {

type severity-level;

mandatory true;

description "This is Writable only if producer supports consumer

to set perceivedSeverity to CLEARED";

yext3gpp:notNotifyable;

}

leaf backedUpStatus {

type boolean;

config false ;

description "Indicates if an object (the MonitoredEntity) has a back

up. See definition in ITU-T Recommendation X.733 clause 8.1.2.4.";

yext3gpp:notNotifyable;

}

leaf backUpObject {

type types3gpp:DistinguishedName;

config false ;

description "Backup object of the alarmed object as defined in

ITU-T Rec. X. 733";

yext3gpp:notNotifyable;

}

leaf trendIndication {

type enumeration {

enum MORE_SEVERE;

enum NO_CHANGE;

enum LESS_SEVERE;

}

config false ;

description "Indicates if some observed condition is getting better,

worse, or not changing. ";

reference "ITU-T Recommendation X.733 clause 8.1.2.6.";

yext3gpp:notNotifyable;

}

list thresholdInfo {

config false ;

yext3gpp:notNotifyable;

description "Indicates the crossed threshold";

key idx;

leaf idx { type int32; }

uses ThresholdCrossingGrp;

}

list stateChangeDefinition {

key attributeName;

config false ;

description "Indicates MO attribute value changes associated with the

alarm for state attributes of the monitored entity (state transitions).

The change is reported with the name of the state attribute, the new

value and an optional old value.

See definition in ITU-T Recommendation X.733 [4] clause 8.1.2.10.";

yext3gpp:notNotifyable;

leaf attributeName {

type string;

}

anydata newValue {

mandatory true;

description "The new value of the attribute. The content of this data

node shall be in accordance with the data model for the attribute.";

}

anydata oldValue{

description "The old value of the attribute. The content of this data

node shall be in accordance with the data model for the attribute.";

}

}

list monitoredAttributes {

key attributeName;

config false ;

yext3gpp:notNotifyable;

description "Attributes of the monitored entity and their

values at the time the alarm occurred that are of interest for the

alarm report.";

reference "ITU-T Recommendation X.733 clause 8.1.2.11.";

leaf attributeName {

type string;

}

anydata value {

mandatory true;

description "The value of the attribute. The content of this data

node shall be in accordance with the data model for the attribute.";

}

}

leaf proposedRepairActions {

type string;

config false ;

description "Indicates proposed repair actions. See definition in

ITU-T Recommendation X.733 clause 8.1.2.12.";

yext3gpp:notNotifyable;

}

leaf additionalText {

type string;

config false ;

yext3gpp:notNotifyable;

}

list additionalInformation {

key name;

config false ;

yext3gpp:notNotifyable;

description "Vendor specific alarm information in the alarm.";

uses types3gpp:nameValuePair;

}

leaf rootCauseIndicator {

type boolean;

default false;

config false ;

description "It indicates that this AlarmInformation is the root cause

of the events captured by the notifications whose identifiers are in

the related CorrelatedNotification instances.";

yext3gpp:notNotifyable;

}

list comments {

yext3gpp:notNotifyable;

description "List of comments and data about the comments.";

key idx;

leaf idx { type uint32; }

uses AlarmCommentGrp;

}

leaf ackTime  {

if-feature AcknowledgeByConsumer;

type yang:date-and-time ;

config false ;

description "It identifies the time when the alarm has been

acknowledged or unacknowledged the last time, i.e. it registers the

time when ackState changes.";

yext3gpp:notNotifyable;

}

leaf ackUserId  {

if-feature AcknowledgeByConsumer;

type string;

description "It identifies the last user who has changed the

Acknowledgement State.";

yext3gpp:notNotifyable;

}

leaf ackSystemId  {

if-feature AcknowledgeByConsumer;

type string;

description "It identifies the system (Management System) that last

changed the ackState of an alarm, i.e. acknowledged or unacknowledged

the alarm.";

yext3gpp:notNotifyable;

}

leaf ackState  {

if-feature AcknowledgeByConsumer;

type enumeration {

enum ACKNOWLEDGED {

description "The alarm has been acknowledged.";

}

enum UNACKNOWLEDGED {

description "The alarm has unacknowledged or the alarm has never

been acknowledged.";

}

}

yext3gpp:notNotifyable;

}

leaf clearUserId {

type string;

description "Carries the identity of the user who invokes the

clearAlarms operation.";

yext3gpp:notNotifyable;

}

leaf clearSystemId {

type string;

yext3gpp:notNotifyable;

}

leaf serviceUser {

type string;

config false ;

description "It identifies the service-user whose request for service

provided by the serviceProvider led to the generation of the

security alarm.";

yext3gpp:notNotifyable;

}

leaf serviceProvider {

type string;

config false ;

description "It identifies the service-provider whose service is

requested by the serviceUser and the service request provokes the

generation of the security alarm.";

yext3gpp:notNotifyable;

}

leaf securityAlarmDetector {

type string;

config false ;

yext3gpp:notNotifyable;

}

list correlatedNotifications {

key sourceObjectInstance;

description "List of correlated notifications";

leaf sourceObjectInstance {

type types3gpp:DistinguishedName;

}

leaf-list notificationIds {

type int32;

min-elements 1;

}

}

}

grouping AlarmListGrp {

description "Represents the AlarmList IOC.";

leaf administrativeState {

type types3gpp:BasicAdministrativeState ;

default LOCKED;

description "When set to UNLOCKED, the alarm list is updated.

When the set to LOCKED, the existing alarm records are not

updated, and new alarm records are not added to the alarm list.";

}

leaf operationalState {

type types3gpp:OperationalState ;

default DISABLED;

config false;

description "The producer sets this attribute to ENABLED, indicating

that it has the resource and ability to record alarm in AlarmList

else, it sets the attribute to DISABLED.";

}

leaf numOfAlarmRecords {

type uint32 ;

config false;

mandatory true;

description "The number of alarm records in the AlarmList";

yext3gpp:notNotifyable;

}

leaf lastModification {

type yang:date-and-time ;

config false;

description "The last time when an alarm record was modified";

yext3gpp:notNotifyable;

}

list alarmRecords {

key alarmId;

description "List of alarmRecords";

yext3gpp:notNotifyable;

uses AlarmRecordGrp;

}

leaf-list unreliableAlarmScope {

type types3gpp:DistinguishedName;

config false;

yext3gpp:notNotifyable;

description "Identifies, the part of the alarm scope that may not be

reliable.

If this parameter is equal to the instance carried in systemDN,

then all AlarmRecord instances in the AlarmList may not be reliable.

If this parameter is equal to some instance represented by

MonitoredEntity, then only AlarmRecord related to this instance and

its descendants may not be reliable.";

}

}

grouping FmSubtree {

description "Contains FM related classes.

Should be used in all classes (or classes inheriting from)

- SubNetwork

- ManagedElement

If some YAM wants to augment these classes/list/groupings they must

augment all user classes!";

list AlarmList {

key id;

max-elements 1;

yext3gpp:only-system-created;

description "The AlarmList represents the capability to store and manage

alarm records. The management scope of an AlarmList is defined by all

descendant objects of the base managed object, which is the object

name-containing the AlarmList, and the base object itself.

AlarmList instances are created by the system or are pre-installed.

They cannot be created nor deleted by MnS consumers.

When the alarm list is locked or disabled, the existing alarm records

are not updated, and new alarm records are not added to the alarm list";

uses top3gpp:Top_Grp ;

container attributes {

uses AlarmListGrp ;

}

}

}

}

<CODE ENDS>

*** END OF CHANGE 1 ***