3GPP TSG-SA5 Meeting #161	S5-252363

Fukuoka, Japan, 19 - 23 May 2025

# Source:	Nokia

# Title:		Rel-19 DP on modelling CCL goals and targets

# Document for:	   Approval

# Agenda Item:	6.19.4.1

# 1	Decision/action requested

The group is asked to discuss and endorse the proposal.

# 2	References

[1] 	3GPP TS 28.536: "Management and orchestration; Management services for communication service assurance; Stage 2 and stage 3".

# 3	Discussion

## 3.1	Basic principles

A CCL needs to have an objective that it aims to achieve – that is named a goal. For example, the ACCL [1] has an assurance goal which contains the set of targets which the ACCL intend to achieve. The CCL should also have a goal.

## 3.2	Information model NRM options for CCL goals

### 3.2.1	Option A1: Goal as an IOC.

The CCL goal is instantiable, e.g., to be reused by one or more CCL instances. It contains a set of  goalTarget(s) which are of type  expectationTarget, GoalTarget (new dataType) or string – see clause 3.2 below

### 3.2.1	Option A2: Goal as information existing on the CCL

The CCL goal is not instantiable but is information (an attribute) on each CCL instance. The CCL goal could then be a list whose entries are of type  expectationTarget, GoalTarget(new dataType) or string – see clause 3.2 below.

## 3.3	Information model definition options for a CCL goal

### 3.3.1	Option B1: Goal as a reference to an intent

##### 6.3.Y.A	CCL

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| cCLGoals | M | T | T | F | T |

### 6.4.1	Attribute properties

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| cCLGoals | It indicates the set of targets to be achieved by the CCL. A target may be similar to an intent expectationTarget or another targets | type: expectationTarget multiplicity: 1..* … |

The goal is explicitly tied to an intent, the goal is a reference to an intent. This implies that every CCL only lives to realize an intent and CCLs cannot be implemented without intents. This is extremely limiting the value of CCLs. For example, SON functions (which are the most basic example of CCLs) are still useful without intents, but this solution implies all SON functions must be related to intent handlers.

### 3.3.2	Option B2:Goal as a set of targets each as an intent expectation target

The goal is a list of targets. Each target is of the same type as an intent expectation target, which implies that:

- Either the CCL goal target may be related to an intent and to specific intent expectation targets in the intent.

- Or the CCL goal target may not be related to any intent but can contain freely formed targets which happens to be formatted in a way similar to intent targets.

This option allows for CCLs that are instantiated to fulfil intents and well as CCLs that have no relation to intents.

##### 6.3.Y.A	Goal

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| goalTargetList | M | T | T | F | T |

### 6.4.1	Attribute properties

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| goalTargetList | It indicates the set of targets to be achieved by the CCL. A target may be similar to an intent expectationTarget or another targets | type: expectationTarget multiplicity: 1..* … |

### 3.3.3	Option B3:Goal as a set of targets each a new data type

##### 6.3.Y.A	Goal

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| goalTargetList | M | T | T | F | T |

##### 6.3.Y.B	GoalTarget <<dataType>>

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| goalTargetName | M | T | T | F | T |
| goalTargetValue | M | T | T | F | T |

### 6.4.1	Attribute properties

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| goalTargetList | It indicates the set of targets to be achieved by the CCL. A target may be similar to an intent expectationTarget or another targets | type: GoalTarget multiplicity: 1..* … |
| goalTargetName | It indicates the name of the attribute for which specific values are desired to be achieved by the CCL. | type: String multiplicity: 1 … |
| goalTargetValue | The value desired to be achieved by the CCL on the attribute named goalTargetName . The string can be a single value (e.g. a float) or a value range. | type: String multiplicity: 1 … |

The goal is a list of targets. Each target is a pair of targetName and targetValue. The goal may be related to an intent, with the targets referring to specific intent expectation targets. But since not all CCLs must relate to intents, the relation is not made explicit, allowing for CCs (like SON functions)  that do not relate to intents.

### 3.3.4	Option B4: Goal as a list of targets each of type string

The goal is a list of targets and each target is of type string. This is a compromise that can represent any of the other options above but has the disadvantage that it is not fully interoperable.

##### 6.3.Y.A	Goal

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| goalTargetList | M | T | T | F | T |

### 6.4.1	Attribute properties

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| goalTargetList | It indicates the set of targets to be achieved by the CCL. A target may be similar to an intent expectationTarget or a pair of <targetName and desired value> or any other format of targets. | type: string multiplicity: 1..* … |

## 3.4	Proposed way forward for SA5

It is proposed to endorse the observations (from above section 3.1 – 3.3)

It is proposed to endorse option A1 as the approach to apply for the CCL Goals NRM. The CCL goal as an IOC which enables a CCL goal to be instantiable and can for example be instantiated once and used by several CCL instances.

It is proposed to endorse option B3 as the approach to apply for modelling CCL Goals. The CCL goal as an IOC containing a list of CCL goal targets.  Each CCL goal target is a pair of targetName and targetValue.

# Detailed proposal

It is requested to endorse the following observations and NRM changes:

-	Observation A1: Modelling a CCL goal as an IOC allows it to be reusable among several CCL instances.

-	Observation A2: Modelling a CCL goal as an attribute without an IOC makes it CCL-specific information enabling each  CCL to be instantiated with its own characteristic goal.

-	Observation B1: Making a CCL goal to be a reference to an intent limits all CCLs to only be means for fulfilling intents.

-	Observation B2: Making a CCL goal target to be of type intent expectation target limits any other definition of CCL target.

-	Observation B3: Making a CCL goal target to be of type string is an oversimplification that is fully standardized (not fully inter-operable).

-	Proposal 1: The CCL goal is modelled as an IOC.

-	Proposal 2: The CCL goal is a list of CCL goal targets, each CCL goal target a pair of targetName and targetValue.