3GPP TSG-SA5 Meeting #161	S5-252854

Fukuoka, Japan, 19 - 23 May 2025	revision of S5-252365

Source:	Nokia

Title:	Pseudo-CR on Procedure for CCL-impact assessment and resolution

Document for:	Approval

Agenda item:	6.19.4.1

Spec:	3GPP TS28.567

Version:	0.2.0

Work Item:	Closed Control Loop Management

Comments

This pCR is to add procedure for CCL-impact assessment and resolution as was agreed in the CCL study in TR28. 867

Proposed Changes

* * * First Change * * * *

# 7	Procedures

## 7.X	CCL-impact assessment and metric conflicts resolution

A CCL (called the actor-CCL) may not know the full scope that its actions will impact. And this may also not be known by the CCL coordination entity, In that case, the impact can be collected from the entities that have been affected by the CCL’s actions - jointly called impacted entities. The CCL contains an attribute, called executedAction attribute, which contains information indicating that an action has been taken that may affect the other CCLs (thus requesting feedback on how much impact there has been); and the CCL-action-impact time indicating the time when the affected entities should provide feedback. Any entity which may be impacted by the CCL actions (e.g. e.g. the CCL coordination entity or other CCLs) subscribes to be notified of changes to the executedAction and the related CCL-action-impact time.

After an action, the CCL updates the executedAction so that notifications are sent to the subscribed entities to indicate that if the entity is affected, it should provide its feedback on the effect in a time not exceeding the CCL-action-impact time. The notification may also be sent to the CCL coordination entity which then notifies that respective affected entities, e.g. other CCLs or other management functions.

The Impacted entity computes its observed impact in form of an index, called the Action Quality Indicator (AQI), that describes and quantifies the observed impact, i.e. it indicates the degree to which the action was good or bad to their objectives. The Action Quality Indicator is an integer in the range [0,10] where "0" indicates that the action was completely unacceptable and should never be reused in that context while "10" indicates that the action had very good outcomes for the reporting Impacted entity (e.g. the affected CCL). An index is used instead of sending specific metrics measured by each Impacted entity because specific metrics would require the actor-CCL to understand all the different metrics in exactly the same way as the Impacted entities do, which is not guaranteed to always be true. The AQI is specific to each CCL and to each scenario thar the CCL evaluates - since it is used to check how good or bad an action was for that CCL in that scenario. Accordingly, its computation would vary depending on the CCL and scenario but can be computed in a uniform way as a weighted sum of normalized KPIs) of that CCL.

Figure 7.X-1: CCL-impact assessment and actual metric-value conflicts resolution

Step 0. The set of CCLs are composed, configured and instantiated.

Step 1 The CCLs register their scopes of interest to the coordination entity including the scopes where they take measurements, take control actions as well as where their actions are expected to impact. Where applicable, the scope have also been coordinated to ensure there are no conflicts for desired impacted scopes, the desired outcomes on the impacted scopes, cross impacts between measurement and control scopes.

Step 2. The acting CCL derives and executes an action plan onto the network.

Step 3. After an action, the CCL updates the executedAction so that notifications are sent to the CCL coordination entity to indicate that if an entity is affected, it should provide its feedback on the effect in a time not exceeding the CCL-action-impact time.

Step 4. The CCL coordination entity then notifies that respective affected entities, e.g. other CCLs or other management functions

Step 5. The impacted entity collects information on its metrics, (e.g., a using PM job), based on which, it computes its observed impact in form of an index, called the Action Quality Indicator (AQI), that describes and quantifies the observed impact, i.e. it indicates the degree to which the action was good or bad to their objectives.

Step 6. The impacted entity sends the AQI to the coordination entity within the CCL-action-impact time that was notified by the actor-CCL.

The Action Quality Indicator is delivered to the actor-CCL in one of two ways:

-	Impacted entity writes the AQI into an equivalent attribute called “reported AQIs” on the coordination entity, i.e., the coordination entity contains an attribute for the Action Quality Indicator to which each impacted entity writes its computed AQI. The reported AQIs attribute is a list to which each affected entity appends a value

-	The impacted entity contains an attribute for an observed AQI in which it writes the computed AQI. This can then be notified to the coordination entity which subsequently reads it to obtain the AQI for that impacted entity.

Step 7. The coordination entity determines the aggregate impact on all affected entities. To enable the CCL coordination entity to determine how much impact actor-CCL had on all the other CCLs together, the CCL coordination entity aggregates the impact based on the reported Action Quality Indicators from the respective impacted entities. The AQI can be computed as a weighted average of the AQI sent by the individual impacted entities. The AQI is delivered to the actor-CCL in one of two ways:

-	The coordination entity writes the AQI into an equivalent attribute called “reported AQIs” on the actor-CCL, i.e., the actor-CCL contains an attribute for the Action Quality Indicator to which coordination entity writes the computed aggregate AQI.

-	The CCL coordination entity contains an attribute for the aggregate AQI in which the coordination entity writes the computed aggregate AQI that is computed from the AQIs reported by the multiple affected CCLs. On modifying this aggregate AQI attribute, this can then be notified to the actor-CCL which subsequently reads it to obtain the aggregate AQI.

Step 8. The coordination entity sends the aggregate AQI to actor-CCL which is then used by the actor-CCL to decide an appropriate action to minimise the impact.

Step 9. The actor-CCL evaluates the impacts and if needed, the way it makes its decisions. For example, the actor-CCL can adjust the control scope (i.e., the acceptable range of values) on a given parameter.

Step 10. actor-CCL can revise the previous actions if needed. If it is computed by the CCL coordination entity, the coordination entity notifies it to the actor-CCL and may also propose a response action, e.g. to reverse the action that was taken.

NOTE: The data models for executedAction, reportedAQIs need to be extended.

* * * Next Change * * * *

Annex X (informative):
UML code for procedure diagrams

# X.2		UML code for CCL coordination procedure diagrams

This annex contains the PlantUML source code for the procedure diagrams in clause 7 of the present document.

## A.2.11	CCL-impact assessment and metric conflicts resolution on unknown or unbounded impact-scope (Figure 7.X-1)

@startuml CCL-impact assessment and metric conflicts resolution on unknown or unbounded impact-scope

skinparam Shadowing false

autonumber

skinparam monochrome true

participant "Actor-CCL \n (CCL MnS producer & \n Coordination MnS Consumer)" as CL1

collections "other-CCLs \n (CCL MnS producer & \n other functions)" as CL2

participant "CCL Coordination MnS producer \n (scope coordination)" as xCL

participant "Network" as Net

Note over CL1, xCL: Actor-CCL and other-CCLs are composed, instantiated and configured as required.

CL2 -> xCL: Register measurement, control, \n& impact scopes of interest

CL1 -> Net: execute derived action plan A

CL1 -> xCL: notify executed action plan A [incl. impact time of action, time for feedback

xCL -> CL2: notify execution of action plan A from \nCCL1 [indicate feedback time]

CL2 -> CL2: evaluate impacts of \naction A to own metrics

CL2 -> xCL: notify impact of action plan A on other CCLs

xCL -> xCL: compute aggregate AQI\n as aggregate impact on\n all affetced entities

xCL -> CL1: notify aggregate impact of action plan A on other CCLs

Alt

CL1 -> CL1: modify own decisions, e.g.,  the control scope

end

Alt

CL1 -> Net: undo/revise executed action plan A

end

@enduml

* * * End of Changes * * * *