3GPP TSG-SA5 Meeting #161	S5-252290

# Fukuoka, Japan, 19 - 23 May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.104 | CR | 0216 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.104 adding stage 3 for analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMDAS_Ph3 |  |  |  |  |  | Date: |  |  | 2025-05-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This is to add stage 3 according to the proposed solution in discussion paper: S5-252287, for agreed use cases (correlation analytics, predicted fault) in SA5#160 meeting. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the stage3 solution for the MDA output of correlation analytics, predicted fault. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The stage 3 for the analytics are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | <Normative stage 3 is in forge> |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1727 at commit f5915a53ce092011cf3c355cfa5a7c90f2e69638 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of First change

*** START OF CHANGE 1 ***

*** OpenAPI/TS28104_MdaReport.yaml ***

<CODE BEGINS>

openapi: 3.0.1

info:

title: MDA Report

version: 19.1.0

description: >-

OAS 3.0.1 specification of the MDA Report

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.104; MDA Report

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.104/

paths: {}

components:

schemas:

#-------- Definition of types-----------------------------------------------------

Recommended3GPPAction:

type: object

properties:

mOInstance:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

path:

type: string

op:

type: string

enum:

- ADD

- REMOVE

- REPLACE

value:

type: array

uniqueItems: true

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeNameValuePairSet'

additionalText:

type: array

items:

type: string

RecommendedAction:

type: object

properties:

recommended3GPPActions:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/Recommended3GPPAction'

recommendedNon3GPPActions:

type: array

uniqueItems: true

items:

type: string

recommendedHumanReadableActions:

type: array

uniqueItems: true

items:

type: string

actionInterval:

type: integer

timeWindow:

type: array

uniqueItems: false

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/TimeWindow'

MeasurementDataCorrelationRecommendation:

type: object

properties:

recommendedMeasurementDataToCollect:

type: array

uniqueItems: true

items:

type: string

recommendedMeasurementDataNotToCollect:

type: array

uniqueItems: true

items:

type: string

modelPerformanceImpact:

type: integer

minimum: 0

maximum: 100

TrainingDataAnalysisOutput:

type: object

properties:

measurementDataCorrelationRecommendation:

$ref: '#/components/schemas/MeasurementDataCorrelationRecommendation'

NFScalingDimensioningDataAnalysisOutput:

type: object

properties:

recommendedActions:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/RecommendedAction'

ThresholdAssessment:

type: object

properties:

performanceMetrics:

description: >-

It indicates list of performance metrics with threshold

configuration issue.

Performance metrics include measurements defined in TS 28.552

and KPIs defined in TS 28.554.

For non-3GPP specified measurements the name is defined elsewhere.

type: array

uniqueItems: true

items:

type: string

timeWindow:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/TimeWindow'

confidenceScore:

type: number

format: float

PmPrediction:

type: object

properties:

pmName:

type: string

pmPredictedValue:

description: >-

This specifies the predicted value of the PM specified by “pmName”

attribute.

oneOf:

- type: integer

- type: number

format: float

PMDataOutput:

type: object

properties:

pmPredictions:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/PmPrediction'

minItems: 1

thresholdAssessment:

$ref: '#/components/schemas/ThresholdAssessment'

thresholdAdjustmentRecommendations:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/RecommendedAction'

ManagementDataCollectionInfo:

type: object

properties:

managementDataType:

type: string

enum:

- MEASUREMENT

- KPI

- TRACE_MDT

- QOE

managementData:

$ref: 'TS28623_ManagementDataCollectionNrm.yaml#/components/schemas/ManagementData'

targetEntities:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

collectionDuration:

type: array

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/TimeWindow'

FailurePredictionOutput:

type: object

properties:

failurePredictionObject:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

potentialFailureType:

type: string

eventTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

issueID:

type: string

perceivedSeverity:

type: string

enum:

- CRITICAL

- MAJOR

- MINOR

- WARNING

- INTERMEDIATE

- CLEARED

managementDataCollectionRecommendations:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/ManagementDataCollectionInfo'

recommendedActions:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/RecommendedAction'

MDAOutputs:

type: object

properties:

mDAType:

$ref: 'TS28104_MdaNrm.yaml#/components/schemas/MDAType'

mDAOutputList:

oneOf:

- $ref: '#/components/schemas/TrainingDataAnalysisOutput'

- $ref: '#/components/schemas/NFScalingDimensioningDataAnalysisOutput'

- $ref: '#/components/schemas/PMDataOutput'

- $ref: '#/components/schemas/FailurePredictionOutput'

- type: array

uniqueItems: true

items:

$ref: '#/components/schemas/MDAOutputEntry'

analyticsWindow:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/TimeWindowRo'

confidenceDegree:

type: number

format: float

MDAOutputEntry:

type: object

properties:

mDAOutputIEName:

type: string

readOnly: true

mDAOutputIEValue: {}

#-------- Definition of MDA Report --------------------------------------------

MDAReport:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- type: object

properties:

mDAReportID:

type: string

mDAOutputs:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/MDAOutputs'

mDARequestRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnRo'

<CODE ENDS>

*** END OF CHANGE 1 ***

End of change