3GPP TSG-SA5 Meeting #161	S5-252320

# Fukuoka, Japan, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.532 | CR | 0381 | rev | 2 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 28.532 Add new notifications for reliability |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson India Private Limited, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | SBMA_Ph3 |  |  |  |  |  | Date: |  |  | 2025-05-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As decribed in TR 28.871 V19.0.0 (2024-12) clause 5.13 in some cases the producer is not able to provide reliable information to the consumer and may not be able to reliably prepare and send all the notifications that the consumer requested in NTFSubscriptionControl. Reasons for this may include, overload or internal errors of the producer or limitations in generating data change notifications in case of big configuration updates. If the producer detects such problems it should notify the consumer about the onset of the problems, but even more importantly when normal, reliable operations are resumed. The latter is important as this is the point where the consumer may take corrective actions to retrieve the data lost in the notifications. 2 notifications are defined to serve this purpose. Based on TR 28.871 V19.0.0 (2024-12) clauses 5.13. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new notifications notifyDataNodeTreeUnreliable and notifyDataNodeTreeSynchRecommended |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No way to indicate unreliable notification transport caused by producer issues |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1.1.x, 11.1.1.x.1, 11.1.1.x.2, 11.1.1.y, 11.1.1.y.1, 11.1.1.y.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Forge: TBD |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

#### 11.1.1.x	Notification notifyPotentialFaultyDataNodeTree

##### 11.1.1.x.1	Definition

This notification is sent to subscribed MnS consumer(s) when the information in the data node tree is potentially faulty, and that subsequently data node tree change notifications may be faulty as well. The reported problem may be caused for example by internal errors of the MnS producer or a loss of connection between the MnS producer and underlying data sources or managed entities.

The notification allows to specify the parts of the data node tree that are potentially faulty, referred to as (faulty) scope, using the parameters "objectInstance" and "scope".

When only the "objectInstance" parameter in the notification header is present, then the complete subtree, whose root data node is identified by the "objectInstance", is potentially faulty.

The "scope" parameter allows to specify more complex faulty scopes. It is expressed using the "scopeType" and "scopeLevel" parameters or the "dataNodeSelector" parameter. The "objectInstance" parameter in the notification header specifies the base object for the scope expressions. The notification shall use the method that is used in the subscription for this notification.

The reported scope should not be broader than the scope in the subscription for this notification. In other words, the reported scope shall be equal to the common subset of the data nodes included in the scope of the subscription and those included in the potentially faulty scope. Note that, when using the "scopeType" and "scopeLevel" parameters to define the scope, this may require to send multiple notifications with different scopes. For example, when the subscription scope includes a complete subnetwork with multiple base stations, and two base stations are not reachable, then the scope in one notification identifies the data node subtree representing one base station, and the scope in the second notification identifies the data node subtree representing the other base station. The expressions for selecting data nodes used with the "dataNodeSelector" parameter are so powerful that usually only one notification is required.

##### 11.1.1.x.2	Input parameters

| Parameter Name | S | Documentation and Allowed Values | Properties |
| --- | --- | --- | --- |
| objectClass | M | See clause 11.0.2 | Type: String multiplicity: 1 isOrdered: N/A isUnique: N/A |
| objectInstance | M | If the "scope" parameter is absent, this parameter identifies the root object of the subtree that is potentially faulty. If the "scope" parameter is present, this parameter identifies the base object of the scope. | Type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A |
| notificationId | M | See clause 11.0.2 | Type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A |
| notificationType | M | "notifyPotentialFaultyDataNodeTree" | Type: String multiplicity: 1 isOrdered: N/A isUnique: N/A |
| eventTime | M | See clause 11.0.2 | Type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A |
| systemDN | M | See clause 11.0.2 | Type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A |
| sequenceNo | O | See clause 11.0.2 | Type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A |
| subscriptionId | O | See clause 11.0.2 | Type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A |
| scope | O | This parameter specifies the set of data nodes that is potentially faulty. The base object of the scope is specified by the "objectInstance" parameter. The parameter has the same structure and semantics as the "Scope" data type in TS 28.622 [11], clause 4.3.23 CHOICE_1.1 scopeType CHOICE_1.2 scopeLevel CHOICE_2.1 dataNodeSelector If the parameter is absent, the scope is the full subtree under "objectInstance". | Type: Scope multiplicity: 0..1 isOrdered: N/A isUnique: N/A |

Next change

#### 11.1.1.y	Notification notifyDataNodeTreeSynchRecommended

##### 11.1.1.y.1	Definition

This notification is sent to subscribed MnS consumer(s) if the MnS producer recommends to the MnS consumer to synchronize his copy of the data node tree on the MnS producer with the current data node tree on the MnS producer.

The notification should be sent in numerous scenarios and for various reasons:

- If the data node tree on the MnS producer becomes reliable again after it was unreliable, and a "notifyPotentialFaultyDataNodeTree" notification was sent.

- If the MnS producer realizes that the data node tree was unreliable only when it is reliable again, and no previous "notifyPotentialFaultyDataNodeTree" notification was sent.

- If, for whatever reason, not all data node tree change notifications, that should have been sent based on the changes in the data node tree, are sent, and the fault causing this erroneous behaviour is gone.

- If the connection from the MnS producer to a MnS consumer was down and the MnS producer determines it is up again.

The notification allows to specify the parts of the data node tree, that should be synchronized, using the parameters "objectInstance" and "scope". The usage of these parameters is defined in clause 11.1.1.x.1.

If the "notifyPotentialFaultyDataNodeTree" notification is supported, the "notifyDataNodeTreeSynchRecommended" notification shall be supported as well. Furthermore, if a "notifyPotentialFaultyDataNodeTree" is sent to a subscribed consumer, a subsequent "notifyDataNodeTreeSynchRecommended" notifications shall be sent to the consumer, when the data node tree is reliable again.

##### 11.1.1.y.2	Input parameters

| Parameter Name | S | Documentation and Allowed Values | Properties |
| --- | --- | --- | --- |
| objectClass | M | See clause 11.0.2 | Type: String multiplicity: 1 isOrdered: N/A isUnique: N/A |
| objectInstance | M | If the "scope" parameter is absent, this parameter identifies the root object of the subtree that should be synchronized. If the "scope" parameter is present, this parameter identifies the base object of the scope. | Type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A |
| notificationId | M | See clause 11.0.2 | Type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A |
| notificationType | M | "notifyPotentialFaultyDataNodeTree" | Type: String multiplicity: 1 isOrdered: N/A isUnique: N/A |
| eventTime | M | See clause 11.0.2 | Type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A |
| systemDN | M | See clause 11.0.2 | Type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A |
| sequenceNo | O | See clause 11.0.2 | Type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A |
| subscriptionId | O | See clause 11.0.2 | Type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A |
| scope | O | This parameter specifies the set of data nodes that should be synchronized. The base object of the scope is specified by the "objectInstance" parameter. The parameter has the same structure and semantics as the "Scope" data type in TS 28.622 [11], clause 4.3.23 CHOICE_1.1 scopeType CHOICE_1.2 scopeLevel CHOICE_2.1 dataNodeSelector If the parameter is absent, the scope is the full subtree under "objectInstance". | Type: Scope multiplicity: 0..1 isOrdered: N/A isUnique: N/A |

End of  changes