3GPP TSG-SA5 Meeting #161	S5-252364

Fukuoka, Japan, 19 - 23 May 2025

Source:	Nokia

Title:	Pseudo-CR on CCL goal conflicts

Document for:	Approval

Agenda item:	6.19.4.1

Spec:	3GPP TS28.567

Version:	0.2.0

Work Item:	Closed Control Loop Management

Comments

This pCR is to add use case and requirements for CCL goal conflicts handling as was agreed in the CCL study in TR28. 867

Proposed Changes

* * * First Change * * * *

# 5	Management Capabilities

## 5.7	CCL Conflict management Capability - CONF

### 5.7.1	Description

A CCL may experience direct conflicts on its goals, targets, scopes, trigger time and execution time. The management system needs to support capabilities to avoid, detect and resolve the conflicts.

### 5.7.2	Use Cases

#### 5.7.2.1	CCL Trigger-time conflicts handling - CONF_01

Typically, a CCL whose start is triggered based on conditions, needs to be triggered to run at a specific time and terminate when certain conditions are met, to run when a certain performance threshold is crossed. If triggered independently, there may be conflicts among the CCLs. The triggers for different CCLs to be executed need to be coordinated to avoid conflicts among the CCLs.

The management system should support the capability for avoidance of action-execution-time conflicts. Since each CCL focuses on a smaller scope of the network problem space, several CCLs may need to be executed.  For actions in a given network scope, the CCLs can be explicitly scheduled by the management system. Where the scopes overlap, the CCLs need to align the action plans, for example, which action plan to execute and when. There is a need to assess each plan and choose the most appropriate combination of action plan(s) based on the selection policy and then notify the selected action plan(s) to the related CCLs. The MnS consumer may also be notified when it is safe to ignore the conflict. The MnS consumer may configure the criteria for evaluating the severity of conflicts.

#### 5.7.2.2	CCL action conflicts handling –CONF_02

When two (or more) CCLs attempt to adjust the same network parameter but with different and contradicting values, the desired actions of the 2 CCL will be in conflict. For example, a CCL assuring throughput of a slice may be scaling-out the virtual resources of the slice. Whereas a CCL minimizing the energy consumption may be scaling-in the virtual resource of the same slice. It can be when the CCLs execute actions at the same time. However, it also happens when the CCLs execute at different times, and the scenario for actions to be separated in time is the more likely than actions occurring simultaneously. casein these conflict scenarios, the network parameter continuously ping-pongs between the two values. Such a conflict may be called an action conflict.

NOTE:	A potential conflict can for example be detected if a CCL observed that PMs on a certain object keep flipping between two values. The constant flipping can be an indication that 2 CCL instances are attempting to change the same scope.

The CCL may detect or observe events that identify the conflicts. The conflict can be avoided using some information or the policies (e.g. priority) provided by the consumer. If the conflict actually occurs, the CCL MnS producer should support services to inform MnS consumers the confirmed detected conflicts. It is needed to maximize the avoidance of conflict, including “requesting” information from MnS consumer and to inform MnS consumer about the potential conflict. CCL MnS Producer may also provide recommendations, for updating/deleting the conflicting CCLs, that would result in the resolution of detected conflict. The recommendation for update may include suggestions for modified targets.

Editor’s Note:	The exact information that can be exchanged is FFS

#### 5.7.2.3	CCL Indirect-target conflicts handling – CONF_03

Two (or more) CCLs configuring different control parameter may all influence the same metric. In other cases, the two CCLs influence two metrics Y1 and Y2 that are couple, i.e., which have a logical relationship between them. E.g. handover (HO) failure and SINR are coupled since a bad SINR can lead to more HO failures. If the two CL desire different values for the metric, or different values for two target metrics Y1 and Y2 but the targets are coupled, the CCLs are in conflict for the metric resulting into a target-metric-value conflict or indirect target conflicts.

Two target metrics Y1 and Y2 may be coupled such that actions to optimize any of them lead to correlated oscillations/degradations in Y1 or Y2, e.g. Y1 ensuring "HO failure is < 2 %" and Y2 wanting "SINR > 10dB". The correlated oscillations indicate a potential conflict, but the CCLs may not see the oscillations in the metric that is not of their interest. The management system should support the capability for detecting potential indirect targets conflicts. An MnS consumer may analyse the correlations to detect the potential conflict between CCL1 and CCL2. The MnS consumer should be able to inform CCL1 and CCL2 about the detected potential conflict represented by the correlated oscillations.

This severity of degradation in the performance metrics of the related CCLs could be the confirmation that a detected potential conflict is an actual harmful conflict. The management system should support the capability for detecting or confirming actual indirect targets conflicts. The threshold to determine the severity may be defined by the MnS consumer (e.g. the operator) so that if the degree of degradation is higher than the threshold then it is a confirmed conflict that requires resolution.

The management system should support the capability for avoiding potential indirect target conflicts. CCLs need to avoid large and frequent changes to network parameters which may affect network stability since they increase the probability of occurrence of conflicts. CCLs should take small smooth changes in the cases where the impact is not so clear and only make the large changes when the CCL is sure that the impact is positive. It is desirable for the CCL to notify to the MnS consumer the planned change, its claimed/predicted performance improvement and reliability/confidence in that action/decision. The MnS consumer may evaluate the claimed performance improvement and reliability/confidence to determine if the action should be allowed or not. The MnS consumer should be enabled notify the decision and possibly the failed criteria to the CCL - to either be executed or to be used to compute better decisions. Based on the inputs, the CCL may update its decision-making and repeat the decision evaluation process. If the CCL has consistently made good large-action-decisions, the MnS consumer should be enabled to inform the CCL that the CCL has consistently made good decisions and achieved its ultimate trust and that no more coordination of its decisions is needed.

The management system should support the capability for resolving detected indirect targets conflict. The MnS consumer should be enabled to trigger one or more CCLs to respond to the detected potential conflict. And if the triggered CCLs is unable to resolve that conflict, the CCL should inform the MnS consumer about the failure to resolve the problem. The MnS consumer can set the thresholds for performance degradation that triggers conflict detection and resolution.

Editor’s Note 1: The criteria for accurately setting the thresholds for performance degradation is FFS.

Editor’s Note 2: The name and description of this type of conflict will be revisited.

#### 5.A.2.K	CCL goal conflicts handling – CONF_0x

CCL instances will be responsible for related, adjacent or in some cases overlapping scopes. In such cases it is good to ensure that the goals of two CCLs are not contradictory or conflicting or leading to contradictory or conflicting outcomes, i.e. the targets in the goals of Closed Control Loop should not contradict one another within that goal or contradict with other targets in goals of related CCLs, otherwise a goal conflict is observed.  It is expected that a given network scope may have a set of  general objectives to which several CCL instances contribute, where the general objective describe the priorities among the different target values on the specific network metrics. The goal may be seen as a set of values of network metrics (measurements and KPIs) to be achieved by the CCL.

It is desirable to have a CCL coordination entity with a goal management capability responsible for managing and optimizing the goals of the CCLs based on For example, as illustrated by Figure 5.6.3.3.1-1, the input network objectives may simply require ensuring, for KPI K1, that with priorities, p1, p2, p3, the value of K1 should respectively be less than values V1, V2, V3. This may be provided for different KPIs on a network level (e.g. by the operator). Then, the goal management functionality compares these objectives to choose the appropriate KPI targets for each CCL. In the example, the CCL goals are set as K1 < 0.1 and K2 > 85 %.

Figure 7.1.1.2-1: Illustration of example prioritized goal targets on a set of KPIs that need to be coordinated among a group of CCL instances. This solution assumes that the scopes are prefixed but the outcomes of the solution may a recommendation to adjust the scopes

The goal management functionality should support capabilities to detect and avoid potential goal targets conflicts, detection and resolution of actual goal targets conflicts. The CCL instances should have the capability to inform the CCL coordination entity about a candidate goal conflict, e.g. about the values of the goal's targets that are in conflict with the targets of another goal. In response, the CCL coordination entity should support the capability to propose revisions of the goals of that CCL instance, to terminate the execution of the CCL instance or to delete the CCL instance.

### 5.7.3	Requirements

Table 5.7.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-CONF_01-01 | The 3GPP Management System should support a capability to detect and inform an authorized MnS consumer about a potential or actual CCL trigger-time conflicts. | CONF_01 |
| REQ-CONF_01-02 | The 3GPP Management System should support a capability to confirm and inform an authorized MnS consumer about a detected CCL trigger-time conflict after it is confirmed. | CONF_01 |
| REQ-CONF_01-03 | The 3GPP Management System should enable authorized MnS Consumer to provide information that can be used to support a capability to avoid or resolve a CCL trigger-time conflict. | CONF_01 |
| REQ-CONF-CONF_02-01 | The 3GPP Management System should support a capability to detect and inform an authorized MnS consumer about a potential action conflict. | CONF-CONF_02 |
| REQ-CONF-CONF_02-02 | The 3GPP Management System should support a capability to confirm and inform an authorized MnS consumer about an actual action conflict. | CONF-CONF_02 |
| REQ-CONF-CONF_02-03 | The 3GPP Management System should enable authorized MnS consumers to provide information that can be used to resolve a CCL action conflict. | CONF-CONF_02 |
| REQ-CONF-CONF_02-04 | The 3GPP Management System should enable authorized MnS consumers to provide information that can be used to avoid the action conflict. | CONF-CONF_02 |
| REQ-CONF-CONF_02-05 | The 3GPP Management System should support a capability to coordinate the resolution of CCL action conflicts among multiple CCLs | CONF-CONF_02 |
| REQ-CONF_03-01 | The 3GPP Management System should support a capability to detect and inform an authorized MnS consumer about a potential or actual CCL Indirect targets conflicts. | CONF-CONF_03 |
| REQ-CONF_03-02 | The 3GPP Management System should support a capability to confirm and inform an authorized MnS consumer about a detected CCL Indirect targets conflict after it is confirmed. | CONF-CONF_03 |
| REQ-CONF_03-03 | The 3GPP Management System should support a capability to avoid or resolve a CCL Indirect targets conflict that has been detected | CONF-CONF_03 |
| REQ-CONF_0x-01 | The 3GPP Management System should support a capability to detect and inform an authorized MnS consumer about a potential or actual CCL goal conflicts. | CONF-CONF_0x |
| REQ-CONF_0x-02 | The 3GPP Management System should support a capability to confirm and inform an authorized MnS consumer about a detected CCL goal conflict after it is confirmed. | CONF-CONF_0x |
| REQ-CONF_0x-03 | The 3GPP Management System should support a capability to avoid or resolve a CCL goal conflict that has been detected | CONF-CONF_0x |
| REQ-CONF_0x-04 | The 3GPP Management System should support a capability to coordinate the resolution of CCL goal conflicts among multiple CCLs | CONF-CONF_0x |

* * * End of Changes * * * *