3GPP TSG-SA5 Meeting #161	S5-252373

Fukuoka, Japan, 19 - 23 May 2025

Source:	Nokia

Title:	Pseudo-CR on NRM for Collaboration between NDTs

Document for:	Approval

Agenda item:	6.19.5.1

Spec:	3GPP TS28.561

Version:	0.2.0

Work Item:	Management aspects of Network Digital Twins

Comments

This pCR is to add NRM for the NDT  following agreements in the NDT study.

Proposed Changes

* * * First Change * * * *

## 5.5	Advanced NDT capabilities - NDTADV

### 5.5.1	Description

This clause describes advanced uses of Network digital twins.

### 5.5.2	Use Cases

#### 5.5.2.1	Collaboration between NDTs - NDTADV_01

A single NDT might not be able to fulfil a task by itself and may be dependent on or need to use the service or outputs of another NDT during the simulation/emulation activity. Thus, the MnS consumer should be able to configure (define or modify) the relation between NDTs during simulation/emulation.

The NDTs that are used by another NDTs are called component NDTs as illustrated by figure 5.5.2.1-1. An NDT (NDT-LES) that simulates load prediction, say for the RAN energy saving purposes may rely on 2 component NDTs - "cNDT-1" that models network traffic and "cNDT-2" that models the active equipment of the cell. Recursively,  cNDT-1 may rely on another component NDT (cNDT-11) that models user movement while cNDT-2  may also be composed of two other component NDTs - "cNDT-21" that that models antenna equipment and"cNDT-22" that that models a MIMO transceiver. The MnS consumer of NDT MnS for NDT-LES should be enabled get information on the component NDTs.

Figure 5.5.2.1-1: An example of nesting NDTs for load prediction

The benefits of collaboration between NDTs are:

-	NDTs are able to interact with each other, supporting information exchange and coordinated behavior.

-	NDTs can dynamically adapt their behavior according to the information exchanged and collaboration in place with other NDTs.

-	NDTs collaborations enhance their capabilities and system awareness.

#### 5.7.2.1	Information on NDT composition - NDTADV_02

Due to the dependency and nested characteristics of NDTs, any errors, malfunctions, or degradation of service may propagate and effect the services of the overall NDT. The MnS consumer should be enabled to obtain information on the NDT's components. For example, the MnS consumer may want to know whether the traffic modelling NDT models traffic for a city or for a village, i.e. whether the NDT has capabilities for a city or for a village. The MnS consumer can then request a city NDT to simulate a specific city.

If the MnS consumer configure the composition or relationships of the DTs with respect to each other, any modifications would change the context of the NDT. The MnS consumer may want to get information on validation and feasibility of the configuration. The NDT should provide information assessing whether the configurations are valid and feasible for the NDT composition and whether they conform with the simulation/emulation services of the NDT.

### 5.5.3	Requirements

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ- NDTADV_01-01 | The 3GPP management system should support a capability enabling an MnS consumer to configure relation between NDTs during simulation/emulation. | NDTADV_01 |
| REQ- NDTADV_02-01 | The 3GPP management system should have a capability enabling an authorized MnS consumer to subscribe to be informed on the characteristics of an NDT component. | NDTADV_02 |
| REQ- NDTADV_02-02 | The 3GPP management system should have a capability enabling an authorized MnS consumer to be informed of the validity and feasibility of the modifications in NDT component. | NDTADV_02 |

* * * Next Change * * * *

## 6.2	Information model for NDT (MnS component typeB)

#### 6.2.1.3	Class definition

Editor’s Note: The attributes of IOCs below are FFS.

##### 6.2.1.3.1	NDTFunction <<InformationObjectClass>>

###### 6.2.1.3.1.1	Definition

This IOC represents the properties of an NDT Function.

The NDTFunction includes a references to one or more NDTFunction instances which act as component NDTs contributing to the functionality of the NDTFunction.

###### 6.2.1.3.1.2	Attributes

The NDTFunction IOC includes attributes inherited from Top IOC (defined in 3GPP TS 28.622 [7]) and the following attributes.

Table 6.2.1.3.1.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| nDTCompositionInfo | O | T | T | F | T |
| Attribute related roles |  |  |  |  |  |
| ndtRef | M | T | T | F | T |

##### 6.2.1.3.2	NDTJob <<InformationObjectClass>>

###### 6.2.1.3.2.1	Definition

This IOC represents the properties of an NDT job demand created by an MnS consumer.

###### 6.2.1.3.2.2	Attributes

The NDTJob IOC includes attributes inherited from Top IOC (defined in 3GPP TS 28.622 [7]) and the following attributes.

Table 6.2.1.3.2.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| Attribute related roles |  |  |  |  |  |
| ndtFunctionRef | M | T | T | F | T |
| ndtReportRefList | M | T | T | F | T |

##### 6.2.1.3.3	NDTReport <<InformationObjectClass>>

###### 6.2.1.3.3.1	Definition

This IOC represents the properties of an NDT report corresponding to an NDT job.

The attribute "PerformanceData" is specified performance metrics and/or alarm types that are collected and reported by NDT after the behaviour is modelled in NDT.

###### 6.2.1.3.3.2	Attributes

The NDTReport IOC includes attributes inherited from Top IOC (defined in 3GPP TS 28.622 [7]) and the following attributes.

Table 6.2.1.3.3.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| Attribute related roles |  |  |  |  |  |
| ndtJobRef | M | T | T | F | T |
| ndtFunctionRef | M | T | T | F | T |

##### 6.2.1.3.Z	NDTCompositionInfo <<dataType>>

###### 6.2.1.3.Z.1	Definition

This dataType represents the properties of one NDT composition.

###### 6.2.1.3.Z.2	Attributes

The nDTCompositionInfo <<datatype>> includes attributes inherited from Top IOC (defined in 3GPP TS 28.622 [7]) and the following attributes.

Table 6.2.1.3.Z.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| nDTCompositionInfoId | M | T | T | F | T |
| nDTCompositionMap | M | T | T | F | T |

###### 6.2.1.3.Z.3	Attribute constraints

None.

###### 6.2.1.3.Z.4	Notifications

The common notifications defined in clauses 6.1 are valid for this IOC, without exceptions.

## 6.3	Attribute definitions

### 6.3.1	Attribute properties

Table 6.3.1-1

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| ndtJobRef | It indicates an DN of a NDTJob Instance. allowedValues: N/A | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ndtFunctionRef | It indicates an DN of a NDTFunction Instance. allowedValues: N/A | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ndtReportRefList | It indicates a list of DN for NDTReport Instances. allowedValues: N/A | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| nDTCompositionInfo | It indicates a description of the compositional information required to compose the components to provide a meaningful operational NDT service. An NDTfunction may be associated to more than one nDTCompositionInfo(s) indicating multiple potential compositional relations. allowedValues: N/A | type: NDTCompositionInfo multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| nDTCompositionInfoId | It indicates an identifier of the nDTCompositionInfo to be used to differentiate the multiple nDTCompositionInfo associated to one NDT. | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| nDTCompositionMap | It indicates a map or graph describing the relations among the components, i.e. which component can provide input to which other component. It is a pair <DN1, DN2> where DN1 where DN2 provides modelling input to DN1 | type: pair < DN, DN > multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |

* * * End of Changes * * * *