3GPP SA WG5 Meeting #161	S5-252375

Fukuoka, Japan, 19-23 May 2025

# Title:	[Draft] Further reply LS on signalling feasibility of dataset and parameter sharing

# Response to:	R2-2503169 Reply LS on signalling feasibility of dataset and parameter sharing

# Release:	Release 19

# Work Item:	NR_AIML_air-Core

Source:	SA5

To:	RAN1, RAN2, RAN3

Cc:	SA3, SA2

Contact Person:

Name:	Juan Zhang

E-mail Address:	juanzhan@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	-

1. Overall Description:

SA5 thanks RAN2 for the Reply LS on signalling feasibility of dataset and parameter sharing in R2-2503169. RAN2 listed the following non-OTA candidate solutions and asked SA5 to confirm if the solutions that impacted SA5 can be supported within Rel-19 existing architecture framework.

Table 1. non-OTA candidate solutions

| Option | Impacted WG | Specification impact/Implementation impact |
| --- | --- | --- |
| OAM -> UE-side training entity (a server inside MNO or an OTT server), where OAM is NW-side dataset/model parameter collection entity | SA5, SA3 | Up to SA5 (any intermediate node between OAM and UE-side OTT server is up to SA5; CN involvement if needed is up to SA2/SA5 discussion) |
| CN -> UE-side training entity (a server inside MNO or an OTT server), where CN is NW-side dataset/model parameter collection entity | SA2, SA3 | Up to SA2 (any intermediate node between CN and UE-side OTT server is up to SA2) |
| gNB -> OAM/CN -> UE-side training entity (a server inside MNO or an OTT server), where gNB is NW-side dataset/model parameter collection entity | RAN3, SA2, SA5, SA3 | Up to RAN3, SA2, SA5 (any intermediate node between gNB/OAM, OAM/UE-side OTT server, CN/UE-side OTT server is up to RAN3/SA2/SA5) |

SA5 discussed the corresponding solutions and approved the following conclusions.

OAM -> UE-side training entity

The UE-side training entity is the AF in trusted domain or untrusted domain.

SA5 supports the Service Based Management Architecture (SBMA) in TS 28.533 and an MnS is a set of offered capabilities for management and orchestration of network and services. SA5 also supports the Management Data Analytics (MDA) in TS 28.104 to provide a capability of processing and analysing data related to network and service events and status in TS 28.104. SA5 agrees that it is feasible for the AF in trusted domain works as an MnS Consumer to access the management capability offered by MnS producer or MDA MnS Consumer to receive the analytics output from MDA MnS producers.

There is an ongoing WID in Rel-19 to support the enhanced OAM for management service exposure to external consumers through CAPIF (MExpo) in TS 28.579, to provide a MnS to be consumed by the external MnS consumer through CAPIF.

gNB -> OAM/CN -> UE-side training entity

gNB -> CN -> UE-side training entity is out of SA5’s scope.

For gNB -> OAM -> UE-side training entity, SA5 agrees it is feasible to reuse the MDT procedure from gNB to OAM.

The status of OAM -> UE-side training entity is described above.

SA5 agrees that it is feasible to support dataset and parameter sharing within the Rel-19 architecture framework. SA5 is discussing the scope for Rel-20 MExpo and SA5 may discuss the details in Rel-20 based on the requirements from RAN WGs.

2. Actions:

To RAN1, RAN2, RAN3:

ACTION: 	SA5 asks RAN1, RAN2 and RAN3 to consider the above information.

3. Date of Next TSG SA WG5 Meetings:

TSG-SA5 Meeting #162		25-29 August 2025	Goteborg, Sweden

TSG-SA5 Meeting #163		13-17 October 2025	China (TBD)