3GPP TSG-SA5 Meeting #161	S5-252397

Fukuoka, Japan, 19 - 23 May 2025

Source:	Samsung

Title:	Solution for conflict management

Document for:	Approval

Agenda item:	6.19.4.1

Spec:	3GPP TS 28.567

Version:	0.2.0

Work Item:	CCL

Comments

This contribution proposes the solution for the use case of CCL instantiation based on conditions.

Proposed Changes

* * * First Change * * * *

## 6.2	Class diagram

### 6.2.1	Relationships

Figure 6.2.1-1: Relations for common information models for AI/ML management

Editor’s Note: The handling of Goal, targets or objectives for the general closed control loops is FFS

Figure 6.2.1-x: Conflict Management Control NRM fragment

### 6.2.2	Inheritance

Figure 6.2.2-1: Inheritance Hierarchy for Closed Control Loops

* * * Next Change * * * *

##### 4.1.2.3.x	ConflictManagementCtrl

4.1.2.3.x.1	Definition

This defines the conflict management functionality.

4.1.2.3.x.2	Attributes

The ConflictManagementCtrl IOC includes attributes inherited from Top IOC (defined TS 28.622[5]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| cCLActionConflictsHandling | M | T | T | F | T |

4.1.2.3.x.3	Attribute constraints

None

4.1.2.3.x.4	Notifications

The common notifications defined in subclause 4.1.2.5 are valid for this IOC, without exceptions or additions.

##### 4.1.2.3.x	CCLActionConflictsHandling <<datatype>>

4.1.2.3.x.1	Definition

This defines the handling of CCL action conflict between the two existing CCLs.

4.1.2.3.x.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| conflictInformation | M | T | T | F | T |
| conflictResolution | M | T | T | F | T |
| targetCCL | M | T | F | F | T |

4.1.2.3.x.3	Attribute constraints

None

4.1.2.3.x.4	Notifications

The common notifications defined in subclause 4.1.2.5 are valid for this IOC, without exceptions or additions.

##### 4.1.2.3.x	ConflictInformation <<datatype>>

4.1.2.3.x.1	Definition

This defines the information related with a conflicting CCL.

4.1.2.3.x.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| conflicitingCCLId | M | T | T | F | T |
| conflicitingGoalsTarget | M | T | T | F | T |
| conflicitingActions | M | T | T | F | T |

4.1.2.3.x.3	Attribute constraints

None

4.1.2.3.x.4	Notifications

The common notifications defined in subclause 4.1.2.5 are valid for this IOC, without exceptions or additions.

##### 4.1.2.3.x	ConflictResolution <<datatype>>

4.1.2.3.x.1	Definition

This defines the information related with conflict resolution.

4.1.2.3.x.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| conflicitingCCLId | M | T | T | F | T |
| cCLPriority | M | T | T | F | T |
| cCLGoalBreachPercentage | M | T | F | F | T |

4.1.2.3.x.3	Attribute constraints

None

4.1.2.3.x.4	Notifications

The common notifications defined in subclause 4.1.2.5 are valid for this IOC, without exceptions or additions.

* * * Next Change * * * *

## 6.4	Attribute definitions

### 6.4.1	Attribute properties

Table 6.4.1-1

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| scopeType | It indicates the type of scope that represented by the particular scope instance. allowedValues: CCL_MEASUREMENT_SCOPE, CCL_TARGET_SCOPE, CCL_CONTROL_SCOPE, CCL_IMPACT_SCOPE Editor’s Note: The allowed values will be revisited | type: Enum multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| cCLActionConflictsHandling | This defines the handling of CCL action conflict between the two existing CCLs. | Type: cCLActionConflictsHandling multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| conflictInformation | This defines the information related with a conflicting CCL. | Type: ConflictInformation multiplicity: * isOrdered: True isUnique: False defaultValue: None isNullable: False |
| conflictResolution | This defines the information related with conflict resolution. | Type: ConflictResolution multiplicity: * isOrdered: True isUnique: False defaultValue: None isNullable: False |
| targetCCL | The identification of the CCL that need to be deleted or updated to resolve conflict. This will be decided as per the information ConflictResolution. | Type: Dn multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| conflicitingCCLId | This indicates the CCL identification | Type: Dn multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| conflicitingGoalsTarget | This provides the goals and target of the CCL that are in conflict. | Type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| conflicitingActions | This provides the set of actions that have been taken by the CCL as part of the Execute step. | Type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| cCLPriority | This provides the priority of the CCL. This will be the numerical value between 1 to 10, with 1 being the least priority. | Type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| cCLGoalBreachPercentage | It defines the breach percentage per goal in terms of how bad the goal(s) is breached. For example, if the goal of guaranteed throughput is 200mbps and the actual throughput is coming to be 100mbps then the breach percentage would be 50%. The CCL that have higher percentage of breach will be prioritized | Type: TBD multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

* * * Next Change * * * *

## A.1.2	CCL inheritance relationships (Figure 6.2.1.1-2)

@startuml

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide members

hide circle

class Top << InformationObjectClass >>

class ClosedControlLoop <<InformationObjectClass>>

class CCLReport <<InformationObjectClass>>

class CCLScope <<InformationObjectClass>>

class ConflictManagementCtrl <<InformationObjectClass>>

Top <|-- ClosedControlLoop

Top <|-- CCLScope

Top <|-- CCLReport

Top <|-- ConflictManagementCtrl

@enduml

Source code for Figure 6.2.1.1-2 CCL inheritance relationships

* * * Next Change * * * *

# A.1.x	UML code for Figure 6.2.1-x

@startuml

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide members

hide circle

class ManagedEntity <<ProxyClass>>

class ConflictManagementCtrl<<InformationObjectClass>>

ManagedEntity "1" *-- "*" ConflictManagementCtrl: <<names>>

note left of ManagedEntity

Represents the following IOCs:

SubNetwork or

ManagedElement

end note

@enduml

* * * Next Change * * * *

* * * End of Changes * * * *