3GPP TSG-SA5 Meeting #161	S5-252418

Fukuoka, Japan, 19 - 23 May 2025

Source:	China Mobile

Title:	Pseudo-CR on TS 28.561 Rapporteur clean-up on abbreviations in clause 5

Document for:	Approval

Agenda item:	6.19.5.1

Spec:	3GPP TS 28.561

Version:	0.2.0

Work Item:	NDT

Comments

The abbreviation of each clause title is only used for associating requirements with use cases, which can be easily achieved by using clause number. In addition, the abbreviation decreases the readbility of the sepc and doesn’t align with clause 5.2 and 5.4.

Proposed Changes

* * * First Change * * * *

5	NDT Management capabilities

5.1	Control and life cycle management of NDTs

5.1.1	Description

This describes uses on control and life cycle management of NDT instances.

5.1.2	Use cases

5.1.2.1	Life cycle management of NDT instances

NDTs can be used for many application use cases. In each case, the NDT needs to implement a defined network scenario, run a simulation or emulation of that scenario, and subsequently provide an output representing the statuses of different network for the simulated network scenario. The lifecycle management of Network Digital Twins (NDT) refers to the management processes that control and transition an NDT instance through the processes of creation, activation, operation, deactivation, deletion and any updates during these processes. At each step, it is critical to ensure that the NDT instance accurately reflects the status of physical entities and support efficient simulation/emulation.

-	For creation, the NDT MnS Consumer requests the creation of an NDT instance based on the capabilities of the NDT producer. The NDT MnS consumer may request the NDT producer to create a new NDT from scratch, to update existing NDT models or to combine existing NDT models to generate a new NDT.

-	At any point during the life of an NDT instance, the MnS consumer may refine and adjust the NDT instance to accurately reflect the status of the physical entity and adapt it to specific application scenarios. For example, the consumer may trigger data collection from the network and synchronization of the state with the NDT.

5.1.2.2	Control of NDT instances

The 3GPP management system should support capabilities enabling consumers to define the network scenario, to control (start, suspend, resume or stop) the simulation or emulation of that network scenario and to provide outcomes representing measurements and counters from metrics in the simulated or emulated network scenario.

An NDT may have dependencies on other NDTs or DTs, each of which implements a specific network scope. The MnS consumer should also be enabled to configure or modify the dependencies e.g., to select which environment or radio propagation DT to be applied.

5.1.2.3	Synchronization with network

The NDT needs to obtain information from network functions and entities. The collected information includes the following:

-	Performance data: Performance measurement or the KPI as defined in 3GPP TS 28.552 [2] and 3GPP TS 28.554 [3].

-	MDT/Trace data: MDT measurements as defined in 3GPP TS 32.422 [4].

-	Fault management data: Fault management data as defined in 3GPP TS 28.111[5]

-	Configuration data: Configuration data defined in 3GPP TS 28.541 [6] and 3GPP TS 28.622 [7], any of the available MOIs;

Mobile networks are generally composed of numerous network elements. The data about these numerous network elements can potentially be vast. When collecting this vast amount of data to NDT, the collection might cause congestion in the network bandwidth used for data collection. The MnS producer should enable the MnS consumer to control the frequency of data to be collected by the NDT to minimize unnecessary collection of data and minimal congestion.

5.1.3	Requirements

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-DTLCM-01: | The 3GPP management system should support a capability enabling an MnS consumer to define the network scenario that should be modelled and simulated/emulated. | Life cycle management of NDT instances (See clause 5.1.2.1) |
| REQ-DTLCM-02: | The 3GPP management system should support a capability enabling an MnS consumer to control the simulation/emulation of a network scenario, e.g. to start, or stop the simulation/emulation. | Life cycle management of NDT instances (See clause 5.1.2.1) |
| REQ-DTLCM-03: | The 3GPP management system should support a capability to provide to the MnS consumer an output of a modelling or simulation process, the output representing the status of the network for the simulated scenario. | Life cycle management of NDT instances (See clause 5.1.2.1) |
| REQ-DTLCM-04: | The 3GPP management system should have the capability enabling the MnS consumer to control the frequency of network data collection. | Synchronization with network (See clause 5.1.2.3) |

* * * Next Change * * * *

5.3	NDT support for verification

5.3.1	Description

This describes NDT use in support of verification.

5.3.2	Use cases

5.3.2.1	General verification

To evaluate a network scenario, the NDT needs to be configured with data defining a network scenario, and where applicable, the configurations and functionalities to be evaluated. The MnS consumer should be enabled to configure the NDT accordingly. Given the scenario, configurations and functionalities, the consumer can then control the simulation (e.g. to start or stop the simulation) according to the LCM capabilities as described in clause 5.1. The NDT simulates the defined scenario configurations and functionalities, and collects data, which is then compiled into an NDT report. The MnS consumer should be enabled to configure the information that should be included in the simulation report. The NDT MnS producer should be enabled to send the simulation report to MnS consumer to indicate the network simulation results.

5.3.2.2	Verification of network response to events

The network can experience different kinds of events, including failure and non-failure events. A failure event refers to the situation where the network does not behave as expected, e.g., where users in the network are completely unable to get service from the network. A non-failure event refers to the situation where the network experiences an unusual occurrence but users are still able to get service. The NDT can be used to evaluate how the network responds any of these events. The NDT can model network behaviors and provide information on potential impact of network events. The NDT can be used to evaluate if a particular event will cause a network failure or whether the network can withstand the event.

The following is an example of events that can be supported:

- 	Signalling storms: A signalling storm is a failure events where a large number of signalling messages suddenly surge in the mobile communication network, resulting in the network processing capacity overload, thus affecting the network performance and stability. During this period, users will repeatedly try to establish the connection until reconnected, thus generating a large number of signalling messages surge suddenly, causing signalling storm. For a signalling storm, the NDT provides information on potential impact of surges in signalling requests on the network. The information can enable the MnS consumer to identify the optimal flow control parameters for each signalling impact point.

5.3.2.3	Verification of network configurations

Several configurations can be provided to the network, either as modifications to single parameters or as bulk configurations, e.g., as a configuration plan. There may be multiple configurations that are iteratively executed in NDT until the network performance, e.g. energy efficiency of NG-RAN, UE throughput in gNB, etc., meets the operators’ requirements. The MnS consumer should be enabled to configure the NDT with data defining a network scenario to be evaluated as well as the input information that should be applied to network in that scenario. The information may include configurations, simulated network scenarios or simulated traffic patterns.

The configurations can be evaluated for any network scenarios including the event listed in 5.3.2.1

5.3.2.4	Verification of automation-function configurations

Besides network configurations, network automation functionality (e.g., SON functions) may also be configured., e.g. configurations of functionality say for energy savings. The NDT can be used to evaluate the configurations of the automation functionality. The MnS consumer should be enabled to configure the NDT with data defining a network scenario to be evaluated for one or more automation functionality with the corresponding input information that should be applied to automation functionality to be evaluated. The information may include configurations, simulated network scenarios or simulated traffic patterns.

An example scenario of automation configuration verification is verification of Configuration of RAN energy saving management function (RAN ES MnF). A RAN ES MnF derives and executes network management operations via provisioning MnS to maximize energy saving for a group of cells e.g. based on the degree to which the cells overlap with each other. The NDT can be used to evaluate the configurations of the RAN ES MnF, i.e., the MnS consumer may instantiate an NDT simulation/emulation to verify the RAN ES configuration. The verification can be for configuration of behavior including which cells are grouped for ES and their degree of overlap. The verification can also be for configuration of performance requirements, which is MnS consumer’s requirements for network simulation performance, e.g., energy efficiency, RAN UE throughput, precision, maximum run time, etc. NDT performs network simulation/emulation, i.e., simulating/emulating the execution of RAN ES according to the configured behavior and performance requirements and collects its impact on the simulated network.

5.3.3	Requirements

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-NDTVER-01: | The management system should support a capability enabling the MnS consumer to define the information that should be included in the report/output on the simulation/emulation of verification of network scenarios and configurations. | General verification (Clause 5.3.2.1) |
| REQ-NDTVER-02: | The management system should have a capability enabling the MnS consumer to define one or more network events to be modelled and evaluated by the NDT. | Verification of network response to events (Clause 5.3.2.2) |
| REQ-NDTVER-03: | The management system should have the capability to report the simulation/emulation outcomes on the verification of one or more network events to be modelled and evaluated by the NDT | Verification of network response to events (Clause 5.3.2.2) |
| REQ-NDTVER-04: | The management system should have a capability enabling the MnS consumer to define one or more network configurations to be modelled and evaluated by the NDT. | Verification of network configurations (Clause 5.3.2.3) |
| REQ-NDTVER-05: | The management system should have the capability to report the simulation/emulation outcomes on the verification of network configurations modelled and evaluated by the NDT. | Verification of network configurations (Clause 5.3.2.3) |
| REQ-NDTVER-06: | The management system should have a capability enabling the MnS consumer to define one or more automation functionality and configurations for the automation functionality to be modelled and evaluated by the NDT. | Verification of automation-function configurations (Clause 5.3.2.4) |
| REQ-NDTVER-07: | The management system should have the capability to report the outcomes on the verification of one or more automation functionality and configurations modelled and evaluated by the NDT. | Verification of automation-function configurations (Clause 5.3.2.4) |

* * * Next Change * * * *

5.5	Advanced NDT capabilities

5.5.1	Description

This clause describes advanced uses of Network digital twins.

5.5.2	Use Cases

5.5.2.1	Collaboration between NDTs

A single NDT might not be able to fulfil a task by itself and may be dependent on or need to use the service or outputs of another NDT during the simulation/emulation activity. Thus, the MnS consumer should be able to configure the relation between NDTs during simulation/emulation.

The benefits of collaboration between NDTs are:

-	NDTs are able to interact with each other, supporting information exchange and coordinated behavior.

-	NDTs can dynamically adapt their behavior according to the information exchanged and collaboration in place with other NDTs.

-	NDTs collaborations enhance their capabilities and system awareness.

5.5.3	Requirements

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ- NDTAUT-01 | The 3GPP management system should support a capability enabling an MnS consumer to configure relation between NDTs during simulation/emulation. | Collaboration between NDTs (Clause 5.5.2.1) |

* * * End of Changes * * * *