3GPP TSG-SA5 Meeting #161	S5-252559

Fukuoka, Japan, 19 - 23 May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.105 | CR | draftCR | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Input to DraftCR Rel-19 TS 28.105 Add Procedure for ML Model Training |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_MGT_Ph2 |  |  |  |  |  | Date: |  |  | 2024-05-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Add Procedure for ML Model Training |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add Procedure for ML Model Training |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Procedure for ML Model Training is missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.x (new), 7.x (Annex A) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | No impact on Stage 3 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of the First Change

7.x	Generic Procedure for AI/ML management

7.x.1	Procedure for ML Training

Step 1. ML Training MnS Consumer sends a request to create an MLTrainingRequest instance (see createMOI operation defined in TS 28.532 [11]) to MnS Producer with the training information. The detailed training information sees attributes of the concrete MLTrainingRequest IOC defined in clause 7.3a.

- When the ML training request is for initial training, the ML Training Consumer shall specify the value of aIMLInferenceName in the training information.

- When the ML training request is for retraining, the ML Training Consumer shall specify the mLModelRef or mLModelCoordinationGroupRef in the training information.

Step 2. Based on the received request, the ML Training MnS Producer creates the concrete MLTrainingRequest instance (i.e. instance of MLTrainingRequest IOC) and configure the new created MLTrainingRequest MOI with the received training information.

Step 3.	MnS Producer sends a response (see createMOI operation defined in TS 28.532[11]) to the MnS Consumer.

Step 4.	Based on the created MLTrainingRequest instance, MnS Producer performs the ML model training for specific aIMLInferenceName.

Step 5. ML Training MnS Producer creates the concrete MLTrainingProcess instance (i.e. instance of MLTrainingProcess IOC) and MLTrainingReport instance (i.e. instance of MLTrainingReport IOC)., and configure the new created MLTrainingProcess MOI and MLTrainingReport MOI based on ML training process and ML training results, respectively.

NOTE 1:	No sequence restriction for above step 4 and step 5.

Step 6.	ML Training MnS Producer notifies the ML training report to the ML Training MnS consumer (see notifyMOIAttributeValueChanges notification in TS 28.532 [11]).

Start of the Secondt Change

Annex A (informative):
PlantUML source code for NRM class diagrams

A.X	Generic Procedure for AI/ML management

A.X.1	Procedure for ML Training

@startuml

'https://plantuml.com/sequence-diagram

hide footbox

autonumber

participant "ML Training MnS Consumer" #white

participant "ML Training MnS Producer" #white

"ML Training MnS Consumer" -> "ML Training MnS Producer": Request to create an MLTrainingRequest instance

"ML Training MnS Producer" -> "ML Training MnS Producer": Create and configure MLTrainingRequest MOI

"ML Training MnS Producer" -> "ML Training MnS Consumer": Response for create an MLTrainingRequest instance

"ML Training MnS Producer" -> "ML Training MnS Producer": Perform ML training

"ML Training MnS Producer" -> "ML Training MnS Producer": Create and configure MLTrainingProcess MOI and MLTrainingReport MOI

"ML Training MnS Producer" -> "ML Training MnS Consumer": Notify the ML Training Report

@enduml