3GPP TSG-SA5 Meeting #161	S5-252667

Fukuoka, JAPAN 19 - 23 May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.298 | CR | 1042 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add charging information for UE-satellite-UE charging to CHF CDR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3-CH |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Since the Satellite ID list has been specified in the TS32.260, and the IMS charging information here also needs to be updated |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Define the Satellite ID list in the ASN.1 CHF CDR in Forge |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not possible to charge the UE-satellite-UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Forge |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | x |  | O&M Specifications |  |  |  | TS 32.260 ... CR 0441 TS 32.291 ... CR 1035 |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

<CODE BEGINS>

IMSChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) imsChargingDataTypes (4) asn1Module (0) version2 (1)}

DEFINITIONS IMPLICIT TAGS ::=

BEGIN

-- EXPORTS everything

IMPORTS

InvolvedParty,

IPAddress,

LocalSequenceNumber,

ManagementExtensions,

NodeAddress,

MSCAddress,

MSTimeZone,

RecordType,

ServiceContextID,

ServiceSpecificInfo,

Session-Id,

SubscriberEquipmentNumber,

SubscriptionID,

ThreeGPPPSDataOffStatus,

TimeStamp

FROM GenericChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) genericChargingDataTypes (0) asn1Module (0) version2 (1)}

;

--

-- IMS RECORDS

--

IMSRecord ::= CHOICE

--

-- Record values 63-69, 82, 89, ,90, 91 are IMS specific

--

{

sCSCFRecord			[63] SCSCFRecord,

pCSCFRecord			[64] PCSCFRecord,

iCSCFRecord			[65] ICSCFRecord,

mRFCRecord			[66] MRFCRecord,

mGCFRecord			[67] MGCFRecord,

bGCFRecord			[68] BGCFRecord,

aSRecord				[69] ASRecord,

eCSCFRecord			[70] ECSCFRecord,

iBCFRecord			[82] IBCFRecord,

tRFRecord			[89] TRFRecord,

tFRecord				[90] TFRecord,

aTCFRecord			[91] ATCFRecord

}

SCSCFRecord		::= SET

{

recordType							[0] RecordType,

retransmission						[1] NULL OPTIONAL,

sIP-Method							[2] SIP-Method OPTIONAL,

role-of-Node							[3] Role-of-Node OPTIONAL,

nodeAddress							[4] NodeAddress OPTIONAL,

session-Id							[5] Session-Id OPTIONAL,

list-Of-Calling-Party-Address		[6] ListOfInvolvedParties OPTIONAL,

called-Party-Address					[7] InvolvedParty OPTIONAL,

privateUserID						[8] GraphicString OPTIONAL,

serviceRequestTimeStamp				[9] TimeStamp OPTIONAL,

serviceDeliveryStartTimeStamp		[10] TimeStamp OPTIONAL,

serviceDeliveryEndTimeStamp			[11] TimeStamp OPTIONAL,

recordOpeningTime					[12] TimeStamp OPTIONAL,

recordClosureTime					[13] TimeStamp OPTIONAL,

interOperatorIdentifiers				[14] InterOperatorIdentifierList OPTIONAL,

localRecordSequenceNumber			[15] LocalSequenceNumber OPTIONAL,

recordSequenceNumber					[16] INTEGER OPTIONAL,

causeForRecordClosing				[17] CauseForRecordClosing OPTIONAL,

incomplete-CDR-Indication			[18] Incomplete-CDR-Indication OPTIONAL,

iMS-Charging-Identifier				[19] IMS-Charging-Identifier OPTIONAL,

list-Of-SDP-Media-Components			[21] SEQUENCE OF Media-Components-List OPTIONAL,

gGSNaddress							[22] NodeAddress OPTIONAL,

serviceReasonReturnCode				[23] UTF8String OPTIONAL,

list-Of-Message-Bodies				[24] SEQUENCE OF MessageBody OPTIONAL,

recordExtensions						[25] ManagementExtensions OPTIONAL,

expiresInformation					[26] INTEGER OPTIONAL,

list-Of-Associated-URI				[27] ListOfInvolvedParties OPTIONAL,

event								[28] UTF8String OPTIONAL,

accessNetworkInformation				[29] OCTET STRING OPTIONAL,

serviceContextID						[30] ServiceContextID OPTIONAL,

list-of-subscription-ID				[31] SEQUENCE OF SubscriptionID OPTIONAL,

list-Of-Early-SDP-Media-Components	[32] SEQUENCE OF Early-Media-Components-List OPTIONAL,

iMSCommunicationServiceIdentifier	[33] IMSCommunicationServiceIdentifier OPTIONAL,

numberPortabilityRouting				[34] NumberPortabilityRouting OPTIONAL,

carrierSelectRouting					[35] CarrierSelectRouting OPTIONAL,

sessionPriority						[36] SessionPriority OPTIONAL,

serviceRequestTimeStampFraction		[37] Milliseconds OPTIONAL,

serviceDeliveryStartTimeStampFraction	[38] Milliseconds OPTIONAL,

serviceDeliveryEndTimeStampFraction		[39] Milliseconds OPTIONAL,

applicationServersInformation			[40] SEQUENCE OF ApplicationServersInformation OPTIONAL,

requested-Party-Address					[41] InvolvedParty OPTIONAL,

list-Of-Called-Asserted-Identity 		[42] ListOfInvolvedParties OPTIONAL,

online-charging-flag						[43] NULL OPTIONAL,

realTimeTariffInformation				[44] SEQUENCE OF RealTimeTariffInformation OPTIONAL,

userLocationInformation					[47] OCTET STRING OPTIONAL,

mSTimeZone 								[48] MSTimeZone OPTIONAL,

nNI-Information     						[46] NNI-Information OPTIONAL,

fromAddress								[51] OCTET STRING OPTIONAL,

iMSEmergencyIndicator					[52] NULL OPTIONAL,

transit-IOI-Lists						[53] TransitIOILists OPTIONAL,

iMSVisitedNetworkIdentifier				[54] OCTET STRING OPTIONAL,

listOfReasonHeader						[55] ListOfReasonHeader OPTIONAL,

additionalAccessNetworkInformation		[56] OCTET STRING OPTIONAL,

instanceId								[57] OCTET STRING OPTIONAL,

subscriberEquipmentNumber				[58] SubscriberEquipmentNumber OPTIONAL,

routeHeaderReceived						[59] OCTET STRING OPTIONAL,

routeHeaderTransmitted					[60] OCTET STRING OPTIONAL,

list-Of-AccessNetworkInfoChange			[62] SEQUENCE OF AccessNetworkInfoChange OPTIONAL,

listOfCalledIdentityChanges				[63] SEQUENCE OF CalledIdentityChange OPTIONAL,

cellularNetworkInformation				[64] OCTET STRING OPTIONAL,

fEIdentifierList                        [65] FEIdentifierList OPTIONAL

}

PCSCFRecord		::= SET

--

-- This record is also applicable for P-CSCF with collocated ATCF

-- ATCF-specific fields which are not applicable to P-CSCF are indicated.

--

{

recordType								[0] RecordType,

retransmission							[1] NULL OPTIONAL,

sIP-Method								[2] SIP-Method OPTIONAL,

role-of-Node							[3] Role-of-Node OPTIONAL,

nodeAddress								[4] NodeAddress OPTIONAL,

session-Id								[5] Session-Id OPTIONAL,

list-Of-Calling-Party-Address			[6] ListOfInvolvedParties OPTIONAL,

called-Party-Address					[7] InvolvedParty OPTIONAL,

privateUserID							[8] GraphicString OPTIONAL,

serviceRequestTimeStamp					[9] TimeStamp OPTIONAL,

serviceDeliveryStartTimeStamp			[10] TimeStamp OPTIONAL,

serviceDeliveryEndTimeStamp				[11] TimeStamp OPTIONAL,

recordOpeningTime						[12] TimeStamp OPTIONAL,

recordClosureTime						[13] TimeStamp OPTIONAL,

interOperatorIdentifiers				[14] InterOperatorIdentifiers OPTIONAL,

localRecordSequenceNumber				[15] LocalSequenceNumber OPTIONAL,

recordSequenceNumber					[16] INTEGER OPTIONAL,

causeForRecordClosing					[17] CauseForRecordClosing OPTIONAL,

incomplete-CDR-Indication				[18] Incomplete-CDR-Indication OPTIONAL,

iMS-Charging-Identifier					[19] IMS-Charging-Identifier OPTIONAL,

list-Of-SDP-Media-Components			[21] SEQUENCE OF Media-Components-List OPTIONAL,

gGSNaddress								[22] NodeAddress OPTIONAL,

serviceReasonReturnCode					[23] UTF8String OPTIONAL,

list-Of-Message-Bodies					[24] SEQUENCE OF MessageBody OPTIONAL,

recordExtensions						[25] ManagementExtensions OPTIONAL,

expiresInformation						[26] INTEGER OPTIONAL,

list-Of-Associated-URI					[27] ListOfInvolvedParties OPTIONAL,

event									[28] UTF8String OPTIONAL,

accessNetworkInformation				[29] OCTET STRING OPTIONAL,

serviceContextID						[30] ServiceContextID OPTIONAL,

list-of-subscription-ID					[31] SEQUENCE OF SubscriptionID OPTIONAL,

list-Of-Early-SDP-Media-Components 		[32] SEQUENCE OF Early-Media-Components-List OPTIONAL,

iMSCommunicationServiceIdentifier		[33] IMSCommunicationServiceIdentifier OPTIONAL,

sessionPriority							[36] SessionPriority OPTIONAL,

serviceRequestTimeStampFraction			[37] Milliseconds OPTIONAL,

serviceDeliveryStartTimeStampFraction	[38] Milliseconds OPTIONAL,

serviceDeliveryEndTimeStampFraction		[39] Milliseconds OPTIONAL,

list-of-Requested-Party-Address			[41] ListOfInvolvedParties OPTIONAL, -- ATCF only

list-Of-Called-Asserted-Identity 		[42] ListOfInvolvedParties OPTIONAL,

nNI-Information							[46] NNI-Information OPTIONAL,

userLocationInformation					[47] OCTET STRING OPTIONAL,

mSTimeZone 								[48] MSTimeZone OPTIONAL,

servedPartyIPAddress					[50] ServedPartyIPAddress OPTIONAL,

fromAddress								[51] OCTET STRING OPTIONAL,

iMSEmergencyIndicator					[52] NULL OPTIONAL,

transit-IOI-Lists						[53] TransitIOILists OPTIONAL,

iMSVisitedNetworkIdentifier				[54] OCTET STRING OPTIONAL,

listOfReasonHeader						[55] ListOfReasonHeader OPTIONAL,

additionalAccessNetworkInformation		[56] OCTET STRING OPTIONAL,

instanceId								[57] OCTET STRING OPTIONAL,

subscriberEquipmentNumber				[58] SubscriberEquipmentNumber OPTIONAL,

routeHeaderReceived						[59] OCTET STRING OPTIONAL,

routeHeaderTransmitted					[60] OCTET STRING OPTIONAL,

list-Of-AccessNetworkInfoChange			[62] SEQUENCE OF AccessNetworkInfoChange OPTIONAL,

listOfCalledIdentityChanges				[63] SEQUENCE OF CalledIdentityChange OPTIONAL,

cellularNetworkInformation				[64] OCTET STRING OPTIONAL,

initialIMS-Charging-Identifier			[105] IMS-Charging-Identifier OPTIONAL, -- ATCF only

list-Of-AccessTransferInformation		[106] SEQUENCE OF AccessTransferInformation OPTIONAL,

-- ATCF only

relatedICID								[107] IMS-Charging-Identifier OPTIONAL,

relatedICIDGenerationNode				[108] NodeAddress OPTIONAL,

fEIdentifierList                        [109] FEIdentifierList OPTIONAL,

satelliteIDList							[110] SEQUENCE Of SatelliteID OPTIONAL

}

ICSCFRecord		::= SET

{

recordType						[0] RecordType,

retransmission					[1] NULL OPTIONAL,

sIP-Method						[2] SIP-Method OPTIONAL,

role-of-Node						[3] Role-of-Node OPTIONAL,

nodeAddress						[4] NodeAddress OPTIONAL,

session-Id						[5] Session-Id OPTIONAL,

list-Of-Calling-Party-Address	[6] ListOfInvolvedParties OPTIONAL,

called-Party-Address				[7] InvolvedParty OPTIONAL,

serviceRequestTimeStamp			[9] TimeStamp OPTIONAL,

interOperatorIdentifiers			[14] InterOperatorIdentifiers OPTIONAL,

localRecordSequenceNumber		[15] LocalSequenceNumber OPTIONAL,

causeForRecordClosing			[17] CauseForRecordClosing OPTIONAL,

incomplete-CDR-Indication		[18] Incomplete-CDR-Indication OPTIONAL,

iMS-Charging-Identifier			[19] IMS-Charging-Identifier OPTIONAL,

serviceReasonReturnCode			[23] UTF8String OPTIONAL,

recordExtensions					[25] ManagementExtensions OPTIONAL,

expiresInformation				[26] INTEGER OPTIONAL,

list-Of-Associated-URI			[27] ListOfInvolvedParties OPTIONAL,

event							[28] UTF8String OPTIONAL,

accessNetworkInformation			[29] OCTET STRING OPTIONAL,

serviceContextID					[30] ServiceContextID OPTIONAL,

numberPortabilityRouting			[34] NumberPortabilityRouting OPTIONAL,

carrierSelectRouting				[35] CarrierSelectRouting OPTIONAL,

sessionPriority					[36] SessionPriority OPTIONAL,

serviceRequestTimeStampFraction	[37] Milliseconds OPTIONAL,

transit-IOI-List					[45] GraphicString OPTIONAL,

userLocationInformation			[47] OCTET STRING OPTIONAL,

mSTimeZone 						[48] MSTimeZone OPTIONAL,

fromAddress						[51] OCTET STRING OPTIONAL,

iMSEmergencyIndicator			[52] NULL OPTIONAL,

listOfReasonHeader				[55] ListOfReasonHeader OPTIONAL,

additionalAccessNetworkInformation	[56] OCTET STRING OPTIONAL,

routeHeaderTransmitted				[60] OCTET STRING OPTIONAL,

s-CSCF-Information					[61] S-CSCF-Information OPTIONAL,

cellularNetworkInformation			[64] OCTET STRING OPTIONAL,

fEIdentifierList                    [65] FEIdentifierList OPTIONAL

}

MRFCRecord		::= SET

{

recordType							[0] RecordType,

retransmission						[1] NULL OPTIONAL,

sIP-Method							[2] SIP-Method OPTIONAL,

nodeAddress							[4] NodeAddress OPTIONAL,

session-Id							[5] Session-Id OPTIONAL,

list-Of-Calling-Party-Address		[6] ListOfInvolvedParties OPTIONAL,

called-Party-Address					[7] InvolvedParty OPTIONAL,

serviceRequestTimeStamp				[9] TimeStamp OPTIONAL,

serviceDeliveryStartTimeStamp		[10] TimeStamp OPTIONAL,

serviceDeliveryEndTimeStamp			[11] TimeStamp OPTIONAL,

recordOpeningTime					[12] TimeStamp OPTIONAL,

recordClosureTime					[13] TimeStamp OPTIONAL,

interOperatorIdentifiers				[14] InterOperatorIdentifiers OPTIONAL,

localRecordSequenceNumber			[15] LocalSequenceNumber OPTIONAL,

recordSequenceNumber					[16] INTEGER OPTIONAL,

causeForRecordClosing				[17] CauseForRecordClosing OPTIONAL,

incomplete-CDR-Indication			[18] Incomplete-CDR-Indication OPTIONAL,

iMS-Charging-Identifier				[19] IMS-Charging-Identifier OPTIONAL,

list-Of-SDP-Media-Components			[21] SEQUENCE OF Media-Components-List OPTIONAL,

gGSNaddress							[22] NodeAddress OPTIONAL,

serviceReasonReturnCode				[23] UTF8String OPTIONAL,

recordExtensions						[25] ManagementExtensions OPTIONAL,

expiresInformation					[26] INTEGER OPTIONAL,

event								[28] UTF8String OPTIONAL,

accessNetworkInformation				[29] OCTET STRING OPTIONAL,

serviceContextID						[30] ServiceContextID OPTIONAL,

list-of-subscription-ID				[31] SEQUENCE OF SubscriptionID OPTIONAL,

list-Of-Early-SDP-Media-Components	[32] SEQUENCE OF Early-Media-Components-List OPTIONAL,

sessionPriority						[36] SessionPriority OPTIONAL,

serviceRequestTimeStampFraction		[37] Milliseconds OPTIONAL,

serviceDeliveryStartTimeStampFraction	[38] Milliseconds OPTIONAL,

serviceDeliveryEndTimeStampFraction		[39] Milliseconds OPTIONAL,

applicationServersInformation			[40] SEQUENCE OF ApplicationServersInformation OPTIONAL,

online-charging-flag						[43] NULL OPTIONAL,

transit-IOI-List							[45] GraphicString OPTIONAL,

userLocationInformation					[47] OCTET STRING OPTIONAL,

mSTimeZone 								[48] MSTimeZone OPTIONAL,

fromAddress								[51] OCTET STRING OPTIONAL,

listOfReasonHeader						[55] ListOfReasonHeader OPTIONAL,

additionalAccessNetworkInformation		[56] OCTET STRING OPTIONAL,

list-Of-AccessNetworkInfoChange			[62] SEQUENCE OF AccessNetworkInfoChange OPTIONAL,

cellularNetworkInformation				[64] OCTET STRING OPTIONAL,

service-Id								[70] Service-Id OPTIONAL,

requested-Party-Address					[71] InvolvedParty OPTIONAL,

list-Of-Called-Asserted-Identity		[72] ListOfInvolvedParties OPTIONAL,

fEIdentifierList                        [73] FEIdentifierList OPTIONAL

}

MGCFRecord		::= SET

{

recordType							[0] RecordType,

retransmission						[1] NULL OPTIONAL,

sIP-Method							[2] SIP-Method OPTIONAL,

role-of-Node							[3] Role-of-Node OPTIONAL,

nodeAddress							[4] NodeAddress OPTIONAL,

session-Id							[5] Session-Id OPTIONAL,

list-Of-Calling-Party-Address		[6] ListOfInvolvedParties OPTIONAL,

called-Party-Address					[7] InvolvedParty OPTIONAL,

serviceRequestTimeStamp				[9] TimeStamp OPTIONAL,

serviceDeliveryStartTimeStamp		[10] TimeStamp OPTIONAL,

serviceDeliveryEndTimeStamp			[11] TimeStamp OPTIONAL,

recordOpeningTime					[12] TimeStamp OPTIONAL,

recordClosureTime					[13] TimeStamp OPTIONAL,

interOperatorIdentifiers				[14] InterOperatorIdentifiers OPTIONAL,

localRecordSequenceNumber			[15] LocalSequenceNumber OPTIONAL,

recordSequenceNumber					[16] INTEGER OPTIONAL,

causeForRecordClosing				[17] CauseForRecordClosing OPTIONAL,

incomplete-CDR-Indication			[18] Incomplete-CDR-Indication OPTIONAL,

iMS-Charging-Identifier				[19] IMS-Charging-Identifier OPTIONAL,

list-Of-SDP-Media-Components			[21] SEQUENCE OF Media-Components-List OPTIONAL,

serviceReasonReturnCode				[23] UTF8String OPTIONAL,

recordExtensions						[25] ManagementExtensions OPTIONAL,

expiresInformation					[26] INTEGER OPTIONAL,

event								[28] UTF8String OPTIONAL,

accessNetworkInformation				[29] OCTET STRING OPTIONAL,

serviceContextID						[30] ServiceContextID OPTIONAL,

list-Of-Early-SDP-Media-Components	[32] SEQUENCE OF Early-Media-Components-List OPTIONAL,

numberPortabilityRouting				[34] NumberPortabilityRouting OPTIONAL,

carrierSelectRouting					[35] CarrierSelectRouting OPTIONAL,

sessionPriority						[36] SessionPriority OPTIONAL,

serviceRequestTimeStampFraction		[37] Milliseconds OPTIONAL,

serviceDeliveryStartTimeStampFraction	[38] Milliseconds OPTIONAL,

serviceDeliveryEndTimeStampFraction		[39] Milliseconds OPTIONAL,

realTimeTariffInformation				[44] SEQUENCE OF RealTimeTariffInformation OPTIONAL,

transit-IOI-List							[45] GraphicString OPTIONAL,

fromAddress								[51] OCTET STRING OPTIONAL,

listOfReasonHeader						[55] ListOfReasonHeader OPTIONAL,

additionalAccessNetworkInformation		[56] OCTET STRING OPTIONAL,

list-Of-AccessNetworkInfoChange			[62] SEQUENCE OF AccessNetworkInfoChange OPTIONAL,

cellularNetworkInformation				[64] OCTET STRING OPTIONAL,

trunkGroupID								[80] TrunkGroupID OPTIONAL,

bearerService							[81] TransmissionMedium OPTIONAL,

iSUPCause								[82] ISUPCause OPTIONAL,

fEIdentifierList                        [83] FEIdentifierList OPTIONAL

}

BGCFRecord		::= SET

{

recordType							[0] RecordType,

retransmission						[1] NULL OPTIONAL,

sIP-Method							[2] SIP-Method OPTIONAL,

role-of-Node							[3] Role-of-Node OPTIONAL,

nodeAddress							[4] NodeAddress OPTIONAL,

session-Id							[5] Session-Id OPTIONAL,

list-Of-Calling-Party-Address		[6] ListOfInvolvedParties OPTIONAL,

called-Party-Address					[7] InvolvedParty OPTIONAL,

serviceRequestTimeStamp				[9] TimeStamp OPTIONAL,

interOperatorIdentifiers			[14] InterOperatorIdentifiers OPTIONAL,

localRecordSequenceNumber			[15] LocalSequenceNumber OPTIONAL,

causeForRecordClosing				[17] CauseForRecordClosing OPTIONAL,

incomplete-CDR-Indication			[18] Incomplete-CDR-Indication OPTIONAL,

iMS-Charging-Identifier				[19] IMS-Charging-Identifier OPTIONAL,

serviceReasonReturnCode				[23] UTF8String OPTIONAL,

recordExtensions					[25] ManagementExtensions OPTIONAL,

expiresInformation					[26] INTEGER OPTIONAL,

event								[28] UTF8String OPTIONAL,

accessNetworkInformation				[29] OCTET STRING OPTIONAL,

serviceContextID						[30] ServiceContextID OPTIONAL,

numberPortabilityRouting				[34] NumberPortabilityRouting OPTIONAL,

carrierSelectRouting					[35] CarrierSelectRouting OPTIONAL,

sessionPriority						[36] SessionPriority OPTIONAL,

serviceRequestTimeStampFraction		[37] Milliseconds OPTIONAL,

serviceDeliveryStartTimeStampFraction	[38] Milliseconds OPTIONAL,

serviceDeliveryEndTimeStampFraction		[39] Milliseconds OPTIONAL,

transit-IOI-List							[45] GraphicString OPTIONAL,

nNI-Information     						[46] NNI-Information OPTIONAL,

fromAddress								[51] OCTET STRING OPTIONAL,

listOfReasonHeader						[55] ListOfReasonHeader OPTIONAL,

additionalAccessNetworkInformation		[56] OCTET STRING OPTIONAL,

cellularNetworkInformation				[64] OCTET STRING OPTIONAL,

fEIdentifierList                        [65] FEIdentifierList OPTIONAL

}

ASRecord		::= SET

{

recordType							[0] RecordType,

retransmission						[1] NULL OPTIONAL,

sIP-Method							[2] SIP-Method OPTIONAL,

role-of-Node							[3] Role-of-Node OPTIONAL,

nodeAddress							[4] NodeAddress OPTIONAL,

session-Id							[5] Session-Id OPTIONAL,

list-Of-Calling-Party-Address		[6] ListOfInvolvedParties OPTIONAL,

called-Party-Address					[7] InvolvedParty OPTIONAL,

privateUserID						[8] GraphicString OPTIONAL,

serviceRequestTimeStamp				[9] TimeStamp OPTIONAL,

serviceDeliveryStartTimeStamp		[10] TimeStamp OPTIONAL,

serviceDeliveryEndTimeStamp			[11] TimeStamp OPTIONAL,

recordOpeningTime					[12] TimeStamp OPTIONAL,

recordClosureTime					[13] TimeStamp OPTIONAL,

interOperatorIdentifiers				[14] InterOperatorIdentifiers OPTIONAL,

localRecordSequenceNumber			[15] LocalSequenceNumber OPTIONAL,

recordSequenceNumber					[16] INTEGER OPTIONAL,

causeForRecordClosing				[17] CauseForRecordClosing OPTIONAL,

incomplete-CDR-Indication			[18] Incomplete-CDR-Indication OPTIONAL,

iMS-Charging-Identifier				[19] IMS-Charging-Identifier OPTIONAL,

list-Of-SDP-Media-Components			[21] SEQUENCE OF Media-Components-List OPTIONAL,

gGSNaddress							[22] NodeAddress OPTIONAL,

serviceReasonReturnCode				[23] UTF8String OPTIONAL,

list-Of-Message-Bodies				[24] SEQUENCE OF MessageBody OPTIONAL,

recordExtensions						[25] ManagementExtensions OPTIONAL,

expiresInformation					[26] INTEGER OPTIONAL,

event								[28] UTF8String OPTIONAL,

accessNetworkInformation				[29] OCTET STRING OPTIONAL,

serviceContextID						[30] ServiceContextID OPTIONAL,

list-of-subscription-ID				[31] SEQUENCE OF SubscriptionID OPTIONAL,

list-Of-Early-SDP-Media-Components 	[32] SEQUENCE OF Early-Media-Components-List OPTIONAL,

iMSCommunicationServiceIdentifier	[33] IMSCommunicationServiceIdentifier OPTIONAL,

numberPortabilityRouting				[34] NumberPortabilityRouting OPTIONAL,

carrierSelectRouting						[35] CarrierSelectRouting OPTIONAL,

sessionPriority							[36] SessionPriority OPTIONAL,

serviceRequestTimeStampFraction			[37] Milliseconds OPTIONAL,

serviceDeliveryStartTimeStampFraction	[38] Milliseconds OPTIONAL,

serviceDeliveryEndTimeStampFraction		[39] Milliseconds OPTIONAL,

list-of-Requested-Party-Address			[41] ListOfInvolvedParties OPTIONAL,

online-charging-flag						[43] NULL OPTIONAL,

realTimeTariffInformation				[44] SEQUENCE OF RealTimeTariffInformation OPTIONAL,

nNI-Information							[46] NNI-Information OPTIONAL,

userLocationInformation					[47] OCTET STRING OPTIONAL,

mSTimeZone 								[48] MSTimeZone OPTIONAL,

fromAddress								[51] OCTET STRING OPTIONAL,

transit-IOI-Lists						[53] TransitIOILists OPTIONAL,

iMSVisitedNetworkIdentifier				[54] OCTET STRING OPTIONAL,

listOfReasonHeader						[55] ListOfReasonHeader OPTIONAL,

additionalAccessNetworkInformation		[56] OCTET STRING OPTIONAL,

instanceId								[57] OCTET STRING OPTIONAL,

subscriberEquipmentNumber				[58] SubscriberEquipmentNumber OPTIONAL,

list-Of-AccessNetworkInfoChange			[62] SEQUENCE OF AccessNetworkInfoChange OPTIONAL,

listOfCalledIdentityChanges				[63] SEQUENCE OF CalledIdentityChange OPTIONAL,

cellularNetworkInformation				[64] OCTET STRING OPTIONAL,

serviceSpecificInfo						[100] SEQUENCE OF ServiceSpecificInfo OPTIONAL,

requested-Party-Address					[101] InvolvedParty OPTIONAL,

list-Of-Called-Asserted-Identity			[102] ListOfInvolvedParties OPTIONAL,

alternateChargedPartyAddress				[103] UTF8String OPTIONAL,

outgoingSessionId						[104] Session-Id OPTIONAL,

initialIMS-Charging-Identifier			[105] IMS-Charging-Identifier OPTIONAL,

list-Of-AccessTransferInformation		[106] SEQUENCE OF AccessTransferInformation OPTIONAL,

tADS-Identifier							[109] TADIdentifier OPTIONAL,

vlr-Number								[110] MSCAddress OPTIONAL,

msc-Address								[111] MSCAddress OPTIONAL,

threeGPPPSDataOffStatus					[112] ThreeGPPPSDataOffStatus OPTIONAL,

fEIdentifierList                        [113] FEIdentifierList OPTIONAL

}

ECSCFRecord		::= SET

{

recordType							[0] RecordType,

retransmission						[1] NULL OPTIONAL,

sIP-Method							[2] SIP-Method OPTIONAL,

role-of-Node							[3] Role-of-Node OPTIONAL,

nodeAddress							[4] NodeAddress OPTIONAL,

session-Id							[5] Session-Id OPTIONAL,

list-Of-Calling-Party-Address		[6] ListOfInvolvedParties OPTIONAL,

called-Party-Address					[7] InvolvedParty OPTIONAL,

serviceRequestTimeStamp				[9] TimeStamp OPTIONAL,

serviceDeliveryStartTimeStamp		[10] TimeStamp OPTIONAL,

serviceDeliveryEndTimeStamp			[11] TimeStamp OPTIONAL,

recordOpeningTime					[12] TimeStamp OPTIONAL,

recordClosureTime					[13] TimeStamp OPTIONAL,

interOperatorIdentifiers				[14] InterOperatorIdentifierList OPTIONAL,

localRecordSequenceNumber			[15] LocalSequenceNumber OPTIONAL,

recordSequenceNumber					[16] INTEGER OPTIONAL,

causeForRecordClosing				[17] CauseForRecordClosing OPTIONAL,

incomplete-CDR-Indication			[18] Incomplete-CDR-Indication OPTIONAL,

iMS-Charging-Identifier				[19] IMS-Charging-Identifier OPTIONAL,

list-Of-SDP-Media-Components			[21] SEQUENCE OF Media-Components-List OPTIONAL,

gGSNaddress							[22] NodeAddress OPTIONAL,

serviceReasonReturnCode				[23] UTF8String OPTIONAL,

list-Of-Message-Bodies				[24] SEQUENCE OF MessageBody OPTIONAL,

recordExtensions						[25] ManagementExtensions OPTIONAL,

expiresInformation					[26] INTEGER OPTIONAL,

event								[28] UTF8String OPTIONAL,

accessNetworkInformation				[29] OCTET STRING OPTIONAL,

serviceContextID						[30] ServiceContextID OPTIONAL,

list-of-subscription-ID				[31] SEQUENCE OF SubscriptionID OPTIONAL,

list-Of-Early-SDP-Media-Components	[32] SEQUENCE OF Early-Media-Components-List OPTIONAL,

iMSCommunicationServiceIdentifier	[33] IMSCommunicationServiceIdentifier OPTIONAL,

sessionPriority						[36] SessionPriority OPTIONAL,

serviceRequestTimeStampFraction		[37] Milliseconds OPTIONAL,

serviceDeliveryStartTimeStampFraction	[38] Milliseconds OPTIONAL,

serviceDeliveryEndTimeStampFraction		[39] Milliseconds OPTIONAL,

applicationServersInformation			[40] SEQUENCE OF ApplicationServersInformation OPTIONAL,

requested-Party-Address					[41] InvolvedParty OPTIONAL,

list-Of-Called-Asserted-Identity			[42] ListOfInvolvedParties OPTIONAL,

userLocationInformation					[47] OCTET STRING OPTIONAL,

mSTimeZone 								[48] MSTimeZone OPTIONAL,

fromAddress								[51] OCTET STRING OPTIONAL,

transit-IOI-Lists						[53] TransitIOILists OPTIONAL,

listOfReasonHeader						[55] ListOfReasonHeader OPTIONAL,

additionalAccessNetworkInformation		[56] OCTET STRING OPTIONAL,

list-Of-AccessNetworkInfoChange			[62] SEQUENCE OF AccessNetworkInfoChange OPTIONAL,

listOfCalledIdentityChanges				[63] SEQUENCE OF CalledIdentityChange OPTIONAL,

cellularNetworkInformation				[64] OCTET STRING OPTIONAL,

fEIdentifierList                        [65] FEIdentifierList OPTIONAL

}

IBCFRecord		::= SET

--

-- This record is also applicable for IBCF with collocated ATCF

-- ATCF-specific fields which are not applicable to IBCF are indicated.

--

{

recordType								[0] RecordType,

retransmission							[1] NULL OPTIONAL,

sIP-Method								[2] SIP-Method OPTIONAL,

role-of-Node							[3] Role-of-Node OPTIONAL,

nodeAddress								[4] NodeAddress OPTIONAL,

session-Id								[5] Session-Id OPTIONAL,

list-Of-Calling-Party-Address			[6] ListOfInvolvedParties OPTIONAL,

called-Party-Address					[7] InvolvedParty OPTIONAL,

privateUserID							[8] GraphicString OPTIONAL, -- ATCF only

serviceRequestTimeStamp					[9] TimeStamp OPTIONAL,

serviceDeliveryStartTimeStamp			[10] TimeStamp OPTIONAL,

serviceDeliveryEndTimeStamp				[11] TimeStamp OPTIONAL,

recordOpeningTime						[12] TimeStamp OPTIONAL,

recordClosureTime						[13] TimeStamp OPTIONAL,

interOperatorIdentifiers					[14] InterOperatorIdentifiers OPTIONAL,

localRecordSequenceNumber			[	15] LocalSequenceNumber OPTIONAL,

recordSequenceNumber						[16] INTEGER OPTIONAL,

causeForRecordClosing					[17] CauseForRecordClosing OPTIONAL,

incomplete-CDR-Indication				[18] Incomplete-CDR-Indication OPTIONAL,

iMS-Charging-Identifier					[19] IMS-Charging-Identifier OPTIONAL,

list-Of-SDP-Media-Components			[21] SEQUENCE OF Media-Components-List OPTIONAL,

gGSNaddress								[22] NodeAddress OPTIONAL, -- ATCF only

serviceReasonReturnCode					[23] UTF8String OPTIONAL,

list-Of-Message-Bodies					[24] SEQUENCE OF MessageBody OPTIONAL,

recordExtensions						[25] ManagementExtensions OPTIONAL,

expiresInformation						[26] INTEGER OPTIONAL,

event									[28] UTF8String OPTIONAL,

accessNetworkInformation				[29] OCTET STRING OPTIONAL,

serviceContextID						[30] ServiceContextID OPTIONAL,

list-of-subscription-ID					[31] SEQUENCE OF SubscriptionID OPTIONAL, -- ATCF only

list-Of-Early-SDP-Media-Components 		[32] SEQUENCE OF Early-Media-Components-List OPTIONAL,

iMSCommunicationServiceIdentifier		[33] IMSCommunicationServiceIdentifier OPTIONAL,

sessionPriority							[36] SessionPriority OPTIONAL,

serviceRequestTimeStampFraction			[37] Milliseconds OPTIONAL,

serviceDeliveryStartTimeStampFraction	[38] Milliseconds OPTIONAL,

serviceDeliveryEndTimeStampFraction		[39] Milliseconds OPTIONAL,

list-of-Requested-Party-Address			[41] ListOfInvolvedParties OPTIONAL, -- ATCF only

list-Of-Called-Asserted-Identity		[42] ListOfInvolvedParties OPTIONAL, -- ATCF only	realTimeTariffInformation				[44] SEQUENCE OF RealTimeTariffInformation OPTIONAL,

transit-IOI-List						[45] GraphicString OPTIONAL,

nNI-Information							[46] SEQUENCE OF NNI-Information OPTIONAL,

userLocationInformation					[47] OCTET STRING OPTIONAL,

mSTimeZone								[48] MSTimeZone OPTIONAL,

fromAddress								[51] OCTET STRING OPTIONAL,

listOfReasonHeader						[55] ListOfReasonHeader OPTIONAL,

additionalAccessNetworkInformation		[56] OCTET STRING OPTIONAL,

routeHeaderReceived						[59] OCTET STRING OPTIONAL,

list-Of-AccessNetworkInfoChange			[62] SEQUENCE OF AccessNetworkInfoChange OPTIONAL,

cellularNetworkInformation				[64] OCTET STRING OPTIONAL,

initialIMS-Charging-Identifier			[105] IMS-Charging-Identifier OPTIONAL, -- ATCF only

list-Of-AccessTransferInformation		[106] SEQUENCE OF AccessTransferInformation OPTIONAL,

-- ATCF only

fEIdentifierList                        [107] FEIdentifierList OPTIONAL

}

TRFRecord		::= SET

{

recordType							[0] RecordType,

retransmission						[1] NULL OPTIONAL,

sIP-Method							[2] SIP-Method OPTIONAL,

role-of-Node							[3] Role-of-Node OPTIONAL,

nodeAddress							[4] NodeAddress OPTIONAL,

session-Id							[5] Session-Id OPTIONAL,

list-Of-Calling-Party-Address		[6] ListOfInvolvedParties OPTIONAL,

called-Party-Address					[7] InvolvedParty OPTIONAL,

serviceRequestTimeStamp				[9] TimeStamp OPTIONAL,

serviceDeliveryStartTimeStamp		[10] TimeStamp OPTIONAL,

serviceDeliveryEndTimeStamp			[11] TimeStamp OPTIONAL,

recordOpeningTime					[12] TimeStamp OPTIONAL,

recordClosureTime					[13] TimeStamp OPTIONAL,

interOperatorIdentifiers				[14] InterOperatorIdentifierList OPTIONAL,

localRecordSequenceNumber			[15] LocalSequenceNumber OPTIONAL,

recordSequenceNumber					[16] INTEGER OPTIONAL,

causeForRecordClosing				[17] CauseForRecordClosing OPTIONAL,

incomplete-CDR-Indication			[18] Incomplete-CDR-Indication OPTIONAL,

iMS-Charging-Identifier				[19] IMS-Charging-Identifier OPTIONAL,

list-Of-SDP-Media-Components			[21] SEQUENCE OF Media-Components-List OPTIONAL,

serviceReasonReturnCode				[23] UTF8String OPTIONAL,

list-Of-Message-Bodies				[24] SEQUENCE OF MessageBody OPTIONAL,

recordExtensions						[25] ManagementExtensions OPTIONAL,

expiresInformation					[26] INTEGER OPTIONAL,

event								[28] UTF8String OPTIONAL,

serviceContextID						[30] ServiceContextID OPTIONAL,

list-of-subscription-ID				[31] SEQUENCE OF SubscriptionID OPTIONAL,

list-Of-Early-SDP-Media-Components 	[32] SEQUENCE OF Early-Media-Components-List OPTIONAL,

iMSCommunicationServiceIdentifier	[33] IMSCommunicationServiceIdentifier OPTIONAL,

numberPortabilityRouting				[34] NumberPortabilityRouting OPTIONAL,

carrierSelectRouting					[35] CarrierSelectRouting OPTIONAL,

sessionPriority						[36] SessionPriority OPTIONAL,

serviceRequestTimeStampFraction			[37] Milliseconds OPTIONAL,

serviceDeliveryStartTimeStampFraction	[38] Milliseconds OPTIONAL,

serviceDeliveryEndTimeStampFraction		[39] Milliseconds OPTIONAL,

applicationServersInformation			[40] SEQUENCE OF ApplicationServersInformation OPTIONAL,

requested-Party-Address					[41] InvolvedParty OPTIONAL,

list-Of-Called-Asserted-Identity			[42] ListOfInvolvedParties OPTIONAL,

nNI-Information							[46] SEQUENCE OF NNI-Information OPTIONAL,

userLocationInformation					[47] OCTET STRING OPTIONAL,

mSTimeZone 								[48] MSTimeZone OPTIONAL,

transit-IOI-Lists						[53] TransitIOILists OPTIONAL,

listOfReasonHeader						[55] ListOfReasonHeader OPTIONAL,

routeHeaderReceived						[59] OCTET STRING OPTIONAL,

listOfCalledIdentityChanges				[63] SEQUENCE OF CalledIdentityChange OPTIONAL,

fEIdentifierList                        [64] FEIdentifierList OPTIONAL

}

ATCFRecord 	::= SET

{

recordType							[0] RecordType,

retransmission						[1] NULL OPTIONAL,

sIP-Method							[2] SIP-Method OPTIONAL,

role-of-Node							[3] Role-of-Node OPTIONAL,

nodeAddress							[4] NodeAddress OPTIONAL,

session-Id							[5] Session-Id OPTIONAL,

list-Of-Calling-Party-Address		[6] ListOfInvolvedParties OPTIONAL,

called-Party-Address					[7] InvolvedParty OPTIONAL,

privateUserID						[8] GraphicString OPTIONAL,

serviceRequestTimeStamp				[9] TimeStamp OPTIONAL,

serviceDeliveryStartTimeStamp		[10] TimeStamp OPTIONAL,

serviceDeliveryEndTimeStamp			[11] TimeStamp OPTIONAL,

recordOpeningTime					[12] TimeStamp OPTIONAL,

recordClosureTime					[13] TimeStamp OPTIONAL,

interOperatorIdentifiers				[14] InterOperatorIdentifiers OPTIONAL,

localRecordSequenceNumber			[15] LocalSequenceNumber OPTIONAL,

recordSequenceNumber					[16] INTEGER OPTIONAL,

causeForRecordClosing				[17] CauseForRecordClosing OPTIONAL,

incomplete-CDR-Indication			[18] Incomplete-CDR-Indication OPTIONAL,

iMS-Charging-Identifier				[19] IMS-Charging-Identifier OPTIONAL,

list-Of-SDP-Media-Components			[21] SEQUENCE OF Media-Components-List OPTIONAL,

gGSNaddress							[22] NodeAddress OPTIONAL,

serviceReasonReturnCode				[23] UTF8String OPTIONAL,

list-Of-Message-Bodies				[24] SEQUENCE OF MessageBody OPTIONAL,

recordExtensions						[25] ManagementExtensions OPTIONAL,

expiresInformation					[26] INTEGER OPTIONAL,

event								[28] UTF8String OPTIONAL,

accessNetworkInformation				[29] OCTET STRING OPTIONAL,

serviceContextID						[30] ServiceContextID OPTIONAL,

list-of-subscription-ID				[31] SEQUENCE OF SubscriptionID OPTIONAL,

list-Of-Early-SDP-Media-Components	[32] SEQUENCE OF Early-Media-Components-List OPTIONAL,

iMSCommunicationServiceIdentifier	[33] IMSCommunicationServiceIdentifier OPTIONAL,

sessionPriority						[36] SessionPriority OPTIONAL,

serviceRequestTimeStampFraction		[37] Milliseconds OPTIONAL,

serviceDeliveryStartTimeStampFraction	[38] Milliseconds OPTIONAL,

serviceDeliveryEndTimeStampFraction		[39] Milliseconds OPTIONAL,

list-of-Requested-Party-Address			[41] ListOfInvolvedParties OPTIONAL,

list-Of-Called-Asserted-Identity			[42] ListOfInvolvedParties OPTIONAL,

nNI-Information							[46] NNI-Information OPTIONAL,

userLocationInformation					[47] OCTET STRING OPTIONAL,

mSTimeZone								[48] MSTimeZone OPTIONAL,

fromAddress								[51] OCTET STRING OPTIONAL,

listOfReasonHeader						[55] ListOfReasonHeader OPTIONAL,

additionalAccessNetworkInformation		[56] OCTET STRING OPTIONAL,

routeHeaderReceived						[59] OCTET STRING OPTIONAL,

routeHeaderTransmitted					[60] OCTET STRING OPTIONAL,

list-Of-AccessNetworkInfoChange			[62] SEQUENCE OF AccessNetworkInfoChange OPTIONAL,

listOfCalledIdentityChanges				[63] SEQUENCE OF CalledIdentityChange OPTIONAL,

cellularNetworkInformation				[64] OCTET STRING OPTIONAL,

initialIMS-Charging-Identifier			[105] IMS-Charging-Identifier OPTIONAL,

list-Of-AccessTransferInformation		[106] SEQUENCE OF AccessTransferInformation OPTIONAL,

fEIdentifierList                        [107] FEIdentifierList OPTIONAL

}

TFRecord		::= SET

{

recordType							[0] RecordType,

retransmission						[1] NULL OPTIONAL,

sIP-Method							[2] SIP-Method OPTIONAL,

role-of-Node							[3] Role-of-Node OPTIONAL,

nodeAddress							[4] NodeAddress OPTIONAL,

session-Id							[5] Session-Id OPTIONAL,

list-Of-Calling-Party-Address		[6] ListOfInvolvedParties OPTIONAL,

called-Party-Address					[7] InvolvedParty OPTIONAL,

serviceRequestTimeStamp				[9] TimeStamp OPTIONAL,

serviceDeliveryStartTimeStamp		[10] TimeStamp OPTIONAL,

serviceDeliveryEndTimeStamp			[11] TimeStamp OPTIONAL,

recordOpeningTime					[12] TimeStamp OPTIONAL,

recordClosureTime					[13] TimeStamp OPTIONAL,

interOperatorIdentifiers				[14] InterOperatorIdentifierList OPTIONAL,

localRecordSequenceNumber			[15] LocalSequenceNumber OPTIONAL,

recordSequenceNumber					[16] INTEGER OPTIONAL,

causeForRecordClosing				[17] CauseForRecordClosing OPTIONAL,

incomplete-CDR-Indication			[18] Incomplete-CDR-Indication OPTIONAL,

iMS-Charging-Identifier				[19] IMS-Charging-Identifier OPTIONAL,

list-Of-SDP-Media-Components			[21] SEQUENCE OF Media-Components-List OPTIONAL,

serviceReasonReturnCode				[23] UTF8String OPTIONAL,

list-Of-Message-Bodies				[24] SEQUENCE OF MessageBody OPTIONAL,

recordExtensions						[25] ManagementExtensions OPTIONAL,

expiresInformation					[26] INTEGER OPTIONAL,

event								[28] UTF8String OPTIONAL,

serviceContextID						[30] ServiceContextID OPTIONAL,

list-Of-Early-SDP-Media-Components	[32] SEQUENCE OF Early-Media-Components-List OPTIONAL,

iMSCommunicationServiceIdentifier	[33] IMSCommunicationServiceIdentifier OPTIONAL,

numberPortabilityRouting				[34] NumberPortabilityRouting OPTIONAL,

carrierSelectRouting					[35] CarrierSelectRouting OPTIONAL,

sessionPriority						[36] SessionPriority OPTIONAL,

serviceRequestTimeStampFraction		[37] Milliseconds OPTIONAL,

serviceDeliveryStartTimeStampFraction	[38] Milliseconds OPTIONAL,

serviceDeliveryEndTimeStampFraction		[39] Milliseconds OPTIONAL,

applicationServersInformation			[40] SEQUENCE OF ApplicationServersInformation OPTIONAL,

requested-Party-Address					[41] InvolvedParty OPTIONAL,

list-Of-Called-Asserted-Identity 		[42] ListOfInvolvedParties OPTIONAL,

nNI-Information							[46] NNI-Information OPTIONAL,

fromAddress								[51] OCTET STRING OPTIONAL,

transit-IOI-Lists						[53] TransitIOILists OPTIONAL,

listOfReasonHeader						[55] ListOfReasonHeader OPTIONAL,

routeHeaderReceived						[59] OCTET STRING OPTIONAL,

routeHeaderTransmitted					[60] OCTET STRING OPTIONAL,

listOfCalledIdentityChanges				[63] SEQUENCE OF CalledIdentityChange OPTIONAL,

fEIdentifierList                        [64] FEIdentifierList OPTIONAL

}

--

-- IMS DATA TYPES

--

--

-- A

--

AccessCorrelationID		::= CHOICE

--

-- gPRS-Charging-Id is used for GPRS, EPS and 5GS

--

{

gPRS-Charging-Id					[2] INTEGER (0..4294967295),

accessNetworkChargingIdentifier	[4] GraphicString

}

AccessNetworkInfoChange		::= SEQUENCE

{

accessNetworkInformation 			 [0] OCTET STRING OPTIONAL,

additionalAccessNetworkInformation	 [1] OCTET STRING OPTIONAL,

accessChangeTime					 [2] TimeStamp OPTIONAL,

cellularNetworkInformation			 [3] OCTET STRING OPTIONAL

}

AccessTransferType	::= ENUMERATED

{

pSToCS (0),

cSToPS (1) ,

pSToPS (2),

cSToCS (3)

}

AccessTransferInformation		::= SEQUENCE

{

accessTransferType 		 			 [0] AccessTransferType OPTIONAL,

accessNetworkInformation 			 [1] OCTET STRING OPTIONAL,

additionalAccessNetworkInformation	 [2] OCTET STRING OPTIONAL,

inter-UE-Transfer					 [3] NULL OPTIONAL,

relatedICID							 [4] IMS-Charging-Identifier OPTIONAL,

relatedICIDGenerationNode			 [5] NodeAddress OPTIONAL,

accessTransferTime                 	 [6] TimeStamp OPTIONAL,

subscriberEquipmentNumber			 [7] SubscriberEquipmentNumber OPTIONAL,

instanceId							 [8] OCTET STRING OPTIONAL,

cellularNetworkInformation			 [9] OCTET STRING OPTIONAL

}

ACRInterimLost		::= ENUMERATED

{

no		(0),

yes		(1),

unknown	(2)

}

AoCCostInformation		::= SEQUENCE

{

accumulatedCost		[0] REAL,

incrementalCost		[1] REAL,

currencyCode			[2] INTEGER

}

AoCInformation ::= SET

{

tariffInformation		[0] TariffInformation OPTIONAL,

aoCCostInformation		[1] AoCCostInformation OPTIONAL

}

ApplicationServersInformation	::= SEQUENCE

{

applicationServersInvolved 			[0] NodeAddress OPTIONAL,

applicationProvidedCalledParties		[1] SEQUENCE OF InvolvedParty OPTIONAL,

sTatus								[2] Status OPTIONAL

}

--

-- C

--

CalledIdentityChange	::= SEQUENCE

{

calledIdentity	[0]	InvolvedParty OPTIONAL,

changeTime		[1] TimeStamp OPTIONAL

}

CarrierSelectRouting	::= GraphicString

CauseForRecordClosing	::= ENUMERATED

{

serviceDeliveryEndSuccessfully	(0),

unSuccessfulServiceDelivery		(1),

timeLimit						(3),

serviceChange					(4), -- e.g. change in media due to Re-Invite,

-- Access Transfer

managementIntervention			(5)  -- partial record generation reasons to be added

--  Additional codes are for further study

}

--

-- E

--

Early-Media-Components-List	::= SEQUENCE

{

sDP-Offer-Timestamp 		[0] TimeStamp OPTIONAL,

sDP-Answer-Timestamp		[1] TimeStamp OPTIONAL,

sDP-Media-Components		[2] SEQUENCE OF SDP-Media-Component OPTIONAL,

mediaInitiatorFlag		[3] NULL OPTIONAL,

sDP-Session-Description	[4] SEQUENCE OF GraphicString OPTIONAL,

sDP-Type					[5] SDP-Type OPTIONAL

}

--

-- F

--

FEIdentifierList ::= SEQUENCE OF GraphicString

--

-- I

--

IMS-Charging-Identifier	::= OCTET STRING

IMSCommunicationServiceIdentifier	::= OCTET STRING

Incomplete-CDR-Indication	::= SET

{

aCRStartLost	[0] BOOLEAN,		-- TRUE if ACR[Start] was lost, FALSE otherwise

aCRInterimLost	[1] ACRInterimLost,

aCRStopLost		[2] BOOLEAN			-- TRUE if ACR[Stop] was lost, FALSE otherwise

}

InterOperatorIdentifierList ::= SEQUENCE OF InterOperatorIdentifiers

InterOperatorIdentifiers ::= SEQUENCE

{

originatingIOI	[0] GraphicString OPTIONAL,

terminatingIOI	[1] GraphicString OPTIONAL

}

ISUPCause	::= SEQUENCE

{

iSUPCauseLocation 		[0] INTEGER OPTIONAL,

iSUPCauseValue			[1] INTEGER OPTIONAL,

iSUPCauseDiagnostics		[2] OCTET STRING OPTIONAL,

enhancedDiagnostics		[3] OCTET STRING OPTIONAL

}

--

-- L

--

ListOfInvolvedParties	::= SEQUENCE OF InvolvedParty

ListOfReasonHeader	::= SEQUENCE OF ReasonHeaderInformation

--

-- M

--

Media-Components-List	::= SEQUENCE

--

--	MediaInitiatorParty is used to identify the initiator of the media

--	multi-participants session e.g. in AS PoC Server

--

{

sIP-Request-Timestamp 			[0] TimeStamp OPTIONAL,

sIP-Response-Timestamp			[1] TimeStamp OPTIONAL,

sDP-Media-Components			[2] SEQUENCE OF SDP-Media-Component OPTIONAL,

mediaInitiatorFlag				[3] NULL OPTIONAL,

sDP-Session-Description			[4] SEQUENCE OF GraphicString OPTIONAL,

mediaInitiatorParty				[5] InvolvedParty OPTIONAL,

sIP-Request-Timestamp-Fraction	[6] Milliseconds OPTIONAL,

sIP-Response-Timestamp-Fraction	[7] Milliseconds OPTIONAL,

sDP-Type						[8] SDP-Type OPTIONAL

}

MessageBody	::= SEQUENCE

{

content-Type				[0] GraphicString,

content-Disposition		[1] GraphicString OPTIONAL,

content-Length			[2] INTEGER,

originator				[3] InvolvedParty OPTIONAL

}

Milliseconds	::= INTEGER (0..999)

--

-- N

--

NNI-Information		::= SEQUENCE

{

sessionDirection		[0] SessionDirection OPTIONAL,

nNIType				[1] NNIType OPTIONAL,

relationshipMode		[2] RelationshipMode OPTIONAL,

neighbourNodeAddress	[3] IPAddress OPTIONAL

}

NNIType		::= ENUMERATED

{

non-roaming					(0),

roaming-without-loopback		(1),

roaming-with-loopback		(2)

}

NumberPortabilityRouting		::= GraphicString

--

-- R

--

RateElement		::= SEQUENCE

{

unitType				[0] INTEGER,

unitValue			[1] REAL,

unitCost				[2] REAL,

unitQuotaThreshold	[3] REAL

}

RealTimeTariffInformation ::= CHOICE

{

tariffInformation		[0] TariffInformation,

tariffXml				[1] UTF8String

}

ReasonHeaderInformation ::= GraphicString

RelationshipMode ::= ENUMERATED

{

trusted		(0),

non-trusted	(1)

}

Role-of-Node	::= ENUMERATED

{

originating	(0),

terminating	(1)

}

--

-- S

--

S-CSCF-Information	::= SEQUENCE

{

mandatoryCapabilities 	[0] SEQUENCE OF GraphicString OPTIONAL,

optionalCapabilities	[1] SEQUENCE OF GraphicString OPTIONAL,

serverName 				[2] GraphicString OPTIONAL

}

SDP-Media-Component ::=  SEQUENCE

{

sDP-Media-Name        			[0] GraphicString OPTIONAL,

sDP-Media-Descriptions			[1] SDP-Media-Description OPTIONAL,

accessCorrelationID					AccessCorrelationID OPTIONAL,	-- not used in MGCF

-- [2] is used by gPRS-Charging-Id

-- [4] is used by accessNetworkChargingIdentifier

localGWInsertedIndication		[5] BOOLEAN OPTIONAL,

iPRealmDefaultIndication		[6] BOOLEAN OPTIONAL,

transcoderInsertedIndication	[7] BOOLEAN OPTIONAL

}

SDP-Media-Description	::= SEQUENCE OF GraphicString

ServedPartyIPAddress	::=  IPAddress

Service-Id	::= GraphicString

SessionDirection	::= ENUMERATED

{

inbound		(0),

outbound		(1)

}

SessionPriority	::= ENUMERATED

--

-- PRIORITY-0 is the highest priority and Priority-4 is the lowest priority. See TS 29.229[232]

--

{

pRIORITY-0 (0),

pRIORITY-1 (1),

pRIORITY-2 (2),

pRIORITY-3 (3),

pRIORITY-4 (4)

}

SIP-Method	::= GraphicString

SDP-Type	::= ENUMERATED

{

sDP-offer	(0),

sDP-answer	(1)

}

Status		::= ENUMERATED

{

fourxx			(0),

fivexx			(1),

time-out			(2)

}

SatelliteID 	::= UTF8String OPTIONAL

--

-- T

--

TADIdentifier		::= ENUMERATED

{

cS		(0),

pS		(1)

}

TariffInformation ::= SEQUENCE

{

currencyCode			[0] INTEGER,

scaleFactor			[1] REAL,

rateElements			[2] SEQUENCE OF RateElement OPTIONAL

}

TransitIOILists ::= SEQUENCE OF GraphicString

TransmissionMedium ::= SEQUENCE

{

tMR	[0] OCTET STRING (SIZE (1)) OPTIONAL, -- required TM, refer to Q.763

tMU	[1] OCTET STRING (SIZE (1)) OPTIONAL  -- used TM, refer to Q.763

}

TrunkGroupID ::= CHOICE

{

incoming	[0] GraphicString,

outgoing	[1] GraphicString

}

END

<CODE ENDS>

End of Change