3GPP TSG-SA5 Meeting #161 	S5-252702

Fukuoka City, Fukuoka, Japan, 19th – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0699 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.552 DRB measurement with filter |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph4 |  |  |  |  |  | Date: |  |  | 2025-05-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | SNSSAI filter is available for most of the DRB related measurements. However, there are a few DRB measurements which does not have SNSSAI filter defined. This is inconsistent definition in SA5 specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding SNSSAI filter for DRB.PDCP.UEThpDl_Filter Adding SNSSAI filter for DRB.PdcpSduVolumeX2DL_Filter Adding SNSSAI filter for DRB.PdcpSduVolumeX2UL_Filter Wording correction on DRB.PdcpSduVolumeXnDL_Filter |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing SNSSAI filter capability for some PM |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.3.7, 5.1.2.1.1.2, 5.1.2.1.1.3, 5.1.2.1.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** START OF NEXT CHANGE ***

##### 5.1.1.3.7	Average DL UE buffered Throughput per DRB

a)	This measurement provides the average down link buffered UE throughput per DRB on NRCellCU. The DRBs are mapped with the same 5QI for NR SA or mapped with the same QCI for EN-DC. This measurement is intended for throughput per UE and bearer independent of traffic patterns and packet size. The measurement is based on Desired buffer size communicated within DDDS from DU to CU UP and is intended for services with burst duration spanning over the time interval of a couple of consequent DDDSs. For very bursty traffic with burst duration within the interval of one DDDS the measured values can be unprecise and cannot be compared across vendors. Initial buffering time in CU and on F1, meant as time interval the first PDCP SDU of the new burst is received in CU until this first part is received in DU, is excluded. The monitoring is supported also in DC scenario and in NSA option3a and 3x.

b)	DER(N=1);

c)	This measurement is obtained by the following formula for a measurement period:

ThroughputVolumeThroughputTime [kbits/s]

where each ThroughputVolume and ThroughputTime is intended to represent one DL burst as explained in the Fig. 5.1.1.3.7-1, Fig. 5.1.1.3.7-3, Fig. 5.1.1.3.7-5 and Table 5.1.1.3.7-2, Table 5.1.1.3.7-4, Table 5.1.1.3.7-6 for DRB (SA, NSA option 3a), split DRB (DC), split DRB (NSA option 3x), respectively. Separate counters are maintained for each mapped 5QI (or QCI for EN-DC).

Figure 5.1.1.3.7-1 Average DL buffered UE throughput per DRB (SA, NSA option 3a)

Table 5.1.1.3.7-2 DRB (SA, NSA option 3a)

| T0’ | First PDCP SDU of the new burst arrived to CU and there are not any other PDCP SDUs in CU UP waiting for transmission to DU nor the ones mapped to PDCP PDUs and sent to DU are in the DU buffer. |
| --- | --- |
| T1’ | First PDCP PDU has been received in DU after T0’ (can be obtained as point in time when PDCP PDU sent from CU to DU plus F1 delay). |
| T2’ | The buffer in DU gets empty after T0’. |
| ThroughputTime | T2’ – T1’ [ms] See NOTE 1. The Achievable DRB throughput is obtained as the “Desired buffer size for data radio bearer” as part of last DDDS feedback [TS 38.425[56]] divided with the DDDS reporting period time interval. In case the desired buffer size is 0 and an PDCP PDU sent to DU it is considered it will be spent the whole time interval in the buffer of DU until desired buffer size >0 is reported in the next DDDS feedback. See NOTE 2. |
| ThroughptVolume | The PDCP SDU volume in bits successfully transmitted (acknowledged by DDDS) in DL to UE per bearer and one burst (consisting of PDCP SDU 1, 2 and 3 in example in Fig. 5.1.1.3.7-1). |
| NOTE 1: Contribution of the given PDCP SDUi to ThroughputTime, i.e. the time period the PDCP PDUi related to the PDCP SDUi will spend in the buffer of DU enitity, is obtained as volume of the PDCP PDUi divided with Achievable DRB throughput of the UE.In case in the point of time the PDCP PDUi is sent to DU while the previous one still kept in the DU buffer the time period these two PDCP SDUs will spend in DU buffer is cumulated, i.e. obtained as sum of volume of the PDCP PDUi and PDCP PDUi-1 divided with Achievable DRB throughput. NOTE 2: The precision of the measured ThroughputTime may be impacted with the precision of the measured F1 delay in case of not time synchronized CU and DU when it is obtained as F1 RTT/2. |  |

Figure 5.1.1.3.7-3: Average DL buffered UE throughput per split DRB (DC)

Table 5.1.1.3.7-4 Split DRB (DC)

| T0 | First PDCP SDU of the new burst arrived to CU and there are not any other PDCP SDUs in CU UP waiting for transmission to DU nor the ones mapped to PDCP PDUs and sent to DU are in the DU buffer for any of the legs relevant to the bearer. |
| --- | --- |
| T1 | First PDCP PDU has been received in DU1 of the first leg after T0 (can be obtained as point in time when PDCP PDU sent from CU to DU1 plus F1 delay). |
| T2 | First PDCP PDU has been received in DU2 of the second leg after T0 (can be obtained as point in time when PDCP PDU sent from CU to DU2 plus F1 delay). |
| T3 | The buffer in DU1 of the first leg gets empty after T0. |
| T4 | The buffer in DU2 of the second leg gets empty after T0 |
| ThroughputTime | T4 – T1 [ms] See NOTE 1. The Achievable DRB throughput is obtained as the “Desired buffer size for data radio bearer” as part of last DDDS feedback [TS 38.425 [56]] divided with the DDDS reporting period time interval. In case the desired buffer size is 0 and an PDCP PDU sent to DU it is considered it will be spent the whole time interval in the buffer of DU until desired buffer size >0 is reported in the next DDDS feedback. See NOTE 2. |
| ThroughputVolume | The PDCP SDU volume in bits successfully transmitted (acknowledged by DDDS) in DL to UE per bearer and one burst (consisting of PDCP SDU 1, 2, 3 and 4 in example in Fig. 5.1.1.3.7-3). |
| NOTE 1: Contribution of the given PDCP SDUi to ThroughputTime, i.e. the time period the PDCP PDUi related to the PDCP SDUi will spend in the buffer of DU related leg is obtained as volume of the PDCP PDUi divided with Achievable DRB throughput. In case in the point of time the PDCP PDUi is sent to DU while the previous one still kept in the DU buffer of the same leg the time period these two PDCP SDUs will spend in DU buffer is cumulated, i.e. obtained as sum of volume of the PDCP PDUi and PDCP PDUi-1 divided with Achievable DRB throughput. NOTE 2: The precision of the measured ThroughputTime may be impacted with the precision of the measured F1 delay in case of not time synchronized CU and DU when it is obtained as F1 RTT/2. |  |

Figure 5.1.1.3.7-5: Average DL buffered UE throughput per split DRB (NSA option 3x)

Table 5.1.1.3.7-6 Split DRB (NSA option 3x)

| T0 | First PDCP SDU of the new burst arrived to CU and there are not any other PDCP SDUs in CU UP waiting for transmission to DU/MeNB nor the ones mapped to PDCP PDUs and sent to DU/MeNB are in the DU/MeNB buffer for any of the legs relevant to the bearer. |
| --- | --- |
| T1 | First PDCP PDU has been received in DU of the first leg after T0 (can be obtained as point in time when PDCP PDU sent from CU to DU plus F1 delay). |
| T2 | First PDCP PDU has been received in MeNB of the second leg after T0 (can be obtained as point in time when PDCP PDU sent from CU to MeNB plus X2 delay). |
| T5 | The buffer in DU of the first leg gets empty after T0. |
| T6 | The buffer in MeNB of the second leg gets empty after T0 |
| ThroughputTime | T6 – T1 [ms] The Achievable DRB throughput is obtained as the “Desired buffer size for data radio bearer” as part of last DDDS feedback [TS 38.425[56]] divided with the DDDS reporting period time interval. In case the desired buffer size is 0 and an PDCP PDU sent to DU it is considered it will be spent the whole time interval in the buffer of DU until desired buffer size >0 is reported in the next DDDS feedback. In case of split DRB NSA option 3x to calculate the Achievable DRB throughput for LTE leg the “Desired buffer size for E-RAB” as part of last DDDS feedback [TS 36.425 [57]] reported from MeNB to CU UP via X2 interface is considered. Note; The precision of the measured ThroughputTime may be impacted with the precision of the measured F1/X2 delay in case of not time synchronized CU and DU when it is obtained as F1/X2 RTT/2. |
| ThroughputVolume | The PDCP SDU volume in bits successfully transmitted (acknowledged by DDDS) in DL to UE per bearer and one burst (consisting of PDCP SDU 1, 2, 3 and 4 in example in Fig. 5.1.1.3.7-5). |
| NOTE 1: Contribution of the given PDCP SDUi to ThroughputTime, i.e. the time period the PDCP PDUi related to the PDCP SDUi will spend in the buffer of DU related leg is obtained as volume of the PDCP PDUi divided with Achievable DRB throughput. In case in the point of time the PDCP PDUi is sent to DU while the previous one still kept in the DU buffer of the same leg the time period these two PDCP SDUs will spend in DU buffer is cumulated, i.e. obtained as sum of volume of the PDCP PDUi and PDCP PDUi-1 divided with Achievable DRB throughput. |  |

d)	Each measurement is a real value representing the throughput in kbit per second. The number of measurements is equal to one. If the optional QoS level subcounter is performed, the number of measurements is equal to the number of mapped 5QI and QCIs for EN-DC.

e)	The measurement name has the form 
DRB.PDCP.UEThpDl, 
DRB.PDCP.UEThpDl_QOS, where QOS represents the mapped 5QI or QCI level, or
DRB.PDCP.UEThpDl_SNSSAI, where SNSSAI represents the S-NSSAI.

f)	GNBCUUPFunction.

NRCellCU.

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality).

*** START OF NEXT CHANGE ***

###### 5.1.2.1.1.2	DL Cell PDCP SDU Data Volume on X2 Interface

a)	This measurement provides the Data Volume (amount of PDCP SDU bits) in the downlink delivered on X2 interface in DC-scenarios. The measurement is calculated per PLMN ID, per S-NSSAI, and per QoS level (mapped 5QI or QCI in EN-DC).
The unit is Mbit.

b)	CC.

c)	This measurement is obtained by counting the number of bits transferred in the downlink through X2 interface. The measurement is performed at the PDCP SDU level. The measurement is performed per configured PLMN ID, per S-NSSAI, and per QoS level (mapped 5QI or QCI in EN-DC).

d)	Each measurement is an integer value representing the number of bits measured in Mbits (1MBits=1000*1000 bits). The number of measurements is equal to the number of PLMNs multiplied by the number of QoS levels multiplied by the number of SNSSAI.
[Total no. of measurement instances] x [no. of filter values for all measurements] (DL and UL) ≤ 100.

e)	The measurement name has the form DRB.PdcpSduVolumeX2DL_Filter.

Where Filteris a combination of PLMN, SNSSAI, and QoS.

Where PLMN represents the PLMN ID, QoS representes the mapped 5QI or the QCI level, SNSSAI represents the S-NSSAI.

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality) and in the energy efficency (EE) area.

NRCellCU in non-split NG-RAN deployment scenarios represents NRCell.

*** START OF NEXT CHANGE ***

###### 5.1.2.1.1.3	DL Cell PDCP SDU Data Volume on Xn Interface

a)	This measurement provides the Data Volume (amount of PDCP SDU bits) in the downlink delivered on Xn interface . The measurement is calculated per PLMN ID and per QoS level (mapped 5QI) and per S-NSSAI. 
The unit is Mbit.

b)	CC.

c)	This measurement is obtained by counting the number of bits transferred in the downlink through Xn interface. The measurement is performed at the PDCP SDU level. The measurement is performed per configured PLMN ID and per QoS level (mapped 5QI) and per S-NSSAI.

d)	Each measurement is an integer value representing the number of bits measured in Mbits (1MBits=1000*1000 bits). The number of measurements is equal to the number of PLMNs multiplied by the number of QoS levels multiplied by the number of S-NSSAIs.
[Total no. of measurement instances] x [no. of filter values for all measurements] (DL and UL) ≤ 100.

e)	The measurement name has the form DRB.PdcpSduVolumeXnDL_Filter. 
Where Filter is a combination of PLMN and QoS and SNSSAI.

Where PLMN represents the PLMN ID, QoS representes the mapped 5QI or the QCI level, and SNSSAI represents S-NSSAI.

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality) and in the energy efficency (EE) area.

NRCellCU in non-split NG-RAN deployment scenarios represents NRCell.

*** START OF NEXT CHANGE ***

###### 5.1.2.1.2.2	UL Cell PDCP SDU Data Volume on X2 Interface

a)	This measurement provides the Data Volume (amount of PDCP SDU bits) in the uplink delivered on X2 interface in NSA scenarios. The measurement is calculated per PLMN ID, per S-NSSAI, and per QoS level (mapped 5QI or QCI in EN-DC).
The unit is Mbit.

b)	CC

c)	This measurement is obtained by counting the number of bits transferred in the uplink through X2 interface. The measurement is performed at the PDCP SDU level. The measurement is performed per configured PLMN ID,  per S-NSSAI, and per QoS level (mapped 5QI or QCI in EN-DC).

d)	Each measurement is an integer value representing the number of bits measured in Mbits (1MBits=1000*1000 bits). The number of measurements is equal to the number of PLMNs multiplied by the number of QoS levels multiplied by the number of SNSSAI.
[Total no. of measurement instances] x [no. of filter values for all measurements] (DL and UL) ≤ 100.

e)	The measurement name has the form DRB.PdcpSduVolumeX2UL_Filter.

Where Filter is a combination of PLMN, SNSSAI, and QoS.

Where PLMN represents the PLMN ID, QoS representes the mapped 5QI or the QCI level, SNSSAI represents the S-NSSAI.

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality) and in the energy efficency (EE) area.

NRCellCU in non-split NG-RAN deployment scenarios represents NRCell.

*** END OF CHANGE ***