3GPP TSG-SA5 Meeting #162	S5-253322

Gothenburg, Sweeden,  25 - 29 August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.533 | CR | 0165 | Rev |  | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.533 addition of architecture reference model for management and orchestration in support of SBMA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NEC, Intel, Huawei, AT&T, ZTE, CATT, MATRIXX Software, Deutsche Telekom, Verizon, China Unicom, China Mobile, Orange |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | SBMA_Ph3 |  |  |  |  |  | Date: |  |  | 2025-08-11 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The current specification does not explicitly define logical Management Functions (MnFs) as producers or consumers of Management Services (MnSs). This lack of clarity creates architectural gaps that lead to ambiguity, inconsistent interpretation across 3GPP Working Groups, and increasing functional overlap, especially as advanced features such as AI/ML, data analytics, and automation expand in 5G Advanced and future 6G systems. The essential, streamlined reference architecture provides the minimum structure needed to ensure architectural traceability and cross-WG alignment while preserving flexibility. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This change introduces a complementary management framework that strengthens the Service-Based Management Architecture (SBMA) with an essential, streamlined reference architecture (Rel-19 baseline). It explicitly defines logical MnFs, clarifies their roles in relation to MnSs/capabilities, and establishes clear traceability between MnFs and their supported capabilities and MnSs they produce or consume. This Rel-19 baseline reference architecture model enables a consistent, modular foundation that can evolve to support increasing intelligence, autonomy, and cross-domain orchestration in Rel-20 and beyond. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Without this baseline, the architectural role of MnFs will remain unclear, and the SBMA concept as defined in SA5 will continue to be poorly understood by other Working Groups. This lack of clarity risks continuing ambiguity and fragmentation across 3GPP, leading to overlapping or inconsistent functions and hindering effective coordination. Consequently, SA5’s ability to provide clear guidance for end-to-end management and orchestration standardisation for 5G Advanced and the transition towards 6G would be significantly weakened. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New clauses 5.1a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | For further background including rational for the change request please check the discussion paper in S5-253320, and a supporting CR in S5-253323. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

# 5	Architecture reference model

## 5.1a	Architecture reference model for management and orchestration

The architecture reference model for management and orchestration in Figure 5.1a-x defines a set of Management Functions (MnFs). Each MnF produces zero, one or more Management Services (MnSs) and/or consumes zero, one or more MnSs (see clause 4.5).

Interaction, including the production and consumption of management services, is exclusively performed via the management service interface which represents the standardised interface that interconnects MnFs for the production and consumption of MnSs, enabling consistent, service-based interactions in an implementation-agnostic manner.

MnFs are logical, implementation-agnostic entities, defined independently of any specific deployment scenario. They are characterised by their overall management responsibilities and functional scope (see clause 5.1b). Each MnF is defined by the set of MnSs it produces and/or consumes, and, where applicable, by the functional management capabilities it logically encompasses to support those MnSs, as specified in the corresponding reference specifications.

Figure 5.1a-x: Architecture reference model for management and orchestration.

Table 5.1a-y defines each MnF in the architecture reference model for management and orchestration along with the corresponding reference specifications that define their functionality.

Table 5.1a-y: MnFs definitions and corresponding reference specifications

| MnF | Description | Functionality | Reference specification |
| --- | --- | --- | --- |
| MDAF | Management Data Analytics Function | Management data analytics | TS 28.104 [57] |
| MLTRF | ML Training Function | ML model training | TS 28.105 [66] |
| MLTEF | ML Testing Function | ML model testing | TS 28.105 [66] |
| MLEMF | ML Emulation Function | AI/ML emulation | TS 28.105 [66] |
| IHF | Intent Handling Function | Intent driven management | TS 28.312 [46] |
| UDHF | UE Data Handling Function | Collection and handling of Trace data, MDT measurements, QoE data, and UE level measurements | TS 32.422 [44], 28.405 [49], TS 28.558 [x1] |
| PMF | Performance Management Function | Collection of performance measurements | TS 28.550 [42], TS 28.552 [5] |
| FMF | Fault Management Function | Fault management | TS 28.530 [3], 28.111 [68] |
| DMF | Data Management Function | Data management | TS 28.537 [39], TS 28.622 [32], TS 28.623 [54]). |
| PRF | Provisioning Function | Provisioning | TS 28.531 [8] |
| NDTMF | Network Digital Twin Management Function | Management of network digital twin | TS 28.561 [x2] |
| CCLMF | Close Control Loop Management Function | Management of close control loop | TS 28.567 [x3] |

NOTE 1:	For details on the 5G System reference architecture including entities/functions and interfaces, see TS 23.501, clause 4.2 [31].

NOTE 2:  The management interface represents the logical abstraction for the MnF-to-NF interactions.

NOTE 3:	The reference architecture does not assume or restrict the physical locality of Management Functions (MnFs). MnFs can be deployed centrally, distributed, or co-located with Network Functions (NFs) depending on operator’s deployment choices (see clause 4.5).

End of changes