3GPP TSG-SA5 Meeting #162	S5-253346

# Stor-Göteborg, Sweden, 25th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.111 | CR | 0053 | rev | - | Current version: | 18.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR 28.111 Probable Cause codes |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson Hungary Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSBMA |  |  |  |  |  | Date: |  |  | 2025-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Probable Cause (PC) value “Intrusion Detection” was defined by two different stadards: M.3100:126 - X.736:710 with different meanings. Already today the definition of probableCause states: “The producer should choose the most specific probableCause applicable.” This is ambigious, difficult if multiple definitions exist for the same string. It is proposed to changed the string value (for 710) to make the values unique. Probable Cause (PC) values from Annex B are not captured in stage 3. Customers who use the string representation of probableCause complained that comparing probableCause strings supplied by different vendors don’t match. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - PC “Intrusion Detection” (710) changed to “Intrusion Detected” (710) PC values added as enumeration to stage 3, precisely defining the probableCause string values. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Duplicate PC value leads to confusion as these have separate semantics (environmental vs. physical violation); no defined stage 3 values for PC values defined at stage 2 leading to interoperability issues. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex B, Forge |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | YANG Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1844 at commit a149770375b54fbe2d40c32ee2d893e1ffe5c8de YAML Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1843 at commit ce60df8fb1a2313f3265964fb28aaae257ef717c |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

Annex B (informative):
Probable Causes

This annex lists probable causes.

Sources of these probable causes are ITU-T Recommendation M.3100 [7], ITU-T Recommendation X.733 [8], and ITU-T Recommendation X.736 [13]. In addition, probable causes for wireless systems are listed in ETSI TS 101 251 V6.3.0 (1999-07) [3].

The listed probable cause strings (or alternatively integers) should be used. If none of them represents the real probable cause appropriately, probable cause strings (or alternatively integers) not listed below may be used as well.

Table B.1: Probable Causes from ITU-T Recommendation M.3100 [7]

| M.3100 Probable cause (string) | (integer) | alarmType |
| --- | --- | --- |
| Indeterminate | 0 | Other |
| Alarm Indication Signal (AIS) | 1 | Communications |
| Call Setup Failure | 2 | Communications |
| Degraded Signal | 3 | Communications |
| Far End Receiver Failure (FERF) | 4 | Communications |
| Framing Error | 5 | Communications |
| Loss Of Frame (LOF) | 6 | Communications |
| Loss Of Pointer (LOP) | 7 | Communications |
| Loss Of Signal (LOS) | 8 | Communications |
| Payload Type Mismatch | 9 | Communications |
| Reserved | 10 |  |
| Remote Alarm Interface | 11 | Communications |
| Excessive Bit Error Rate (EBER) | 12 | Communications |
| Path Trace Mismatch | 13 | Communications |
| Unavailable | 14 | Communications |
| Signal Label Mismatch | 15 | Communications |
| Loss Of Multi Frame | 16 | Communications |
| Communications Receive Failure | 17 | Communications |
| Communications Transmit Failure | 18 | Communications |
| Modulation Failure | 19 | Communications |
| Demodulation Failure | 20 | Communications |
| Reserved | 21-26 |  |
| Reserved for M.3100 potential future extensions. | 27-50 |  |
| Back Plane Failure | 51 | Equipment |
| Data Set Problem | 52 | Equipment |
| Equipment Identifier Duplication | 53 | Equipment |
| External IF Device Problem | 54 | Equipment |
| Line Card Problem | 55 | Equipment |
| Multiplexer Problem | 56 | Equipment |
| NE Identifier Duplication | 57 | Equipment |
| Power Problem | 58 | Equipment |
| Processor Problem | 59 | Equipment |
| Protection Path Failure | 60 | Equipment |
| Receiver Failure | 61 | Equipment |
| Replaceable Unit Missing | 62 | Equipment |
| Replaceable Unit Type Mismatch | 63 | Equipment |
| Synchronization Source Mismatch | 64 | Equipment |
| Terminal Problem | 65 | Equipment |
| Timing Problem | 66 | Equipment |
| Transmitter Failure | 67 | Equipment |
| Trunk Card Problem | 68 | Equipment |
| Replaceable Unit Problem | 69 | Equipment |
| Real Time Clock Failure | 70 | Equipment |
| Reserved | 71-80 |  |
| Protection Mechanism Failure | 81 | Equipment |
| Protecting Resource Failure | 82 | Equipment |
| Reserved for M.3100 potential future extensions. | 83-100 |  |
| Air Compressor Failure | 101 | Environmental |
| Air Conditioning Failure | 102 | Environmental |
| Air Dryer Failure | 103 | Environmental |
| Battery Discharging | 104 | Environmental |
| Battery Failure | 105 | Environmental |
| Commercial Power Failure | 106 | Environmental |
| Cooling Fan Failure | 107 | Environmental |
| Engine Failure | 108 | Environmental |
| Fire Detector Failure | 109 | Environmental |
| Fuse Failure | 110 | Environmental |
| Generator Failure | 111 | Environmental |
| Low Battery Threshold | 112 | Environmental |
| Pump Failure | 113 | Environmental |
| Rectifier Failure | 114 | Environmental |
| Rectifier High Voltage | 115 | Environmental |
| Rectifier Low F Voltage | 116 | Environmental |
| Ventilation System Failure | 117 | Environmental |
| Enclosure Door Open | 118 | Environmental |
| Explosive Gas | 119 | Environmental |
| Fire | 120 | Environmental |
| Flood | 121 | Environmental |
| High Humidity | 122 | Environmental |
| High Temperature | 123 | Environmental |
| High Wind | 124 | Environmental |
| Ice Build Up | 125 | Environmental |
| Intrusion Detection | 126 | Environmental |
| Low Fuel | 127 | Environmental |
| Low Humidity | 128 | Environmental |
| Low Cable Pressure | 129 | Environmental |
| Low Temperature | 130 | Environmental |
| Low Water | 131 | Environmental |
| Smoke | 132 | Environmental |
| Toxic Gas | 133 | Environmental |
| Reserved for M.3100 potential future extensions. | 134-150 |  |
| Storage Capacity Problem | 151 | Processing Error |
| Memory Mismatch | 152 | Processing Error |
| Corrupt Data | 153 | Processing Error |
| Out Of CPU Cycles | 154 | Processing Error |
| Software Environment Problem | 155 | Processing Error |
| Software Download Failure | 156 | Processing Error |
| Loss of Real Time | 157 | Processing Error |
| Reinitialized | 158 | Processing Error |
| Reserved | 159-167 |  |
| Reserved for M.3100 potential future extensions. | 168-200 |  |
| Reserved | 201-202 |  |
| Excessive Error Rate | 203 | Quality of service |
| Reserved | 204-207 |  |
| Reserved for M.3100 potential future extensions. | 208-300 |  |

Table B.2: Probable Causes from ITU-T Recommendation X.733 [8]

| X.733 Probable Cause (string) | (integer) | alarmType |
| --- | --- | --- |
| Adapter Error | 301 | Equipment |
| Application Subsystem Failure | 302 | Processing error |
| Bandwidth Reduction | 303 | Security Service or Mechanism Violation |
| Reserved | 304 |  |
| Communication Protocol Error | 305 | Communications |
| Communication Subsystem Failure | 306 | Communications |
| Configuration or Customizing Error | 307 | Processing error |
| Congestion | 308 | Quality of service |
| Reserved | 309 |  |
| CPU Cycles Limit Exceeded | 310 | Processing error |
| Data Set or Modem Error | 311 | Equipment |
| Reserved | 312 |  |
| DTE-DCE Interface Error | 313 | Communications |
| Reserved | 314 |  |
| Equipment Malfunction | 315 | Communications |
| Excessive Vibration | 316 | Integrity Violation |
| File Error | 317 | Environmental |
| Reserved | 318-320 |  |
| Heating or Ventilation or Cooling System Problem | 321 | Environmental |
| Humidity Unacceptable | 322 | Environmental |
| Input/Output Device Error | 323 | Equipment |
| Input Device Error | 324 | Environmental |
| LAN Error | 325 | Processing error |
| Leak Detection | 326 | Environmental |
| Local Node Transmission Error | 327 | Communications |
| Reserved | 328-329 |  |
| Material Supply Exhausted | 330 | Environmental |
| Reserved | 331 |  |
| Out of Memory | 332 | Processing error |
| Output Device Error | 333 | Equipment |
| Performance Degraded | 334 | Quality of service |
| Reserved | 335 |  |
| Pressure Unacceptable | 336 | Operational Violation |
| Reserved | 337-338 |  |
| Queue Size Exceeded | 339 | Quality of service |
| Receive Failure | 340 | Equipment |
| Reserved | 341 |  |
| Remote Node Transmission Error | 342 | Communications |
| Resource at or Nearing Capacity | 343 | Quality of service |
| Response Time Excessive | 344 | Quality of service |
| Re-transmission Rate Excessive | 345 | Quality of service |
| Software Error | 346 | Processing error |
| Software Program Abnormally Terminated | 347 | Processing error |
| Software Program Error | 348 | Processing error |
| Reserved | 349 |  |
| Temperature Unacceptable | 350 | Environmental |
| Threshold Crossed | 351 | Quality of service |
| Reserved | 352 |  |
| Toxic Leak Detected | 353 | Environmental |
| Transmit Failure | 354 | Equipment |
| Reserved | 355 |  |
| Underlying Resource Unavailable | 356 | Processing error |
| Version Mismatch | 357 | Processing error |
| Reserved for potential future X.721/X.733 extensions | 358-500 |  |

Table B.3: Probable Causes for Wireless Systems from ETSI TS 101 251 V6.3.0 (1999-07) [3]

| Wireless Systems (string) | (integer) | alarmType |
| --- | --- | --- |
| A-bis to BTS interface failure | 501 | Equipment |
| A-bis to TRX interface failure | 502 | Equipment |
| Antenna problem | 503 | Equipment |
| Battery breakdown | 504 | Equipment |
| Battery charging fault | 505 | Equipment |
| Clock synchronization problem | 506 | Equipment |
| Combiner problem | 507 | Equipment |
| Disk problem | 508 | Equipment |
| Reserved | 509 |  |
| Excessive receiver temperature | 510 | Equipment |
| Excessive transmitter output power | 511 | Equipment |
| Excessive transmitter temperature | 512 | Equipment |
| Frequency hopping degraded | 513 | Equipment |
| Frequency hopping failure | 514 | Equipment |
| Frequency redefinition failed | 515 | Equipment |
| Line interface failure | 516 | Equipment |
| Link failure | 517 | Equipment |
| Loss of synchronization | 518 | Equipment |
| Lost redundancy | 519 | Equipment |
| Mains breakdown with battery back-up | 520 | Equipment |
| Mains breakdown without battery back-up | 521 | Equipment |
| Power supply failure | 522 | Equipment |
| Receiver antenna fault | 523 | Equipment |
| Reserved | 524 |  |
| Receiver multicoupler failure | 525 | Equipment |
| Reduced transmitter output power | 526 | Equipment |
| Signal quality evaluation fault | 527 | Equipment |
| Timeslot hardware failure | 528 | Equipment |
| Transceiver problem | 529 | Equipment |
| Transcoder problem | 530 | Equipment |
| Transcoder or rate adapter problem | 531 | Equipment |
| Transmitter antenna failure | 532 | Equipment |
| Transmitter antenna not adjusted | 533 | Equipment |
| Reserved | 534 |  |
| Transmitter low voltage or current | 535 | Equipment |
| Transmitter off frequency | 536 | Equipment |
| Database inconsistency | 537 | Processing error |
| File system call unsuccessful | 538 | Processing error |
| Input parameter out of range | 539 | Processing error |
| Invalid parameter | 540 | Processing error |
| Invalid pointer | 541 | Processing error |
| Message not expected | 542 | Processing error |
| Message not initialized | 543 | Processing error |
| Message out of sequence | 544 | Processing error |
| System call unsuccessful | 545 | Processing error |
| Timeout expired | 546 | Processing error |
| Variable out of range | 547 | Processing error |
| Watch dog timer expired | 548 | Processing error |
| Cooling system failure | 549 | Environmental |
| External equipment failure | 550 | Environmental |
| External power supply failure | 551 | Environmental |
| External transmission device failure | 552 | Environmental |
| Reserved | 553-560 |  |
| Reduced alarm reporting | 561 | Quality of service |
| Reduced event reporting | 562 | Quality of service |
| Reduced logging capability | 563 | Quality of service |
| System resources overload | 564 | Quality of service |
| Broadcast channel failure | 565 | Communications |
| Connection establishment error | 566 | Communications |
| Invalid message received | 567 | Communications |
| Invalid MSU received | 568 | Communications |
| LAPD link protocol failure | 569 | Communications |
| Local alarm indication | 570 | Communications |
| Remote alarm indication | 571 | Communications |
| Routing failure | 572 | Communications |
| SS7 protocol failure | 573 | Communications |
| Transmission error | 574 | Communications |
| Reserved | 575 |  |
| Reserved for potential future ETSI extensions | 576-700 |  |

Table B.4: Probable Causes for Security Alarm from X.736 [13]

| Wireless Systems (string) | (integer) | alarmType |
| --- | --- | --- |
| Authentication Failure | 701 | security service or mechanism violation |
| Breach of Confidentiality | 702 | security service or mechanism violation |
| Cable Tamper | 703 | physical violation |
| Delayed Information | 704 | time domain violation |
| Denial of Service | 705 | operational violation |
| Duplicate Information | 706 | integrity violation |
| Information Missing | 707 | integrity violation |
| Information Modification Detected | 708 | integrity violation |
| Information Out of Sequence | 709 | integrity violation |
| Intrusion Detected | 710 | physical violation |
| Key Expired | 711 | time domain violation |
| Non Repudiation Failure | 712 | security service or mechanism violation |
| Out of Hours Activity | 713 | time domain violation |
| Out of Service | 714 | operational violation |
| Procedural Error | 715 | operational violation |
| Unauthorised Access Attempt | 716 | security service or mechanism violation |
| Unexpected Information | 717 | integrity violation |
| Unspecified Reason | 718 | security service or mechanism violation |
| Reserved for X.736 potential future extensions. | 719-800 |  |

End of changes

Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1844 at commit a149770375b54fbe2d40c32ee2d893e1ffe5c8de

*** START OF CHANGE 1 ***

*** yang-models/_3gpp-common-fm.yang ***

<CODE BEGINS>

module _3gpp-common-fm {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-common-fm";

prefix "fm3gpp";

import ietf-yang-types { prefix yang; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-yang-extensions { prefix yext3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Defines a Fault Management model

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.111";

revision 2025-08-13 { reference "CR-0053 CR-0054"; }

revision 2025-05-01 { reference "CR-0042 CR-0043"; }  // common for R18, R19

revision 2025-03-25 { reference "CR-0025 CR-0026"; }

revision 2024-05-12 {

description "The definition of the module was from TS 28.623 to TS 28.111";

reference CR-0008 ;

}

revision 2024-03-06 { reference CR-0333 ; }

revision 2024-02-24 { reference CR-0346; }

revision 2024-01-18 {

description "The specification of the file is moved from 28.623 to 28.532";

reference "28.623 CR-0315";

}

revision 2023-09-18 { reference CR-0271; }

revision 2023-05-10 { reference CR-0250; }

revision 2022-10-24 { reference CR-0196;   }

revision 2021-08-08 { reference "CR-0132"; }

revision 2021-06-02 { reference "CR-0130"; }

revision 2020-06-03 { reference "CR-0091"; }

revision 2020-02-24 { reference "S5-201365"; }

feature AcknowledgeByConsumer {

description "Indicates whether alarm acknowledgement by the consumer is

supported.";

}

typedef eventType {

type enumeration {

enum OTHER {

value 1;

}

enum COMMUNICATIONS_ALARM {

value 2;

}

enum QUALITY_OF_SERVICE_ALARM {

value 3;

}

enum PROCESSING_ERROR_ALARM {

value 4;

}

enum EQUIPMENT_ALARM {

value 5;

}

enum ENVIRONMENTAL_ALARM {

value 6;

}

enum INTEGRITY_VIOLATION {

value 7;

}

enum OPERATIONAL_VIOLATION {

value 8;

}

enum PHYSICAL_VIOLATION {

value 9;

}

enum SECURITY_SERVICE_OR_MECHANISM_VIOLATION {

value 10;

}

enum TIME_DOMAIN_VIOLATION {

value 11;

}

}

description "General category for the alarm.";

}

typedef severity-level {

type enumeration {

enum CRITICAL { value 3; }

enum MAJOR { value 4; }

enum MINOR { value 5; }

enum WARNING { value 6; }

enum INDETERMINATE { value 7; }

enum CLEARED { value 8; }

}

description "The possible alarm severities";

}

typedef probable-causes {

type enumeration {

enum INDETERMINATE { value 0; }

enum ALARM_INDICATION_SIGNAL { value 1; }

enum CALL_SETUP_FAILURE { value 2; }

enum DEGRADED_SIGNAL { value 3; }

enum FAR_END_RECEIVER_FAILURE { value 4; }

enum FRAMING_ERROR { value 5; }

enum LOSS_OF_FRAME { value 6; }

enum LOSS_OF_POINTER { value 7; }

enum LOSS_OF_SIGNAL { value 8; }

enum PAYLOAD_TYPE_MISMATCH { value 9; }

enum REMOTE_ALARM_INTERFACE { value 11; }

enum EXCESSIVE_BIT_ERROR_RATE { value 12; }

enum PATH_TRACE_MISMATCH { value 13; }

enum UNAVAILABLE { value 14; }

enum SIGNAL_LABEL_MISMATCH { value 15; }

enum LOSS_OF_MULTI_FRAME { value 16; }

enum COMMUNICATIONS_RECEIVE_FAILURE { value 17; }

enum COMMUNICATIONS_TRANSMIT_FAILURE { value 18; }

enum MODULATION_FAILURE { value 19; }

enum DEMODULATION_FAILURE { value 20; }

enum BACK_PLANE_FAILURE { value 51; }

enum DATA_SET_PROBLEM { value 52; }

enum EQUIPMENT_IDENTIFIER_DUPLICATION { value 53; }

enum EXTERNAL_IF_DEVICE_PROBLEM { value 54; }

enum LINE_CARD_PROBLEM { value 55; }

enum MULTIPLEXER_PROBLEM { value 56; }

enum NE_IDENTIFIER_DUPLICATION { value 57; }

enum POWER_PROBLEM { value 58; }

enum PROCESSOR_PROBLEM { value 59; }

enum PROTECTION_PATH_FAILURE { value 60; }

enum RECEIVER_FAILURE { value 61; }

enum REPLACEABLE_UNIT_MISSING { value 62; }

enum REPLACEABLE_UNIT_TYPE_MISMATCH { value 63; }

enum SYNCHRONIZATION_SOURCE_MISMATCH { value 64; }

enum TERMINAL_PROBLEM { value 65; }

enum TIMING_PROBLEM { value 66; }

enum TRANSMITTER_FAILURE { value 67; }

enum TRUNK_CARD_PROBLEM { value 68; }

enum REPLACEABLE_UNIT_PROBLEM { value 69; }

enum REAL_TIME_CLOCK_FAILURE { value 70; }

enum PROTECTION_MECHANISM_FAILURE { value 81; }

enum PROTECTING_RESOURCE_FAILURE { value 82; }

enum AIR_COMPRESSOR_FAILURE { value 101; }

enum AIR_CONDITIONING_FAILURE { value 102; }

enum AIR_DRYER_FAILURE { value 103; }

enum BATTERY_DISCHARGING { value 104; }

enum BATTERY_FAILURE { value 105; }

enum COMMERCIAL_POWER_FAILURE { value 106; }

enum COOLING_FAN_FAILURE { value 107; }

enum ENGINE_FAILURE { value 108; }

enum FIRE_DETECTOR_FAILURE { value 109; }

enum FUSE_FAILURE { value 110; }

enum GENERATOR_FAILURE { value 111; }

enum LOW_BATTERY_THRESHOLD { value 112; }

enum PUMP_FAILURE { value 113; }

enum RECTIFIER_FAILURE { value 114; }

enum RECTIFIER_HIGH_VOLTAGE { value 115; }

enum RECTIFIER_LOW_F_VOLTAGE { value 116; }

enum VENTILATION_SYSTEM_FAILURE { value 117; }

enum ENCLOSURE_DOOR_OPEN { value 118; }

enum EXPLOSIVE_GAS { value 119; }

enum FIRE { value 120; }

enum FLOOD { value 121; }

enum HIGH_HUMIDITY { value 122; }

enum HIGH_TEMPERATURE { value 123; }

enum HIGH_WIND { value 124; }

enum ICE_BUILD_UP { value 125; }

enum INTRUSION_DETECTION { value 126; }

enum LOW_FUEL { value 127; }

enum LOW_HUMIDITY { value 128; }

enum LOW_CABLE_PRESSURE { value 129; }

enum LOW_TEMPERATURE { value 130; }

enum LOW_WATER { value 131; }

enum SMOKE { value 132; }

enum TOXIC_GAS { value 133; }

enum EXTERNAL_POINT_FAILURE { value 136; }

enum STORAGE_CAPACITY_PROBLEM { value 151; }

enum MEMORY_MISMATCH { value 152; }

enum CORRUPT_DATA { value 153; }

enum OUT_OF_CPU_CYCLES { value 154; }

enum SOFTWARE_ENVIRONMENT_PROBLEM { value 155; }

enum SOFTWARE_DOWNLOAD_FAILURE { value 156; }

enum LOSS_OF_REAL_TIME { value 157; }

enum REINITIALIZED { value 158; }

enum EXCESSIVE_ERROR_RATE { value 203; }

enum ADAPTER_ERROR { value 301; }

enum APPLICATION_SUBSYSTEM_FAILURE { value 302; }

enum BANDWIDTH_REDUCED { value 303; }

enum COMMUNICATIONS_PROTOCOL_ERROR { value 305; }

enum COMMUNICATIONS_SUBSYSTEM_FAILURE { value 306; }

enum CONFIGURATION_OR_CUSTOMIZATION_ERROR { value 307; }

enum CONGESTION { value 308; }

enum CPU_CYCLES_LIMIT_EXCEEDED { value 310; }

enum DATA_SET_OR_MODEM_ERROR { value 311; }

enum DTE_DCE_INTERFACE_ERROR { value 313; }

enum EQUIPMENT_MALFUNCTION { value 315; }

enum EXCESSIVE_VIBRATION { value 316; }

enum FILE_ERROR { value 317; }

enum HEATING_OR_VENTILATION_OR_COOLING_SYSTEM_PROBLEM { value 321; }

enum HUMIDITY_UNACCEPTABLE { value 322; }

enum INPUT_OUTPUT_DEVICE_ERROR { value 323; }

enum INPUT_DEVICE_ERROR { value 324; }

enum LAN_ERROR { value 325; }

enum LEAK_DETECTED { value 326; }

enum LOCAL_NODE_TRANSMISSION_ERROR { value 327; }

enum MATERIAL_SUPPLY_EXHAUSTED { value 330; }

enum OUT_OF_MEMORY { value 332; }

enum OUTPUT_DEVICE_ERROR { value 333; }

enum PERFORMANCE_DEGRADED { value 334; }

enum PRESSURE_UNACCEPTABLE { value 336; }

enum QUEUE_SIZE_EXCEEDED { value 339; }

enum RECEIVE_FAILURE { value 340; }

enum REMOTE_NODE_TRANSMISSION_ERROR { value 342; }

enum RESOURCE_AT_OR_NEARING_CAPACITY { value 343; }

enum RESPONSE_TIME_EXCESSIVE { value 344; }

enum RETRANSMISSION_RATE_EXCESSIVE { value 345; }

enum SOFTWARE_ERROR { value 346; }

enum SOFTWARE_PROGRAM_ABNORMALLY_TERMINATED { value 347; }

enum SOFTWARE_PROGRAM_ERROR { value 348; }

enum TEMPERATURE_UNACCEPTABLE { value 350; }

enum THRESHOLD_CROSSED { value 351; }

enum TOXIC_LEAK_DETECTED { value 353; }

enum TRANSMIT_FAILURE { value 354; }

enum UNDERLYING_RESOURCE_UNAVAILABLE { value 356; }

enum VERSION_MISMATCH { value 357; }

enum A_BIS_TO_BTS_INTERFACE_FAILURE { value 501; }

enum A_BIS_TO_TRX_INTERFACE_FAILURE { value 502; }

enum ANTENNA_PROBLEM { value 503; }

enum BATTERY_BREAKDOWN { value 504; }

enum BATTERY_CHARGING_FAULT { value 505; }

enum CLOCK_SYNCHRONIZATION_PROBLEM { value 506; }

enum COMBINER_PROBLEM { value 507; }

enum DISK_PROBLEM { value 508; }

enum EXCESSIVE_RECEIVER_TEMPERATURE { value 510; }

enum EXCESSIVE_TRANSMITTER_OUTPUT_POWER { value 511; }

enum EXCESSIVE_TRANSMITTER_TEMPERATURE { value 512; }

enum FREQUENCY_HOPPING_DEGRADED { value 513; }

enum FREQUENCY_HOPPING_FAILURE { value 514; }

enum FREQUENCY_REDEFINITION_FAILED { value 515; }

enum LINE_INTERFACE_FAILURE { value 516; }

enum LINK_FAILURE { value 517; }

enum LOSS_OF_SYNCHRONIZATION { value 518; }

enum LOST_REDUNDANCY { value 519; }

enum MAINS_BREAKDOWN_WITH_BATTERY_BACKUP { value 520; }

enum MAINS_BREAKDOWN_WITHOUT_BATTERY_BACKUP { value 521; }

enum POWER_SUPPLY_FAILURE { value 522; }

enum RECEIVER_ANTENNA_FAULT { value 523; }

enum RECEIVER_MULTICOUPLER_FAILURE { value 525; }

enum REDUCED_TRANSMITTER_OUTPUT_POWER { value 526; }

enum SIGNAL_QUALITY_EVALUATION_FAULT { value 527; }

enum TIMESLOT_HARDWARE_FAILURE { value 528; }

enum TRANSCEIVER_PROBLEM { value 529; }

enum TRANSCODER_PROBLEM { value 530; }

enum TRANSCODER_OR_RATE_ADAPTER_PROBLEM { value 531; }

enum TRANSMITTER_ANTENNA_FAILURE { value 532; }

enum TRANSMITTER_ANTENNA_NOT_ADJUSTED { value 533; }

enum TRANSMITTER_LOW_VOLTAGE_OR_CURRENT { value 535; }

enum TRANSMITTER_OFF_FREQUENCY { value 536; }

enum DATABASE_INCONSISTENCY { value 537; }

enum FILE_SYSTEM_CALL_UNSUCCESSFUL { value 538; }

enum INPUT_PARAMETER_OUT_OF_RANGE { value 539; }

enum INVALID_PARAMETER { value 540; }

enum INVALID_POINTER { value 541; }

enum MESSAGE_NOT_EXPECTED { value 542; }

enum MESSAGE_NOT_INITIALIZED { value 543; }

enum MESSAGE_OUT_OF_SEQUENCE { value 544; }

enum SYSTEM_CALL_UNSUCCESSFUL { value 545; }

enum TIMEOUT_EXPIRED { value 546; }

enum VARIABLE_OUT_OF_RANGE { value 547; }

enum WATCH_DOG_TIMER_EXPIRED { value 548; }

enum COOLING_SYSTEM_FAILURE { value 549; }

enum EXTERNAL_EQUIPMENT_FAILURE { value 550; }

enum EXTERNAL_POWER_SUPPLY_FAILURE { value 551; }

enum EXTERNAL_TRANSMISSION_DEVICE_FAILURE { value 552; }

enum REDUCED_ALARM_REPORTING { value 561; }

enum REDUCED_EVENT_REPORTING { value 562; }

enum RECUCED_LOGGING_CAPABILITY { value 563; }

enum SYSTEM_RESOURCES_OVERLOAD { value 564; }

enum BROADCAST_CHANNEL_FAILURE { value 565; }

enum CONNECTION_ESTABLISHMENT_ERROR { value 566; }

enum INVALID_MESSAGE_RECEIVED { value 567; }

enum INVALID_MSU_RECEIVED { value 568; }

enum LAPD_LINK_PROTOCOL_FAILURE { value 569; }

enum LOCAL_ALARM_INDICATION { value 570; }

enum REMOTE_ALARM_INDICATION { value 571; }

enum ROUTING_FAILURE { value 572; }

enum SS7_PROTOCOL_FAILURE { value 573; }

enum TRANSMISSION_ERROR { value 574; }

enum AUTHENTICATION_FAILURE { value 701; }

enum BREACH_OF_CONFIDENTIALITY { value 702; }

enum CABLE_TAMPER { value 703; }

enum DELAYED_INFORMATION { value 704; }

enum DENIAL_OF_SERVICE { value 705; }

enum DUPLICATE_INFORMATION { value 706; }

enum INFORMATION_MISSING { value 707; }

enum INFORMATION_MODIFICATION_DETECTED { value 708; }

enum INFORMATION_OUT_OF_SEQUENCE { value 709; }

enum INTRUSION_DETECTED { value 710; }

enum KEY_EXPIRED { value 711; }

enum NON_REPUDIATION_FAILURE { value 712; }

enum OUT_OF_HOURS_ACTIVITY { value 713; }

enum OUT_OF_SERVICE { value 714; }

enum PROCEDURAL_ERROR { value 715; }

enum UNAUTHORISED_ACCESS_ATTEMPT { value 716; }

enum UNEXPECTED_INFORMATION { value 717; }

enum UNSPECIFIED_REASON { value 718; }

}

}

grouping AlarmCommentGrp {

leaf commentTime {

type yang:date-and-time;

config false;

mandatory true;

yext3gpp:inVariant;

description "Date and Time the comment was created.";

}

leaf commentUserId {

type string;

mandatory true;

yext3gpp:inVariant;

description "It carries the identification of the user who made the

comment.";

}

leaf commentSystemId {

type string;

mandatory true;

yext3gpp:inVariant;

description "It carries the identification of the system (

Management System) from which the comment is made. That system

supports the user that made the comment.";

}

leaf commentText {

type string;

mandatory true;

yext3gpp:inVariant;

description "It carries the textual comment.";

}

}

grouping ThresholdHysteresisGrp {

description "The ThresholdHysteresis defines the threshold boundaries to

control the hysteresis mechanism.

The high attribute of ThresholdHysteresis identifies the higher value of

a threshold with hysteris, the integer type is used for counter

thresholds and the float type for gauge thresholds. The low attribute

of ThresholdHysteresis identifies the lower value of a threshold with

hysteresis, applicable only to gauge thresholds.";

leaf high {

type union {

type int64;

type decimal64 {

fraction-digits 7;

}

}

mandatory true;

description "Higher value of a threshold with hysteris, the integer type

is used for counter thresholds and the float type for gauge

thresholds.";

}

leaf low {

type decimal64 {

fraction-digits 7;

}

description "Lower value of a threshold with hysteresis, applicable

only to gauge thresholds.";

}

}

grouping ThresholdLevelIndGrp {

description "The up attribute indicates for counter and gauge thresholds

that the threshold crossing occurred when going up. The down attribute

only indicates for gauge thresholds that the threshold crossing occurred

when going down, applicable only to gauge thresholds.";

list up {

description "Indicates for counter and gauge thresholds that the

threshold crossing occurred when going up.";

max-elements 1;

key idx;

leaf idx { type int32; }

uses ThresholdHysteresisGrp;

}

list down {

description "Indicates for gauge thresholds that the threshold crossing

occurred when going down, applicable only to gauge thresholds.";

max-elements 1;

key idx;

leaf idx { type int32; }

uses ThresholdHysteresisGrp;

}

}

grouping ThresholdCrossingGrp {

description "The datatype indicates the crossed threshold

information regardless of the gauge threshold, which represents an

instantaneous value that changes over time, or the counter threshold,

which represents monotonically increasing cumulative quantity.

The observedMeasurement attribute of TheresholdInfo specifies the name

of the monitored measurement that crossed the threshold and that

caused the notification (Rec. ITU-T X. 733[8]). The observedValue

attribute indicates the value of the gauge or counter which crossed

the threshold. This may be different from the threshold value if, for

example, the gauge may only take on discrete values. Integer values

are used for counters and float values for gauges (Rec. ITU-T X. 733).

Note that a 'number' type property can contain both integers and

floating point numbers.

For the thresholdLevel attribute, in the case of a gauge, it specifies

a pair of threshold values, the first being the value of the crossed

threshold and the second, its corresponding hysteresis; in the case of

a counter, it specifies only the threshold value (Rec. ITU-T X. 733).

For the armTime attribute, for a gauge threshold, it specifies the

time at which the threshold was last re-armed, namely the time after

the previous threshold crossing at which the hysteresis value of the

threshold was exceeded thus again permitting generation of

notifications when the threshold is crossed; for a counter threshold,

the later of the time at which the threshold offset was last applied,

or the time at which the counter was last initialized (for resettable

counters) (Rec. ITU-T X. 733).";

leaf observedMeasurement {

type string;

mandatory true;

description "The name of the monitored measurement that crossed the

threshold and that caused the notification (Rec. ITU-T X. 733 ";

}

leaf observedValue {

type union {

type int64;

type decimal64 {

fraction-digits 7;

}

}

mandatory true;

description "The value of the gauge or counter which crossed the

threshold. This may be different from the threshold value if, for

example, the gauge may only take on discrete values.

Integer values are used for counters and float values for gauges

(Rec. ITU-T X. 733). Note that a 'number' type property can contain

both integers and floating point numbers.";

}

list thresholdLevel {

description "In the case of a gauge the threshold level specifies

a pair of threshold values, the first being the value of the crossed

threshold and the second, its corresponding hysteresis; in the case

of a counter the threshold level specifies only the threshold value

(Rec. ITU-T X. 733).";

max-elements 1;

key idx;

leaf idx { type int32; }

uses ThresholdLevelIndGrp;

}

leaf armTime {

type yang:date-and-time;

description "For a gauge threshold, the time at which the threshold

was last re-armed, namely the time after the previous threshold

crossing at which the hysteresis value of the threshold was

exceeded thus again permitting generation of notifications when the

threshold is crossed. For a counter threshold, the later of the time

at which the threshold offset was last applied, or the time at

which the counter was last initialized (for resettable counters)

(Rec. ITU-T X. 733)";

}

}

grouping AlarmRecordGrp {

description "Contains alarm information of an alarmed object instance.

A new record is created in the alarm list when an alarmed object

instance generates an alarm and no alarm record exists with the same

values for objectInstance, alarmType, probableCause and specificProblem.

When a new record is created the MnS producer creates an alarmId, that

unambiguously identifies an alarm record in the AlarmList.

Alarm records are maintained only for active alarms. Inactive alarms are

automatically deleted by the MnS producer from the AlarmList.

Active alarms are alarms whose

a)  perceivedSeverity is not CLEARED, or whose

b)  perceivedSeverity is CLEARED and its ackState is not ACKNOWLEDED.";

leaf alarmId {

type string;

mandatory true;

description "Identifies the alarmRecord";

yext3gpp:notNotifyable;

yext3gpp:inVariant;

}

leaf objectInstance {

type types3gpp:DistinguishedName;

config false ;

mandatory true;

yext3gpp:notNotifyable;

yext3gpp:inVariant;

}

leaf notificationId {

type int32;

config false ;

mandatory true;

description "The Id of the last notification updating the AlarmRecord.";

yext3gpp:notNotifyable;

}

leaf alarmRaisedTime {

type yang:date-and-time ;

mandatory true;

config false ;

yext3gpp:notNotifyable;

}

leaf alarmChangedTime {

type yang:date-and-time ;

config false ;

description "not applicable if related alarm has not changed";

yext3gpp:notNotifyable;

}

leaf alarmClearedTime {

type yang:date-and-time ;

config false ;

description "not applicable if related alarm was not cleared";

yext3gpp:notNotifyable;

}

leaf alarmType {

type eventType;

config false ;

mandatory true;

description "General category for the alarm.";

yext3gpp:notNotifyable;

yext3gpp:inVariant;

}

leaf probableCause {

type union {

type probable-causes;

type int32;

type string;

}

config false ;

mandatory true;

yext3gpp:notNotifyable;

yext3gpp:inVariant;

}

leaf specificProblem {

type union {

type int32;

type string;

}

config false ;

reference "ITU-T Recommendation X.733 clause 8.1.2.2.";

yext3gpp:notNotifyable;

yext3gpp:inVariant;

}

leaf perceivedSeverity {

type severity-level;

mandatory true;

description "This is Writable only if producer supports consumer

to set perceivedSeverity to CLEARED";

yext3gpp:notNotifyable;

}

leaf backedUpStatus {

type boolean;

config false ;

description "Indicates if an object (the MonitoredEntity) has a back

up. See definition in ITU-T Recommendation X.733 clause 8.1.2.4.";

yext3gpp:notNotifyable;

}

leaf backUpObject {

type types3gpp:DistinguishedName;

config false ;

description "Backup object of the alarmed object as defined in

ITU-T Rec. X. 733";

yext3gpp:notNotifyable;

}

leaf trendIndication {

type enumeration {

enum MORE_SEVERE;

enum NO_CHANGE;

enum LESS_SEVERE;

}

config false ;

description "Indicates if some observed condition is getting better,

worse, or not changing. ";

reference "ITU-T Recommendation X.733 clause 8.1.2.6.";

yext3gpp:notNotifyable;

}

list thresholdInfo {

config false ;

yext3gpp:notNotifyable;

description "Indicates the crossed threshold";

key idx;

leaf idx { type int32; }

uses ThresholdCrossingGrp;

}

list stateChangeDefinition {

key attributeName;

config false ;

description "Indicates MO attribute value changes associated with the

alarm for state attributes of the monitored entity (state transitions).

The change is reported with the name of the state attribute, the new

value and an optional old value.

See definition in ITU-T Recommendation X.733 [4] clause 8.1.2.10.";

yext3gpp:notNotifyable;

leaf attributeName {

type string;

}

anydata newValue {

mandatory true;

description "The new value of the attribute. The content of this data

node shall be in accordance with the data model for the attribute.";

}

anydata oldValue{

description "The old value of the attribute. The content of this data

node shall be in accordance with the data model for the attribute.";

}

}

list monitoredAttributes {

key attributeName;

config false ;

yext3gpp:notNotifyable;

description "Attributes of the monitored entity and their

values at the time the alarm occurred that are of interest for the

alarm report.";

reference "ITU-T Recommendation X.733 clause 8.1.2.11.";

leaf attributeName {

type string;

}

anydata value {

mandatory true;

description "The value of the attribute. The content of this data

node shall be in accordance with the data model for the attribute.";

}

}

leaf proposedRepairActions {

type string;

config false ;

description "Indicates proposed repair actions. See definition in

ITU-T Recommendation X.733 clause 8.1.2.12.";

yext3gpp:notNotifyable;

}

leaf additionalText {

type string;

config false ;

yext3gpp:notNotifyable;

}

list additionalInformation {

key name;

config false ;

yext3gpp:notNotifyable;

description "Vendor specific alarm information in the alarm.";

uses types3gpp:nameValuePair;

}

leaf rootCauseIndicator {

type boolean;

default false;

config false ;

description "It indicates that this AlarmInformation is the root cause

of the events captured by the notifications whose identifiers are in

the related CorrelatedNotification instances.";

yext3gpp:notNotifyable;

}

list comments {

yext3gpp:notNotifyable;

description "List of comments and data about the comments.";

key idx;

leaf idx { type uint32; }

uses AlarmCommentGrp;

}

leaf ackTime  {

if-feature AcknowledgeByConsumer;

type yang:date-and-time ;

config false ;

description "It identifies the time when the alarm has been

acknowledged or unacknowledged the last time, i.e. it registers the

time when ackState changes.";

yext3gpp:notNotifyable;

}

leaf ackUserId  {

if-feature AcknowledgeByConsumer;

type string;

description "It identifies the last user who has changed the

Acknowledgement State.";

yext3gpp:notNotifyable;

}

leaf ackSystemId  {

if-feature AcknowledgeByConsumer;

type string;

description "It identifies the system (Management System) that last

changed the ackState of an alarm, i.e. acknowledged or unacknowledged

the alarm.";

yext3gpp:notNotifyable;

}

leaf ackState  {

if-feature AcknowledgeByConsumer;

type enumeration {

enum ACKNOWLEDGED {

description "The alarm has been acknowledged.";

}

enum UNACKNOWLEDGED {

description "The alarm has unacknowledged or the alarm has never

been acknowledged.";

}

}

yext3gpp:notNotifyable;

}

leaf clearUserId {

type string;

description "Carries the identity of the user who invokes the

clearAlarms operation.";

yext3gpp:notNotifyable;

}

leaf clearSystemId {

type string;

yext3gpp:notNotifyable;

}

leaf serviceUser {

type string;

config false ;

description "It identifies the service-user whose request for service

provided by the serviceProvider led to the generation of the

security alarm.";

yext3gpp:notNotifyable;

}

leaf serviceProvider {

type string;

config false ;

description "It identifies the service-provider whose service is

requested by the serviceUser and the service request provokes the

generation of the security alarm.";

yext3gpp:notNotifyable;

}

leaf securityAlarmDetector {

type string;

config false ;

yext3gpp:notNotifyable;

}

list correlatedNotifications {

key sourceObjectInstance;

description "List of correlated notifications";

leaf sourceObjectInstance {

type types3gpp:DistinguishedName;

}

leaf-list notificationIds {

type int32;

min-elements 1;

}

}

}

grouping AlarmListGrp {

description "Represents the AlarmList IOC.";

leaf administrativeState {

type types3gpp:BasicAdministrativeState ;

default LOCKED;

description "When set to UNLOCKED, the alarm list is updated.

When the set to LOCKED, the existing alarm records are not

updated, and new alarm records are not added to the alarm list.";

}

leaf operationalState {

type types3gpp:OperationalState ;

default DISABLED;

config false;

description "The producer sets this attribute to ENABLED, indicating

that it has the resource and ability to record alarm in AlarmList

else, it sets the attribute to DISABLED.";

}

leaf numOfAlarmRecords {

type uint32 ;

config false;

mandatory true;

description "The number of alarm records in the AlarmList";

yext3gpp:notNotifyable;

}

leaf lastModification {

type yang:date-and-time ;

config false;

description "The last time when an alarm record was modified";

yext3gpp:notNotifyable;

}

list alarmRecords {

key alarmId;

description "List of alarmRecords";

yext3gpp:notNotifyable;

uses AlarmRecordGrp;

}

leaf-list unreliableAlarmScope {

type types3gpp:DistinguishedName;

config false;

yext3gpp:notNotifyable;

description "Identifies, the part of the alarm scope that may not be

reliable.

If this parameter is equal to the instance carried in systemDN,

then all AlarmRecord instances in the AlarmList may not be reliable.

If this parameter is equal to some instance represented by

MonitoredEntity, then only AlarmRecord related to this instance and

its descendants may not be reliable.";

}

}

grouping FmSubtree {

description "Contains FM related classes.

Should be used in all classes (or classes inheriting from)

- SubNetwork

- ManagedElement

If some YAM wants to augment these classes/list/groupings they must

augment all user classes!";

list AlarmList {

key id;

max-elements 1;

yext3gpp:only-system-created;

description "The AlarmList represents the capability to store and manage

alarm records. The management scope of an AlarmList is defined by all

descendant objects of the base managed object, which is the object

name-containing the AlarmList, and the base object itself.

AlarmList instances are created by the system or are pre-installed.

They cannot be created nor deleted by MnS consumers.

When the alarm list is locked or disabled, the existing alarm records

are not updated, and new alarm records are not added to the alarm list";

uses top3gpp:Top_Grp ;

container attributes {

uses AlarmListGrp ;

}

}

}

}

<CODE ENDS>

*** END OF CHANGE 1 ***

Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1843 at commit ce60df8fb1a2313f3265964fb28aaae257ef717c

*** START OF CHANGE 1 ***

*** OpenAPI/TS28111_FaultNrm.yaml ***

<CODE BEGINS>

openapi: 3.0.1

info:

title: Fault Management NRM

version: 18.5.0

description: >-

OAS 3.0.1 definition of the Fault Supervision MnS

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.111; Fault Management

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.111/

servers:

- url: '{MnSRoot}/FaultSupervisionMnS/{MnSversion}'

variables:

MnSRoot:

description: See subclause 4.4.3 of TS 32.158

default: http://example.com/3GPPManagement

MnSversion:

description: Version number of the OpenAPI definition

default: XXX

paths: {}

components:

schemas:

#---- Definition of AlarmRecord ----------------------------------------------------#

AlarmId:

type: string

AlarmType:

type: string

enum:

- COMMUNICATIONS_ALARM

- QUALITY_OF_SERVICE_ALARM

- PROCESSING_ERROR_ALARM

- EQUIPMENT_ALARM

- ENVIRONMENTAL_ALARM

- INTEGRITY_VIOLATION

- OPERATIONAL_VIOLATION

- PHYSICAL_VIOLATION

- SECURITY_SERVICE_OR_MECHANISM_VIOLATION

- TIME_DOMAIN_VIOLATION

- OTHER

ProbableCause:

description: >-

The value of the probable cause may be a specific standardized string, or any

vendor provided string.

The value of the probable cause may also be an integer. The mapping of integer

values to probable causes is vendor specific.

oneOf:

- anyOf:

- type: string

enum:

- INDETERMINATE

- ALARM_INDICATION_SIGNAL

- CALL_SETUP_FAILURE

- DEGRADED_SIGNAL

- FAR_END_RECEIVER_FAILURE

- FRAMING_ERROR

- LOSS_OF_FRAME

- LOSS_OF_POINTER

- LOSS_OF_SIGNAL

- PAYLOAD_TYPE_MISMATCH

- REMOTE_ALARM_INTERFACE

- EXCESSIVE_BIT_ERROR_RATE

- PATH_TRACE_MISMATCH

- UNAVAILABLE

- SIGNAL_LABEL_MISMATCH

- LOSS_OF_MULTI_FRAME

- COMMUNICATIONS_RECEIVE_FAILURE

- COMMUNICATIONS_TRANSMIT_FAILURE

- MODULATION_FAILURE

- DEMODULATION_FAILURE

- BACK_PLANE_FAILURE

- DATA_SET_PROBLEM

- EQUIPMENT_IDENTIFIER_DUPLICATION

- EXTERNAL_IF_DEVICE_PROBLEM

- LINE_CARD_PROBLEM

- MULTIPLEXER_PROBLEM

- NE_IDENTIFIER_DUPLICATION

- POWER_PROBLEM

- PROCESSOR_PROBLEM

- PROTECTION_PATH_FAILURE

- RECEIVER_FAILURE

- REPLACEABLE_UNIT_MISSING

- REPLACEABLE_UNIT_TYPE_MISMATCH

- SYNCHRONIZATION_SOURCE_MISMATCH

- TERMINAL_PROBLEM

- TIMING_PROBLEM

- TRANSMITTER_FAILURE

- TRUNK_CARD_PROBLEM

- REPLACEABLE_UNIT_PROBLEM

- REAL_TIME_CLOCK_FAILURE

- PROTECTION_MECHANISM_FAILURE

- PROTECTING_RESOURCE_FAILURE

- AIR_COMPRESSOR_FAILURE

- AIR_CONDITIONING_FAILURE

- AIR_DRYER_FAILURE

- BATTERY_DISCHARGING

- BATTERY_FAILURE

- COMMERCIAL_POWER_FAILURE

- COOLING_FAN_FAILURE

- ENGINE_FAILURE

- FIRE_DETECTOR_FAILURE

- FUSE_FAILURE

- GENERATOR_FAILURE

- LOW_BATTERY_THRESHOLD

- PUMP_FAILURE

- RECTIFIER_FAILURE

- RECTIFIER_HIGH_VOLTAGE

- RECTIFIER_LOW_F_VOLTAGE

- VENTILATION_SYSTEM_FAILURE

- ENCLOSURE_DOOR_OPEN

- EXPLOSIVE_GAS

- FIRE

- FLOOD

- HIGH_HUMIDITY

- HIGH_TEMPERATURE

- HIGH_WIND

- ICE_BUILD_UP

- INTRUSION_DETECTION

- LOW_FUEL

- LOW_HUMIDITY

- LOW_CABLE_PRESSURE

- LOW_TEMPERATURE

- LOW_WATER

- SMOKE

- TOXIC_GAS

- EXTERNAL_POINT_FAILURE

- STORAGE_CAPACITY_PROBLEM

- MEMORY_MISMATCH

- CORRUPT_DATA

- OUT_OF_CPU_CYCLES

- SOFTWARE_ENVIRONMENT_PROBLEM

- SOFTWARE_DOWNLOAD_FAILURE

- LOSS_OF_REAL_TIME

- REINITIALIZED

- EXCESSIVE_ERROR_RATE

- ADAPTER_ERROR

- APPLICATION_SUBSYSTEM_FAILURE

- BANDWIDTH_REDUCED

- COMMUNICATIONS_PROTOCOL_ERROR

- COMMUNICATIONS_SUBSYSTEM_FAILURE

- CONFIGURATION_OR_CUSTOMIZATION_ERROR

- CONGESTION

- CPU_CYCLES_LIMIT_EXCEEDED

- DATA_SET_OR_MODEM_ERROR

- DTE_DCE_INTERFACE_ERROR

- EQUIPMENT_MALFUNCTION

- EXCESSIVE_VIBRATION

- FILE_ERROR

- HEATING_OR_VENTILATION_OR_COOLING_SYSTEM_PROBLEM

- HUMIDITY_UNACCEPTABLE

- INPUT_OUTPUT_DEVICE_ERROR

- INPUT_DEVICE_ERROR

- LAN_ERROR

- LEAK_DETECTED

- LOCAL_NODE_TRANSMISSION_ERROR

- MATERIAL_SUPPLY_EXHAUSTED

- OUT_OF_MEMORY

- OUTPUT_DEVICE_ERROR

- PERFORMANCE_DEGRADED

- PRESSURE_UNACCEPTABLE

- QUEUE_SIZE_EXCEEDED

- RECEIVE_FAILURE

- REMOTE_NODE_TRANSMISSION_ERROR

- RESOURCE_AT_OR_NEARING_CAPACITY

- RESPONSE_TIME_EXCESSIVE

- RETRANSMISSION_RATE_EXCESSIVE

- SOFTWARE_ERROR

- SOFTWARE_PROGRAM_ABNORMALLY_TERMINATED

- SOFTWARE_PROGRAM_ERROR

- TEMPERATURE_UNACCEPTABLE

- THRESHOLD_CROSSED

- TOXIC_LEAK_DETECTED

- TRANSMIT_FAILURE

- UNDERLYING_RESOURCE_UNAVAILABLE

- VERSION_MISMATCH

- A_BIS_TO_BTS_INTERFACE_FAILURE

- A_BIS_TO_TRX_INTERFACE_FAILURE

- ANTENNA_PROBLEM

- BATTERY_BREAKDOWN

- BATTERY_CHARGING_FAULT

- CLOCK_SYNCHRONIZATION_PROBLEM

- COMBINER_PROBLEM

- DISK_PROBLEM

- EXCESSIVE_RECEIVER_TEMPERATURE

- EXCESSIVE_TRANSMITTER_OUTPUT_POWER

- EXCESSIVE_TRANSMITTER_TEMPERATURE

- FREQUENCY_HOPPING_DEGRADED

- FREQUENCY_HOPPING_FAILURE

- FREQUENCY_REDEFINITION_FAILED

- LINE_INTERFACE_FAILURE

- LINK_FAILURE

- LOSS_OF_SYNCHRONIZATION

- LOST_REDUNDANCY

- MAINS_BREAKDOWN_WITH_BATTERY_BACKUP

- MAINS_BREAKDOWN_WITHOUT_BATTERY_BACKUP

- POWER_SUPPLY_FAILURE

- RECEIVER_ANTENNA_FAULT

- RECEIVER_MULTICOUPLER_FAILURE

- REDUCED_TRANSMITTER_OUTPUT_POWER

- SIGNAL_QUALITY_EVALUATION_FAULT

- TIMESLOT_HARDWARE_FAILURE

- TRANSCEIVER_PROBLEM

- TRANSCODER_PROBLEM

- TRANSCODER_OR_RATE_ADAPTER_PROBLEM

- TRANSMITTER_ANTENNA_FAILURE

- TRANSMITTER_ANTENNA_NOT_ADJUSTED

- TRANSMITTER_LOW_VOLTAGE_OR_CURRENT

- TRANSMITTER_OFF_FREQUENCY

- DATABASE_INCONSISTENCY

- FILE_SYSTEM_CALL_UNSUCCESSFUL

- INPUT_PARAMETER_OUT_OF_RANGE

- INVALID_PARAMETER

- INVALID_POINTER

- MESSAGE_NOT_EXPECTED

- MESSAGE_NOT_INITIALIZED

- MESSAGE_OUT_OF_SEQUENCE

- SYSTEM_CALL_UNSUCCESSFUL

- TIMEOUT_EXPIRED

- VARIABLE_OUT_OF_RANGE

- WATCH_DOG_TIMER_EXPIRED

- COOLING_SYSTEM_FAILURE

- EXTERNAL_EQUIPMENT_FAILURE

- EXTERNAL_POWER_SUPPLY_FAILURE

- EXTERNAL_TRANSMISSION_DEVICE_FAILURE

- REDUCED_ALARM_REPORTING

- REDUCED_EVENT_REPORTING

- RECUCED_LOGGING_CAPABILITY

- SYSTEM_RESOURCES_OVERLOAD

- BROADCAST_CHANNEL_FAILURE

- CONNECTION_ESTABLISHMENT_ERROR

- INVALID_MESSAGE_RECEIVED

- INVALID_MSU_RECEIVED

- LAPD_LINK_PROTOCOL_FAILURE

- LOCAL_ALARM_INDICATION

- REMOTE_ALARM_INDICATION

- ROUTING_FAILURE

- SS7_PROTOCOL_FAILURE

- TRANSMISSION_ERROR

- AUTHENTICATION_FAILURE

- BREACH_OF_CONFIDENTIALITY

- CABLE_TAMPER

- DELAYED_INFORMATION

- DENIAL_OF_SERVICE

- DUPLICATE_INFORMATION

- INFORMATION_MISSING

- INFORMATION_MODIFICATION_DETECTED

- INFORMATION_OUT_OF_SEQUENCE

- INTRUSION_DETECTED

- KEY_EXPIRED

- NON_REPUDIATION_FAILURE

- OUT_OF_HOURS_ACTIVITY

- OUT_OF_SERVICE

- PROCEDURAL_ERROR

- UNAUTHORISED_ACCESS_ATTEMPT

- UNEXPECTED_INFORMATION

- UNSPECIFIED_REASON

- type: string

- type: integer

SpecificProblem:

oneOf:

- type: string

- type: integer

PerceivedSeverity:

type: string

enum:

- INDETERMINATE

- CRITICAL

- MAJOR

- MINOR

- WARNING

- CLEARED

TrendIndication:

type: string

enum:

- MORE_SEVERE

- NO_CHANGE

- LESS_SEVERE

ThresholdHysteresis:

type: object

required:

- high

properties:

high:

oneOf:

- type: integer

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/Float'

low:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Float'

ThresholdLevelInd:

oneOf:

- type: object

properties:

up:

$ref: '#/components/schemas/ThresholdHysteresis'

- type: object

properties:

down:

$ref: '#/components/schemas/ThresholdHysteresis'

ThresholdCrossing:

type: object

properties:

observedMeasurement:

type: string

observedValue:

type: number

thresholdLevel:

$ref: '#/components/schemas/ThresholdLevelInd'

armTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

required:

- observedMeasurement

- observedValue

CorrelatedNotification:

type: object

properties:

sourceObjectInstance:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

notificationIds:

type: array

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationId'

required:

- sourceObjectInstance

- notificationIds

CorrelatedNotifications:

type: array

items:

$ref: '#/components/schemas/CorrelatedNotification'

AckState:

type: string

enum:

- ACKNOWLEDGED

- UNACKNOWLEDGED

AlarmRecord:

description: >-

The alarmId is not a property of an alarm record. It is used as key

in the map of alarm records instead.

type: object

properties:

# alarmId:

#  $ref: '#/components/schemas/AlarmId'

objectInstance:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

notificationId:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationId'

alarmRaisedTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

alarmChangedTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

alarmClearedTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

alarmType:

$ref: '#/components/schemas/AlarmType'

probableCause:

$ref: '#/components/schemas/ProbableCause'

specificProblem:

$ref: '#/components/schemas/SpecificProblem'

perceivedSeverity:

$ref: '#/components/schemas/PerceivedSeverity'

backedUpStatus:

type: boolean

backUpObject:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

trendIndication:

$ref: '#/components/schemas/TrendIndication'

thresholdinfo:

$ref: '#/components/schemas/ThresholdCrossing'

correlatedNotifications:

$ref: '#/components/schemas/CorrelatedNotifications'

stateChangeDefinition:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeValueChangeSet'

monitoredAttributes:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeNameValuePairSet'

proposedRepairActions:

type: string

additionalText:

type: string

additionalInformation:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeNameValuePairSet'

rootCauseIndicator:

type: boolean

ackTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

ackUserId:

type: string

ackSystemId:

type: string

ackState:

$ref: '#/components/schemas/AckState'

clearUserId:

type: string

clearSystemId:

type: string

serviceUser:

type: string

serviceProvider:

type: string

securityAlarmDetector:

type: string

AlarmList-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

administrativeState:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AdministrativeState'

operationalState:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/OperationalState'

numOfAlarmRecords:

type: integer

lastModification:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

alarmRecords:

description: >-

This resource represents a map of alarm records.

The alarmIds are used as keys in the map.

type: object

additionalProperties:

$ref: '#/components/schemas/AlarmRecord'

unreliableAlarmScope:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

#---- Definition of alarm notifications --------------------------------------------#

AlarmNotificationTypes:

type: string

enum:

- notifyNewAlarm

- notifyChangedAlarm

- notifyChangedAlarmGeneral

- notifyAckStateChanged

- notifyCorrelatedNotificationChanged

- notifyComments

- notifyClearedAlarm

- notifyAlarmListRebuilt

- notifyPotentialFaultyAlarmList

AlarmListAlignmentRequirement:

type: string

enum:

- ALIGNMENT_REQUIRED

- ALIGNMENT_NOT_REQUIRED

NotifyNewAlarm:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

required:

- alarmId

- alarmType

- probableCause

- perceivedSeverity

properties:

alarmId:

$ref: '#/components/schemas/AlarmId'

alarmType:

$ref: '#/components/schemas/AlarmType'

probableCause:

$ref: '#/components/schemas/ProbableCause'

specificProblem:

$ref: '#/components/schemas/SpecificProblem'

perceivedSeverity:

$ref: '#/components/schemas/PerceivedSeverity'

backedUpStatus:

type: boolean

backUpObject:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

trendIndication:

$ref: '#/components/schemas/TrendIndication'

thresholdInfo:

$ref: '#/components/schemas/ThresholdCrossing'

correlatedNotifications:

$ref: '#/components/schemas/CorrelatedNotifications'

stateChangeDefinition:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeValueChangeSet'

monitoredAttributes:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeNameValuePairSet'

proposedRepairActions:

type: string

additionalText:

type: string

additionalInformation:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeNameValuePairSet'

rootCauseIndicator:

type: boolean

NotifyNewSecAlarm:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

required:

- alarmId

- alarmType

- probableCause

- perceivedSeverity

- serviceUser

- serviceProvider

- securityAlarmDetector

properties:

alarmId:

$ref: '#/components/schemas/AlarmId'

alarmType:

$ref: '#/components/schemas/AlarmType'

probableCause:

$ref: '#/components/schemas/ProbableCause'

specificProblem:

$ref: '#/components/schemas/SpecificProblem'

perceivedSeverity:

$ref: '#/components/schemas/PerceivedSeverity'

correlatedNotifications:

$ref: '#/components/schemas/CorrelatedNotifications'

additionalText:

type: string

additionalInformation:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeNameValuePairSet'

rootCauseIndicator:

type: boolean

serviceUser:

type: string

serviceProvider:

type: string

securityAlarmDetector:

type: string

NotifyClearedAlarm:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

required:

- alarmId

- alarmType

- probableCause

- perceivedSeverity

properties:

alarmId:

$ref: '#/components/schemas/AlarmId'

alarmType:

$ref: '#/components/schemas/AlarmType'

probableCause:

$ref: '#/components/schemas/ProbableCause'

perceivedSeverity:

$ref: '#/components/schemas/PerceivedSeverity'

correlatedNotifications:

$ref: '#/components/schemas/CorrelatedNotifications'

clearUserId:

type: string

clearSystemId:

type: string

NotifyChangedAlarm:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

required:

- alarmId

- alarmType

- probableCause

- perceivedSeverity

properties:

alarmId:

$ref: '#/components/schemas/AlarmId'

alarmType:

$ref: '#/components/schemas/AlarmType'

probableCause:

$ref: '#/components/schemas/ProbableCause'

perceivedSeverity:

$ref: '#/components/schemas/PerceivedSeverity'

NotifyChangedAlarmGeneral:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

required:

- alarmId

- alarmType

properties:

alarmId:

$ref: '#/components/schemas/AlarmId'

alarmType:

$ref: '#/components/schemas/AlarmType'

probableCause:

$ref: '#/components/schemas/ProbableCause'

specificProblem:

$ref: '#/components/schemas/SpecificProblem'

perceivedSeverity:

$ref: '#/components/schemas/PerceivedSeverity'

correlatedNotifications:

$ref: '#/components/schemas/CorrelatedNotifications'

backedUpStatus:

type: boolean

backUpObject:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

trendIndication:

$ref: '#/components/schemas/TrendIndication'

thresholdInfo:

$ref: '#/components/schemas/ThresholdCrossing'

stateChangeDefinition:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeValueChangeSet'

monitoredAttributes:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeNameValuePairSet'

proposedRepairActions:

type: string

additionalText:

type: string

additionalInformation:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeNameValuePairSet'

rootCauseIndicator:

type: boolean

changedAlarmAttributes:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeNameValuePairSet'

NotifyChangedSecAlarmGeneral:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

required:

- alarmId

- alarmType

- serviceUser

- serviceProvider

- securityAlarmDetector

properties:

alarmId:

$ref: '#/components/schemas/AlarmId'

alarmType:

$ref: '#/components/schemas/AlarmType'

probableCause:

$ref: '#/components/schemas/ProbableCause'

perceivedSeverity:

$ref: '#/components/schemas/PerceivedSeverity'

correlatedNotifications:

$ref: '#/components/schemas/CorrelatedNotifications'

additionalText:

type: string

additionalInformation:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeNameValuePairSet'

rootCauseIndicator:

type: boolean

serviceUser:

type: string

serviceProvider:

type: string

securityAlarmDetector:

type: string

changedAlarmAttributes:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeNameValuePairSet'

NotifyCorrelatedNotificationChanged:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

required:

- alarmId

- correlatedNotifications

properties:

alarmId:

$ref: '#/components/schemas/AlarmId'

correlatedNotifications:

$ref: '#/components/schemas/CorrelatedNotifications'

rootCauseIndicator:

type: boolean

NotifyAckStateChanged:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

required:

- alarmId

- alarmType

- probableCause

- perceivedSeverity

- ackState

- ackUserId

properties:

alarmId:

$ref: '#/components/schemas/AlarmId'

alarmType:

$ref: '#/components/schemas/AlarmType'

probableCause:

$ref: '#/components/schemas/ProbableCause'

perceivedSeverity:

$ref: '#/components/schemas/PerceivedSeverity'

ackState:

$ref: '#/components/schemas/AckState'

ackUserId:

type: string

ackSystemId:

type: string

NotifyComments:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

required:

- alarmId

- alarmType

- probableCause

- perceivedSeverity

- comments

properties:

alarmId:

$ref: '#/components/schemas/AlarmId'

alarmType:

$ref: '#/components/schemas/AlarmType'

probableCause:

$ref: '#/components/schemas/ProbableCause'

perceivedSeverity:

$ref: '#/components/schemas/PerceivedSeverity'

comments:

$ref: '#/components/schemas/Comments'

NotifyPotentialFaultyAlarmList:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

required:

- reason

properties:

reason:

type: string

NotifyAlarmListRebuilt:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

required:

- reason

properties:

reason:

type: string

alarmListAlignmentRequirement:

$ref: '#/components/schemas/AlarmListAlignmentRequirement'

#---- Definition of resources ------------------------------------------------------#

Comment:

type: object

properties:

commentTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

commentUserId:

type: string

commentSystemId:

type: string

commentText:

type: string

Comments:

description: >-

Collection of comments. The comment identifiers are allocated by the

MnS producer and used as key in the map.

type: object

additionalProperties:

$ref: '#/components/schemas/Comment'

#----- Definitions in TS 28.111 for TS 28.532 --------------------------#

resources-faultNrm:

oneOf:

- $ref: '#/components/schemas/AlarmList-Single'

#----- Definitions in TS 28.111 for TS 28.532 --------------------------#

<CODE ENDS>

*** END OF CHANGE 1 ***