3GPP TSG-SA5 Meeting #162	S5-253355

Goteborg, Sweden, 25 - 29 August 2025

Source:	NTT Docomo

Title:	Pseudo-CR on TR28.869 VNF generic OAM functions evaluation

Document for:	Approval

Agenda item:	6.19.6

Spec:	3GPP TR 28.869

Version:	1.5.1

Work Item:	FS_Cloud_OAM

Comments

<Proposals, reason for change, abstract, comments if necessary (optional)>

Proposed Changes

* * * First Change * * * *

### 5.1.1	Use case #1: Cloud-native VNF configuration management

#### …

#### 5.1.1.4	Evaluation of solutions

The solutions described in clause 5.1.1.3 can be used to support the configuration of the application and non- application parameters of NF Deployments.

The MnS provisioning service producer can and interact with VNF generic OAM functions to convey/apply configuration to NF Deployments. Communication between MnS Producers and the VNF generic OAM functions is through the interfaces specified in ETSI GS NFV-IFA 049 [2].

- The specification of these interfaces is out of scope of 3GPP.

- The MnS provisioning service producer needs to be able to be aware of the VNF generic OAM functions endpoints (i.e., serviceInstanceId – unique ID for that VNF Generic OAM function instance) and the serviceUri – the accessible endpoint for the VNF Generic OAM function.

For NF Deployments configuration of non-application parameters 3GPP specifications like 28.531, 28.533 need to consider also the interfaces specified in ETSI GS NFV-IFA 049, besides ETSI GS NFV-IFA008.

* * * Next Change * * * *

### 5.1.2	Use case #2：Cloud-native VNF policy management

…

#### 5.1.2.4	Evaluation of solutions

The solutions described in clause 5.1.2.3 can be used to support the policy enforcement for NF Deployments.

The MnS provisioning service producer can interact with VNF generic OAM functions to convey/apply NF deployment policies to NF Deployments. Communication between MnS Producers and the VNF generic OAM functions is through the interfaces specified in ETSI GS NFV-IFA 049 [2].

- The specification of these interfaces is out of scope of 3GPP.

- The MnS provisioning service producer needs to be able to be aware of the VNF generic OAM functions endpoints (i.e., serviceInstanceId – unique ID for that VNF Generic OAM function instance) and the serviceUri – the accessible endpoint for the VNF Generic OAM function.

* * * Next Change * * * *

### 5.1.3	Use case #3: Cloud-native VNF Traffic management

…

#### 5.1.3.4	Evaluation of solutions

The solutions described in clause 5.1.3.3 can be used to support the traffic management for NF Deployments.

The MnS provisioning service producer can interact with VNF generic functions for traffic management of NF Deployments. Communication between MnS Producers and the VNF generic OAM functions is through the interfaces specified in ETSI GS NFV-IFA 049 [2].

- The specification of these interfaces is out of scope of 3GPP.

- The MnS provisioning service producer needs to be able to be aware of the VNF generic OAM functions endpoints (i.e., serviceInstanceId – unique ID for that VNF Generic OAM function instance) and the serviceUri – the accessible endpoint for the VNF Generic OAM function.

The relation with 3GPP signalling traffic needs to be clarified.

* * * Next Change * * * *

### 5.1.4	Use case #4: Cloud-native VNF Upgrade

…

#### 5.1.4.4	Evaluation of solutions

The solutions described in clause 5.1.4.3 can be used to support the upgrade management for NF Deployments.

The MnS provisioning service producer can interact with VNF generic functions for upgrade management of NF Deployments. Communication between MnS Producers and the VNF generic OAM functions is through the interfaces specified in ETSI GS NFV-IFA 049 [2].

- The specification of these interfaces is out of scope of 3GPP.

- The MnS provisioning service producer needs to be able to be aware of the VNF generic OAM functions endpoints (i.e., serviceInstanceId – unique ID for that VNF Generic OAM function instance) and the serviceUri – the accessible endpoint for the VNF Generic OAM function.

* * * End of Changes * * * *