3GPP TSG-SA5 Meeting #162	S5-253442

Goteborg, Sweden, 25 - 29 August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0720 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-20 CR 28.552 Add a new measurement related to the time consumed by the RE-NWDAF to provide roaming data collection service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NWDAF_Ph3-OAM |  |  |  |  |  | Date: |  |  | 2025-8-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The time consumption of RE-NWDAF providing roaming data collection service needs to be monitored. It reflects how much time is consumed by an RE-NWDAF to prepare and provide the data information. With this measurement, the operators are able to understand the working status of the RE-NWDAF and make correct decisions on whether they need to provide the management actions to optimize the RE-NWDAF service performance. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new measurement “Time consumed by the RE-NWDAF to provide roaming data collection service” is introduced. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Operator will not be able to monitor the time consumed by the RE-NWDAF to provide roaming data collection service and to make correct decisions on whether they need to provide the management actions to optimize the RE-NWDAF service performance. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.18 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of First change

5.18	Performance measurements for NWDAF

5.18.X    Measurements related to RE-NWDAF Data Collection

5.18.X.5   Time consumed by the RE-NWDAF to provide roaming data collection service

a)	This measurement provides the time consumed by the RE-NWDAF to provide the roaming data collection service.

b)	DER(n=1)

c)	This measurement is obtained by the following method:

the time when the RE-NWDAF sends the notification related to the roaming data collection service subscription to the RE-NWDAF roaming data collection consumer, minus the time when the RE-NWDAF receives the corresponding roaming data collection service subscription from the RE-NWDAF roaming data collection consumer. In the case where there are multiple notifications for single subscription, this measurement is obtained by calculating the time when the RE-NWDAF sends the first notification related to the roaming data collection service subscription to the RE-NWDAF data collection consumer, minus the time when the RE-NWDAF receives the corresponding roaming data collection service subscription from the RE-NWDAF roaming data collection consumer.

d)	A real value

e)	DANS.RoamingDataSerTimeCons

f)	NWDAFFunction

g)	Valid for packet switched traffic

h)	5GS.

End of Changes