TSG-SA5 Meeting #162	S5-253478

# Goteborg, Sweden, 25 – 29 August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.623 | CR | 0563 | rev | - | Current version: | 18.11.0 |  |
| For HELP on using this form: compr ehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 28.623 Update area scope definition for NPN support (stage 3) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMDT_Ph2 |  |  |  |  |  | Date: |  |  | 2025-08-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Clause 9.3.1.169 of TS 38.413 specifies the MDT configuration parameters for NR. There are two relevant IEs in this datatype regarding area scope: - Area Scope of MDT (CHOICE), which includes 8 choices: cell based, TA based, TAI based, PLMN wide, PNI-NPN based MDT, SNPN Cell Based MDT, SNPN TAI Based MDT, and SNPN Based MDT. - PNI-NPN Area Scope of MDT, which includes list of CAG cells. This IE can be combined with the choices above, when these represent non-CAG cells (cells only providing public access). In the context of area scope for MDT, when comparing attributes between MDT Configuration – NR (TS 38.413, under RAN3 remit) and AreaScope (TS 28.622, under SA5 remit), one can notice that the AreaScope presents different problems: - It does not allow supporting RAN3 scenarios involving only cells providing non-public access, i.e. when the choice is one of the following: “PNI-“NPN based MDT, “SNPN Cell Based MDT”, “SNPN TAI Based MDT”, and “SNPN Based MDT”. The reason is that AreaScope definition always forces the consumer to specify a value for one of the CHOICE attributes: nrCellIdList (which corresponds to CHOICE = cell based in 38.413), tacList (which corresponds to CHOICE = TA based in 38.413) or taiList (which corresponds to CHOICE = TAI based in 38.413). - It does not allow supporting RAN3 scenarios involving - It allows combining “tacList” with SNPN feature, which is a RAN3 scenario that is not supported. - It allows combining EUTRA with NPN features, which is not permitted. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to reformat the AreaScope datatype, so that the scenarios and combinations allowed align with RAN3 scenarios. SS in YANG and YAML. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment between RAN3 and SA5 for area scope, with SA5 lacking support of one RAN3 scenarios, while enabling non-allowed scenarios. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Forge |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | X |  | O&M Specifications |  |  |  | TS 28.622 CR 0583 |  |  |
| Other comments: |  | YAML: Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1854 at commit 03cc6ef19f00fa9f6c0bfd7eb981a0746cefc121 YANG: Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1855 at commit c28826f3bb62d3c6d40dd3eff2665be25987a867 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

*** START OF CHANGE 1 ***

*** OpenAPI/TS28623_GenericNrm.yaml ***

<CODE BEGINS>

openapi: 3.0.1

info:

title: Generic NRM

version: 18.9.0

description: >-

OAS 3.0.1 definition of the Generic NRM

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.623; Generic NRM

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.623/

paths: {}

components:

schemas:

#-------- Definition of types-----------------------------------------------------

RegistrationState:

type: string

enum:

- REGISTERED

- DEREGISTERED

VnfParameter:

type: object

properties:

vnfInstanceId:

type: string

vnfdId:

type: string

flavourId:

type: string

autoScalable:

type: boolean

PeeParameter:

type: object

properties:

siteIdentification:

type: string

siteDescription:

type: string

siteLatitude:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Latitude'

siteLongitude:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Longitude'

siteAltitude:

type: number

format: float

equipmentType:

type: string

environmentType:

type: string

powerInterface:

type: string

Operation:

type: object

properties:

name:

type: string

allowedNFTypes:

$ref: '#/components/schemas/NFType'

operationSemantics:

$ref: '#/components/schemas/OperationSemantics'

NFType:

type: string

description: ' NF name defined in TS 23.501 or TS 29.510'

enum:

- NRF

- UDM

- AMF

- SMF

- AUSF

- NEF

- PCF

- SMSF

- NSSF

- UDR

- LMF

- GMLC

- 5G_EIR

- SEPP

- UPF

- N3IWF

- AF

- UDSF

- DN

- BSF

- CHF

- NWDAF

- PCSCF

- CBCF

- HSS

- UCMF

- SOR_AF

- SPAF

- MME

- SCSAS

- SCEF

- SCP

- NSSAAF

- ICSCF

- SCSCF

- DRA

- IMS_AS

- AANF

- 5G_DDNMF

- NSACF

- MFAF

- EASDF

- DCCF

- MB_SMF

- TSCTSF

- ADRF

- GBA_BSF

- CEF

- MB_UPF

- NSWOF

- PKMF

- MNPF

- SMS_GMSC

- SMS_IWMSC

- MBSF

- MBSTF

- PANF

- TNGF

- W_AGF

- TWIF

- TSN_AF

OperationSemantics:

type: string

enum:

- REQUEST_RESPONSE

- SUBSCRIBE_NOTIFY

SAP:

type: object

properties:

host:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/HostAddr'

port:

type: integer

NFServiceType:

type: string

enum:

- Namf_Communication

- Namf_EventExposure

- Namf_MT

- Namf_Location

- Nsmf_PDUSession

- Nsmf_EventExposure

- Others

TransportProtocol:

anyOf:

- type: string

enum:

- TCP

- type: string

SupportedPerfMetricGroup:

type: object

properties:

performanceMetrics:

type: array

items:

type: string

granularityPeriods:

type: array

items:

type: integer

minimum: 1

reportingMethods:

type: array

items:

type: string

enum:

- FILE_BASED_LOC_SET_BY_PRODUCER

- FILE_BASED_LOC_SET_BY_CONSUMER

- STREAM_BASED

reportingPeriods:

type: array

items:

type: integer

minimum: 1

ReportingCtrl:

oneOf:

- type: object

properties:

fileReportingPeriod:

type: integer

- type: object

properties:

fileReportingPeriod:

type: integer

notificationRecipientAddress:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Uri'

- type: object

properties:

fileReportingPeriod:

type: integer

fileLocation:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Uri'

- type: object

properties:

streamTarget:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Uri'

ProcessMonitor:

description: >-

This data type is the "ProcessMonitor" data type without specialisations.

type: object

properties:

id:

type: string

status:

type: string

enum:

- NOT_STARTED

- RUNNING

- FINSHED

- FAILED

- PARTIALLY_FAILED

- CANCELLING

- CANCELLED

progressPercentage:

type: integer

minimum: 0

maximum: 100

progressStateInfo:

type: string

resultStateInfo:

type: string

startTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

endTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

timer:

type: integer

AreaScope:

oneOf:

- type: array

items:

$ref: '#/components/schemas/EutraCellId'

minItems: 1

maxItems: 32

- type: object

properties:

nrCellId:

type: array

items:

$ref: '#/components/schemas/NrCellId'

minItems: 1

maxItems: 32

nPNIdentityList:

type: array

items:

$ref: '#/components/schemas/NpnId-Type'

- type: object

properties:

tacList:

type: array

items:

$ref: '#/components/schemas/Tac'

minItems: 1

maxItems: 8

cAGIdList:

type: array

items:

type: string

maxItems: 256

- type: object

properties:

taiList:

type: array

items:

$ref: '#/components/schemas/Tai'

minItems: 1

maxItems: 8

nPNIdentityList:

type: array

items:

$ref: '#/components/schemas/NpnId-Type'

- type: array

items:

$ref: '#/components/schemas/NpnId-Type'

minItems: 1

Tai:

type: object

properties:

mcc:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Mcc'

mnc:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Mnc'

tac:

$ref: '#/components/schemas/Tac'

Tac:

type: string

pattern: '(^[A-Fa-f0-9]{4}$)|(^[A-Fa-f0-9]{6}$)'

EutraCellId:

type: string

pattern: '^[A-Fa-f0-9]{7}$'

NrCellId:

type: string

pattern: '^[A-Fa-f0-9]{9}$'

IpAddr:

oneOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/Ipv4Addr'

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/Ipv6Addr'

SchedulingTime:

oneOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/TimeWindow'

- type: object

properties:

timeIntervals:

type: array

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/TimeInterval'

- type: object

properties:

timeIntervals:

type: array

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/TimeInterval'

daysOfWeek:

type: array

items:

type: string

enum:

- MONDAY

- TUESDAY

- WEDNESDAY

- THURSDAY

- FRIDAY

- SATURDAY

- SUNDAY

- type: object

properties:

timeIntervals:

type: array

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/TimeInterval'

daysOfMonth:

type: array

items:

type: integer

minimum: 0

maximum: 31

NpnId-Type:

type: object

properties:

plmnId:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/PlmnId'

cAGIdList:

type: array

items:

type: string

minItems: 1

maxItems: 256

nIDList:

type: array

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Nid'

minItems: 1

maxItems: 16

oneOf:

- required: [plmnId, cAGIdList]

- required: [plmnId, nIDList]

#-------- Definition of abstract IOC Top -----------------------------------------

Top-Attr:

#  This definition will be deprecated, when all occurances of Top-Attr

#  are replaced by Top.

type: object

properties:

id:

type: string

nullable: true

objectClass:

type: string

objectInstance:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

VsDataContainer:

$ref: '#/components/schemas/VsDataContainer-Multiple'

required:

- id

Top:

type: object

properties:

id:

type: string

nullable: true

objectClass:

type: string

objectInstance:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

VsDataContainer:

$ref: '#/components/schemas/VsDataContainer-Multiple'

required:

- id

#-------- Definition of IOCs with new name-containments defined in other TS ------

SubNetwork-Attr:

type: object

properties:

dnPrefix:

type: string

userLabel:

type: string

userDefinedNetworkType:

type: string

setOfMcc:

type: array

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Mcc'

priorityLabel:

type: integer

supportedPerfMetricGroups:

type: array

items:

$ref: '#/components/schemas/SupportedPerfMetricGroup'

supportedTraceMetrics:

type: array

items:

type: string

ManagedElement-Attr:

type: object

properties:

dnPrefix:

type: string

managedElementTypeList:

type: array

items:

type: string

userLabel:

type: string

locationName:

type: string

managedBy:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

vendorName:

type: string

userDefinedState:

type: string

swVersion:

type: string

priorityLabel:

type: integer

supportedPerfMetricGroups:

type: array

items:

$ref: '#/components/schemas/SupportedPerfMetricGroup'

supportedTraceMetrics:

type: array

items:

type: string

SubNetwork-ncO:

type: object

properties:

SubNetwork:

$ref: '#/components/schemas/SubNetwork-Multiple'

ManagedElement:

$ref: '#/components/schemas/ManagedElement-Multiple'

ManagementNode:

$ref: '#/components/schemas/ManagementNode-Multiple'

MnsAgent:

$ref: '#/components/schemas/MnsAgent-Multiple'

MeContext:

$ref: '#/components/schemas/MeContext-Multiple'

PerfMetricJob:

$ref: 'TS28623_PmControlNrm.yaml#/components/schemas/PerfMetricJob-Multiple'

ThresholdMonitor:

$ref: 'TS28623_ThresholdMonitorNrm.yaml#/components/schemas/ThresholdMonitor-Multiple'

TraceJob:

$ref: 'TS28623_TraceControlNrm.yaml#/components/schemas/TraceJob-Multiple'

ManagementDataCollection:

$ref: 'TS28623_ManagementDataCollectionNrm.yaml#/components/schemas/ManagementDataCollection-Multiple'

NtfSubscriptionControl:

$ref: 'TS28623_SubscriptionControlNrm.yaml#/components/schemas/NtfSubscriptionControl-Multiple'

AlarmList:

$ref: 'TS28111_FaultNrm.yaml#/components/schemas/AlarmList-Single'

FileDownloadJob:

$ref: 'TS28623_FileManagementNrm.yaml#/components/schemas/FileDownloadJob-Multiple'

Files:

$ref: 'TS28623_FileManagementNrm.yaml#/components/schemas/Files-Multiple'

MnsRegistry:

$ref: 'TS28623_MnSRegistryNrm.yaml#/components/schemas/MnsRegistry-Single'

Scheduler:

$ref: '#/components/schemas/Scheduler-Multiple'

ConditionMonitor:

$ref: '#/components/schemas/ConditionMonitor-Multiple'

SupportedNotifications:

$ref: '#/components/schemas/SupportedNotifications-Single'

QMCJobs:

$ref: 'TS28623_QoEMeasurementCollectionNrm.yaml#/components/schemas/QMCJob-Multiple'

ManagedElement-ncO:

type: object

properties:

MnsAgent:

$ref: '#/components/schemas/MnsAgent-Multiple'

PerfMetricJob:

$ref: 'TS28623_PmControlNrm.yaml#/components/schemas/PerfMetricJob-Multiple'

ThresholdMonitor:

$ref: 'TS28623_ThresholdMonitorNrm.yaml#/components/schemas/ThresholdMonitor-Multiple'

TraceJob:

$ref: 'TS28623_TraceControlNrm.yaml#/components/schemas/TraceJob-Multiple'

NtfSubscriptionControl:

$ref: 'TS28623_SubscriptionControlNrm.yaml#/components/schemas/NtfSubscriptionControl-Multiple'

AlarmList:

$ref: 'TS28111_FaultNrm.yaml#/components/schemas/AlarmList-Single'

FileDownloadJob:

$ref: 'TS28623_FileManagementNrm.yaml#/components/schemas/FileDownloadJob-Multiple'

Files:

$ref: 'TS28623_FileManagementNrm.yaml#/components/schemas/Files-Multiple'

Scheduler:

$ref: '#/components/schemas/Scheduler-Multiple'

ConditionMonitor:

$ref: '#/components/schemas/ConditionMonitor-Multiple'

SupportedNotifications:

$ref: '#/components/schemas/SupportedNotifications-Single'

QMCJobs:

$ref: 'TS28623_QoEMeasurementCollectionNrm.yaml#/components/schemas/QMCJob-Multiple'

#-------- Definition of abstract IOCs --------------------------------------------

ManagedFunction-Attr:

type: object

properties:

userLabel:

type: string

vnfParametersList:

type: array

items:

$ref: '#/components/schemas/VnfParameter'

peeParametersList:

type: array

items:

$ref: '#/components/schemas/PeeParameter'

priorityLabel:

type: integer

supportedPerfMetricGroups:

type: array

items:

$ref: '#/components/schemas/SupportedPerfMetricGroup'

supportedTraceMetrics:

type: array

items:

type: string

EP_RP-Attr:

type: object

properties:

userLabel:

type: string

farEndEntity:

type: string

supportedPerfMetricGroups:

type: array

items:

$ref: '#/components/schemas/SupportedPerfMetricGroup'

ManagedFunction-ncO:

type: object

properties:

PerfMetricJob:

$ref: 'TS28623_PmControlNrm.yaml#/components/schemas/PerfMetricJob-Multiple'

ThresholdMonitor:

$ref: 'TS28623_ThresholdMonitorNrm.yaml#/components/schemas/ThresholdMonitor-Multiple'

ManagedNFService:

$ref: '#/components/schemas/ManagedNFService-Multiple'

TraceJob:

$ref: 'TS28623_TraceControlNrm.yaml#/components/schemas/TraceJob-Multiple'

#-------- Definition of concrete IOCs --------------------------------------------

VsDataContainer-Single:

type: object

properties:

id:

type: string

attributes:

type: object

properties:

vsDataType:

type: string

vsDataFormatVersion:

type: string

vsData:

nullable: true

VsDataContainer:

$ref: '#/components/schemas/VsDataContainer-Multiple'

ManagedNFService-Single:

allOf:

- $ref: '#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

userLabel:

type: string

nFServiceType:

$ref: '#/components/schemas/NFServiceType'

sAP:

$ref: '#/components/schemas/SAP'

operations:

type: array

items:

$ref: '#/components/schemas/Operation'

administrativeState:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AdministrativeState'

operationalState:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/OperationalState'

usageState:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/UsageState'

registrationState:

$ref: '#/components/schemas/RegistrationState'

ManagementNode-Single:

allOf:

- $ref: '#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

userLabel:

type: string

managedElements:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

vendorName:

type: string

userDefinedState:

type: string

locationName:

type: string

swVersion:

type: string

MnsAgent:

$ref: '#/components/schemas/MnsAgent-Multiple'

MnsAgent-Single:

allOf:

- $ref: '#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

systemDN:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

MeContext-Single:

allOf:

- $ref: '#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

dnPrefix:

type: string

Scheduler-Single:

allOf:

- $ref: '#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

schedulingTimes:

type: array

items:

$ref: '#/components/schemas/SchedulingTime'

minItems: 1

schedulerStatus:

type: boolean

ConditionMonitor-Single:

allOf:

- $ref: '#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

condition:

type: string

conditionStatus:

type: boolean

SupportedNotifications-Single:

allOf:

- $ref: '#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

notificationTypes:

type: array

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationType'

notificationProtocols:

type: array

items:

type: string

enum:

- HTTP

- HTTP_VES_ENCAPS

minItems: 1

SubNetwork-Single:

allOf:

- $ref: '#/components/schemas/Top'

- $ref: '#/components/schemas/SubNetwork-Attr'

- $ref: '#/components/schemas/SubNetwork-ncO'

- $ref: 'TS28104_MdaNrm.yaml#/components/schemas/SubNetwork-ncO-MdaNrm'

- $ref: 'TS28105_AiMlNrm.yaml#/components/schemas/SubNetwork-ncO-AiMlNrm'

- $ref: 'TS28312_IntentNrm.yaml#/components/schemas/SubNetwork-ncO-IntentNrm'

- $ref: 'TS28317_RanScNrm.yaml#/components/schemas/SubNetwork-ncO-RanScNrm'

- $ref: 'TS28536_CoslaNrm.yaml#/components/schemas/SubNetwork-ncO-CoslaNrm'

- $ref: 'TS28538_EdgeNrm.yaml#/components/schemas/SubNetwork-ncO-EdgeNrm'

- $ref: 'TS28541_SliceNrm.yaml#/components/schemas/SubNetwork-ncO-SliceNrm'

- $ref: 'TS28541_NrNrm.yaml#/components/schemas/SubNetwork-ncO-NrNrm'

- $ref: 'TS28541_5GcNrm.yaml#/components/schemas/SubNetwork-ncO-5GcNrm'

- $ref: 'TS28318_DsoNrm.yaml#/components/schemas/SubNetwork-ncO-DsoNrm'

ManagedElement-Single:

allOf:

- $ref: '#/components/schemas/Top'

- $ref: '#/components/schemas/ManagedElement-Attr'

- $ref: '#/components/schemas/ManagedElement-ncO'

- $ref: 'TS28104_MdaNrm.yaml#/components/schemas/ManagedElement-ncO-MdaNrm'

- $ref: 'TS28105_AiMlNrm.yaml#/components/schemas/ManagedElement-ncO-AiMlNrm'

- $ref: 'TS28536_CoslaNrm.yaml#/components/schemas/ManagedElement-ncO-CoslaNrm'

- $ref: 'TS28541_NrNrm.yaml#/components/schemas/ManagedElement-ncO-NrNrm'

- $ref: 'TS28541_5GcNrm.yaml#/components/schemas/ManagedElement-ncO-5GcNrm'

NrmRoot:

oneOf:

- type: object

properties:

SubNetwork:

$ref: '#/components/schemas/SubNetwork-Multiple'

- type: object

properties:

ManagedElement:

$ref: '#/components/schemas/ManagedElement-Multiple'

#-------- Definition of YAML arrays for name-contained IOCs ----------------------

VsDataContainer-Multiple:

type: array

items:

$ref: '#/components/schemas/VsDataContainer-Single'

ManagedNFService-Multiple:

type: array

items:

$ref: '#/components/schemas/ManagedNFService-Single'

ManagementNode-Multiple:

type: array

items:

$ref: '#/components/schemas/ManagementNode-Single'

MnsAgent-Multiple:

type: array

items:

$ref: '#/components/schemas/MnsAgent-Single'

MeContext-Multiple:

type: array

items:

$ref: '#/components/schemas/MeContext-Single'

Scheduler-Multiple:

type: array

items:

$ref: '#/components/schemas/Scheduler-Single'

ConditionMonitor-Multiple:

type: array

items:

$ref: '#/components/schemas/ConditionMonitor-Single'

SubNetwork-Multiple:

type: array

items:

$ref: '#/components/schemas/SubNetwork-Single'

ManagedElement-Multiple:

type: array

items:

$ref: '#/components/schemas/ManagedElement-Single'

#-------- Definitions in TS 28.623 for TS 28.532 ---------------------------------

resources-genericNrm:

oneOf:

- $ref: '#/components/schemas/NrmRoot'

- $ref: '#/components/schemas/VsDataContainer-Single'

- $ref: '#/components/schemas/ManagementNode-Single'

- $ref: '#/components/schemas/MnsAgent-Single'

- $ref: '#/components/schemas/MeContext-Single'

- $ref: '#/components/schemas/ManagedNFService-Single'

- $ref: '#/components/schemas/Scheduler-Single'

- $ref: '#/components/schemas/ConditionMonitor-Single'

- $ref: '#/components/schemas/SupportedNotifications-Single'

- $ref: '#/components/schemas/SubNetwork-Single'

- $ref: '#/components/schemas/ManagedElement-Single'

<CODE ENDS>

*** END OF CHANGE 1 ***

Next Change

*** START OF CHANGE 1 ***

*** yang-models/_3gpp-common-trace.yang ***

<CODE BEGINS>

module _3gpp-common-trace {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-common-trace";

prefix "trace3gpp";

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-yang-types {prefix types3gpp; }

import _3gpp-common-yang-extensions {prefix yext3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-files { prefix files3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Trace handling

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.623

Generic Network Resource Model (NRM)

Integration Reference Point (IRP);

Solution Set (SS) definitions

3GPP TS 28.622

Generic Network Resource Model (NRM)

Integration Reference Point (IRP);

Information Service (IS)" ;

revision 2025-08-06 { reference "CR-0563" ; }

revision 2025-05-06 { reference "CR-0531 CR-0535" ; }

revision 2025-02-06 { reference "CR-0503 CR-0507" ; }

revision 2025-02-05 { reference "CR-0460" ; }

revision 2024-11-05 { reference "CR-0470" ; }

revision 2024-10-06 { reference "CR-0459"; }

revision 2024-08-06 { reference "CR-0376 CR-0378 CR-0420"; }

revision 2024-05-12 { reference "CR-0401"; }

revision 2024-05-06 { reference CR-0359; }

revision 2024-04-06 { reference "CR-0342"; }

revision 2024-01-29 { reference "CR-0316"; }

revision 2023-11-06 { reference "CR-0290 CR-0294"; }

revision 2023-11-03 { reference CR-0302 ; }

revision 2023-09-18 { reference CR-0271 ; }

revision 2023-08-10 { reference CR-0261; }

revision 2023-04-26 { reference CR-0250; }

revision 2023-02-18 { reference "CR-0234"; }

revision 2023-02-16 { reference "CR-0233"; }

revision 2022-09-30 { reference CR-0191 ; }

revision 2022-04-27 { reference "CR-0159"; }

revision 2021-10-18 { reference "CR-0139"; }

revision 2021-07-22 { reference "CR-0137"; }

revision 2021-01-25 { reference "CR-0122"; }

revision 2020-11-16 { reference "CR-0117"; }

revision 2020-08-06 { reference "CR-0102"; }

feature FilesUnderTraceJob {

description "Files shall be contained under TraceJob";

}

grouping FreqInfoGrp {

description "Represents the FreqInfo dataType.

This <<dataType>> defines the RF reference frequency and the frequency

operating bands used in a cell for a given direction (UL or DL) in FDD

or for both UL and DL directions in TDD";

leaf arfcn {

type uint32 {

range 0..3279165;

}

mandatory true;

description "RF Reference Frequency as defined in TS 38.104,

clause 5.4.2.1. The frequency provided identifies the absolute

frequency position of the reference resource block (Common RB 0)

of the carrier. Its lowest subcarrier is also known as Point A.";

}

leaf-list freqBands {

type uint32 {

range 1..1024;

}

min-elements 1;

description "List of NR frequency operating bands. Primary NR

Operating Band as defined in TS 38.104, clause 5.4.2.3.

The value 1 corresponds to n1, value 2 corresponds to NR operating

band n2, etc.";

}

}

grouping AreaConfigGrp {

description "Represents the AreaConfig dataType.

This <<dataType>> defines the area for which measurement logging should

be performed. It is described by a list of cells and a list of

frequencies.";

list freqInfo {

key arfcn;

min-elements 1;

max-elements 32;

description "It specifies the carrier frequency and bands used in

a cell.";

uses FreqInfoGrp ;

}

leaf-list pciList {

type uint32 {

range 0..1007;

}

min-elements 1;

max-elements 32;

description "List of neighbour cells subject for MDT scope.";

}

}

grouping AreaScopeGrp {

description "Represents the AreaScope dataType.

This <<dataType>> defines an area scope";

choice AreaScopeChoice {

case eutran-only {

leaf-list eutraCellIdList {

type string;

min-elements 1;

max-elements 32;

description "List of E-UTRAN cells identified by E-UTRAN-CGI";

}

}

case tac {

leaf-list tacList {

type types3gpp:Tac;

min-elements 1;

max-elements 8;

description "Tracking Area Code list";

}

leaf-list cAGIdList {

type string;

max-elements 256;

description "It identifies a CAG list containing up to

256 CAG-identifiers per UE or up to 12 CAG-identifiers

per cell, see TS 38.331 [38]. CAG ID is used to combine

with PLMN ID to identify a PNI-NPN.AG ID is a hexadecimal

range with size 32 bit.";

}

}

case else {

list nPNIdentityList {

description "list of NPN IDs of in NR. It is either

a list of PNI-NPNs identified by CAG ID with

associated plmn-Identity or a list of SNPN

identified by Network ID with associated plmn-Identity";

key idx;

min-elements 1;

uses types3gpp:NpnIdGrp;

leaf idx { type string; }

}

leaf-list nrCellIdList {

must 'not(../taiList)';

type string;

max-elements 32;

description "List of NR cells identified by NG-RAN CGI";

}

list taiList {

must 'not(../nrCellIdList)';

description "Tracking Area Identity list";

key idx;

max-elements 8;

leaf idx { type string; }

uses types3gpp:TaiGrp;

}

}

}

}

grouping NpnIdGrp {

description "Represents the NpnId dataType.";

list plmnId {

key "mcc mnc";

description "It specifies the PLMN Id of the NPN network.";

uses types3gpp:PLMNId;

max-elements 1;

}

choice npnidChoice {

leaf-list cAGIdList {

type string;

max-elements 256;

description "It identifies a CAG list containing up to 256

CAG-identifiers per UE or up to 12 CAG-identifiers per cell,

see TS 38.331.

CAG ID is used to combine with PLMN ID to identify a PNI-NPN.

CAG ID is a hexadecimal range with size 32 bit.";

}

leaf-list nIDList {

type string;

max-elements 16;

description "It identifies a list of NIDs containing up to 16 NIDs,

see TS 38.331.

NID is used to combine with PLMN ID to identify an SNPN.

NID is a hexadecimal range with size 44 bit.";

}

}

}

grouping ExcessPacketDelayThresholdsGrp {

description "Represents the ExcessPacketDelayThresholds dataType.

This <<dataType>> defines a excess packet delay threshold information

to enable the calculation of the PDCP Excess Packet Delay in the

uplink in case of M6 uplink measurements are requested. The excess

packet delay threshold information is specified with the 5QI value

and excess packet delay threshold value.";

leaf fiveQIValue {

type uint8;

mandatory true;

description "It indicates 5QI value.";

}

leaf excessPacketDelayThresholdValue {

type decimal64 {

fraction-digits 2;

range 0.25|0.5|1|2|4|5|10|20|30|40|50|60|70|80|90|100|150|300|500 ;

}

mandatory true;

units milliseconds;

description "Value of excess packet delay threshold

for M6 UL measurement in milliseconds.";

}

}

grouping TraceReferenceGrp {

description "Represents the TraceReference dataType.

This <<dataType>> defines a globally unique identifier, which uniquely

identifies the Trace Session that is created by the TraceJob. It is

composed of the MCC, MNC (resulting in PLMN identifier) and the

trace identifier.";

uses types3gpp:PLMNId;  // mcc+mnc

leaf traceId {

type string;

mandatory true;

description "An identifier, which identifies the Trace

(together with MCC and MNC). This is a 3 byte Octet String.";

}

}

grouping MbsfnAreaGrp {

description "Represents the MbsfnArea dataType.

This <<dataType>> defines a MBSFN area. It is composed of the MBSFN Area

identifier and the carrier frequency (EARFCN).";

leaf mbsfnAreaId {

type uint32 {

range 1..max;

}

mandatory true;

description "MBSFN Area Identifier";

}

leaf earfcn{

type uint32 {

range 1..max;

}

mandatory true;

description "Carrier Frequency";

}

}

grouping TraceConfigGrp {

description "Defines the configuration parameters of TraceJob

which are specific for Trace or combined Trace and Immediate MDT.

The attribute listOfNeTypes specifies the network elements to be

traced. The optional attribute listOfInterfaces allows to specify

the individual interfaces of the network elements to be recorded.

The attribute traceDepth allows to configure the level of detail

of the information which shall be recorded.  For trace the reporting

is event based, where the triggering event is configured with

attribute triggeringEvents. For each triggering event the first and

last message (start/stop triggering event) to record  are specified.";

list listOfInterfaces {

key idx;

description "Specifies the interfaces that need to be traced in the given

ManagedEntityFunction.The attribute is applicable only for Trace. In

case this attribute is not used, it carries a null semantic.";

reference "Clause 5.5 of 3GPP TS 32.422 for additional details on the

allowed values.";

leaf idx { type uint32 ; }

leaf-list MSCServerInterfaces {

type enumeration {

enum A ;

enum Iu-CS ;

enum Mc ;

enum MAP-G ;

enum MAP-B ;

enum MAP-E ;

enum MAP-F ;

enum MAP-D ;

enum MAP-C ;

enum CAP ;

}

}

leaf-list MGWInterfaces {

type enumeration {

enum Mc ;

enum Nb-UP ;

enum Iu-UP ;

}

}

leaf-list RNCInterfaces {

type enumeration {

enum Iu-CS ;

enum Iu-PS ;

enum Iur ;

enum Iub ;

enum Uu ;

}

}

leaf-list SGSNInterfaces {

type enumeration {

enum Gb ;

enum Iu-PS ;

enum Gn ;

enum MAP-Gr ;

enum MAP-Gd ;

enum MAP-Gf ;

enum Ge ;

enum Gs ;

enum S6d ;

enum S4 ;

enum S3 ;

enum S13 ;

}

}

leaf-list GGSNInterfaces {

type enumeration {

enum Gn ;

enum Gi ;

enum Gmb ;

}

}

leaf-list S-CSCFInterfaces {

type enumeration {

enum Mw ;

enum Mg ;

enum Mr ;

enum Mi ;

}

}

leaf-list P-CSCFInterfaces {

type enumeration {

enum Gm ;

enum Mw ;

}

}

leaf-list I-CSCFInterfaces {

type enumeration {

enum Cx ;

enum Dx ;

enum Mg ;

enum Mw ;

}

}

leaf-list MRFCInterfaces {

type enumeration {

enum Mp ;

enum Mr ;

}

}

leaf-list MGCFInterfaces {

type enumeration {

enum Mg ;

enum Mj ;

enum Mn ;

}

}

leaf-list IBCFInterfaces {

type enumeration {

enum Ix ;

enum Mx ;

}

}

leaf-list E-CSCFInterfaces {

type enumeration {

enum Mw ;

enum Ml ;

enum Mm ;

enum Mi-Mg ;

}

}

leaf-list BGCFInterfaces {

type enumeration {

enum Mi ;

enum Mj ;

enum Mk ;

}

}

leaf-list ASInterfaces {

type enumeration {

enum Dh ;

enum Sh ;

enum ISC ;

enum Ut ;

}

}

leaf-list HSSInterfaces {

type enumeration {

enum MAP-C ;

enum MAP-D ;

enum Gc ;

enum Gr ;

enum Cx ;

enum S6d ;

enum S6a ;

enum Sh ;

}

}

leaf-list EIRInterfaces {

type enumeration {

enum MAP-F ;

enum S13 ;

enum MAP-Gf ;

}

}

leaf-list BM-SCInterfaces {

type enumeration {

enum Gmb ;

}

}

leaf-list MMEInterfaces {

type enumeration {

enum S1-MME ;

enum S3 ;

enum S6a ;

enum S10 ;

enum S11 ;

enum S13 ;

}

}

leaf-list SGWInterfaces {

type enumeration {

enum S4 ;

enum S5 ;

enum S8 ;

enum S11 ;

enum Gxc ;

}

}

leaf-list PDN_GWInterfaces {

type enumeration {

enum S2a ;

enum S2b ;

enum S2c ;

enum S5 ;

enum S6b ;

enum Gx ;

enum S8 ;

enum SGi ;

}

}

leaf-list eNBInterfaces {

type enumeration {

enum S1-MME ;

enum X2 ;

}

}

leaf-list en-gNBInterfaces {

type enumeration {

enum S1-MME ;

enum X2 ;

enum Uu ;

enum F1-C ;

enum E1 ;

}

}

leaf-list AMFInterfaces {

type enumeration {

enum N1 ;

enum N2 ;

enum N8 ;

enum N11 ;

enum N12 ;

enum N14 ;

enum N15 ;

enum N20 ;

enum N22 ;

enum N26 ;

}

}

leaf-list AUSFInterfaces {

type enumeration {

enum N12 ;

enum N13 ;

}

}

leaf-list NEFInterfaces {

type enumeration {

enum N29 ;

enum N30 ;

enum N33 ;

}

}

leaf-list NRFInterfaces {

type enumeration {

enum N27 ;

}

}

leaf-list NSSFInterfaces {

type enumeration {

enum N22 ;

enum N31 ;

}

}

leaf-list PCFInterfaces {

type enumeration {

enum N5 ;

enum N7 ;

enum N15 ;

}

}

leaf-list SMFInterfaces {

type enumeration {

enum N4 ;

enum N7 ;

enum N10 ;

enum N11 ;

enum S5-C ;

enum N38 ;

enum N16 ;

enum N16a ;

}

}

leaf-list SMSFInterfaces {

type enumeration {

enum N20 ;

enum N21 ;

}

}

leaf-list UDMInterfaces {

type enumeration {

enum N8 ;

enum N10 ;

enum N13 ;

enum N21 ;

}

}

leaf-list UPFInterfaces {

type enumeration {

enum N4 ;

}

}

leaf-list ng-eNBInterfaces {

type enumeration {

enum NG-C ;

enum Xn-C ;

enum Uu ;

}

}

leaf-list gNB-CU-CPInterfaces {

type enumeration {

enum NG-C ;

enum Xn-C ;

enum Uu ;

enum F1-C ;

enum E1 ;

enum X2-C ;

}

}

leaf-list gNB-CU-UPInterfaces {

type enumeration {

enum E1 ;

}

}

leaf-list gNB-DUInterfaces {

type enumeration {

enum F1-C ;

}

}

}

leaf-list listOfNeTypes {

type enumeration {

enum MSC_SERVER;

enum SGSN;

enum MGW;

enum GGSN;

enum RNC;

enum BM_SC;

enum MME;

enum SGW;

enum PGW;

enum ENB;

enum EN_GNB;

enum GNB_CU_CP;

enum GNB_CU_UP;

enum GNB_DU;

}

description "Specifies the network element types where the trace should

be activated. The attribute is applicable only for Trace with

Signalling Based Trace activation. ";

reference "Clause 5.4 of 3GPP TS 32.422.";

}

leaf traceDepth {

when '../../jobType = "TRACE_ONLY"'

+  ' or ../../jobType = "IMMEDIATE_MDT_AND_TRACE"';

type enumeration  {

enum MINIMUM;

enum MEDIUM;

enum MAXIMUM;

enum VENDORMINIMUM;

enum VENDORMEDIUM;

enum VENDORMAXIMUM;

}

default MAXIMUM;

description "It specifies the trace depth. The attribute is

applicable only for Trace.";

reference "Clause 5.3 of 3GPP TS 32.422.";

}

list triggeringEvents {

key idx;

description "It specifies the triggering event parameter of the trace

session. The attribute is applicable only for Trace.";

reference "Clause 5.1 of 3GPP TS 32.422";

leaf idx { type uint32; }

max-elements 1;

leaf-list MSC_SERVER {

type enumeration {

enum MO_MT_CALLS;

enum MO_MT_SMS;

enum LU_IMSIattach_IMSIdetach;

enum SS;

}

}

leaf-list SGSN {

type enumeration {

enum PDPcontext;

enum MO_MT_SMS;

enum RAU_GPRSattach_GPRSdetach;

enum MBMScontext;

}

}

leaf-list MGW {

type enumeration {

enum CONTEXT;

}

}

leaf-list GGSN {

type enumeration {

enum PDPcontext;

enum MBMScontext;

}

}

leaf-list IMS {

type enumeration {

enum SIPsession_StandaloenTransaction;

}

}

leaf-list BM_SC {

type enumeration {

enum MBMSactivation;

}

}

leaf-list MME {

type enumeration {

enum UEinitiatedPDNconnectivityRequest;

enum ServiceRequest;

enum InitialAttach_TAU_Detach;

enum UEInitiatedPDNdisconnection;

enum BearerActivationModificationDeletion;

enum Handover;

}

}

leaf-list SGW {

type enumeration {

enum PDNconnectionCreation;

enum PDNconnectionTermination;

enum BearerActivationModificationDeletion;

}

}

leaf-list PGW {

type enumeration {

enum PDNconnectionCreation;

enum PDNconnectionTermination;

enum BearerActivationModificationDeletion;

}

}

leaf-list AMF {

type enumeration {

enum Registration;

enum ServiceRequest;

enum Handover;

enum UEderegistration;

enum NetworkDeregistration;

enum UEMobilityFromEPC;

enum UEMobilityToEPC;

enum CovergedChargingService;

}

}

leaf-list SMF {

type enumeration {

enum PDUsessionEstablishment;

enum PDUsessionModification;

enum PDUsessionRelease;

enum PDUsessionUPactivationDeactivation;

enum MobilityBtw3gppAndN3gppTo5GC;

enum MobilityFromEpc;

enum MobilityBtwISMForVSMF;

enum CovergedChargingService;

enum OfflineOnlyCharging;

}

}

leaf-list PCF {

type enumeration {

enum AMpolicy;

enum SMpolicy;

enum Authorization;

enum BDTpolicy;

enum SpendingLimitControl;

}

}

leaf-list UPF {

type enumeration {

enum N4Session;

}

}

leaf-list AUSF {

type enumeration {

enum UEauthentication;

}

}

leaf-list NEF {

type enumeration {

enum EventExposure;

enum PFDmanagement;

enum ParameterProvisiong;

enum Trigger;

}

}

leaf-list NRF {

type enumeration {

enum NFmanagement;

enum NFdiscovery;

}

}

leaf-list NSSF {

type enumeration {

enum NSSelection;

enum NSSAI;

}

}

leaf-list SMSF {

type enumeration {

enum SMService;

}

}

leaf-list UDM {

type enumeration {

enum UEcontext;

enum SubscriberData;

enum UEauthentication;

enum EventExposure;

}

}

}

}

grouping ImmediateMdtConfigGrp {

description "Represents the ImmediateMdtConfig dataType.

This <<dataType>> defines the configuration parameters of

IOC TraceJob which are specific for Immediate MDT or combine

Trace and Immediate MDT.

The optional attribute positioningMethod allows to specify

the positioning methods to use.

The following attributes are conditional available based on the

measurements configured in listOfMeasurements:

-reportInterval (conditional for M1 in LTE or NR and M1/M2 in UMTS),

-reportAmount (conditional for M1/M2 in UMTS),

-reportAmountM1LTE (conditional for M1 in LTE),

-reportAmountM4LTE (conditional for M4 in LTE),

-reportAmountM5LTE (conditional for M5 in LTE),

-reportAmountM6LTE (conditional for M6 in LTE),

-reportAmountM7LTE (conditional for M7 in LTE),

-reportAmountM1NR (conditional for M1 in NR),

-reportAmountM4NR (conditional for M4 in NR),

-reportAmountM5NR (conditional for M5 in NR),

-reportAmountM6NR (conditional for M6 in NR),

-reportAmountM7NR (conditional for M7 in NR),

-reportingTrigger (conditional for M1 in LTE or NR and M1/M2 in UMTS),

-eventThreshold (conditional for A2 event reporting or A2 event

triggered periodic reporting),

-collectionPeriodRRMNR (conditional for M4 and M5 in NR),

-collectionPeriodM6NR (conditional for M6 in NR),

-collectionPeriodM7NR (conditional for M7 in NR),

-collectionPeriodRRMLTE (conditional for M3 in LTE),

-measurementPeriodLTE (conditional for M4 and M5 in LTE),

-collectionPeriodM6LTE (conditional for M6 in LTE),

-collectionPeriodM7LTE (conditional for M7 in LTE),

-collectionPeriodRRMUMTS (conditional for M4 and M5 in UMTS),

-measurementPeriodUMTS (conditional for M6 and M7 in UMTS),

-measurementQuantity (conditional for 1F event reporting),

-beamLevelMeasurement (conditional for M1 in NR),

-excessPacketDelayThresholds (conditional for M6 UL measurement in NR).

For immediate MDT, the measurement reporting is dependent on the

configured measurements:

- For measurement M1 in LTE or NR, it is possible to select between

periodical, event triggered, event triggered periodic reporting or

reporting according to all configured RRM event triggers. For M1 and M2

measurement in UMTS, it is possible to select between periodical, event

triggered reporting or reporting according to all configured RRM event

triggers. Parameter reportingTrigger determines which of the reporting

methods is selected and in case of event triggered or event-triggered

periodic, which is the decisive event type. For periodical reporting,

parameters reportInterval and reportAmount determine the interval between

two successive reports and the number of reports. This means the

periodical reporting terminates after reportAmount reports have been

sent as long as reportAmount is configured with a value different from

infinity. For event-triggered periodic reporting, these two parameters

apply in addition to parameter eventThreshold which determines the

threshold of the event. In this case up to reportAmount reports are

sent with a periodicity of reportInterval after the entering condition

is fulfilled. The reporting is stopped, if the leaving condition is

fulfulled and is restarted if the configured event reoccurs. For event

based reporting, there is only one report sent after the event occurs.

The parameters to configure are reportingTrigger and eventThreshold.

In case of UMTS and 1f event reporting, additionally parameter

measurementQuantity is necessary in order to determine for which

measurement(s) the event threshold is applicable.  Parameter

beamLevelMeasurement determines whether beam level measurements shall

be included in case of NR.

- For measurement M2 in LTE or NR, reporting is according to RRM

configuration, see TS 38.321, TS 36.321 and TS 38.331, TS 36.331.

- For measurement M4 in UMTS, reporting is either according to RRM

configuration, see TS 25.321 and TS 25.331 or periodic or event

triggered periodic using parameter collectionPeriodRRMUMTS and

eventThresholdUphUMTS.

- For measurement M3 in UMTS, the reporting is done upon

availability, see TS 37.320.

- For measurements M4, M5, M6 and M7 in NR, for measurements

M3, M4, M5, M6 and M7 in LTE and for measurements M5, M6 and M7

in UMTS periodical reporting is applied. The configurable parameter

is the interval between two measurements (collectionPeriodRrmNr,

collectionPeriodM6NR, collectionPeriodM7NR, collectionPeriodRRMLTE,

measurementPeriodLte, collectionPeriodM6LTE, collectionPeriodM7LTE,

collectionPeriodRRMUMTS, measurementPeriodUMTS) and the number of

reports (reportAmountM4NR, reportAmountM5NR, reportAmountM6NR,

reportAmountM7NR, reportAmountM4LTE, reportAmountM5LTE,

reportAmountM6LTE, reportAmountM7LTE).  If no collection

period is configured for M5 in UMTS, all available measurements

ar elogged according to RRM configuration.

- Measurements M8 and M9 in NR or LTE are reported according to

configured M1 and/or M6 related UE measurement reporting.

";

leaf listOfMeasurements {

when '../../../jobType = "IMMEDIATE_MDT_ONLY"';

type enumeration {

enum M1;

enum M2;

enum M3;

enum M4;

enum M5;

enum M6_DL;

enum M6_UL;

enum M7_DL;

enum M7_UL;

enum M1_EVENT_TRIGGERED;

enum M6;

enum M7;

enum M8;

enum M9;

}

description "It specifies the UE measurements that shall be

collected in an Immediate MDT job. The attribute is

applicable only for Immediate MDT.";

reference "Clause 5.10.3 of 3GPP TS 32.422.";

}

leaf reportingTrigger {

when '../../../jobType = "IMMEDIATE_MDT_ONLY"';

type enumeration {

enum PERIODICAL;

enum A2_FOR_LTE;

enum 1F_FOR_UMTS;

enum 1I_FOR_UMTS_MCPS_TDD;

enum A2_TRIGGERED_PERIODIC_FOR_LTE;

enum ALL_CONFIGURED_RRM_FOR_LTE;

enum ALL_CONFIGURED_RRM_FOR_UMTS;

}

description "It specifies whether periodic or event based

measurements should be collected.

The attribute is applicable only for Immediate MDT and when the

listOfMeasurements is configured for M1 (for both UMTS and LTE)

or M2 (only for UMTS).";

reference "Clause 5.10.4 of 3GPP TS 32.422.";

}

leaf reportInterval {

when '../../../jobType = "IMMEDIATE_MDT_ONLY"'

+  ' and ../reportingTrigger = "PERIODICAL"';

type uint32 {

range "120|240|250|480|500|640|1000|1024|2000|2048|3000|4000|"

+"5120|6000|8000|10240|12000|16000|20000|"

+"20480|24000|28000|32000|40960|60000|64000|"

+"360000|720000|1800000|3600000";

}

units milliseconds;

description "It specifies the interval between the periodical measurements

that shall be taken when the UE is in connected mode.

The attribute is applicable only for Immediate MDT and when

reportingTrigger is configured for periodical measurements.";

reference "Clause 5.10.5 of 3GPP TS 32.422.";

}

leaf reportAmount {

when '../../../jobType = "IMMEDIATE_MDT_ONLY"'

+  ' and ../reportingTrigger = "PERIODICAL"';

type union {

type uint32 {

range "1|4|8|16|32|64" ;

}

type enumeration {

enum INFINITY;

}

}

description "It specifies the number of measurement reports that shall be

taken for periodic reporting while the UE is in connected.

The attribute is applicable only for Immediate MDT and when

reportingTrigger is configured for periodical measurements.";

reference "Clause 5.10.6 of 3GPP TS 32.422.";

}

leaf eventThreshold {

when '../../../jobType = "IMMEDIATE_MDT_ONLY"';

type int64;

description "Specifies the threshold which should trigger the reporting

in case A2 event reporting in LTE or 1F/1l event in UMTS. The attribute

is applicable only for Immediate MDT and when reportingTrigger is

configured for A2 event in LTE or 1F event or 1l event in UMTS.";

reference "Clauses 5.10.7 and 5.10.7a of 3GPP TS 32.422.";

}

leaf collectionPeriodRRMNR {

when '../../../jobType = "IMMEDIATE_MDT_ONLY"'

+ ' or ../../../jobType = "IMMEDIATE_MDT_AND_TRACE"';

type uint32 {

range "1024|2048|5120|10240|60000";

}

units milliseconds;

description "Specifies the collection period for collecting RRM

configured measurement samples for M4, M5 in NR. The attribute is

applicable only for Immediate MDT.";

reference "Clause 5.10.30 of 3GPP TS 32.422.";

}

leaf collectionPeriodM6NR {

when '../../../jobType = "IMMEDIATE_MDT_ONLY"'

+ ' or ../../../jobType = "IMMEDIATE_MDT_AND_TRACE"';

type enumeration {

enum 120ms;

enum 240ms;

enum 480ms;

enum 640ms;

enum 1024ms;

enum 2048ms;

enum 5120ms;

enum 10240ms;

enum 20480ms;

enum 40960ms;

enum 1min;

enum 6min;

enum 12min;

enum 30min;

}

description "It specifies the collection period for the Packet Delay

measurement (M6) for NR MDT taken by the gNB. The attribute is

applicable only for Immediate MDT.";

reference "clause 5.10.34 of  TS 32.422.";

}

leaf collectionPeriodM7NR {

when '../../../jobType = "IMMEDIATE_MDT_ONLY"'

+ ' or ../../../jobType = "IMMEDIATE_MDT_AND_TRACE"';

type uint32 {

range "1..60";

}

description "It specifies the collection period for the Packet Loss Rate

measurement (M7) for NR MDT taken by the gNB. The attribute is

applicable only for Immediate MDT. ";

reference "clause 5.10.35 of  TS 32.422.";

}

leaf collectionPeriodRRMLTE {

when '../../../jobType = "IMMEDIATE_MDT_ONLY"'

+  ' or ../../../jobType = "IMMEDIATE_MDT_AND_TRACE"';

type enumeration {

enum 100ms;

enum 1000ms;

enum 1024ms;

enum 1280ms;

enum 2048ms;

enum 2560ms;

enum 5120ms;

enum 10000ms;

enum 10240ms;

enum 1min;

}

description "Specifies the collection period for collecting RRM configured

measurement samples for M3 in LTE. The attribute is applicable only

for Immediate MDT.";

reference "Clause 5.10.20 of 3GPP TS 32.422.";

}

leaf measurementPeriodLTE {

when '../../../jobType = "IMMEDIATE_MDT_ONLY"'

+  ' or ../../../jobType = "IMMEDIATE_MDT_AND_TRACE"';

type enumeration {

enum 1024ms;

enum 2048ms;

enum 5120ms;

enum 10240ms;

enum 1min;

}

mandatory true;

description "It specifies the measurement period for the

Data Volume (M4) and Scheduled IP throughput measurements

(M5) for LTE MDT taken by the eNB. The attribute is

applicable only for Immediate MDT.";

reference "Clause 5.10.23 of 3GPP TS 32.422.";

}

leaf collectionPeriodM6LTE {

when '../../../jobType = "IMMEDIATE_MDT_ONLY"'

+  ' or ../../../jobType = "IMMEDIATE_MDT_AND_TRACE"';

type uint32 {

range "1024|2048|5120|10240";

}

units milliseconds;

description "Specifies the collection period for the Packet Delay

measurement (M6) for MDT taken by the eNB. The attribute is applicable

only for Immediate MDT.";

reference "Clause 5.10.32 of  TS 32.422.";

}

leaf collectionPeriodM7LTE {

when '../../../jobType = "IMMEDIATE_MDT_ONLY"'

+  ' or ../../../jobType = "IMMEDIATE_MDT_AND_TRACE"';

type uint16 {

range 1..60 ;

}

description "It specifies the collection period for the Packet Loss Rate

measurement (M7) for LTE MDT taken by the eNB. The attribute is

applicable only for Immediate MDT.";

reference "Clause 5.10.33 of  TS 32.422.";

}

leaf eventThresholdUphUMTS {

when '../../../jobType = "IMMEDIATE_MDT_ONLY"'

+  ' or ../../../jobType = "IMMEDIATE_MDT_AND_TRACE"';

type uint16 {

range 0..31 ;

}

description "It specifies the threshold which should trigger

the reporting in case of event-triggered periodic reporting for M4

(UE power headroom measurement) in UMTS.";

reference "Clause 5.10.39 of TS 32.422.";

}

leaf collectionPeriodRRMUMTS {

when '../../../jobType = "IMMEDIATE_MDT_ONLY"'

+  ' or ../../../jobType = "IMMEDIATE_MDT_AND_TRACE"';

type uint32 {

range "100|250|500|1000|2000|"

+"3000|4000|6000";

}

units milliseconds;

description "Specifies the collection period for collecting RRM configured

measurement samples for M3, M4, M5 in UMTS. The attribute is applicable

only for Immediate MDT.";

reference "Clause 5.10.21 of 3GPP TS 32.422.";

}

leaf measurementPeriodUMTS {

when '../../../jobType = "IMMEDIATE_MDT_ONLY"'

+  'or ../../../jobType = "IMMEDIATE_MDT_AND_TRACE"';

type uint32 {

range "1000|2000|3000|4000|6000|8000|12000|16000|20000|"

+"24000|28000|32000|64000";

}

units milliseconds;

description "It specifies the measurement period for the Data Volume and

Throughput measurements for MDT taken by RNC.

The attribute is applicable only for Immediate MDT.";

reference "Clause 5.10.22 of 3GPP TS 32.422.";

}

leaf measurementQuantity {

when '../../../jobType = "IMMEDIATE_MDT_ONLY"'

+  ' or ../../../jobType = "IMMEDIATE_MDT_AND_TRACE"';

type enumeration {

enum CPICH_ECNO;

enum CPICH_RSCP;

enum PATHLOSS;

}

description "It specifies the measurements that are collected in an MDT

job for a UMTS MDT configured for event triggered reporting.";

reference "Clause 5.10.15 of 3GPP TS 32.422.";

}

leaf beamLevelMeasurement {

when '../../../jobType = "IMMEDIATE_MDT_ONLY"'

+  ' or ../../../jobType = "IMMEDIATE_MDT_AND_TRACE"';

type boolean;

default false;

description "Indicates whether the NR M1 beam level measurements shall

be included or not.";

reference "Clause 5.10.40 of TS 32.422.";

}

leaf positioningMethod {

when '../../../jobType = "IMMEDIATE_MDT_ONLY"'

+  ' or ../../../jobType = "IMMEDIATE_MDT_AND_TRACE"';

type enumeration {

enum GNSS;

enum E_CELL_ID;

}

description "It specifies what positioning method should be used in the

MDT job.";

reference "Clause 5.10.19 of 3GPP TS 32.422.";

}

list excessPacketDelayThresholds {

description "Excess packet delay thresholds info for M6 UL measurement.";

min-elements 1;

key idx;

leaf idx { type string; }

uses ExcessPacketDelayThresholdsGrp;

}

leaf reportAmountM1LTE {

type enumeration {

enum 1;

enum 2;

enum 4;

enum 8;

enum 16;

enum 32;

enum 64;

enum INFINITY;

}

description "It specifies the number of measurement reports that shall be

taken for periodic reporting while the UE is in connected mode.

The attribute is applicable only for Immediate MDT and combined Trace

and Immediate MDT and when reportingTrigger is configured for periodical

measurements and applicable only for LTE.";

reference "Clause 5.10.6 of TS 32.422.";

}

leaf reportAmountM4LTE {

type enumeration {

enum 1;

enum 2;

enum 4;

enum 8;

enum 16;

enum 32;

enum 64;

enum INFINITY;

}

description "It specifies the number of measurement reports that shall be

taken for periodic reporting while the UE is in connected mode.

The attribute is applicable only for Immediate MDT and combined Trace

and Immediate MDT and when reportingTrigger is configured for periodical

measurements and applicable only for LTE.";

reference "Clause 5.10.6 of TS 32.422.";

}

leaf reportAmountM5LTE {

type enumeration {

enum 1;

enum 2;

enum 4;

enum 8;

enum 16;

enum 32;

enum 64;

enum INFINITY;

}

description "It specifies the number of measurement reports that shall be

taken for periodic reporting while the UE is in connected mode.

The attribute is applicable only for Immediate MDT and combined Trace

and Immediate MDT and when reportingTrigger is configured for periodical

measurements and applicable only for LTE.";

reference "Clause 5.10.6 of TS 32.422.";

}

leaf reportAmountM6LTE {

type enumeration {

enum 1;

enum 2;

enum 4;

enum 8;

enum 16;

enum 32;

enum 64;

enum INFINITY;

}

description "It specifies the number of measurement reports that shall be

taken for periodic reporting while the UE is in connected mode.

The attribute is applicable only for Immediate MDT and combined Trace

and Immediate MDT and when reportingTrigger is configured for periodical

measurements and applicable only for LTE.";

reference "Clause 5.10.6 of TS 32.422.";

}

leaf reportAmountM7LTE {

type enumeration {

enum 1;

enum 2;

enum 4;

enum 8;

enum 16;

enum 32;

enum 64;

enum INFINITY;

}

description "It specifies the number of measurement reports that shall be

taken for periodic reporting while the UE is in connected mode.

The attribute is applicable only for Immediate MDT and combined Trace

and Immediate MDT and when reportingTrigger is configured for periodical

measurements and applicable only for LTE.";

reference "Clause 5.10.6 of TS 32.422.";

}

leaf reportAmountM1NR {

type enumeration {

enum 1;

enum 2;

enum 4;

enum 8;

enum 16;

enum 32;

enum 64;

enum INFINITY;

}

description "It specifies the number of measurement reports that shall be

taken for periodic reporting while the UE is in connected mode.

The attribute is applicable only for Immediate MDT and combined Trace

and Immediate MDT and when reportingTrigger is configured for periodical

measurements and applicable only for NR.";

reference "Clause 5.10.6 of TS 32.422.";

}

leaf reportAmountM4NR {

type enumeration {

enum 1;

enum 2;

enum 4;

enum 8;

enum 16;

enum 32;

enum 64;

enum INFINITY;

}

description "It specifies the number of measurement reports that shall be

taken for periodic reporting while the UE is in connected mode.

The attribute is applicable only for Immediate MDT and combined Trace

and Immediate MDT and when reportingTrigger is configured for periodical

measurements and applicable only for NR.";

reference "Clause 5.10.6 of TS 32.422.";

}

leaf reportAmountM5NR {

type enumeration {

enum 1;

enum 2;

enum 4;

enum 8;

enum 16;

enum 32;

enum 64;

enum INFINITY;

}

description "It specifies the number of measurement reports that shall be

taken for periodic reporting while the UE is in connected mode.

The attribute is applicable only for Immediate MDT and combined Trace

and Immediate MDT and when reportingTrigger is configured for periodical

measurements and applicable only for NR.";

reference "Clause 5.10.6 of TS 32.422.";

}

leaf reportAmountM6NR {

type enumeration {

enum 1;

enum 2;

enum 4;

enum 8;

enum 16;

enum 32;

enum 64;

enum INFINITY;

}

description "It specifies the number of measurement reports that shall be

taken for periodic reporting while the UE is in connected mode.

The attribute is applicable only for Immediate MDT and combined Trace

and Immediate MDT and when reportingTrigger is configured for periodical

measurements and applicable only for NR.";

reference "Clause 5.10.6 of TS 32.422.";

}

leaf reportAmountM7NR {

type enumeration {

enum 1;

enum 2;

enum 4;

enum 8;

enum 16;

enum 32;

enum 64;

enum INFINITY;

}

description "It specifies the number of measurement reports that shall be

taken for periodic reporting while the UE is in connected mode.

The attribute is applicable only for Immediate MDT and combined Trace

and Immediate MDT and when reportingTrigger is configured for periodical

measurements and applicable only for NR.";

reference "Clause 5.10.6 of TS 32.422.";

}

}

grouping LoggedMdtConfigGrp {

description "This <<dataType>> defines the configuration parameters of

IOC TraceJob which are specific for Logged MDT or Logged MBSFN MDT.

Based on the value configured for attribute jobType in IOC TraceJob,

different attributes are available. In case of LOGGED_MDT_ONLY, the

attributes reportType, eventListForEventTriggeredMeasurement,

eventThresholdL1, hysteresisL1, timeToTriggerL1,

areaConfigurationForNeighCells

and npnIdentityList are applicable. In case of LOGGED_MBSFN_MDT,

the attribute mbsfnAreaList is applicable.

The optional attribute plmnList allows to specify the PLMNs where

measurement collection, status indication and log reporting is allowed,

the optional attribute areaConfigurationForNeighCell allows to specify

the area for which UE is requested to perform measurements logging for

neighbour cells which have list of frequencies.  For logged MDT in UMTS

and LTE, the reporting is periodical. Parameter loggingInterval

determines  the interval between the reports and parameter

loggingDuration determines how long the configuration is valid

meaning after this duration has passed no further reports are sent.

In NR, the reporting can be periodical or event

based, determined by parameter reportType. For periodical reporting the

same parameters as in LTE and UMTS apply. For event based reporting,

parameter eventListForEventTriggeredMeasurement configures the event

type, namely 'out of coverage' or 'L1 event'. In case 'L1 event' is

selected as event type, the logging is performed according to

parameter loggingInterval at regular intervals only when the

conditions indicated by eventThresholdL1,

hysteresisL1, timeToTriggerL1 (defining the thresholds, hysteresis

and time to trigger) are met and if UE is 'camped normally' state

(TS 38.331, TS 38.304).  In case 'out of coverage' is selected as

event type, the

logging is performed according to parameter loggingInterval at regular

intervals only when the UE is in 'any cell selection' state.

Furthermore, logging is performed immediately upon transition from the

'any cell selection' state to the 'camped normally' state (TS 38.331,

TS 38.304).";

leaf traceCollectionEntityId {

type int64;

description "It specifies the TCE Id which is sent to the UE in

Logged MDT.";

reference "Clause 5.10.11 of 3GPP TS 32.422.";

}

leaf loggingDuration {

when '../../../jobType = "LOGGED_MDT_ONLY" or'

+ ' ../../../jobType = "LOGGED_MBSFN_MDT"';

type uint32 {

range "600|1200|2400|3600|5400|7200";

}

units seconds;

description "Specifies how long the MDT configuration is valid at the

UE in case of Logged MDT. The attribute is applicable only for

Logged MDT and Logged MBSFN MDT.";

reference "Clause 5.10.9 of 3GPP TS 32.422.";

}

leaf loggingInterval {

when '../../../jobType = "LOGGED_MDT_ONLY" or'

+ ' ../../../jobType = "LOGGED_MBSFN_MDT"';

type enumeration {

enum 320ms;

enum 640ms;

enum 1280ms;

enum 2560ms;

enum 5120ms;

enum 10240ms;

enum 20480ms;

enum 30720ms;

enum 40960ms;

enum 61440ms;

enum INFINITY;

}

description "Specifies the periodicty for Logged MDT. The attribute is

applicable only for Logged MDT and Logged MBSFN MDT.";

reference "Clause 5.10.8 of 3GPP TS 32.422.";

}

leaf reportType {

when '../../../jobType = "IMMEDIATE_MDT_ONLY"';

type enumeration {

enum PERIODICAL;

enum EVENT_TRIGGERED;

}

description "It specifies report type for logged NR MDT";

reference "Clause 5.10.27 of 3GPP TS 32.422.";

}

leaf eventListForEventTriggeredMeasurement {

when '../../../jobType = "LOGGED_MDT_ONLY"';

type enumeration {

enum OUT_OF_COVERAGE ;

enum A2_EVENT ;

}

description "Specifies event types for event triggered measurement in the

case of logged NR MDT.  Each trace session may configure at most one

event. The UE shall perform logging of measurements only upon certain

condition being fulfilled:

- Out of coverage.

- A2 event.";

reference "Clause 5.10.28 of 3GPP TS 32.422.";

}

leaf eventThresholdL1 {

when '../../../jobType = "LOGGED_MDT_ONLY" or'

+ ' ../../../jobType = "LOGGED_MBSFN_MDT"';

type uint32 {

range "0..127";

}

description "It specifies the threshold which should trigger

the reporting in case of event based reporting of logged NR MDT.

The attribute is applicable only for Logged MDT and when reportType

is configured for event triggered reporting and when

eventListForEventTriggeredMeasurement is configured for L1 event.";

reference "clause 5.10.36 of TS 32.422.";

}

leaf hysteresisL1 {

when '../../../jobType = "LOGGED_MDT_ONLY" or '

+ '../../../jobType = "LOGGED_MBSFN_MDT"';

type uint32 {

range "0..30";

}

description "It specifies the hysteresis used within the entry and leave

condition of the L1 event based reporting of logged NR MDT.

The attribute is applicable only for Logged MDT, when reportType

is configured for event triggered reporting and when

eventListForEventTriggeredMeasurement is configured for L1 event. ";

reference "clause 5.10.37 of TS 32.422.";

}

leaf timeToTriggerL1 {

when '../../../jobType = "LOGGED_MDT_ONLY" or '

+ '../../../jobType = "LOGGED_MBSFN_MDT"';

type int32 {

range 0|40|64|80|100|128|160|256|320|480|512|640|1024|1280|2560|5120;

}

units milliseconds;

description "It specifies the threshold which should trigger

the reporting in case of event based reporting of logged NR MDT.

The attribute is applicable only for Logged MDT, when reportType

is configured for event triggered reporting and when

eventListForEventTriggeredMeasurement is configured for L1 event.";

reference "clauses 5.10.38 of TS 32.422.";

}

list areaConfigurationForNeighCell {

when '../../../jobType = "LOGGED_MDT_ONLY"';

key "idx";

leaf idx { type uint32 ; }

description "It specifies the area for which UE is requested to perform

measurement logging for neighbour cells which have list of frequencies.

If it is not configured, the UE shall perform measurement logging for

all the neighbour cells.

Applicable only to NR Logged MDT.";

reference "Clause 5.10.26 of 3GPP TS 32.422.";

uses AreaConfigGrp;

}

list mbsfnAreaList {

when '../../../jobType = "LOGGED_MBSFN_MDT"';

key "mbsfnAreaId earfcn";

max-elements 8;

description "The MBSFN Area consists of a MBSFN Area ID and Carrier

Frequency (EARFCN). The target MBSFN area List can have up to 8 entries.

This parameter is applicable only if the job type is Logged MBSFN MDT.";

reference "Clause 5.10.25 of 3GPP TS 32.422.";

uses MbsfnAreaGrp;

}

list nPNIdentityList {

description "It defines which NPNs that can be served by the NR cell,

and which CAG IDs or NIDs can be supported by the NR cell for

corresponding PNI-NPN or SNPN.";

key idx;

max-elements 1;

uses NpnIdGrp;

leaf idx { type string;}

}

}

grouping MdtConfigGrp {

description "Defines the configuration parameters of IOC

TraceJob which are specific for MDT. The attribute

anonymizationOfMdtData specifies the level of anonymization

of MDT data.  The attribute areaScope defines the area scope of MDT,

which is specified in clause 5.10.2 of TS 32.422.  The

optional attribute sensorInformation allows to

specify  the sensor information to include.  Based on the value

configured for attribute jobType in IOC TraceJob, the attributes

immediateMdtConfig or loggedMdtConfig are available: In case of

IMMEDIATE_MDT_ONLY or IMMEDIATE_MDT_AND_TRACE the attribute

immediateMdtConfig is applicable. In case of LOGGED_MDT_ONLY or

LOGGED_MBSFN_MDT the attribute loggedMdtConfig is applicable.";

leaf anonymizationOfMdtData {

when ../areaScope ;

type enumeration {

enum NO_IDENTITY;

enum TAC_OF_IMEI;

}

default NO_IDENTITY;

description "Specifies the level of anonymization of MDT data.

This attribute is only applicable for management based activation";

reference "Clause 5.10.12 of 3GPP TS 32.422.";

}

list areaScope {

key "idx";

leaf idx { type uint32 ; }

description "It specifies the area where data shall be collected.";

max-elements 1;

uses AreaScopeGrp;

}

leaf-list sensorInformation {

type enumeration {

enum BAROMETRIC_PRESSURE;

enum UE_SPEED;

enum UE_ORIENTATION;

}

description "It specifies which sensor information shall be included in

logged NR MDT and immediate NR MDT measurement if they are available.";

reference "Clause 5.10.29 of 3GPP TS 32.422.";

}

list immediateMdtConfig {

description "The set of parameters specific for Immediate MDT

configuration.";

key idx;

max-elements 1;

leaf idx { type string; }

uses ImmediateMdtConfigGrp;

}

list loggedMdtConfig {

description "The set of parameters specific for Logged MDT and Logged

MBSFN MDT configuration.";

key idx;

max-elements 1;

leaf idx { type string; }

uses LoggedMdtConfigGrp;

}

leaf mNOnly {

type boolean;

default false;

description "Specifies whether the MDT configuration is for MN

only or not. The value FALSE means the MDT configuration is for

both MN and SN. The value TRUE means the MDT configuration is

for MN only.";

}

list pLMNList {

key "mcc mnc";

uses types3gpp:PLMNId;

max-elements 16;

description "It indicates the PLMNs where measurement collection. ";

reference "Clause 5.10.24 of 3GPP TS 32.422.";

}

}

grouping UECoreMeasConfigGrp {

description "Represents the UECoreMeasConfig dataType.

This <<dataType>> defines the aconfiguration parameters of IOC

TraceJob which are specific for 5GC UE level measurements

collection.";

leaf-list ueCoreMeasurements {

type string;

min-elements 1;

description "List of 5GC UE level measurements identified by name.

The list may include 5GC UE level measurements

defined in TS 28.558, or vendor specific measurements.

For 5GC UE level measurements defined in TS 28.558, the

name is constructed as follows:

'family.measurementName.subcounter' for measurement type

with specified subcounter

'family.measurementName.ALL' for measurement type with all

supported subcounters

'family.measurementName' for measurement type without

subcounters

'family' for measurement family, including all measurement

types and

the associated subcounters under this family.

For non-3GPP specified 5GC UE level measurements the name is

defined elsewhere.";

}

leaf ueCoreMeasGranularityPeriod {

type uint32;

description "Granularity period used to produce 5GC UE

level measurements. The period is defined in milliseconds (ms).";

}

leaf nfTypeToMeasure {

type string;

mandatory true;

description "It indicates the type of NE to produce the 5GC UE level

measurements.

allowedValues: The NF types represented by the measured object

classes as defined by f) of the 5GC UE level measurements specified

in TS 28.558.";

}

leaf-list objectInstances {

type string;

description "List of object instances.

Each object instance is identified by its DN.";

}

leaf-list rootObjectInstances {

type string;

description "List of object instances.

Each object instance is identified by its DN and designates

the root of a substree that contains the root object and all

descendant objects.";

}

}

grouping TraceJobGrp {

leaf jobType {

type enumeration {

enum IMMEDIATE_MDT_ONLY;

enum LOGGED_MDT_ONLY;

enum TRACE_ONLY;

enum IMMEDIATE_MDT_AND_TRACE;

enum RLF_REPORT_ONLY;

enum RCEF_REPORT_ONLY;

enum LOGGED_MBSFN_MDT;

enum 5GC_UE_LEVEL_MEASUREMENTS_ONLY;

enum TRACE_AND_5GC_UE_LEVEL_MEASUREMENTS;

enum IMMEDIATE_MDT_AND_5GC_UE_LEVEL_MEASUREMENTS;

enum TRACE_AND_IMMEDIATE_MDT_AND_5GC_UE_LEVEL_MEASUREMENTS;

}

default TRACE_ONLY;

description "It specifies whether the

TraceJob represents only MDT, Logged MBSFN MDT, Trace, or

5GC UE level measurements job, or a combined job. It also

defines the MDT mode.";

reference "Clause 5.9a of 3GPP TS 32.422.";

}

list pLMNTarget {

key "mcc mnc";

description "Specifies which PLMN that the subscriber of the session to

be recorded uses as selected PLMN.";

uses types3gpp:PLMNId;

}

leaf-list listOfTraceMetrics {

when '../jobType = "TRACE_ONLY"'

+  ' or ../jobType = "IMMEDIATE_MDT_AND_TRACE"';

type string;

description "List of trace metrics identified by name.

Includes trace messages, MDT measurements (Immediate MDT,

Logged MDT, Logged MBSFN MDT), RLF and RCEF reports,

see TS 32.422). Trace messages are identified with their message

identifier. Trace metric identifier is constructed as

defined in clause 10 of TS 32.422.

For non-3GPP specified trace metrics the name is defined elsewhere.";

reference "Clause 10 of 3GPP TS 32.422.";

}

leaf traceCollectionEntityIPAddress {

type inet:ip-address;

description "It specifies the address of the Trace Collection Entity

when the attribute traceReportingFormat is configured for the

file-based reporting. The attribute is applicable for both Trace and

MDT.";

reference "Clause 5.9 of 3GPP TS 32.422.";

}

leaf traceReportingConsumerUri {

when '../traceReportingFormat  = "STREAMING"';

type inet:uri;

description "URI of the Streaming Trace data reporting MnS consumer

(a.k.a. streaming target).";

reference "Clause 5.9 of 3GPP TS 32.422.";

}

list traceReference {

key "idx";

min-elements 1;

max-elements 1;

description "A globally unique identifier, which uniquely identifies the

Trace Session that is created by the TraceJob.

In case of shared network, it is the MCC and MNC of the Participating

Operator that request the trace session that shall be provided.

The attribute is applicable for both Trace and MDT.";

reference "Clause 5.6 of 3GPP TS 32.422.";

leaf idx { type uint32 ; }

uses trace3gpp:TraceReferenceGrp ;

}

leaf jobId {

type string;

yext3gpp:inVariant;

description "Identifier of a TraceJob";

}

leaf traceReportingFormat {

type enumeration {

enum FILE-BASED;

enum STREAMING;

}

default FILE-BASED;

description "Specifies the trace reporting format - streaming trace

reporting or file-based trace reporting";

reference "3GPP TS 32.422 clause 5.11";

}

list traceTarget {

key "targetIdType targetIdValue";

max-elements 1;

leaf targetIdType {

type enumeration {

enum IMSI;

enum IMEI;

enum IMEISV;

enum PUBLIC_ID;

enum UTRAN_CELL;

enum E-UTRAN_CELL;

enum NG-RAN_CELL;

enum ENB;

enum RNC;

enum GNB;

enum SUPI;

}

}

leaf targetIdValue {

type string;

}

description "Specifies the target object of the Trace and MDT. The

attribute is applicable for both Trace and MDT. This attribute

includes the ID type of the target as an enumeration and the ID value.

The traceTarget shall be 'PUBLIC_ID' in case of a Management Based

Activation is done to an SCSCFFunction (Serving Call Session

Control Function)  or PCSCFFunction (Proxy Call Session Control

Function).  The traceTarget shall be 'UTRAN_CELL' only in case

of the UTRAN cell traffic trace function.

The traceTarget shall be 'E-UTRAN_CELL' only in case of E-UTRAN cell

traffic trace function.

The traceTarget shall be 'NG-RAN_CELL' only in case of NR cell

traffic trace function.

The traceTarget shall be either 'IMSI', 'IMEI' or

'IMEISV' if the Trace Session is activated to any of the following

ManagedEntity(ies):

- HssFunction

- MscServerFunction

- SgsnFunction

- GgsnFunction

- BmscFunction

- RncFunction

- MmeFunction

- ServingGWFunction

- PGWFunction

The traceTarget shall be either 'SUPI' or 'IMEISV' if the Trace Session

is activated to any of the followed ManagedEntity(ies):

- AFFunction

- AMFFunction

- AUSFFunction

- NEFFunction

- NRFFunction

- NSSFFunction

- PCSCFFunction

- SMFFunction

- UPFFunction

- UDMFunction

In case of signalling based MDT, the traceTarget attribute should

be able to carry 'PUBLIC_ID', 'IMSI', 'IMEI', 'IMEISV' or 'SUPI'.

In case of management based 5GC UE level measurements collection,

the traceTarget attribute shall be able to carry the corresponding

Measured UE Identifier as defined by the bullet g) of the 5GC UE

level measurements (see TS 28.558) when the TraceJob is created

at the subject ManagedEntity.";

reference "3GPP TS 32.422.";

}

list traceConfig {

when '../jobType = "TRACE_ONLY"'

+  ' or ../jobType = "IMMEDIATE_MDT_AND_TRACE"'

+  ' or ../jobType = "TRACE_AND_5GC_UE_LEVEL_MEASUREMENTS"'

+  ' or ../jobType =

"TRACE_AND_IMMEDIATE_MDT_AND_5GC_UE_LEVEL_MEASUREMENTS"';

key idx;

description "Trace config";

max-elements 1;

uses TraceConfigGrp;

leaf idx { type string; }

}

list mdtConfig {

when '../jobType = "IMMEDIATE_MDT_ONLY"'

+ ' or ../jobType = "IMMEDIATE_MDT_AND_TRACE"'

+ ' or ../jobType = "RLF_REPORT_ONLY"'

+ ' or ../jobType = "RCEF_REPORT_ONLY"'

+ ' or ../jobType = "LOGGED_MBSFN_MDT"'

+ ' or ../jobType = "IMMEDIATE_MDT_AND_5GC_UE_LEVEL_MEASUREMENTS"'

+ ' or ../jobType =

"TRACE_AND_IMMEDIATE_MDT_AND_5GC_UE_LEVEL_MEASUREMENTS"';

key idx;

description "MDT config";

max-elements 1;

uses MdtConfigGrp;

leaf idx { type string; }

}

list ueCoreMeasConfig {

when '../jobType = "5GC_UE_LEVEL_MEASUREMENTS_ONLY"'

+ ' or ../jobType = "TRACE_AND_5GC_UE_LEVEL_MEASUREMENTS"'

+ ' or ../jobType = "IMMEDIATE_MDT_AND_5GC_UE_LEVEL_MEASUREMENTS"'

+ ' or ../jobType =

"TRACE_AND_IMMEDIATE_MDT_AND_5GC_UE_LEVEL_MEASUREMENTS"';

key idx;

description "5GC UE level measurements config";

max-elements 1;

uses UECoreMeasConfigGrp;

leaf idx { type string; }

}

list nPNTarget {

description "applicable only for NR and shall be present in case of NPN

either a PNI-NPN or a SNPN) and for management-based activation when

several NPNs are supported in the RAN.";

key idx;

max-elements 1;

uses NpnIdGrp;

leaf idx { type string;}

}

}

grouping TraceSubtree {

description "Contains classes that manage Tracing.

Should be used in all  classes (or classes inheriting from)

- SubNetwork

- ManagedElement

- ManagedFunction

If a YANG module wants to augment these classes/list/groupings they must

augment all user classes!";

list TraceJob {

description "A TraceJob instance represents the Trace Control and

Configuration parameters of a particular Trace Job (see TS 32.421 and

TS 32.422 for details). It can be name-contained by SubNetwork,

ManagedElement, ManagedFunction.

To activate Trace Jobs, a MnS consumer has to create TraceJob object

instances on the MnS producer. A MnS consumer can activate a Trace Job

for another MnS consumer since it is not required the value of

traceCollectionEntityIPAddress or traceReportingConsumerUri to be

his own.

For the details of Trace Job activation see clauses 4.1.1.1.2 and

4.1.2.1.2 of TS 32.422.

When a MnS consumer wishes to deactivate a Trace Job, the MnS consumer

shall delete the corresponding TraceJob instance.

For details of management Trace Job activation/deactivation see clause

4.1.1.1.2 of TS 32.422.

The attribute traceReference specifies a globally unique ID and

identifies a Trace session. One Trace Session may be activated to

multiple Network Elements. The traceReference is populated by the

consumer that makes the request for a Trace Session.

The jobId attribute presents the job identifier of a TraceJob instance.

The jobId can be used to associate  multiple TraceJob instances.

For example, it is possible to configure the same jobId value for

multiple TraceJob instances required to produce the data (e.g. RSRP

values of M1 and RLF reports) for a specific network analysis.

The attribute traceReportingFormat defines the method for reporting

the produced measurements. The selectable options are file-based or

stream-based reporting. In case of file-based reporting the attribute

traceCollectionEntityIPAddress is used to specify the IP address to

which the trace records shall be transferred, while in case of

stream-based reporting the attribute traceReportingConsumerUri

specifies the streaming target.

The mandatory attribute traceTarget determines the target object of

the TraceJob. Dependent on the network element to which the Trace

Session is activated different types of the target object are possible.

The attribute pLMNTarget defines the PLMN for which sessions shall be

selected in the Trace Session in case of management based activation

when several PLMNs are supported in the RAN.

The attribute jobType specifies the kind of data to collect. In case of

Trace only, the configuration parameters of attribute traceConfig shall

be applied. In case of Immediate MDT only, Logged MDT only, RLF reports

only, RCEF reports only and Logged MBSFN MDT, the configuration

parameters of attribute mdtConfig or a subset of these shall be

applied. In case of UE measurements only, the configuration parameters

of attribute ueCoreMeasConfig shall be applied. In case of any

combination of Trace, Immediate MDT, Trace and UE measurements, the

configuration parameters of the corresponding attributes traceConfig,

mdtConfig and ueCoreMeasConfig are applicable.

Creation and deletion of TraceJob instances by MnS consumers is

optional; when not supported, the TraceJob instances may be created

and deleted by the system or be pre-installed.";

key id;

uses top3gpp:Top_Grp ;

container attributes {

uses TraceJobGrp ;

}

uses files3gpp:FilesSubtree {

if-feature FilesUnderTraceJob;

}

}

}

}

<CODE ENDS>

*** END OF CHANGE 1 ***

*** START OF CHANGE 2 ***

*** yang-models/_3gpp-common-yang-types.yang ***

<CODE BEGINS>

module _3gpp-common-yang-types {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-common-yang-types";

prefix "types3gpp";

import ietf-inet-types { prefix inet; }

import ietf-yang-types { prefix yang; }

import _3gpp-common-yang-extensions { prefix yext3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "The model defines a YANG mapping of the top level

information classes used for management of 5G networks and

network slicing.

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.623";

revision 2025-08-06 { reference CR-0563; }

revision 2025-02-18 { reference CR-0511; }

revision 2025-02-06 { reference CR-0507; }

revision 2024-05-18 { reference CR-0359; }

revision 2024-04-12 { reference CR-0346; }

revision 2023-11-06 { reference CR-0305; }

revision 2023-09-18 { reference CR-0271 ; }

revision 2023-08-09 { reference CR-0266; }

revision 2023-05-10 { reference CR-0250; }

revision 2023-02-14 { reference CR-0234; }

revision 2022-11-04 { reference "CR-0194"; }

revision 2022-10-24 { reference CR-0196;  }

revision 2022-07-26 { reference "CR-0180" ; }

revision 2022-02-09 { reference "CR-0144"; }

revision 2021-11-01 { reference "CR-0141"; }

revision 2021-09-30 {

description "Added Longitude, Latitude, TenthOfDegrees, OnOff.";

reference "CR-0138";

}

revision 2020-11-06 {

description "Removed incorrect S-NSSAI definitions.";

reference "CR-0118";

}

revision 2020-03-10 {

description "Removed faulty when statements.";

reference "SP-200229";

}

revision 2019-10-25 {

description "Added ManagedNFProfile.";

reference "S5-194457";

}

revision 2019-10-16 {

description "Added SAP and usageState.";

reference "S5-193518";

}

revision 2019-06-23 {

reference  "Initial version.";

}

typedef EnabledDisabled {

type enumeration {

enum DISABLED ;

enum ENABLED ;

}

}

grouping nameValuePair {

leaf name { type string; }

leaf value { type string; }

}

typedef DayOfWeekT {

type enumeration {

enum Monday;

enum Tuesday;

enum Wednesday;

enum Thursday;

enum Friday;

enum Saturday;

enum Sunday;

}

}

typedef DayOfWeek {

type enumeration {

enum MONDAY;

enum TUESDAY;

enum WEDNESDAY;

enum THURSADY;

enum FRIDAY;

enum SATURDAY;

enum SUNDAY;

}

}

typedef DaysOfMonth {

type uint8 {

range 0..31;

}

}

typedef FullTime {

type yang:time-with-zone-offset;

}

grouping TimeWindowGrp {

leaf startTime {

type yang:date-and-time;

yext3gpp:inVariant;

}

leaf endTime {

type yang:date-and-time;

yext3gpp:inVariant;

}

}

grouping ProcessMonitorGrp {

description "Provides attributes to monitor the progress of processes

with specific purpose and limited lifetime running on MnS producers.

It may be used as data type for dedicated progress monitor attributes

when specifying the management representation of these processes.

The attributes in this clause are defined in a generic way.

For some attributes specialisations may be provided when specifying a

concrete process representation.

If a management operation on some IOCs triggers an associated

asynchronous process (whose progress shall be monitored), this should

also result in creating an attribute named 'processMonitor' (of type

'ProcessMonitor') in these IOC(s). The processMonitor attribute may be

accompanied by use-case specific additional data items.

The progress of the process is described by the 'status' and

'progressPercentage' attributes. Additional textual qualifications for

the 'status' attribute may be provided by the 'progressStateInfo' and

'resultStateInfo' attributes.

When the process is instantiated, the 'status' is set to 'NOT_RUNNING'

and the 'progressPercentage' to '0'. The MnS producer decides when to

start executing the process and to transition into the 'RUNNING' state.

This time is captured in the 'startTime' attribute. Alternatively, the

process may start to execute directly upon its instantiation. One

alternative must be selected when using this data type.

During the 'RUNNING' state the 'progressPercentage' attribute may be

repeatedly updated. The exact semantic of this attribute is subject to

further specialisation. The 'progressInfo' attribute may be used to

provide additional textual information in the 'NOT_RUNNING', 'CANCELLING'

and 'RUNNING' states. Further specialisation of

'progressStateInfo' may be provided where this data type is

used.

Upon successful completion of the process, the 'status' attribute is set

to 'FINISHED', the 'progressPercentage' to 100%. The time is captured in

the 'endTime' attribute. Additional textual information may be provided

in the 'resultStateInfo' attribute. The type of

'resultStateInfo' in this data type definition is 'String'.

Further specialisation of 'resultStateInfo' may be provided

where this data type is used.

In case the process fails to complete successfully, the 'status'

attribute is set to 'FAILED' or 'PARTIALLY_FAILED', the current value of

'progressPercentage' is frozen, and the time captured in 'endTime'. The

'resultStateInfo' specifies the reason for the failure.

Specific failure reasons may be specified where the data type defined in

this clause is used. The exact semantic of failure may be subject for

further specialisation as well.

In case the process is cancelled, the 'status' attribute is first set to

'CANCELLING' and when the process is really cancelled then to 'CANCELLED'.

The transition to 'CANCELLED' is captured in the 'endTime' attribute.

The value of 'progressPercentage' is frozen. Additional textual

information may be provided in the 'resultStateInfo' attribute.

The 'resultStateInfo' attribute is provided only for additional textual

qualification of the states 'FINISHED', 'FAILED', 'PARTIALLY_FAILED' or

'CANCELLED'. It shall not be used for making the outcome, that the

process may produce in case of success, available.

The process may have to be completed within a certain time after its

creation, for example because required data may not be available any

more after a certain time, or the process outcome is needed until a

certain time and when not provided by this time is not needed any more.

The time until the MnS producer automatically cancels the process is

indicated by the 'timer' attribute.";

leaf id {

type string;

mandatory true;

description "Id of the process. It is unique within a single

multivalue attribute of type ProcessMonitor.";

}

leaf status {

type enumeration {

enum NOT_STARTED ;

enum RUNNING ;

enum CANCELLING ;

enum FINISHED ;

enum FAILED ;

enum PARTIALLY_FAILED ;

enum CANCELLED ;

}

config false;

default  RUNNING;

description "Represents the status of the associated process,

whether it fails, succeeds etc.

It does not represent the returned values of a successfully finished

process. ";

}

leaf progressPercentage {

type uint8 {

range 0..100;

}

config false;

description "Progress of the associated process as percentage";

}

leaf-list progressStateInfo {

type string;

config false;

description "Additional textual qualification of the states

'NOT_STARTED', 'CANCELLING' and 'RUNNING'.

For specific processes, specific well-defined strings (e.g. string

patterns or enums) may be defined as a specialisation.";

}

leaf resultStateInfo {

type string;

config false;

description "Additional textual qualification of the states

'FINISHED', 'FAILED', 'PARTIALLY_FAILED and 'CANCELLED'.

For example, in the 'FAILED' or 'PARTIALLY_FAILED' state this

attribute may be used to provide error reasons.

This attribute shall not be used to make the outcome of the process

available for retrieval, if any. For this purpose, dedicated

attributes shall be specified when specifying the representation of

a specific process.

For specific processes, specific well-defined strings (e.g. string

patterns or enums) may be defined as a specialisation.";

}

leaf startTime {

type yang:date-and-time;

config false;

description "Start time of the associated process, i.e. the time when the

status changed from 'NOT_STARTED' to 'RUNNING'.";

}

leaf endTime {

type yang:date-and-time;

config false;

description "Date and time when status changed to 'SUCCESS', 'CANCELLED',

'FAILED' or 'PARTIALLY_FAILED'.

If the time is in the future, it is the estimated time

the process will end.";

}

leaf timer {

type uint32;

units minutes;

description "Time until the associated process is automatically cancelled.

If set, the system decreases the timer with time. When it reaches zero

the cancellation of the associated process is initiated by the

MnS_Producer.

If not set, there is no time limit for the process.

Once the timer is set, the consumer can not change it anymore.

If the consumer has not set the timer the MnS Producer may set it.";

yext3gpp:notNotifyable;

}

}

grouping NpnIdGrp {

description "Represents the NPN supported by the <<IOC>> using this

<<dataType>> as one of its attributes in case of the cell is a

NPN-only cell.";

list plmnId {

key "mcc mnc";

min-elements 1;

description "It specifies the PLMN Id of the NPN network.";

uses types3gpp:PLMNId;

}

choice npnidChoice {

leaf-list cAGIdList {

type string {

pattern '^[A-Fa-f0-9]{8}$';

}

max-elements 256;

description "It identifies a CAG list containing up to 12

CAG-identifiers per Ue or up to 12 CAG-identifiers per cell,

see TS 38.331.

CAG ID is used to combine with PLMN ID to identify a PNI-NPN.CAG ID is

a hexadecimal range with size 32 bit";

}

leaf-list nIDList {

type string {

pattern '^[A-Fa-f0-9]{11}$';

}

max-elements 16;

description "It identifies a list of NIDs containing up to 16 NIDs, see

TS 38.331. NID is used to combine with PLMN ID to identify an SNPN.

NID is a hexadecimal range with size 44 bit.";

}

}

}

typedef TenthOfDegrees {

type uint16 {

range 0..3600;

}

units "0.1 degrees";

description "A single integral value corresponding to an angle in degrees

between 0 and 360 with a resolution of 0.1 degrees.";

}

typedef Latitude {

type decimal64 {

fraction-digits 4;

range "-90.0000..+90.0000";

}

description "Latitude values";

}

typedef Longitude {

type decimal64 {

fraction-digits 4;

range "-180.0000..+180.0000";

}

description "Longitude values";

}

typedef Altitude  {

type decimal64 {

fraction-digits 6;

}

units "meters";

description

"Height from a reference 0 value.";

}

grouping GeographicalCoordinates {

description "This datatype represents the geographical coordinates";

reference "3GPP TS 28.538 clause 6.3.8";

leaf latitude {

type Latitude;

mandatory true;

}

leaf longitude {

type Longitude;

mandatory true;

}

}

typedef OnOff {

type enumeration {

enum ON;

enum OFF;

}

}

// grouping ManagedNFProfile will be removed as it is

//  being moved to _3gpp-5gc-nrm-nfprofile

grouping ManagedNFProfile {

description "Defines profile for managed NF";

reference "3GPP TS 23.501";

leaf idx { type uint32 ; }

leaf nfInstanceID {

config false;

mandatory true;

type yang:uuid ;

description "This parameter defines profile for managed NF.

The format of the NF Instance ID shall be a

Universally Unique Identifier (UUID) version 4,

as described in IETF RFC 4122 " ;

yext3gpp:inVariant;

}

leaf-list nfType {

config false;

min-elements 1;

type NfType;

description "Type of the Network Function" ;

}

leaf hostAddr {

mandatory true;

type inet:host ;

description "Host address of a NF";

}

leaf authzInfo {

type string ;

description "This parameter defines NF Specific Service authorization

information. It shall include the NF type (s) and NF realms/origins

allowed to consume NF Service(s) of NF Service Producer.";

reference "See TS 23.501" ;

}

leaf location {

type string ;

description "Information about the location of the NF instance

(e.g. geographic location, data center) defined by operator";

reference "TS 29.510" ;

}

leaf capacity {

mandatory true;

type uint16 ;

description "This parameter defines static capacity information

in the range of 0-65535, expressed as a weight relative to other

NF instances of the same type; if capacity is also present in the

nfServiceList parameters, those will have precedence over this value.";

reference "TS 29.510" ;

}

leaf nFSrvGroupId {

type string ;

description "This parameter defines identity of the group that is

served by the NF instance.

May be config false or true depending on the ManagedFunction.

Config=true for Udrinfo. Config=false for UdmInfo and AusfInfo.

Shall be present if ../nfType = UDM or AUSF or UDR. ";

reference "TS 29.510" ;

}

leaf-list supportedDataSetIds {

type enumeration {

enum SUBSCRIPTION;

enum POLICY;

enum EXPOSURE;

enum APPLICATION;

}

description "List of supported data sets in the UDR instance.

May be present if ../nfType = UDR";

reference "TS 29.510" ;

}

leaf-list smfServingAreas {

type string ;

description "Defines the SMF service area(s) the UPF can serve.

Shall be present if ../nfType = UPF";

reference "TS 29.510" ;

}

leaf priority {

type uint16;

description "This parameter defines Priority (relative to other NFs

of the same type) in the range of 0-65535, to be used for NF selection;

lower values indicate a higher priority. If priority is also present

in the nfServiceList parameters, those will have precedence over

this value. Shall be present if ../nfType = AMF ";

reference "TS 29.510" ;

}

}

typedef usageState {

type enumeration {

enum IDLE;

enum ACTIVE;

enum BUSY;

}

description "It describes whether or not the resource is actively in

use at a specific instant, and if so, whether or not it has spare

capacity for additional users at that instant. The value is READ-ONLY.";

reference "ITU T Recommendation X.731";

}

grouping SAPGrp {

leaf host {

type inet:host;

mandatory true;

}

leaf port {

type inet:port-number;

mandatory true;

}

description "Service access point.";

reference "TS 28.622";

}

typedef Mcc {

description "The mobile country code consists of three decimal digits,

The first digit of the mobile country code identifies the geographic

region (the digits 1 and 8 are not used):";

type string {

pattern '[02-79][0-9][0-9]';

}

reference "3GPP TS 23.003 subclause 2.2 and 12.1";

}

typedef Mnc {

description "The mobile network code consists of two or three

decimal digits (for example: MNC of 001 is not the same as MNC of 01)";

type string {

pattern '[0-9][0-9][0-9]|[0-9][0-9]';

}

reference "3GPP TS 23.003 subclause 2.2 and 12.1";

}

grouping PLMNId {

leaf mcc {

mandatory true;

type Mcc;

}

leaf mnc {

mandatory true;

type Mnc;

}

reference "TS 38.413 clause 9.3.3.5";

}

typedef Nci {

description "NR Cell Identity. The NCI shall be of fixed length of 36 bits

and shall be coded using full hexadecimal representation.

The exact coding of the NCI is the responsibility of each PLMN operator";

reference "TS 23.003";

type union {

type string {

length 36;

pattern '[01]+';

}

type string {

length 9;

pattern '[a-fA-F0-9]*';

}

}

}

typedef OperationalState {

reference "3GPP TS 28.625 and ITU-T X.731";

type enumeration {

enum DISABLED {

value 0;

description "The resource is totally inoperable.";

}

enum ENABLED {

value 1;

description "The resource is partially or fully operable.";

}

}

}

typedef BasicAdministrativeState {

reference "3GPP TS 28.625 and ITU-T X.731";

type enumeration {

enum LOCKED {

value 0;

description "The resource is administratively prohibited from performing

services for its users.";

}

enum UNLOCKED {

value 1;

description "The resource is administratively permitted to perform

services for its users. This is independent of its inherent

operability.";

}

}

}

typedef AdministrativeState {

reference "3GPP TS 28.625 and ITU-T X.731";

type enumeration {

enum LOCKED {

value 0;

description "The resource is administratively prohibited from performing

services for its users.";

}

enum UNLOCKED {

value 1;

description "The resource is administratively permitted to perform

services for its users. This is independent of its inherent

operability.";

}

enum SHUTTINGDOWN {

value 2;

description "Use of the resource is administratively permitted to

existing instances of use only. While the system remains in

the shutting down state the manager or the managed element

may at any time cause the resource to transition to the

locked state.";

}

}

}

typedef AvailabilityStatus {

type enumeration {

enum IN_TEST;

enum FAILED;

enum POWER_OFF;

enum OFF_LINE;

enum OFF_DUTY;

enum DEPENDENCY;

enum DEGRADED;

enum NOT_INSTALLED;

enum LOG_FULL;

}

}

typedef CellState {

type enumeration {

enum IDLE;

enum INACTIVE;

enum ACTIVE;

}

}

typedef Nrpci {

type uint32;

description "Physical Cell Identity (PCI) of the NR cell.";

reference "TS 36.211 subclause 6.11";

}

typedef Tac {

type int32 {

range 0..16777215 ;

}

description "Tracking Area Code";

reference "TS 23.003 clause 19.4.2.3";

}

grouping TaiGrp {

description "This <<dataType>> defines a Tracking Area Identity (TAI)

as specified in clause 28.6 of TS 23.003, clause 8.2 of TS 38.300

and clause 9.3.3.11 of TS 38.413. It is composed of the PLMN

identifier (PLMN-Id, which is composed of the MCC and MNC) and

the Tracking Area Code (TAC). ";

list plmnId {

description "PLMN Identity.";

min-elements 1;

max-elements 1;

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf tac { type Tac; }

}

grouping GeoCoordinateGrp {

description "Geographical location on earth";

leaf latitude {

type decimal64 {

fraction-digits 4;

range -90..90 ;

}

mandatory true;

description "Latitude based on World Geodetic System (1984 version)

global reference frame (WGS 84). Positive values correspond to the

northern hemisphere.";

}

leaf longitude {

type decimal64 {

fraction-digits 4;

range -180..180 ;

}

mandatory true;

description "Longitude based on World Geodetic System (1984 version)

global reference frame (WGS 84). Positive values correspond to

degrees east of 0 degrees longitude.";

}

leaf altitude {

type Altitude;

}

}

grouping GeoAreaGrp {

description "This data type defines a geographical area.

The geo-area is defined using a convex polygon in the attribute

'convexGeoPolygon'.";

list convexGeoPolygon {

description "Specifies the geographical area with a convex polygon.

The convex polygon is specified by its corners.";

key "latitude longitude";

min-elements 3;

ordered-by user;

uses GeoCoordinateGrp;

}

}

typedef AmfRegionId {

type union {

type uint8 ;

type string {

length 8;

pattern '[01]*';

}

}

reference "clause 2.10.1 of 3GPP TS 23.003";

}

typedef AmfSetId {

type union {

type uint16 {

range '0..1023';

}

type string {

length 8;

pattern '[01]*';

}

}

reference "clause 2.10.1 of 3GPP TS 23.003";

}

typedef AmfPointer {

type union {

type uint8 {

range '0..63';

}

type string {

length 6;

pattern '[01]*';

}

}

reference "clause 2.10.1 of 3GPP TS 23.003";

}

grouping AmfIdentifier {

leaf amfRegionId {

type AmfRegionId;

}

leaf amfSetId {

type AmfSetId;

}

leaf amfPointer {

type AmfPointer;

}

description "The AMFI is constructed from an AMF Region ID,

an AMF Set ID and an AMF Pointer.

The AMF Region ID identifies the region,

the AMF Set ID uniquely identifies the AMF Set within the AMF Region, and

the AMF Pointer uniquely identifies the AMF within the AMF Set. ";

}

// type definitions especially for core NFs

typedef NfType {

type enumeration {

enum NRF;

enum UDM;

enum AMF;

enum SMF;

enum AUSF;

enum NEF;

enum PCF;

enum SMSF;

enum NSSF;

enum UDR;

enum LMF;

enum GMLC;

enum 5G_EIR;

enum SEPP;

enum UPF;

enum N3IWF;

enum AF;

enum UDSF;

enum BSF;

enum CHF;

}

}

typedef NotificationType {

type enumeration {

enum N1_MESSAGES;

enum N2_INFORMATION;

enum LOCATION_NOTIFICATION;

}

}

typedef Load {

description "Latest known load information of the NF, percentage ";

type uint8 {

range 0..100;

}

}

typedef N1MessageClass {

type enumeration {

enum 5GMM;

enum SM;

enum LPP;

enum SMS;

}

}

typedef N2InformationClass {

type enumeration {

enum SM;

enum NRPPA;

enum PWS;

enum PWS_BCAL;

enum PWS_RF;

}

}

grouping DefaultNotificationSubscription {

leaf notificationType {

type NotificationType;

}

leaf callbackUri {

type inet:uri;

}

leaf n1MessageClass {

type N1MessageClass;

}

leaf n2InformationClass {

type N2InformationClass;

}

}

grouping Ipv4AddressRange {

leaf start {

type inet:ipv4-address;

}

leaf end {

type inet:ipv4-address;

}

}

grouping Ipv6PrefixRange {

leaf start {

type inet:ipv6-prefix;

}

leaf end {

type inet:ipv6-prefix;

}

}

typedef NsiId {

type string;

}

typedef UeMobilityLevel {

type enumeration {

enum STATIONARY;

enum NOMADIC;

enum RESTRICTED_MOBILITY;

enum FULLY_MOBILITY;

}

}

typedef ResourceSharingLevel {

type enumeration {

enum SHARED;

enum NOT_SHARED;

}

}

typedef TxDirection {

type enumeration {

enum DL;

enum UL;

enum DL_AND_UL;

}

}

grouping AddressWithVlan {

leaf ipAddress {

type inet:ip-address;

}

leaf vlanId {

type uint16;

}

}

/* DistinguishedName pattern is built up based on the

EBNF in 32.300 clause 7.3  EBNF of DN String Representation

leaf DN { type string {   //  Same pattern as LocalDN

pattern '[A-Z][^,=+<>#;\\"\r\n*.]*=([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2}))(([^,=+<>#;\\"\r\n*]|(\\[a-fA-F0-9]{2}))*([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2})))?(,[A-Z][^,=+<>#;\\"\r\n*.]*=([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2}))(([^,=+<>#;\\"\r\n*]|(\\[a-fA-F0-9]{2}))*([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2})))?)*';

} }

leaf fullLocalDN { type string {   // LocalRDN , { RDNSeparator , LocalRDN }    RDNSeparator is a single , no space or \R allowed   Me.mykey=1 allowed

//  (fullLocalRDN)(,(fullLocalRDN))*

pattern '(([A-Z][^,=+<>#;\\"\r\n*.]*|([A-Z][^,=+<>#;\\"\r\n*.]*\.[a-z][^,=+<>#;\\"\r\n*.]*))=(([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2}))(([^,=+<>#;\\"\r\n*]|(\\[a-fA-F0-9]{2}))*([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2})))?))(,(([A-Z][^,=+<>#;\\"\r\n*.]*|([A-Z][^,=+<>#;\\"\r\n*.]*\.[a-z][^,=+<>#;\\"\r\n*.]*))=(([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2}))(([^,=+<>#;\\"\r\n*]|(\\[a-fA-F0-9]{2}))*([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2})))?)))*';

} }

leaf LocalDN { type string {   // LocalRDN , { RDNSeparator , LocalRDN }    RDNSeparator is a single , no space or \R allowed

//  LocalRDN(,LocalRDN)*

pattern '[A-Z][^,=+<>#;\\"\r\n*.]*=([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2}))(([^,=+<>#;\\"\r\n*]|(\\[a-fA-F0-9]{2}))*([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2})))?(,[A-Z][^,=+<>#;\\"\r\n*.]*=([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2}))(([^,=+<>#;\\"\r\n*]|(\\[a-fA-F0-9]{2}))*([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2})))?)*';

} }

leaf fullLocalRDN { type string {   // same as fullLocalDNAttributeTypeAndValue

pattern '([A-Z][^,=+<>#;\\"\r\n*.]*|([A-Z][^,=+<>#;\\"\r\n*.]*\.[a-z][^,=+<>#;\\"\r\n*.]*))=(([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2}))(([^,=+<>#;\\"\r\n*]|(\\[a-fA-F0-9]{2}))*([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2})))?)';

} }

leaf LocalRDN { type string {   // same as LocalDNAttributeTypeAndValue

pattern '[A-Z][^,=+<>#;\\"\r\n*.]*=([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2}))(([^,=+<>#;\\"\r\n*]|(\\[a-fA-F0-9]{2}))*([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2})))?';

} }

leaf fullLocalDNAttributeTypeAndValue { type string { // LocalDNAttributeType , AttributeTypeAndValueSeparator , RegularAttributeValue

// pattern LocalDNAttributeType=RegularAttributeValue

pattern '([A-Z][^,=+<>#;\\"\r\n*.]*|([A-Z][^,=+<>#;\\"\r\n*.]*\.[a-z][^,=+<>#;\\"\r\n*.]*))=(([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2}))(([^,=+<>#;\\"\r\n*]|(\\[a-fA-F0-9]{2}))*([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2})))?)';

} }

// limitation: NamesOfClassAndNamingAttributenot supported Me.mykey=1

leaf LocalDNAttributeTypeAndValue { type string {

// ebnf1          LocalDNAttributeType , AttributeTypeAndValueSeparator , RegularAttributeValue

// ebnf2-limited  NameOfClassWithIdAttribute , AttributeTypeAndValueSeparator , RegularAttributeValue

// pattern        NameOfClassWithIdAttribute=RegularAttributeValue

pattern '[A-Z][^,=+<>#;\\"\r\n*.]*=([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2}))(([^,=+<>#;\\"\r\n*]|(\\[a-fA-F0-9]{2}))*([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2})))?';

} }

leaf LocalDNAttributeType { type string {   // NameOfClassWithIdAttribute | NamesOfClassAndNamingAttribute  RDNSeparator is a single , no space or \R allowed

//  NameOfClassWithIdAttribute|NamesOfClassAndNamingAttribute

pattern '[A-Z][^,=+<>#;\\"\r\n*.]*|([A-Z][^,=+<>#;\\"\r\n*.]*\.[a-z][^,=+<>#;\\"\r\n*.]*)';

} }

leaf RegularAttributeValue { type string {       // ( AttributeValueChar - SpaceChar ) , [ { AttributeValueChar } , ( AttributeValueChar - SpaceChar ) ]

pattern '([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2}))(([^,=+<>#;\\"\r\n*]|(\\[a-fA-F0-9]{2}))*([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2})))?' ;

} }

leaf NamesOfClassAndNamingAttribute  { type string {  // ClassName , ClassNamingAttributeSeparator , NamingAttributeName

// pattern: ClassName\.NamingAttributeName

pattern '[A-Z][^,=+<>#;\\"\r\n*.]*\.[a-z][^,=+<>#;\\"\r\n*.]*' ;

} }

leaf restrictiveClassName { type string {     //

pattern '[a-zA-Z][a-zA-Z0-9-_]*' ;

} }

leaf ClassName { type string {     // CapitalLetterChar , { LocalDNAttributeTypeChar }

pattern '[A-Z][^,=+<>#;\\"\r\n*.]*' ;

} }

leaf NamingAttributeName { type string {   // SmallLetterChar , { LocalDNAttributeTypeChar }

pattern '[a-z][^,=+<>#;\\"\r\n*.]*' ;

} }

*/

typedef DistinguishedName {

type string {

pattern '[A-Z][^,=+<>#;\\"\r\n*.]*=([^,=+<>#;\\"\r\n* ]|'

+ '(\\[a-fA-F0-9]{2}))(([^,=+<>#;\\"\r\n*]|(\\[a-fA-F0-9]{2}))*'

+ '([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2})))?'

+ '(,[A-Z][^,=+<>#;\\"\r\n*.]*=([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2}))'

+ '(([^,=+<>#;\\"\r\n*]|(\\[a-fA-F0-9]{2}))*'

+ '([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2})))?)*';

}

description "Represents the 3GPP standard for DistinguishedName.

Limitations:

- RDNSeparator: don't allow SpaceChar or CarriageReturnChar

- NullDN: Disallow nullDN that is the same as not providing a DN

- NamesOfClassAndNamingAttribute format not allowed

(eg. ManagedElement.mykey=345436)";

reference  "3GPP TS 32.300";

}

typedef QOffsetRange  {

type int8 {

range "-24 | -22 | -20 | -18 | -16 | -14 | -12 | -10 | -8 | -6 | " +

" -5 | -4 | -3 | -2 | -1 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 8 | 10 | " +

" 12 | 14 | 16 | 18 | 20 | 22 | 24";

}

units dB;

}

grouping ReportingCtrl {

choice reportingCtrl {

mandatory true;

description "

This choice defines the method for reporting collected performance

metrics to MnS consumers as well as the parameters for configuring the

reporting function. It is a choice between the control parameter

required for the reporting methods, whose presence selects the

reporting method as follows:

- When only the fileReportingPeriod attribute is present, the MnS

producer shall store files on the MnS producer at a location selected

by the MnS producer and, on condition that an appropriate subscription

is in place, inform the MnS consumer about the availability of new

files and the file location using the notifyFileReady notification.

In case the preparation of a file fails, 'notifyFilePreparationError'

shall be sent instead.

- When the 'fileReportingPeriod' and 'notificationRecipientAddress'

attributes are present, then the MnS producer shall behave like

described for the case that only the 'fileReportingPeriod' is present.

In addition, the MnS producer shall create on behalf of the MnS

consumer a subscription, using 'NtfSubscriptionControl', for the

notification types 'notifyMOICreation' and 'notifyMOIDeletion' related

to the 'File' instances that will be produced later. In case an existing

subscription does already include the 'File' instances to be produced,

no new subscription shall be created. The

'notificationRecipientAddress' attribute in the created

'NtfSubscriptionControl' instance shall be set to the value of the

'notificationRecipientAddress' in the related 'PerfMetricJob'. This

feature is called implicit notification subscription, as opposed to the

case where the MnS consumer creates the subscription (explicit

notification subscription). When the related 'PerfMetricJob' is

deleted, the 'NtfSubscriptionControl' instance created due to the

request for implicit subscription shall be deleted as well.

- When only the 'fileReportingPeriod' and 'fileLocation' attributes

are present (CHOICE_3), the MnS producer shall store the files at the

location specified by 'fileLocation'. The file location may identify

any entity such as a file server or a MnS consumer. The identified

MnS consumer may or may not be identical to the MnS consumer creating

the 'PerfMetricJob'. As for CHOICE_1 the MnS producer may emit

'notifyFileReady' and 'notifyFilePreparationError' notifications to

inform subscribers that a file has been made available at the location

specified by 'fileLocation'.

- When only the streamTarget attribute is present, the MnS producer

shall stream the data to the location specified by streamTarget.

For the file-based reporting methods the fileReportingPeriod attribute

specifies the time window during which collected measurements are

stored into the same file before the file is closed and a new file is

opened.";

case file-based-reporting {

leaf fileReportingPeriod {

type uint32 {

range 1..max;

}

units minutes;

mandatory true;

description "For the file-based reporting method this is the time

window during which collected measurements are stored into the same

file before the file is closed and a new file is opened.

The time-period must be a multiple of the granularityPeriod.

Applicable when the file-based reporting method is supported.";

}

choice reporting-target {

case file-target {

leaf fileLocation {

type string ;

description "Applicable and must be present when the file-based

reporting method is supported, and the files are stored on the MnS

consumer.";

}

}

case notification-target {

leaf notificationRecipientAddress {

type string;

description "Must be present when the notification-based reporting

method is supported, and the the files are available as

notifications for the MnS consumer to subscribe to.";

}

}

description "When netiher fileLocation or notificationRecipientAddress

are present, the files are stored and available to the MnS consumer

if the MnS subscribes to the notifyFileReady notification.";

}

}

case stream-based-reporting {

leaf streamTarget {

type string;

mandatory true;

description "Applicable when stream-based reporting method is

supported.";

}

}

}

}

}

<CODE ENDS>

*** END OF CHANGE 2 ***

End of Change