3GPP TSG-SA5 Meeting #162	S5-253493

Goteborg, Sweden, 25 - 29 August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.156 | CR | 0117 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 32.156 Add solution to construct conditions on performance metrics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MADCOL_Ph2, CCLM |  |  |  |  |  | Date: |  |  | 2025-08-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Currently it is not possible to construct conditions based on performance metrics. This requirement is discussed in MADCOL_ph2, see current Editor’s Note in TS 28.622 clause 4.3.47.1 as well as in FS_CCLM, see TR 28.867 clause 5.2 Triggered CCL. In TR 28.867 clause 5.2.3.4 the solution of pseudo attribute was evaluated and recommended for normative phase. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce pseudo attribute as property of a class which can only be accessed as part of rules for conditions to allow construction of conditions on performance metrics. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - No solution to construct conditions on performance metrics; - Parts of the definiton of ManagementDataCollection IOC in TS 28.622 needs to be removed according to the Editor’s Note. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.2.X (new), 5.2.X.1 (new), 5.2.X.2 (new), 5.2.X.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

# 2	References

[1]	OMG "Unified Modelling Language (OMG UML), Infrastructure", Version 2. 4.

[2]	OMG "Unified Modelling Language (OMG UML), Superstructure", Version 2. 4.

[3]	3GPP TS 32.300: "Telecommunication management; Configuration Management (CM); Name convention for Managed Objects".

[4]	Void

[5]	Void

[6]	3GPP TS 28.620: " Telecommunication management; Fixed Mobile Convergence (FMC) Federated Network Information Model (FNIM) Umbrella Information Model (UIM)".

[7]	ITU-T X.680,"OSI networking and system aspects – Abstract Syntax Notation One (ASN.1)".

[8]	 Void

[9]	3GPP TS 32.602 "Telecommunication management; Configuration Management (CM); Basic CM Integration Reference Point (IRP): Information Service (IS)".

[10]	3GPP TS 32.612: "Telecommunication management; Configuration Management (CM); Bulk CM Integration Reference Point (IRP): Information Service (IS)".

[11]	3GPP TS 32.111-2: " Telecommunication management; Fault Management; Part 2: Alarm Integration Reference Point (IRP): Information Service (IS)".

[12]	3GPP TS 32.302: "Telecommunication management; Configuration Management (CM); Notification Integration Reference Point (IRP): Information Service (IS)".

[13]	3GPP TS 32.404: "Telecommunication management; Performance Management (PM); Performance measurements - Definitions and template".

[14]	Void

[15]	3GPP TS 28.541 "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[16]	3GPP TS 28.532 "Management and orchestration; Generic management services".

[17]	3GPP TS 28.531 "Management and orchestration; Provisioning".

[18]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[19]	3GPP TS 32.102: “Telecommunication management; Architecture”.

[20]	3GPP TS 28.533: “Management and orchestration; Architecture framework”.

[21]	IETF RFC 20 “ASCII format for Network Interchange”

[22]	IETF RFC 7950 “The YANG 1.1 Data Modeling Language”

[23]	3GPP TS 29.501 “Principles and Guidelines for Services Definition; Stage 3”

[X]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[Y]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[Z]	3GPP TS 32.160: “Management and orchestration; Management service template”.

Next change

#### 5.2.11.2	Removing/Deprecating model elements

When removal or a non backwards compatible change is needed for a model element, it shall be kept in the specification as-is but be marked as deprecated for one release. The deprecated element may be removed in the next release.

A new replacing model element may be defined beside the original. In this case the replacing element shall be indicated in the specification of the old element.

Implementations of the previous release that now implement the current release shall continue to support usage of the deprecated attributes/classes as well as any new replacing attributes/classes, but not at the same time. As soon as the newer (replacing) attributes/classes are used, it may no longer be possible to also support usage of the deprecated elements or show correct values for the deprecated attributes. (E.g. when the type of an attribute is changed from integer to string). Once the replacing attribute/IOC is used, the old attribute/IOC may lose functionality and should not be used anymore.

In case the deprecated or the replacing element was or is intended to have a multiplicity strictly greater than zero (mandatory to configure/report), the model elements should be declared with a multiplicity including zero, as only one of the deprecated and the replacement elements will be used at any one time.

The deprecating procedure shall be used between releases. There is no need to follow it during the development of a single release, as long as the release is not yet frozen.

### 5.2.X Pseudo Attribute

#### 5.2.X.1	Description

A pseudo attribute is an element representing a property of the class. Pseudo attributes cannot be used by regular CRUD operations, not even read. Pseudo attributes can only be evaluated as part of rules for conditions. The properties of a pseudo attribute are described as follows:

-	The attribute value is determined by the value of the represented property.

-	Attribute properties defining valid interactions of managers and agents with attributes values are provided in Table 5.2.X.1-1.

-	Documentation of the attribute is provided in the description of the property.

-	Pseudo attributes are optional supported (supportQualifier: O).

Table 5.2.X.1-1: Pseudo attribute properties defining valid interactions with attributes

| Property name and value | Description |
| --- | --- |
| isInvariant: False | The value of a pseudo attribute is determined by the current value of the represented property. |
| isWritable: False | The value of a pseudo attribute cannot be set by a manager upon object creation nor modified later because it is determined by the current value of the represented property. |
| isReadable: False | The value of a pseudo attribute cannot be read by CRUD operations but only by dedicated methods. |
| isNotifyable: False | There is no notification sent in case of a pseudo attribute value change. |

#### 5.2.X.2	Example

Pseudo attributes are specified in a separate subclause of the class definitions in the template for NRM (TS 32.160 [Z]).

#### 5.2.X.3	Name style

A pseudo attribute name shall use the LCC style.

Well Known Abbreviation (WKA) is treated as a word if used in a name. However, WKA shall be used as defined in the specification document that originally defined the WKA (its letter case cannot be changed) except when it is the first word of a name; and if so, its first letter must be in lower case.

#### 5.2.X.4	Usage of properties of IOC as pseudo attributes

Editor’s Note: It needs to be discussed whether the content of this subclause shall be moved to another specification.

This chapter describes how to map properties of IOCs to names and values of pseudo attributes such that they can be evaluated as part of rules for conditions. Within rules for condition the names and values of pseudo attributes can be used the same way as names and values of attributes as defined by the IOC of any NRM.

The pseudo attribute is accessible over a condition expression. Notations are specified in

- 	Jex for the HTTP/JSON solution.

-	XPath for the NETCONF/YANG solution.

###### 5.2.X.4.1 Usage of performance measurements as pseudo attributes

For performance measurements according to TS 28.552 [X] the association of properties to IOC is given for any performance measurement by bullet f) of the measurement definition in TS 28.552 [X].

The name of a pseudo attribute that represents a performance measurement is given for any measurement by bullet e) of the measurement definition in TS 28.552 [X], including any optional filters.

The value of a pseudo attribute that represents a performance measurement according to TS 28.552 [X] shall evaluate to the performance measurement value at the expiry of the latest completed granularity period, irrespective of any potential reporting period.

###### 5.2.X.4.2 Usage of KPI as pseudo attributes

For key performance indicators (KPI) according to TS 28.554 [X] the association of properties to IOC is given for any KPI by bullet d) of the KPI definition in TS 28.554 [X].

The name of a pseudo attribute that represents a KPI is given for any KPI by bullet a) of the KPI definition in TS 28.554 [X].

The value of a pseudo attribute that represents a KPI according to TS 28.554 [X] shall evaluate to the KPI value at the expiry of the latest completed granularity period, irrespective of any potential reporting period.

End of changes