3GPP TSG-SA5 Meeting #162	S5-253511

Goteborg, Sweden, 25 - 29 August 2025

Title:	Reply LS on Continuous MDT

Response to:	R3-253958/S5-253296

Release:	Release 19

Work Item:	NR_AIML_NGRAN_enh-Core

Source:	SA5

To:	RAN3

Cc:

Contact person:	Shi Xiaoli

shixiaoli@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	S5-253522

- Overall description

SA5 would like to thank RAN3 for the reply LS on Continuous MDT. SA5 has been asked to check the feasibility of the principles agreed by RAN3, to specify a solution that reflects such principles (if feasible) and to provide feedback to RAN3.

RAN3: “For the following principles:

- Information from OAM to the NG-RAN nodes participating to Continuous MDT, allowing for the identification of a continuous MDT process, are provided e.g. in the form of specific Trace Reference(s).

- Solutions for Continuous MDT shall not have an impact on the 5GC in Rel19 from a RAN3 perspective.

- RAN3 agrees to use TR(s) and TRSR(s) to resolve the MDT measurement correlation problem”

Answer from SA5:

SA5 has studied the above principles from the incoming LS (R3-253958) and agreed on the general principle of Continuous Management Based MDT activation.

- SA5 agree on introducing a new Job Type “Immediate MDT and Logged MDT” for the Continuous MDT process which is identified by a single session-specific TR. This single TR, together with the UE-specific TRSR assigned by the serving gNB, is used for solving the trace correlation problem.

- To avoid time interruptions in the measurement process and to ensure measurement continuity across different RRC states it could be possible to rely on the UE context in 5GC/AMF, where at least the single session-specific TR and the UE-specific TRSR are stored. Therefore, from SA5 perspective, measurement continuity across RRC state changes (i.e., after idle/inactive to connected state transition) is achieved by storing in AMF/5GC at least the single TR associated to the Continuous MDT process and the TRSR assigned to the Continuous MDT UE.

- SA5 agree that the (potentially new) serving gNB keeps the mapping relationship of TRSRs allocated to the Continuous MDT UE in case of TRSR allocation conflicts between the (old) serving gNB and the (potentially new) serving gNB. The (potentially new) serving gNB provides the mapping relationship of TRSRs to the TCE for the sake of trace correlation of MDT reports from the same UE.

RAN3: “RAN3 asks SA5 to check the security aspects of the Continuous MDT solution derived.”

Answer from SA5:

From SA5 perspective, the same TRSR (allocated by the old serving gNB) being sent from the new serving gNB to UE when providing the Continuous MDT configuration after an idle/inactive-to-connected RRC state change may represent a security issue.

For the details, please see the attachment.

- Actions

To RAN3

ACTION: 	SA5 kindly asks RAN3 to take the above feedback into account when discussing the solution for Continuous MDT in Rel-19.

- Dates of next TSG SA WG5 meetings

SA5#163		13 October - 17 October 2025		China

SA5#164		17 November - 21 November 2025		Dallas , US