3GPP TSG-SA5 Meeting #162	S5-253515

Goteborg, Sweden, 25 - 29 August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.540 | CR | 0047 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.540 add missing feature concepts and requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In TS 28.541, the NRM fragment to support Prose, UPF capability, NR mobility, ATSSS and SON function management has been already defined. However, there is no corresponding usecase and requirements description in the current TS 28.540 specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add missing Prose, UPF capability, NR mobility, ATSSS and SON function management management use case and requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The use case and requirements for Prose, UPF capability, NR mobility, ATSSS and SON function management will not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.x1(new), 4.x2(new), 4.x3(new), 4.x4(new), 4.x5(new), 5.x1(new) , 5.x2(new) , 5.x3(new) , 5.x4(new) , 5.x5(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System".

[3]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[4]	3GPP TS 38.401: "NG-RAN; Architecture description".

[5]	3GPP TS 37.340: "NR; Multi-connectivity; Overall description; Stage 2".

[6]	3GPP TS 28.531: "Management and orchestration of 5G networks; Provisioning".

[7]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[8]	GSMA NG.116 – "Generic Network Slice Template" v1.0 (2019-05-23).

[9]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[10]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[11]	3GPP TS 23.369: " Ambient power-enabled Internet of Things; Stage 2".

[x1]	3GPP TS 23.304: "Proximity based Services (ProSe) in the 5G System (5GS)".

[x2]	3GPP TS 28.313: "Self-Organizing Networks (SON) for 5G networks".

[x3]	3GPP TS 28.310: " Management and orchestration; Energy efficiency of 5G".

Next Change

4.X1	ProSe capabilities management

The UE includes the ProSe Capability as part of the "5GMM capability" in the Registration Request message. The AMF stores the ProSe Capability for 5G ProSe operation. The ProSe Capability indicates whether the UE supports one or more of the following ProSe capabilities defined in TS 23.304 [x1]. The PCF generates the PC5 QoS parameters used by NG-RAN corresponding to a UE based on the ProSe capabilities. The Operator can indicate the supported ProSe Capability by the PCF. So, the 3GPP management system should have the capability allowing an authorized consumer to configure ProSe capabilities.

4.X2	UPF capabilities management

UPF may support operator defined non-standardized feature(s) (such as hardware features (e.g. xPUs information (e.g. CPU/GPU information)), computing features (e.g. computing resource type, computing capability) and protection features (e.g. Firewall, DDoS)) or partially supported standardized feature(s). See details in clause 5.8.2.21 of 3GPP TS 23.501 [2]. If the UPF supports such features, the UPF indicates the supported features to the SMF using the operator configurable UPF capabilities to the SMF, SMF can consider the operator configurable UPF capabilities for UPF selection. So, the 3GPP management system should have the capability allowing an authorized consumer to configure UPF capabilities.

4.X3	NR mobility management

NR mobility including L1/L2 Triggered Mobility (LTM), Dual Active Protocol Stack (DAPS) and Conditional Handover (CHO).  As defined in TS 38.300[3], LTM is a kind of handover procedure in which a gNB receives L1 or L3 measurement report(s) from a UE, and on their basis the gNB may change UE serving cell by a cell switch command signalled via a MAC CE. In case of DAPS handover, the UE continues the downlink user data reception from the source gNB until releasing the source cell and continues the uplink user data transmission to the source gNB until successful random-access procedure to the target gNB. A Conditional Handover (CHO) is defined as a handover that is executed by the UE when one or more handover execution conditions are met.

The 3GPP management system should have the capability allowing an authorized consumer to enable or disable LTM, DAPS and CHO cell switch from one cell to another cell.

4.X4	Access Traffic Steering management

The functionality in an ATSSS-capable UE that can steer, switch and split the MA PDU Session traffic across 3GPP access and non-3GPP access, is called a "steering functionality" as defined in TS 23.501[2]. The UE indicates to the network its supported steering functionalities and steering modes by including in the UE ATSSS Capability including ATSSS-LL functionality, MPTCP functionality. If the UE includes in its ATSSS capabilities "MPTCP functionality with any steering mode and ATSSS-LL functionality with only Active-Standby steering mode", and if the DNN configuration allows MPTCP and ATSSS-LL with any steering mode (i.e. any Steering Mode allowed for ATSSS-LL), including RTT measurement without using PMF protocol.

Therefore, the 3GPP management system should have the capability to configure the ATSSS capability including ATSSS-LL functionality, MPTCP functionality and RTT measurement without using PMF for the UPF.

4.X5	SON management

In RAN, Self-optimisation including Mobility Load Balancing, Mobility Robustness Optimization, RACH Optimization, Coverage and Capacity Optimisation and Energy saving is the process of optimizing network performance and requires OAM management as defined in TS 38.300 [3]. The 3GPP management system should have the capability to configure the related management parameters for these use case.

The 3GPP management system should have the capability allowing an authorized consumer to switching on/off the SON function, making policies for the SON function, and evaluating the performance of the SON function.

Next Change

5.X1	Requirements for management of ProSe capabilities

REQ-PROSEC_NRM-CON-001 The NRM definitions shall support ProSe capabilities management.

REQ-PROSEC_NRM-CON-002 The NRM definitions shall support 5G DDNMF, 5G ProSe Direct Discovery, 5G ProSe Direct Communication and 5G ProSe UE-to-Network Relay management.

5.X2	Requirements for management of UPF capabilities

REQ-UPFC_NRM-CON-001 The NRM definitions shall support UPF capabilities management.

5.X3	Requirements for management of NR mobility

See the detailed requirements in TS 28.313 [x2]

5.X4	Requirements for management of Access Traffic Steering

REQ-ATSSS_NRM-CON-001 The NRM definitions shall support ATSSS capabilities management.

REQ-ATSSS_NRM-CON-001 The NRM definitions shall support ATSSS rules management.

5.X5	Requirements for management of SON function

See the detailed requirements in TS 28.313[x2] and TS 28.310 [x3]

End of changes