3GPP TSG-SA5 Meeting #162	S5-253522

Goteborg, Sweden, 25 - 29 August 2025

# Source:	Huawei

# Title:	Discussion on management of continuous MDT

# Document for:	Endorsement

Agenda Item:	6.19.14

1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

2	References

- S5-253296 (R3-253958), Reply LS on Continuous MDT, Response to R3-253024/ S5-252114, RAN3 (Ericsson)

- R3-250891, LS on Continuous MDT, RAN3 (Ericsson)

- R3-253024 (S5-252114), Reply LS to LS on Continuous MDT, Response to R3-250891, SA5 (Huawei, Nokia)

3	Rationale

In the last meeting RAN3 agreed to send a reply LS on Continuous MDT for RAN AI/ML purposes in [1].

This contribution analyzes the content of such reply LS for specifying support of Continuous MDT, and provides corresponding proposals for discussion.

An LS exchange involving RAN3 and SA5 for specifying support of Continuous MDT was initiated by RAN3 when it was agreed to send the LS to SA5 in [2]. In this LS RAN3 introduced the new concept of Continuous MDT, explaining which problems need to be addressed to practically enable Continuous MDT – i.e., measurement continuity across RRC state changes and during connected mode mobility, as well as trace correlation at Trace Collection Entity (TCE) of MDT reports provided by the same UE – and which are the requirements for a Continuous MDT solution from RAN3 perspective. The ACTION to SA5 was to check whether it was feasible to fulfil the requirements for Continuous MDT and solve the problem statements outlined, along with providing some guidance from SA5 on possible solutions for Continuous MDT for further RAN3 discussions.

SA5 replied to the RAN3 LS as in [3] with a set of questions for clarification related to the measurement continuity and corresponding OAM requirements, as well as to the trace correlation at TCE. As a consequence, RAN3 discussed further the topic on Continuous MDT based on the reply LS from SA5 in [3] and provided another reply LS in [1].

It is worth noting that, in their initial LS in [2], RAN3 stated the following:

To solve the Measurement Continuity Problem, RAN3 agreed that: *** skip non-relevant text *** - RAN3 will derive solutions for the Measurement Continuity Problem across RRC_Connected mobility. SA5 is requested to focus on the Measurement Continuity Problem across different RRC states. *** skip non-relevant text *** To solve the Trace Correlation Problem, namely to enable correlation of MDT measurements generated by the same UE, it is required to use a temporary identifier for the Continuous MDT, not revealing the UE identity. Such identifier should be made available at the TCE to enable correlation of the management-based MDT reports collected during the Continuous MDT, in a way that minimises the impacts on the Management Based MDT solution.

In our understanding, as a consequence of the above excerpt from [2], SA5 is requested to discuss the following two aspects:

- how to enable trace correlation at TCE of MDT reports provided by the same UE, while minimizing the impacts on the existing management-based MDT framework, and

- measurement continuity across UE’s RRC state changes.

We will discuss the above two aspects in the following subsections.

3.1 Trace correlation of MDT reports provided by the same UE

A UE selected by its serving gNB for Continuous MDT needs to support both Immediate MDT and Logged MDT. The UE performs measurements while in different RRC states, that is, Immediate MDT measurements when in connected mode, and Logged MDT measurements when in idle/inactive mode, and it reports them to the serving gNB in different time instances and anyway only when the UE is in connected mode. As a consequence, the gNB will provide such MDT reports to the TCE within separate Trace Files and in different time instances as well. The TCE then needs to be able to identify that the Immediate MDT and Logged MDT measurement reports are from the same UE, so that the TCE can correlate these MDT reports in order to create a unique dataset that could be used, e.g., for AI/ML model training purposes. It is worth to highlight that there is no need for the TCE to identify the UE which provided the Immediate MDT and Logged MDT reports, but what it is actually needed is that the TCE identifies which MDT reports are provided by the same UE.

Proposal 1: SA5 to agree that, for Continuous MDT, the TCE needs to identify which management-based MDT reports (Logged and Immediate) are provided by the same UE and there is no need for the TCE to identify/recognize that UE.

To enable trace correlation of Immediate MDT and Logged MDT reports from the same UE at TCE level, we think that a new Job Type which allows to configure a management-based MDT session encompassing Immediate MDT and Logged MDT simultaneously may be introduced, and assigning to this MDT session the single Trace Reference (TR). The OAM sends a Trace Session activation request to the gNB with configuration parameters including the Job Type and the TR, and then the gNB activates the MDT functionality for the selected UE and assigns a Trace Recording Session Reference (TRSR) to the selected UE.

Therefore, in management-based Continuous MDT, a single TR could be assigned to an MDT session having a Job Type equal to “Immediate MDT and Logged MDT”, and this configuration is valid for all UEs selected by the NG-RAN. In order to then correlate the Immediate MDT and Logged MDT reports from the same UE, the TCE could perform such correlation by selecting those MDT reports having the same couple TR/TRSR: the TR links the MDT reports one to each other (as they both belong to the same MDT session) while the TRSR links the MDT reports to a specific UE.

In our understanding, this is in line with the following two principles as outlined by RAN3 in their last LS in [1], where one possibility to identify the Continuous MDT process in NG-RAN is achieved in the form of a specific TR configured by the OAM, and where the trace correlation of MDT reports could be resolved by relying on the couple TR and TRSR:

[…]. Based on the replies from SA5, RAN3 had a discussion and agreed to the following principles: - Information from OAM to the NG-RAN nodes participating to Continuous MDT, allowing for the identification of a continuous MDT process, are provided e.g. in the form of specific Trace Reference(s). *** skip non-relevant text *** - RAN3 agrees to use TR(s) and TRSR(s) to resolve the MDT measurement correlation problem *** skip non-relevant text ***

Proposal 2: SA5 to agree on introducing a new Job Type “Immediate MDT and Logged MDT” for the Continuous MDT process; such process is identified by a single session-specific TR and UE-specific TRSR, and the couple TR/TRSR is used for solving the trace correlation problem.

3.2 Measurement continuity across UE’s RRC state changes

In our reply LS in [3], we asked RAN3 to provide further details on the measurement continuity across different RRC states:

RAN3: “- RAN3 will derive solutions for the Measurement Continuity Problem across RRC_Connected mobility. SA5 is requested to focus on the Measurement Continuity Problem across different RRC states.” Answer from SA5: SA5 would like RAN3 to provide some clarifications on what problem is referred by “the Measurement Continuity Problem across different RRC states”.

In their reply LS in [1], RAN3 clarified that:

Concerning the clarifications requested by SA5 on the Measurement Continuity Problem across different RRC states, RAN3 would like to clarify that the Measurement Continuity Problem across different RRC states refers to how to enable correlation of management-based MDT measurements collected while a UE was in RRC_Connected, RRC_Inactive and RRC_Idle.

To be specific, the scenario involves a UE which has been selected by its serving gNB while in connected mode and configured to perform both Immediate MDT and Logged MDT measurements according to the configuration provided by the OAM for the Continuous MDT process, which in our solution is characterized by the new Job Type “Immediate MDT and Logged MDT” and a corresponding single TR. After having performed Logged MDT measurements while in idle/inactive state, the UE re-establishes/resumes its RRC connection in a new cell and it reports the availability of Logged MDT measurements to the (potentially new) serving gNB. In order to avoid time interruptions in the measurement process, the UE should be reselected by the (potentially new) serving gNB immediately after it re-established/resumes its RRC connection and re-configured to perform Immediate MDT immediately after it re-establishes/resumes the RRC connection, i.e., just after it moves to connected state.

In their reply LS in [1], as part of the ACTION, RAN3 asked SA5 to check the feasibility of the principle:

[…]. Based on the replies from SA5, RAN3 had a discussion and agreed to the following principles: *** skip non-relevant text *** - Solutions for Continuous MDT shall not have an impact on the 5GC in Rel19 from a RAN3 perspective. *** skip non-relevant text ***

For the scenario analysed above, we think that, to ensure measurement continuity across different RRC states, it should be possible to rely on a network-based solution where the gNB serving a UE selected for Continuous MDT sends parts of the MDT configuration to the AMF (e.g., within the NG: UE CONTEXT RELEASE REQUEST) which stores such information. These parts of the MDT configuration can be stored within the UE context in the 5GC/AMF by default only for those UEs selected for Continuous MDT, i.e., configured with a management-based MDT configuration with Job Type “Immediate MDT and Logged MDT”. In this manner, once the UE re-establishes/resumes its RRC connection, the (potentially new) serving gNB will be provided by the 5GC/AMF with UE context data including user consent information as well as the stored parts of the MDT configuration (e.g., within the NG: INITIAL UE CONTEXT SETUP REQUEST).

These stored parts of the MDT configuration can be at least the single TR associated to the Continuous MDT process (i.e., the management-based MDT configuration having Job Type “Immediate MDT and Logged MDT”) and the TRSR previously allocated to the Continuous MDT UE by the (old) serving gNB, and these pieces of information can be used as an indication that the UE was previously selected by the (old) serving gNB for Continuous MDT, so that the (potentially new) serving gNB can re-select the same UE immediately and re-configure the UE to continue performing the Immediate MDT measurements in connected mode. This removes the burden of the (potentially new) serving gNB to recognize which UE has been previously selected and configured with the Continuous MDT by the (old) serving gNB, because it will become aware of this by simply checking the presence of the parts of the MDT configuration in the UE context data provided by the 5GC/AMF. If present, this means that the UE was previously selected for Continuous MDT by the (old) serving gNB and needs to be re-selected and re-configured again by the (potentially new) serving gNB.

We see the following drawbacks for solutions that do not affect 5GC, such as the ones relying on the content of the UE MDT measurements report and:

- the UE may provide the Logged MDT report not immediately after it re-established/resumes the RRC connection; the UE, in fact, needs to wait for the (potentially new) serving gNB to request the Logged MDT measurements from the UE, and this request typically occurs after the UE context in the (potentially new) serving gNB is created. As a consequence, the gNB cannot promptly understand that such UE was previously selected by the (old) serving gNB for Continuous MDT and this creates time interruptions in the measurement process which, for Continuous MDT, should be avoided as much as possible;

- even if assuming that the (potentially new) serving gNB requests the Logged MDT measurements from the UE in a very short time after the UE context in the (potentially new) serving gNB is created, the UE may report the Logged MDT measurements in a time instant which is well beyond the reception of the request from the (potentially new) serving gNB due to, e.g., the UE being currently busy with other procedures, such as an ongoing random access procedure. Again, this delays the gNB in re-selecting the UE for Continuous MDT and, as a consequence, creates time interruptions in the measurement process.

Proposal 3: SA5 to discuss and confirm that it is feasible to specify a network-based solution for Continuous MDT for measurement continuity across RRC state changes (i.e., after idle/inactive to connected state transition) based on storing in AMF/5GC at least the single TR associated to the Continuous MDT process and the TRSR assigned to the Continuous MDT UE.

The overall solution description based on the new Job Type “Immediate MDT and Logged MDT” and storing in 5GC/AMF (as part of the UE context information) at least the single TR (allocated by the OAM) for the management-based Continuous MDT configuration and the TRSR allocated by the serving gNB is provided in the Annex of this paper.

3.3 Additional aspects of the measurement continuity problem

In the reply LS from RAN3 in [1] the overlapping issue is analysed and RAN3 provided some feedback which is solution-specific:

In one of their replies SA5 stated: Answer: SA5 agrees that at lease TR and TRSR (which does not revealing UE identity) could be used for the correlation of the management-based MDT reports at TCE. SA5 would like RAN3 to elaborate the details of how to avoid overlapping In response to the question from SA5 to elaborate the details of how to avoid overlapping, RAN3 has discussed based on available solutions and provides the following details: *** skip non-relevant text *** RAN3 would like to ask SA5 to take the above agreements and descriptions from RAN3 into account and to specify a solution for continuous MDT accordingly.

In our understanding the overlapping issue relates to the TRSR allocation conflict that could occur between the (old) serving gNB and the (potentially new) serving gNB for the UE selected for Continuous MDT after idle/inactive-to-connected RRC state change. By assuming that the TRSR previously allocated by the (old) serving gNB cannot be re-assigned by the (potentially new) serving gNB (and received as part of the UE context data provided by the 5GC/AMF) because this TRSR is already assigned to another UE served by the (potentially new) serving gNB, a new TRSR (different from the one allocated by the (old) serving gNB) is allocated to the UE. In doing so, and for the purpose of trace correlation at TCE, it is important to keep the mapping relationship of TRSRs allocated to the UE, and we think that this mapping relationship can be kept by the (potentially new) serving gNB. The new TRSR allocated by the (potentially new) serving gNB is provided to the UE as part of the MDT configuration and, after the UE reports the Immediate MDT measurements to the (potentially new) serving gNB, the (potentially new) serving gNB sends this measurement report to the TCE including both the TR (selected by the OAM as part of the Continuous MDT configuration), the newly allocated TRSR and the mapping relationship of TRSRs allocated to the concerned UE. In this way, the TCE correlates the MDT reports from the (old) serving gNB marked with the couple TR/TRSR with the MDT reports from the (potentially new) serving gNB marked with a different couple TR/TRSR (note that the TR is the same in (old) serving gNB and (potentially new) serving gNB, while only the TRSR is different): this is possible because TCE also receives the mapping relationship of TRSRs allocated to UE by the (old) serving gNB and (potentially new) serving gNB.

Proposal 4: SA5 to discuss and agree on the (potentially new) serving gNB to keep the mapping relationship of TRSRs allocated to the Continuous MDT UE in case of TRSR allocation conflicts between the (old) serving gNB and the (potentially new) serving gNB. The (potentially new) serving gNB provides the mapping relationship of TRSRs to the TCE for the sake of trace correlation of MDT reports from the same UE.

Finally, in their reply LS in [1], RAN3 asked SA5 to check the security aspects of the solution for Continuous MDT:

RAN3 would like to ask SA5 to take the above agreements and descriptions from RAN3 into account and to specify a solution for continuous MDT accordingly. RAN3 asks SA5 to check the security aspects of the Continuous MDT solution derived.

In our understanding there could be a security concern due to the fact that the same TRSR allocated by the (old) serving gNB is reused by the (potentially new) serving gNB when it reselects the UE for Continuous MDT and such TRSR is sent over the Uu interface from the (potentially new) serving gNB to the UE when configuring the UE itself for Immediate MDT and Logged MDT measurements. This could be easily solved by relying on the mechanism illustrated above in Proposal 4 for solving the TRSR allocation conflict: the (potentially new) serving gNB could be mandated to assign a new TRSR at each idle/inactive-to-connected RRC state change and the (potentially new) serving gNB keeps (and reports to TCE) the mapping relationship of the TRSRs allocated to the UE for the sake of trace correlation at TCE.

Proposal 5: SA5 to agree that the security issues due to reusing the same TRSR for the Continuous MDT UE at each idle/inactive-to-connected RRC state change (if acknowledged) can be solved by mandating the (potentially new) serving gNB to re-allocate a new TRSR at each idle/inactive-to-connected RRC state change and the (potentially new) serving gNB keeping the mapping relationship of the TRSRs allocated to the UE.

4	Detailed proposal

It is proposed to endorse the proposal solution as specified in section 3, and response to RAN3 accordingly.

5. Annex – Solution for Continuous MDT

The overall solution description based on the new Job Type “Immediate MDT and Logged MDT” and storing in 5GC/AMF (as part of the UE context information) at least the single TR (allocated by the OAM) for the management-based Continuous MDT configuration and the TRSR allocated by the serving gNB is summarized in the following Figure 1, where user consent of the UE is also handled based on current specifications (e.g., TS 32.422), and changes with respect to legacy management-based MDT are highlighted in yellow. For the sake of simlicity the call-flow in Figure 1 assumes that the same TRSR is used at both gNB1 (old serving gNB) and gNB2 (new serving gNB).

Figure 1 - Network-based solution for management-based Continuous MDT: TCE correlation and measurement continuity across RRC state changes (from idle/inactive-to-connected transition).

- As part of the UE Context Setup procedure the AMF – which previously stored user consent information for UE1 from the UDM, not shown in Figure 1 – provides the gNB1 with user consent information if the user is within its home operator’s PLMNs and the user has given its consent, and user consent information of UE1 are stored in gNB1;

- OAM provides the gNB1 (and other gNBs in the Area scope, i.e., gNB2) with a management-based MDT configuration having Job Type “Immediate MDT and Logged MDT” and a corresponding single TR “TR#1”;

- gNB1 performs user selection among consented users (or users for which consent is not required) and, since it has received user consent information for UE1 in Step 1., UE1 is selected for Continuous MDT;

- gNB1 configures the UE1 to perform MDT measurements based on the management-based MDT configuration previously provided by the OAM and assigns a UE-specific TRSR “TRSR#1”;

- UE1 is in connected mode, hence it performs and reports to gNB1 the Immediate MDT measurements according to the management-based MDT configuration provided by the gNB1;

- gNB1 saves the received Immediate MDT measurements into a Trace Record file;

- The Trace Record file containing the Immediate MDT measurements is sent to the TCE including the couple TR/TRSR, i.e., “TR#1 / TRSR#1”;

- We now consider the case where UE1 moves to idle mode: in this RRC state UE1 performs (and logs) Logged MDT measurements according to the management-based MDT configuration provided by the gNB1 in Step 4.;

- As part of the UE Context Release procedure the gNB1 send at least “TR#1” and “TRSR#1” allocated to UE1 to the AMF which stores such information;

- We now assume that UE1 triggers transition from idle to connected mode under a new serving gNB, i.e., gNB2; at this point gNB2 is not aware that UE1 was previously selected by gNB1 for Continuous MDT measurements;

- At the end of the RRC Setup procedure involving UE1 and gNB2, gNB2 receives the availability indication from UE1 related to Logged MDT measurements available in the UE1;

- As part of the UE Context Setup procedure the AMF prepares UE context data of UE1 (e.g., PDU session context, security Key, UE Radio Capability and UE Security Capabilities, etc.) and sends it to gNB2; AMF also checks if the user is within its home operator’s PLMNs and the user has given its consent: if so, the AMF provides gNB2 with user consent information of UE1 as well as “TR#1” and “TRSR#1” allocated to UE1 by gNB1 and stored in AMF in Step 9.;

- UE context information of UE1, including user consent, “TR#1” and “TRSR#1”, are stored in gNB2;

- UE1 is now in connected mode under the coverage of gNB2;

- gNB2 performs user selection among consented users (or users for which consent is not required) and, since it has received user consent information for UE1 in Step 12., UE1 can be a candidate. The presence of “TR#1” and “TRSR#1” is an indication for gNB2 that UE1 was previously selected by gNB1 for Continuous MDT, implying that UE1 needs to be re-selected and re-configured by gNB2 to continue performing MDT measurements;

- As part of the UE RRC Reconfiguration towards UE1, gNB2 re-configures the UE1 to perform MDT measurements based on the management-based MDT configuration previously provided by the OAM in Step 2. and assigns a UE-specific TRSR: for simplicity, it is assumed that the same “TRSR#1” previously assigned to UE1 by gNB1 is also re-assigned by gNB2 – this is possible if “TRSR#1” was not already assigned to another UE served by gNB2, and how to handle TRSR assignment conflicts is discussed below as part of the measurement continuity during the same RRC state (i.e., at handover); UE1 can re-start Immediate MDT measurements;

NOTE: TRSR assignment conflicts and potential security concerns due to reusing the same TRSR in gNB1 and gNB2 can be solved by mandating the new serving gNB (gNB2) to assign a new TRSR at each idle/inactive-to-connected RRC state change for the UE and the new serving gNB keeping (and reporting to TCE) the mapping relationship of the TRSRs allocated to the UE for the sake of trace correlation at TCE

- gNB2 retrieves the Logged MDT measurements from UE1;

- gNB2 saves the received Logged MDT measurements of UE1 into another Trace Record file;

- The Trace Record file containing the Logged MDT measurements is sent to the TCE including the couple TR/TRSR, i.e., “TR#1 / TRSR#1”;

- The TCE correlates the Trace Record files received from gNB1 in Step 7. (with Immediate MDT measurements of UE1) and from gNB2 in Step 19. (with Logged MDT measurements of UE1) as both Trace Record files are “marked with” (and hence identifiable by) the same couple TR/TRSR, i.e., “TR#1 / TRSR#1”.