3GPP TSG-SA5 Meeting #162	S5-253531

Goteborg, Sweden, 25 - 29 August 2025

Source:	Huawei

Title:	Rel-19 pCR TS 28.567 Update the attribute definition in clause 6.4.1

Document for:	Approval

Agenda item:	6.19.4.1

Spec:	3GPP TS 28.567

Version:	V0.3.0

Work Item:	CCLM

Comments

Following issue exists in clause 6.4.1:

- GeneratedAlarmResult is a datatype, not an attribute.

- The type of cCLActionConflictsHandling should be CCLActionConflictsHandling.

- The type of conflictResolution should be ActionConflictResolution.

- Fix the properties for cCLType.

- Fix the typos.

This contribution aims to update the attribute definition in clause 6.4.1.

Proposed Changes

* * * First Change * * * *

6.4	Attribute definitions

6.4.1	Attribute properties

Table 6.4.1-1

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| scopeType | It indicates the type of scope that represented by the particular scope instance. allowedValues: CCL_MEASUREMENT_SCOPE, CCL_TARGET_SCOPE, CCL_CONTROL_SCOPE, CCL_IMPACT_SCOPE Editor’s Note: The allowed values will be revisited | type: Enum multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| coordinationCapability | It indicates a capability of a coordination entity to coordinate CCL conflicts | type: CoordinationCapability multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| cCLCoordinationCapabilityID | It indicates an identifier for a specific CCL conflicts coordination capability | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| closedControlLoopRefList | It indicates a list of DN for ClosedControlLoop Instances. allowedValues: N/A | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| cCLScopeCoordinationCapability | It indicates a specific type of CCL conflict coordination capacity | type: CCLScopeCoordinationCapability multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| coordinatedCCLsScopes | It indicates the scopes of the CCL that are coordinated by the coordinationEntity It is a pair <string_1, string_2 > where string_1 is the DN of a CCL being coordinated and string_2 the DN of that CCL’s CCLScope. | type: pair <string, string > multiplicity: 2 ..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| operationalState | It indicates the operational state of the ClosedControlLoop instance. It describes whether the resource is installed and partially or fully operable (Enabled) or the resource is not installed or not operable (Disabled). AllowedValues; Enabled/Disabled allowedValues: "ENABLED", "DISABLED". The meaning of these values is as defined in 3GPP TS 28.625 [14] and ITU-T X.731 [15]. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: Disabled isNullable: False |
| administrativeState | It indicates the administrative state of the ClosedControlLoop instance. It describes the permission to use or the prohibition against using the ClosedControlLoop instance. The administrative state is set by the MnS consumer. AllowedValues; Locked/Unlocked allowedValues: "LOCKED", "UNLOCKED". The meaning of these values is as defined in 3GPP TS 28.625 [14] and ITU-T X.731 [15]. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: Locked isNullable: False |
| cCLComponentsInfo | It indicates information on the constituent components of a CCL. allowedValues: N/A | type: CCLComponentInfo multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| cCLComponentId | It indicates the identifier of a CCL component. It is the DN of a object instantiated to act as a component of the CCL | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| cCLSteps | It indicates the CCL steps or functionality that is accomplished by a CCL component. allowedValues: DATA_COLLECTION, ANALYSIS, DECISION, EXECUTION | type: Enum multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| FaultManagementAlarmIdList | It describes the list of IDs of alarms to be managed by Fault Management CCL. allowedValues: A list of alarmIds as specified in TS 28.111 [4], clause 7.4.1 | type: List multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| FaultManagementTimeWindow | It describes the information of a time window (including start and end time) specified by the consumer for fault management to carry out troubleshooting and to clear the alarms. allowedValues: timeWindow as defined in 3GPP TS 28.622 [5], clause 4.4.1 | type: TimeWindow multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| FaultManagementBackUpObjectRequirement | It describes whether to back-up the alarmed object is required by the consumer before fault management. allowedValues: True, False | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| FaultManagementIsolateObjectRequirement | It describes whether to isolate the alarmed object from interaction with other objects is required by the consumer before fault management. allowedValues: True, False | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| clearUserId | It carries the identity of the Fault Management CCL who is the consumer that invokes the clearAlarms operation. allowedValues: clearUserId as defined in 3GPP TS 28.111 [4], clause 7.4.1 | type: string multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| FaultManagementCCLReport | It describes the Fault Management CCL report. allowedValues: Not Applicable | type: FaultManagementCCLReport multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| GeneratedAlarmResultList | It describes the list of generated alarm results allowedValues: A list of GeneratedAlarmResult | type: List multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| FaultManagementCCLReportTime | It describes the time when the FaultManagementCCLReport is created. allowedValues: DateTime as specified in TS 28.622 [5]. | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| alarmId | It identifies an AlarmRecord as specified in TS 28.111 [4] allowedValues: A string as specified in TS 28.111 [4] | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| alarmClearedStatus | It describes whether an alarm is cleared by the Fault Management CCL when the identified root cause is resolved. allowedValues: True, False | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| identifiedRootCauseInformation | It describes root cause information identified by the Fault Management CCL. allowedValues: String | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| enhancedCorrelationInformation | It describes the list of correlated alarm Ids identified by the Fault Management CCL allowedValues: A list of alarmId | type: List multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| cCLActionConflictsHandling | This defines the handling of CCL action conflict between the two existing CCLs. | Type: CCLActionConflictsHandling multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| conflictInformation | This defines the information related with a conflicting CCL. | Type: ConflictInformation multiplicity: * isOrdered: True isUnique: False defaultValue: None isNullable: False |
| conflictResolution | This defines the information related with conflict resolution. | Type: ActionConflictResolution multiplicity: * isOrdered: True isUnique: False defaultValue: None isNullable: False |
| targetCCL | The identification of the CCL that need to be deleted or updated to resolve conflict. This will be decided as per the information ConflictResolution. | Type: Dn multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| conflictingCCLId | This indicates the CCL identification | Type: Dn multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| conflictingActions | This provides the set of actions that have been taken by the CCL as part of the Execute step. | Type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| cCLPriority | This provides the priority of the CCL. This will be the numerical value between 1 to 10, with 1 being the least priority. | Type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| cCLMetricBreachPercentage | It defines the breach percentage per metric in terms of how bad the metric(s) is breached. For example, if the metric of guaranteed throughput is 200mbps and the actual throughput is coming to be 100mbps then the breach percentage would be 50%. The CCL that have higher percentage of breach will be prioritized | Type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| cCLComponentList | It indicates the list of components ating as steps of the CCL, each either a MnF or a MnS producer whose services can be part of the CCL. The cCLComponent may have a role among MONITOR; ANALYSIS; DECISION; EXECUTION. Or OTHER. OTHER. Is used for example in the caes where a components fulfile more than 1 role or where the role can be siml y described by the four options. The cCLComponents are sequenced, i.e., cCLComponents is an ordred list. For example, if there are 2 steps that contribute to the analysis role, it is necessary to show how those steps are sequenced. The order in which they are listed indicates the order in which their services should be chained to complete the CCL | type: CCLComponent multiplicity: 1..* isOrdered: True isUnique: True defaultValue: None isNullable: False |
| cCLType | It indicates a type or Category of CCL that is to be instantiated or dynamically composition. It indicates the kind of capability that will be accomplished by the CCL instance, e.g. ENERGYOPTIMIZATION, SLICEASSURANCE, etc. The specific details, characteristics and behavior of a CCL for a given CCL type are then written into the CCL purpose. Editor’s Note: Documentation and Allowed values will be revisited | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| cCLComponentRole | It indicates a role accomplished by CCL component. AllowedValues: MONITOR; ANALYSIS; DECISION; EXECUTION, OTHER. Is used for example in the caes where a components fulfile more than 1 role or where the role can be siml y described by the four options | type: Enum multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| cCLComponentIdentification | It indicates the entity accomplishing the component. It may be the the DN of an MOI or the combination of URI and DN that can be used to fulfil that role. | Type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| cCLActionTrigger | This defines the criteria/conditions under which the CCL is allowed to take actions. | Type: CCLTrigger multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| desiredBehavior | This will define the corresponding behavior of the CCL. The behaviors can be represented by an ENUM to include: - DECISION_ACTIVATION: The CCL executes the recommendations that it derives on to the network. - NOTIFY_RCOMMENDATION: The CCL starts processing input to derive recommendations but without the corresponding actions executed on the network. Instead, the recommendation is notified to the consumer who then considers whether it should be applied or not. - DO_NOTHING: do not do anything. | Type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

* * * End of Changes * * * *