# 3GPP TSG-SA5 Meeting #162	S5-253553

# Goteborg, Sweden, 25 - 29 Aug 2025

# Source:	Huawei

# Title:	Discussion on NSPA charging information correction

# Document for:	Discussion

Agenda Item:	7.3

1	Decision/action requested

To analyse the gap in NSPA charging information.

2	References

[1]	3GPP TS 28.201 	Charging management; Network slice performance and analytics charging in the 5G System (5GS); Stage 2

[2]	3GPP TS 23.288 Architecture enhancements for 5G System (5GS) to support network data analytics services

[3]	3GPP TS 28.532 Management and orchestration; Generic management services

[4]	3GPP TS 29.520 Network Data Analytics Services, stage3

3	Rationale

In the NSPA charging [1], the performance & analytics in the NSPA container, e.g. load level, are provided by either NWDAF or MnS Producer with a certain subscription condition at CEF. The subscription condition includes, e.g. start time and stop time, as described in clause TS 28.201 [1] clause 5.2.2.3 and 5.2.2.4.

“Subscribe Request: the CEF decides to start the collection of performance and analytics information and subscribes to performance and analytics information from NWDAF with the performance indicator, S-NSSAI and subscription condition (e.g. Start time, stop time).”  --- TS 28.201 [1] clause 5.2.2.3

The start time and stop time are beneficial to the network slice charging, e.g., indicating the applicable time window of each performance analytics, which currently not included in the charging information.

According the NWDAF specification TS 29.520 [4], the subscription request includes "startTs" and "endTs" attributes to indicate the start and end time of the observation period requested by NF (e.g., CEF) as specified in clause 5.1.6.2.7. The event notification includes "start" and "expiry" attribute to indicate the start and expiration time of which the statistics analytics information is applicable, the validity period specified by “start” and “expiry” attributes is a subset of the analytics target period indicated by “startTs" and "endTs" as specified in clause 5.1.6.2.5. The event notification also includes a separate attribute "timeStampGen" to indicate the timestamp of analytics generation. The timeline logic for request time and response time can be summarized as follows:

Figure 1. Example of the timeline for analytics subscription with NWDAF

According the MnS specification in TS 28.532 [3], the subscription request also includes "beginTime" and "endTime" to indicate the period over which the statistics or predictions are requested as specified in clause 11.6.1.5.2. The performance data file which is notified to MNS consumers includes the "collectionBeginTime" and "collectionEndTime" attributes to indicate the period of collected performance metrics stored in the file as specified in clause 11.3.2.1.2. The timeline logic for request time and response time can be summarized as follows:

Figure 2. Example of the timeline analytics subscription with MnS Producer

To this end, we recommend capturing the requested analytics period in the NSPA container to match all notifications within that analysis period. The requested analysis period for the NWDAF is indicated by the "startTs" and "endTs" attributes, the requested analysis period for the MnS is indicated by the "beginTime" and "endTime" attributes.

In addition, given the NSPA container may carry analytics from either NWDAF or MnS Producer, it is also important to indicate the source network identification that provide the analytics.

4	Detailed proposal

Based on the above analysis, it is proposed to extend the NSPA container with three attributes: source NF identification, start timestamp and stop timestamp.

Therefore, a set of stage 2 (TS 28.201) and stage 3 (TS 32.291 and TS 32.298) CRs for Rel-19 are proposed for approval.