3GPP TSG-SA5 Meeting #162	S5-253616

Goteborg, Sweden, 25 - 29 August 2025

# Source:	CSCN

# Title:	New WID on IMS call charging capability optimization

# Document for:	Approval

# Agenda Item:	7.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	New WID on IMS call charging capability optimization

Acronym:

Unique identifier:

Potential target Release:	Rel-19

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  |  |  |
| No |  |  |  |  |  |
| Don't know |  |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
| X | Normative – Stage 2 |
| X | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A |  |  |  |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| N/A |  |  |

Dependency on non-3GPP (draft) specification: None

3	Justification

For offline charging, in TS 32.299 section 6.1.3.4, it is described that the CDF closes a CDR when it detects that expected Diameter ACRs for a particular session have not been received for a period of time. When the expected ACR is ACR[Stop] but it is lost, CDF will close this CDR as an incomplete CDR.  Per TS 32.298, the serviceDeliveryEndTimeStamp field in CDR is retrieved from SIP-Request-Timestamp AVP of ACR[Stop] message indicating a session termination. So in this case,  serviceDeliveryEndTimeStamp field in CDR will be not present and no filed can provide a timestamp indicating the session termination.

For IMS charging, the valid charging information must include both the timestamp indicating the session start and the timestamp indicating the session termination, so it’s necessary to enhance the charging system to provide the session termination timestamp in the ACR[Stop] lost scenario. Actually, per TS 32.260, during a session, CTF will send ACR[Interim] to CDF due to a session modification procedure (i.e. change in media) or expiration of the Charging Data Interim Interval. So CDF can retrieve the timestamp carried in the ACR[Interim] upon reception of an ACR[Interim] message. It is possible to add a new field in CDR based on this information to solve this problem.

The similar scenario may also happen in online charging. That is, CCR[Terminate] is lost for OCS. So it needs to consider how to get the valid session termination timestamp in this scenario, too. Per TS 32.299 section 6.3.5, the last CCR[Update] shall report the used service units till that time, so it is possible for OCS to get the valid session termination time based on this information.

4	Objective

The objective of the work item is to provide valid session termination timestamp in charging system in case of ACR[Stop] or CCR[Terminate] is lost, with following tasks.

-WT-1: Specify the session termination timestamp for ACR[Stop] lost scenario in offline charging;

-WT-2: Specify the session termination timestamp for CCR[Terminate] lost scenario in offline charging.

TU estimates and dependencies

| Work Task ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | SA Dependency (Yes/No/Maybe) | Non-3GPP Dependency |
| --- | --- | --- | --- | --- | --- |
| WT-1 | 0 | 0.5 | No | Yes | No |
| WT-2 | 0 | 0.5 | No | Yes | No |

Total TU estimates for the study phase: 0

Total TU estimates for the normative phase: 1

Total TU estimates: 1

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 32.260 | Enhancements to IMS CDR contents | TSG#109 (Sep. 2025) |  |
| TS 32.298 | Enhancements to IMS CDR parameters and charging information in ASN.1 | TSG#109 (Sep. 2025) |  |
| TS 32.299 | Enhancements to offline and online charging error cases(Diameter procedures) | TSG#109 (Sep. 2025) |  |

6	Work item Rapporteur(s)

To be assigned by 3GPP SA.

7	Work item leadership

SA5

8	Aspects that involve other WGs

9	Supporting Individual Members

Supporting IM name

China Telecom

CATT

ZTE

China Unicom