3GPP TSG-SA5 Meeting #162	S5-253648

Goteborg, Sweden, 25 - 29 August 2025

# Source:	CSCN

# Title:	DP on CDR generation capability optimization

# Document for:	Discussion

Agenda Item:	7.2

1	Decision/action requested

Discussion to optimize CDR generation capability

2	References

[1] 3GPP TS 32.240 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects;

Telecommunication management; Charging management; Charging architecture and principles

[2] 3GPP TS 32.297 3rd Generation Partnership Project;Technical Specification Group Services and System Aspects;

Telecommunication management; Charging management; Charging Data Record (CDR) file format and transfer

3	Rationale

In offline only charging  system, the service-control decision is implemented in billing domain using CDR files. BD collects and processes CDRs files, and based on the charging information provided by CDR, BD charges on the subscriber account. If BD finds the balance in subscriber account is below specific threshold, BD shall make corresponding service-control decisions, such as to decrease the maximum bit rate , or stop service, etc.

Per TS 32.240, CDRs files are generated as below procedure:

Upon receiving a charging event, the CDF uses the event to create/open a CDR. The CDR may be closed due to a number of reasons configured on the CDF or dependent on implementation, including but not limited to:

-	time limit;

-	volume limit;

-	limit of change of charging conditions;

-	end of user session, e.g. reception of the final charging event describing the session termination;

-	limits (e.g. memory size) imposed by implementation.

After the CDR is closed in CDF, it is immediately transferred to the CGF. The CDRs may be routed to one of several simultaneously open files inside the CGF. CDR files are closed on the CGF based on certain operator configured parameters, for example:

-	file size limit,

-	file duration (time) limit,

-	time of day,

-	maximum number of CDRs.

From above procedure, we can see that the threshold to close CDR on CDF and the threshold to close CDR files on CGF have an important impact on the delay that BD makes service-control decisions after subscriber account reaches the service-control threshold. The higher the threshold in CDF and CGF is, the longer the delay for BD to make the service-control decision is.

For example, for IMS charging, if the time limit threshold is set to 30 minutes in CDF to close CDR and the file duration limit is set to 15 minutes in CGF to close CDR files, there will be 45 minutes delay in BD to take credit-control action at most.

So it is necessary to support the threshold configuration by groups on CDF and CGF. With this change, CDF and CGF can configure a lower threshold to close CDR and CDR files for high-risk subscriber group so that BD can make service-control for them more timely. On the other hand. a higher threshold can be configured for low-risk subscriber group, so that CDF and CGF can close CDR  and CDR files more infrequently to save signaling and processing loads.

4	Detailed proposal

When CDF closes CDR  due to a number of reasons, such as

-	time limit;

-	volume limit;

Multiple thresholds for one closure reason can be configured on one CDF. For example, CDF supports to close CDRs due to time limit by 30 minutes for group A and by 15 minutes for group B simultaneously.

When CGF closes CDR files based on certain operator configured parameters, such as

-	file size limit,

-	file duration (time) limit,

Multiple threshold for one close reason can be configured by groups in one CGF. For example, CGF supports to close CDR files due to file duration limit by 15 minutes for group A and by 5 minutes for group B simultaneously.