3GPP TSG-SA5 Meeting #162	S5-253856

Goteborg, Sweden, 25 - 29 August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.104 | CR | 0230 | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.104 Add MDA type definition for ATSSS traffic steering analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMDAS_Ph3 |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | MDA type definition for ATSSS traffic steering analytics is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new MDA type definition for ATSSS traffic steering analytics. Corrected some editorial issues. Changed order of MDA type definitions to align with the structure of clause 8.4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unable to request MDA analysis for ATSSS traffic steering analytics. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.2.9.1, 8.6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 8.4.2.9.1	MDA type

The MDA type for Traffic congestion prediction analysis is: SLSAnalysis.TrafficCongestionPredictionAnalysis.

* * * Next Change * * * *

### 8.6.1	MDAType <<enumeration>>

Table 8.6.1-1: <<enumeration>> MDAType

| Enumeration value | Description |
| --- | --- |
| COVERAGE_ANALYTICS_COVERAGE_PROBLEM_ANALYSIS | Indicates that the MDA type for the Coverage problem analysis defined in clause 8.4.1.1 |
| COVERAGE_ANALYTICS_PAGING_OPTIMIZATION | Indicates that the MDA type for the Paging Optimization defined in clause 8.4.1.2 |
| SLS_ANALYSIS_SERVICE_EXPERIENCE_ANALYSIS | Indicates that the MDA type for the Service experience analysis defined in clause 8.4.2.1 |
| SLS_ANALYSIS_NETWORK_SLICE_THROUGHPUT_ANALYSIS | Indicates that the MDA type for the Network slice throughput analysis defined in clause 8.4.2.2 |
| SLS_ANALYSIS_NETWORK_SLICE_TRAFFIC_ANALYSIS | Indicates that the MDA type for the Network slice traffic prediction defined in clause 8.4.2.3 |
| SLS_ANALYSIS_E2E_LATENCY_ANALYSIS | Indicates that the MDA type for the E2E latency analysis defined in clause 8.4.2.4 |
| SLS_ANALYSIS_NETWORK_SLICE_LOAD_ANALYSIS | Indicates that the MDA type for the Network slice load analysis defined in clause 8.4.2.5 |
| UE_THROUGHPUT_ANALYSIS_TRAFFIC_CONGESTION_PROBLEM_ANALYSIS | Indicates that the MDA type for the UE throughput analysis defined in clause 8.4.2.6 |
| SLSANALYSIS_EDGE_APPLICATION_DEPLOYMENT_LOCATION_ANALYSIS | Indicates that the MDA type for the Edge application deployment location analysis defined in clause 8.4.2.7 |
| SLSANALYSIS_EDGE_COMPUTING_PERFORMANCE_ANALYSIS | Indicates that the MDA type for the Edge Computing Performance Analysis defined in clause 8.4.2.8 |
| SLSANALYSIS_TRAFFIC_CONGESTION_PREDICTION_ANALYSIS | Indicates that the MDA type for the traffic congestion prediction analysis defined in clause 8.4.2.9 |
| MDA_ASSISTED_FAULT_MANAGEMENT_FAILURE_PREDICTION | Indicates that the MDA type for the MDA assisted failure prediction defined in clause 8.4.3.1 |
| MDA_ASSISTED_ENERGY_SAVING_ENERGY_SAVING_ANALYSIS | Indicates that the MDA type for the Energy saving analysis defined in clause 8.4.4.1 |
| MOBILITY_MANAGEMENT_ANALYTICS_MOBILITY_PERFORMANCE_ANALYSIS | Indicates that the MDA type for the Mobility performance analysis defined in clause 8.4.5.1 |
| MOBILITY_MANAGEMENT_ANALYTICS_HANDOVER_OPTIMIZATION | Indicates that the MDA type for the Handover Optimization analysis defined in clause 8.4.5.2 |
| MAINTENANCE_MAINTENANCE_ANALYTICS | Indicates that the MDA type for the Maintenance analytics defined in clause 8.4.6.1 |
| MAINTENANCE_SOFTWARE_UPGRADE_VALIDATION_ANALYTICS | Indicates that the MDA type for the Software upgrade validation analytics defined in clause 8.4.6.2 |
| RESOURCE_ANALYTICS_VIRTUALIZED_RESOURCE_UTILIZATION_ANALYSIS_NF | Indicates that the MDA type for the Virtualized resource utilization analysis defined in clause 8.4.7.1.1 |
| RESOURCE_ANALYTICS_PHYSICAL_RESOURCE_UTILIZATION_ANALYSIS_NF | Indicates that the MDA type for the Physical resource utilization analysis defined in clause 8.4.7.1.2 |
| RESOURCE_ANALYTICS_5GC_CONTROL_PLANE_CONGESTION_ANALYSIS | Indicates that the MDA type for the 5GC Control plane congestion analysis defined in clause 8.4.7.1.3 |
| PREDICTIONS_PM_DATA | Indicates that the MDA type for the MDA assisted PM predictions defined in clause 8.4.8.1 |
| ATSSS_PERFORMANCE_TRAFFIC_STEERING_ANALYTICS | Indicates that the MDA type for the Traffic steering analytics defined in clause 8.4.9.1 |
| CORRELATION_ANALYTICS_TRAINING_DATA_ANALYSIS | Indicates that the MDA type for the Correlation analysis defined in clause 8.4.10.1 |
| CORRELATION_ANALYTICS_NF_SCALING_DIMENSIONING_DATA_ANALYSIS | Indicates that the MDA type for the NF Scaling and dimensioning analysis defined in clause 8.4.10.2 |

* * * Next Change * * * *

Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1858 at commit ba522871d40e50006389d43af255388ba89d0230

*** OpenAPI/TS28104_MdaNrm.yaml ***

<CODE BEGINS>

openapi: 3.0.1

info:

title: MDA NRM

version: 19.2.0

description: >-

OAS 3.0.1 specification of the MDA NRM

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.104; MDA

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.104/

paths: {}

components:

schemas:

#-------- Definition of types-----------------------------------------------------

MDAType:

type: string

enum:

- COVERAGE_ANALYTICS_COVERAGE_PROBLEM_ANALYSIS

- COVERAGE_ANALYTICS_PAGING_OPTIMIZATION

- SLS_ANALYSIS_SERVICE_EXPERIENCE_ANALYSIS

- SLS_ANALYSIS_NETWORK_SLICE_THROUGHPUT_ANALYSIS

- SLS_ANALYSIS_NETWORK_SLICE_TRAFFIC_ANALYSIS

- SLS_ANALYSIS_E2E_LATENCY_ANALYSIS

- SLS_ANALYSIS_NETWORK_SLICE_LOAD_ANALYSIS

- UE_THROUGHPUT_ANALYSIS_TRAFFIC_CONGESTION_PROBLEM_ANALYSIS

- SLS_ANALYSIS_EDGE_APPLICATION_DEPLOYMENT_LOCATION_ANALYSIS

- SLS_ANALYSIS_EDGE_COMPUTING_PERFORMANCE_ANALYSIS

- SLS_ANALYSIS_TRAFFIC_CONGESTION_PREDICTION_ANALYSIS

- MDA_ASSISTED_FAULT_MANAGEMENT_FAILURE_PREDICTION

- MDA_ASSISTED_ENERGY_SAVING_ENERGY_SAVING_ANALYSIS

- MOBILITY_MANAGEMENT_ANALYTICS_MOBILITY_PERFORMANCE_ANALYSIS

- MOBILITY_MANAGEMENT_ANALYTICS_HANDOVER_OPTIMIZATION

- MAINTENANCE_MAINTENANCE_ANALYTICS

- MAINTENANCE_SOFTWARE_UPGRADE_VALIDATION_ANALYTICS

- RESOURCE_ANALYTICS_VIRTUALIZED_RESOURCE_UTILIZATION_ANALYSIS_NF

- RESOURCE_ANALYTICS_PHYSICAL_RESOURCE_UTILIZATION_ANALYSIS_NF

- RESOURCE_ANALYTICS_5GC_CONTROL_PLANE_CONGESTION_ANALYSIS

- PREDICTIONS_PM_DATA

- ATSSS_PERFORMANCE_TRAFFIC_STEERING_ANALYTICS

- CORRELATION_ANALYTICS_TRAINING_DATA_ANALYSIS

- CORRELATION_ANALYTICS_NF_SCALING_DIMENSIONING_DATA_ANALYSIS

MDATypes:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/MDAType'

MDAOutputs:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/MDAOutputPerMDAType'

AnalyticsScopeType:

oneOf:

- type: object

properties:

managedEntitiesScope:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

- type: object

properties:

areaScope:

type: array

uniqueItems: true

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/GeoArea'

MDAOutputPerMDAType:

type: object

properties:

mDAType:

$ref: '#/components/schemas/MDAType'

mDAOutputIEFilters:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/MDAOutputIEFilter'

MDAOutputIEFilter:

type: object

properties:

mDAOutputIEName:

type: string

filterValue:

type: string

threshold:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/ThresholdInfo'

analyticsPeriod:

$ref: '#/components/schemas/AnalyticsSchedule'

timeOut:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

ReportingMethod:

type: string

enum:

- FILE

- STREAMING

- NOTIFICATION

ReportingTarget:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Uri'

AnalyticsSchedule:

oneOf:

- type: object

properties:

timeDurations:

type: array

uniqueItems: true

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/TimeWindow'

- type: object

properties:

granularityPeriod:

type: integer

ThresholdInfo:

type: object

properties:

monitoredMDAOutputIE:

type: string

thresholdDirection:

type: string

enum:

- UP

- DOWN

- UP_AND_DOWN

thresholdValue:

oneOf:

- type: integer

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/Float'

hysteresis:

oneOf:

- type: integer

minimum: 0

- type: number

format: float

minimum: 0

#-------- Definition of types for name-containments ------

SubNetwork-ncO-MdaNrm:

type: object

properties:

MDAFunction:

$ref: '#/components/schemas/MDAFunction-Multiple'

MDAReport:

$ref: '#/components/schemas/MDAReport-Multiple'

ManagedElement-ncO-MdaNrm:

type: object

properties:

MDAFunction:

$ref: '#/components/schemas/MDAFunction-Multiple'

#-------- Definition of abstract IOCs --------------------------------------------

#-------- Definition of concrete IOCs --------------------------------------------

MDAFunction-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

supportedMDACapabilities:

$ref: '#/components/schemas/MDATypes'

mLModelRefList:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnListRo'

aIMLInferenceFunctionRefList:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnListRo'

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-ncO'

- type: object

properties:

MDARequest:

$ref: '#/components/schemas/MDARequest-Multiple'

MDAReport:

$ref: '#/components/schemas/MDAReport-Multiple'

MDARequest-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- type: object

properties:

requestedMDAOutputs:

$ref: '#/components/schemas/MDAOutputs'

reportingMethod:

$ref: '#/components/schemas/ReportingMethod'

reportingTarget:

$ref: '#/components/schemas/ReportingTarget'

analyticsScope:

$ref: '#/components/schemas/AnalyticsScopeType'

startTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

stopTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

recommendationFilter:

$ref: '#/components/schemas/AnalyticsScopeType'

performanceThresholdInfo:

type: array

items:

$ref: 'TS28623_ThresholdMonitorNrm.yaml#/components/schemas/ThresholdInfo'

thresholdMonitorRefList:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

MDAReport-Single:

$ref: 'TS28104_MdaReport.yaml#/components/schemas/MDAReport'

#-------- Definition of JSON arrays for name-contained IOCs ----------------------

MDAFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/MDAFunction-Single'

MDARequest-Multiple:

type: array

items:

$ref: '#/components/schemas/MDARequest-Single'

MDAReport-Multiple:

type: array

items:

$ref: '#/components/schemas/MDAReport-Single'

#-------- Definitions in TS 28.104 for TS 28.532 ---------------------------------

resources-mdaNrm:

oneOf:

- $ref: '#/components/schemas/MDAFunction-Single'

- $ref: '#/components/schemas/MDARequest-Single'

- $ref: '#/components/schemas/MDAReport-Single'

<CODE ENDS>

* * * End of Changes * * * *