3GPP TSG-SA5 Meeting #162	S5-254006

Goteborg, Sweden, 25 - 29 August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.662 | CR | 0023 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | R19 CR 28662 Correct references to align with RAN specifications |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2025-08-25 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | An outdated reference to a RAN specificaiton has been corrected. A reference to a RAN specification has been outdated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Not used references are voided. New references for E-UTRAN and NR are added. A not used abbreviation is deleted. An abbreviation included in 21.905 is deleted. References are changed to be according to 21.801. Attribute constraints that were run time constraints has been changed to be design time constaints. Editorial changes are done. “Negative references” have been removed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Interoperability problems may occur. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.2, 4.1, 4.3.3.3, 4.3.4.1, 4.3.5.1, 4.3.6.3, 4.3.7.1, 4.4.1, 4.5.1and 4.5.2. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of S5-253692 |  |  |  |  |  |  |  |  |

1st change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 32.101: "Telecommunication management; Principles and high level requirements".

[3]	3GPP TS 32.102: "Telecommunication management; Architecture".

[4]	3GPP TS 32.150: "Telecommunication management; Integration Reference Point (IRP) Concept and definitions".

[5]	Void.

[6]	Void.

[7]	3GPP TS 36.104: "Evolved Universal Terrestrial Radio Access (E_UTRA); Base Station (BS) radio transmission and reception".

[8]	Void.

[9]	Void.

[10]	3GPP TS 28.661: "Telecommunication management; Generic Radio Access Network (RAN) Network Resource Model (NRM) Integration Reference Point (IRP); Requirements".

[11]	3GPP TS 32.111-2: "Telecommunication management; Fault Management; Part 2: Alarm Integration Reference Point (IRP): Information Service (IS)".

[12]	3GPP TS 28.652: "Telecommunication management; Universal Terrestrial Radio Access Network (UTRAN) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[13]	3GPP TS 28.658: "Telecommunication management; Evolved Universal Terrestrial Radio Access Network (E-UTRAN) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[14]	3GPP TS 28.655:"Telecommunication management; GSM/EDGE Radio Access Network (GERAN) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[15]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[16]	3GPP TS 32.302: "Telecommunication management; Configuration Management (CM); Notification Integration Reference Point (IRP): Information Service (IS)".

[17]	3GPP TS 32.662: "Telecommunication management; Configuration Management (CM); Kernel CM Information Service (IS)".

[18]	3GPP TS 25.106: "Technical Specification Group Radio Access Network; UTRA repeater radio transmission and reception".

[19]	3GPP TS 45.005: "Radio transmission and reception".

[20]	3GPP TS 45.010: "Radio subsystem synchronization".

[21]	3GPP TS 25.104: "Base Station (BS) radio transmission and reception (FDD)".

[22]	Void.

[23]	3GPP TS 38.104: "NR; Base Station (BS) radio transmission and reception".

[24]	3GPP TS 28.541: "NR and NG-RAN Network Resource Model (NRM) stage 2 and stage 3".

[25] 	3GPP TS 28.652: "UTRAN Network Resource Model (NRM) Integration Reference Point (IRP): Information Service (IS)".

[26]	3GPP TS 37.466: "Iuant Interface: Application Part".

[27]	3GPP TS 28.532: "Management and Orchestration: Generic Management Services".

[y]	3GPP TS 36.106: "FDD repeater radio transmission and reception"

2nd change

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

CM	Configuration Management

IOC	Information Object Class

RDN	Relative Distinguished Name

SS	Solution Set

3rd change

## 4.1	Imported information entities and local labels

| Label reference | Local label |
| --- | --- |
| TS 28.622 [15], IOC, ManagedFunction | ManagedFunction |
| S 28.652 [12], IOC, UtranGenericCell | UtranGenericCell |
| TS 28.658 [13], IOC, EUtranGenericCell | EUtranGenericCell |
| TS 28.655 [14], IOC, GSMCell | GSMCell |
| TS 28.541 [24], IOC, NRSectorCarrier | NRSectorCarrier |
| TS 28.541 [24], IOC, NRCellDU | NRCellDU |
| TS 28.658 [13], IOC, ENBFunction | ENBFunction |
| TS 28.652 [25], IOC, NodeBFunction | NodeBFunction |
| TS 28.655 [14], IOC, BssFunction | BssFunction |

4th change

#### 4.3.3.3	Attribute Constraints

| Name | Definition |
| --- | --- |
| theCellList | Condition: Association between SectorEquipmentFunction and ProxyCellList is absent; and association between SectorEquipmentFunction and NRSectorCarrier is absent. |
| tmaBaseStationId through tmaGainResolution | Condition: The interface towards the TMA subunit supports TS 37.466 [26] |

5th change

#### 4.3.4.1	Definition

A GSM cell can consist of a number of carriers. These carriers can be configured in a number of ways, for example, the carriers can have different propagation properties which are sent with different antenna tilt, with different RF power, different radio band and even possibly different antenna.

The various GSMCellPart instances capture different radio propagation properties allowing different frequency planning schemes, e.g. some GSMCellPart instances can use frequency groups planned for tighter frequency reuse.

Hence, a GSM cell can, and in some cases must, be distributed on more than one SectorEquipmentFunction.

This IOC is required as part of the capability to satisfy the Requirements statement identified below.

| Referenced TS | Requirement label | Comment |
| --- | --- | --- |
| TS 28.661 [10] | REQ-GRAN_NRM-CON-001 |  |
| TS 28.661 [10] | REQ-GRAN_NRM-CON-002 |  |

6th change

#### 4.3.5.1	Definition

This IOC represents common aspects of Base Station (BS) functionality shared by several radio access technologies.

| Referenced TS | Requirement label | Comment |
| --- | --- | --- |
| TS 28.661 [10] | REQ-GRAN_NRM-CON-001 |  |
| TS 28.661 [10] | REQ-GRAN_NRM-CON-002 |  |

7th change

#### 4.3.6.3	Attribute constraints

| Name | Definition |
| --- | --- |
| relatedSectorEquipment | Condition: Supporting the GERAN sharing case. In such case, there shall be at least one GSMCellPart present at one end of this association. |
| relatedAntennaList | Condition: Association between SectorEquipmentFunction and ProxyCell is not supported. |
| relatedTMAList | Condition: Association between SectorEquipmentFunction and ProxyCell is not supported. |

8th change

#### 4.3.7.1	Definition

This IOC represents the management aspect of a repeater. For the information on repeater see 3GPP TS 25.106 [18] for UMTS and TS 36.106 [y] for LTE.

This IOC is required as part of the capability to satisfy the Requirements statement identified below.

| Referenced TS | Requirement label | Comment |
| --- | --- | --- |
| TS 28.661 [10] | REQ-GRAN_NRM-CON-003 |  |

9th change

### 4.4.1	Attribute properties

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| aRFCN | This attribute (Absolute Radio Frequency Channel Number) defines a pair of Radio Frequency (RF) channel frequencies for uplink and downlink use. See TS 45.005 [19] clause 2 for the ARFCN for GSM. ARFCN are based on a 200 kHz channel raster. allowedValues: See TS 45.005 [19] clause 2. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| aTA | This attribute (allowed Timing Advance) defines the signal sent by the BTS to the MS which the MS uses to advance its timings of transmissions to the BTS so as to compensate for propagation delay. allowedValues: See TS 45.010 [20] | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| bearing | The bearing in degrees that the antenna is pointing in. Antenna bearing" in Ref. TS 37.466 [26]. allowedValues: See "Antenna bearing" in TS 37.466 [26]. | type: Integer multiplicity: 0..1 isOrdered: True isUnique: False defaultValue: None isNullable: False |
| confOutputPower | It defines the allowed total power to use for all cells together in this sector. It may be set by the operator and/or set to a value limited by HW limitation or licensed power, e.g.: 20, 40, 60, 80,120 watts allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ctrlConnMode | Remote communication mode used by a repeater to send and receive control message, such as GSM SMS, WCDMA SMS, Circle Switch Data-CSD, Package Switch Dat-IP, Serial port. allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| dlAttenuation | Downlink signal attenuation of the device to change downlink gain. allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| environmentInfo | The repeater device is located either in the building or out of the building. allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| firmwareVer | Version of the device firmware. allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| fqBandList | The list of frequency bands/ranges supported by the hardware associated with the SectorEquipmentFunction. Valid frequency bands/ranges may be found in TS 25.104 (UTRA) [21], 36.104 (E-UTRA) [7] and 38.104 (NR) [23]. AllowedValues: Operating band id or supported <UL,DL,mode> expressed as a string. Examples for NR: Bands: {“n1”, “n12”} Frequencies: {“1920–1980, 2110–2170, FDD”, “699–716, 729–746, FDD”} | type: String multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| elevation | The elevation the antenna function should have, based on World Geodetic System (1984 version) global reference frame (WGS 84). allowedValues: Positive values correspond to meters above sea level. Negative values correspond to meters below sea level. If empty, value is not defined. | type: Real multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| horizBeamwidth | The 3 dB power beamwidth of the antenna pattern in the horizontal plane. A value of 360 indicates an omni-directional antenna. Note: The value of this attribute has no operational impact on the network, e.g. the NE behaviour is not affected by the value setting of this attribute. Note as well that this attribute is not supported over the Iuant interface. A single integral value corresponding to an angle in degrees between 0 and 360. allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| latitude | The latitude of the antenna location based on World Geodetic System (1984 version) global reference frame (WGS 84). Positive values correspond to the northern hemisphere. allowedValues: -90.0000 to +90.0000 | type: Real multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| longitude | The longitude of the antenna location based on World Geodetic System (1984 version) global reference frame (WGS 84). Positive values correspond to degrees east of 0 degrees longitude. allowedValues: -180.0000 to +180.0000 | type: Real multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| maxAzimuthValue | The maximum amount of change of azimuth the RET system can support. This is the change in degrees clockwise from bearing. Note: The value of this attribute has no operational impact on the network, e.g. the NE behaviour is not affected by the value setting of this attribute. Note as well that this attribute is not supported over the Iuant interface. A single integral value corresponding to an angle in degrees between 0 and 360 with a resolution of 0.1 degrees. allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| minAzimuthValue | The minimum amount of change of azimuth the RET system can support. This is the change in degrees counter-clockwise from bearing. Note: The value of this attribute has no operational impact on the network, e.g. the NE behaviour is not affected by the value setting of this attribute. Note as well that this attribute is not supported over the Iuant interface. A single integral value corresponding to an angle in degrees between 0 and 360 with a resolution of 0.1 degrees. allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| priority | The priority of a repeater decided by an operator. allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| powerSwitch | Power switch of device which has two status: ON/OFF. allowedValues: ON, OFF | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| relatedAntennaList | This attribute contains the DNs of one or more AntennaFunction. allowedValues: N/A | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| relatedSectorEquipment | This attribute contains the DN of one SectorEquipmentFunction. allowedValues: N/A | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| relatedTMAList | This attribute contains the DNs of one or more TmaFunction. allowedValues: N/A | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| repeaterType | The repeater type defined by operator, such as wide band, frequency selective, indoor and fiber optic. allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| retGroupName | The group name is a textual, alpha-numeric string to define a logical grouping of antennas which may be in different cells. This attribute permits the definition of a logical grouping of the antennas. This may be defined either at installation time, or by management activity to provisioning the group name via the Itf-N. allowedValues: N/A (String size is bounded to 80 characters.) | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| retTiltValue | The electrical tilt setting of the antenna, "Tilt value" in Ref. 3GPP TS 37.466 [26]. allowedValues: See "Tilt value" in Ref. TS 37.466 [26]. | type: Integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| beamTilt | The beam tilt of the wanted antenna beam in the vertical plane. A positive value on the beam tilt indicates an antenna beam direction below the vertical plane." | type: Integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| sharedTechnologies | This attribute defines the radio access technologies sharing the common functionalities of a Base Station (BS). allowedValues: GSM, UMTS, LTE, or any combination thereof. | type: Integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| tmaAntennaBearing | A data field defined in Table B.3 of 3GPP TS 37.466 [26]. See TS 37.466 [26]. allowedValues: N/A | type: Integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| tmaBaseStationId | A data field defined in Table B.3 of TS 37.466 [26] allowedValues: N/A | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| tmaFunctionFlag | Defined in TS 37.466 [26] allowedValues: N/A | type: Integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| tmaGainFigure | Defined in TS 37.466 [26] allowedValues: N/A | type: Integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| tmaGainResolution | A data field defined in Table B.3 of TS 37.466 [26] allowedValues: N/A | type: Integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| tmaInstalledMechanicalTilt | A data field defined in Table B.3 of TS 37.466 [26] allowedValues: N/A | type: Integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| tmaMaxGain | Defined in TS 37.466 [26] allowedValues: N/A | type: Integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| tmaMinGain | Defined in TS 37.466 [26] allowedValues: N/A | type: Integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| tmaNumberOfSubunits | Defined in TS 37.466 [26] allowedValues: -- | Defined in 3GPP TS 37.466 [26] type: -- multiplicity: -- isOrdered: -- isUnique: -- defaultValue: -- isNullable: -- |
| tmaResolution | Defined in TS 37.466 [26] allowedValues: N/A | type: Integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| tmaSectorId | A data field defined in Table B.3 of TS 37.466 [26] allowedValues: N/A | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| tmaStateFlag | Defined in TS 37.466 [26] allowedValues: N/A | type: Integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| tmaSubunitNumber | Defined in TS 37.466 [26] allowedValues: N/A | type: Integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| tmaSubunitRxFrequencyBand | A data field defined in Table B.3 of TS 37.466 [26] allowedValues: See TS 37.466 [26]. | type: Integer multiplicity: 2 isOrdered: True isUnique: True defaultValue: None isNullable: False |
| tmaSubunitType | A data field defined in Table B.3 of TS 37.466 [26] allowedValues: N/A | type: Integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| tmaSubunitTxFrequencyBand | A data field defined in Table B.3 of TS 37.466 [26] allowedValues: See TS 37.466 [26]. | type: Integer multiplicity: 2 isOrdered: True isUnique: True defaultValue: None isNullable: False |
| tsc | This attribute has the same definition as the one used in GsmCell IOC. The presence of GSMCellPart means the tsc attribute in GsmCell IOC instance is irrelevant (not applicable). allowedValues: N/A | type: Integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ulAttenuation | Uplink signal attenuation of the device to change uplink gain. allowedValues: N/A | type: Integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| vertBeamwidth | The 3 dB power beamwidth of the antenna pattern in the vertical plane. The value of this attribute has no operational impact on the network, e.g. the NE behaviour is not affected by the value setting of this attribute. This attribute is not supported over the Iuant interface. allowedValues: A single integral value corresponding to an angle in degrees between 0 and 180. | type: Integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| Attribute related to role |  |  |
| externalUTRANCell | This role (when present) represents RepeaterFunction capability to identify one ExternalUtranCell. When present, it shall contain one ExternalUtranCell DN. allowedValues: N/A | type: DN multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| theProxyBsList | A CommonBsFunction instance serves a number of ProxyBsFunction instances. This CommonBsFunction role-attribute contains a list of DNs of ENBFunction (TS 28.658 [13]), NodeBFunction (TS 28.652 [12]) and BssFunction (TS 28.655 [14]) that it serves. allowedValues: N/A | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| referencedBy | This attribute contains the DNs of one or more objects (e.g. antenna, cells, sector carrier) if associations between them exist. allowedValues: N/A | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |

10th change

### 4.5.1	Alarm notifications

This subclause presents a list of notifications, defined in 3GPP TS 32.111-2 [11], that IRPManager can receive. The notification header attribute objectClass/objectInstance, defined in TS 32.302 [16], would capture the DN of an instance of an IOC defined in this IRP specification.

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyAckStateChanged | See Alarm IRP (TS 32.111-2 [11]) |  |
| notifyChangedAlarm | See Alarm IRP (TS 32.111-2 [11]) |  |
| notifyClearedAlarm | See Alarm IRP (TS 32.111-2 [11]) |  |
| notifyNewAlarm | See Alarm IRP (TS 32.111-2 [11]) |  |
| notifyComments | See Alarm IRP (TS 32.111-2 [11]) |  |
| notifyAlarmListRebuilt | See Alarm IRP (TS 32.111-2 [11]) |  |
| notifyPotentialFaultyAlarmList | See Alarm IRP (TS 32.111-2 [11]) |  |

11th change

### 4.5.2	Configuration notifications

This subclause presents a list of notifications, defined in TS 32.662 [17], that IRPManager can receive. The notification header attribute objectClass/objectInstance, defined in 3GPP TS 32.302[16], would capture the DN of an instance of an IOC defined in this IRP specification.

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyAttributeValueChange | O |  |
| notifyObjectCreation | O |  |
| notifyObjectDeletion | O |  |

End of changes