3GPP TSG-SA5 Meeting #163	S5-254300

Wuhan, China, 13. - 17. October 2025

Source:	China Mobile, Huawei

Title:	Pseudo-CR on 28.881 Add solution for enhancement of radio service delivering and assurance scenarios

Document for:	Approval

Agenda item:	6.20.1

Spec:	3GPP TR 28.881

Version:	V0.1.0

Work Item:	FS_IDMS_MN_Ph4

Comments

This contribution proposes to enhance the use case #1: Enhancement of radio service delivering and assurance scenarios to support the following scenarios:

- MnS consumer expresses the radio service delivering and assurance expectation for a specified assurance time duration. Using the concert as an example, MnS consumer expresses the intent indicating a radio service to be delivered and assured for two hours, or before concert, MnS consumer request to explore the supported maximum number of UE for the Radio Service which to be validated for two hours after it being delivered.

- MnS consumer expresses the radio service delivering and assurance requirements for a specified number of UEs with percentage value of the amount of simultaneous active UEs.

Proposed Changes

* * * First Change * * * *

4.1 Use case #1: Enhancement of radio service delivering and assurance scenarios

4.1.1 Description

In 3GPP TS 28.312 [1], the existing use case and requirements for intent containing an expectation for delivering a radio service is described in clause 5.1.2. The RadioServiceExpectation is defined to represent MnS consumer's expectations for radio service delivering and assurance in the specified area. However, following scenarios are not supported:

-	MnS consumer expresses the radio service delivering and assurance expectation with service reliability information.

-	MnS consumer expresses the radio service delivering and assurance intent expectation for a specified area described in the form of civic address. For example, MnS consumer wants to ensure the radio service targets (dLThptPerUETarget and dLLatencyTarget) for a specific civic address (e.g., the CivicAddress defined in clause 6.1.6.2.14 in TS 29.572 [3]).

-	MnS consumer expresses the radio service delivering and assurance expectation for a specified assurance time duration. Using the concert as an example, MnS consumer expresses the intent indicating a radio service to be delivered and assured for two hours, or before concert, MnS consumer request to explore the supported maximum number of UE for the Radio Service which to be validated for two hours after it being delivered.

-	MnS consumer expresses the radio service delivering and assurance requirements for a specified number of UEs with percentage value of the amount of simultaneous active UEs.

4.1.2 Potential requirements

REQ-IDMS_RadioServiceIntent -CON-1: The intent driven MnS producer for radio service should have capabilities enabling the MnS consumer to express service reliability requirements.

REQ-IDMS_RadioServiceIntent -CON-2: The intent driven MnS producer for radio service should have capabilities enabling the MnS consumer to express radio service delivering and assurance for a specified area described in the form of civic address.

REQ-IDMS_RadioServiceIntent -CON-3: The intent driven MnS producer for radio service should have capabilities enabling the MnS consumer to express radio service assurance time duration requirements.

4.1.3 Potential solutions

This solution proposes to reuse and enhance the existing RadioServiceExpectation defined in 3GPP TS 28.312 [2].

Enhancement Aspect1: Add following attributes as the ExpectationTargets for the RadioServiceExpectation:

- UeActivityFactorTarget, it represents the UE activity Factor target (the expected percentage value of the amount of simultaneous active UEs to the total number of UEs where active means the UEs are exchanging data with the network.) for the radio service that the intent expectation is applied. The detailed definition forUeActivityFactor see ActivityFactor defined in TS 28.541 [2].

Enhancement Aspect2: Add following attributes as the Expectation Contexts for the RadioServiceExpectation to support assurance duration information:

-	AssuranceDurationContext, it describes the time duration at which the radio service instance is scheduled to be available and guaranteed. The type is integer and unit is hour.

4.1.4 Evaluation of potential solutions

TBD

* * * End of Changes * * * *