3GPP TSG-SA5 Meeting #163	S5-254303

Wuhan, China, 13. - 17. October 2025

Source:	China Mobile

Title:	Pseudo-CR on TR 28.883 Add use case and requirements on enhancement for NDT management and orchestration

Document for:	Approval

Agenda item:	6.20.3

Spec:	3GPP TR 28.883

Version:	V0.0.0

Work Item:	FS_NDT_Ph2

Comments

This contribution proposes to enhance the use case for Dynamic orchestration and management of NDT instances.

Proposed Changes

* * * First Change * * * *

5.X	Use Case #<X>: Dynamic orchestration and management of NDT instances

5.X.1	Description

In 3GPP TS 28.561, the existing use case and requirements for Life cycle management of NDT instances and Control of NDT instances are defined in clause 5.1.2.1 and 5.1.2.2, which describes that the lifecycle management of NDT refers to the management processes that control and transition an NDT instance through the processes of creation, activation, operation, deactivation, deletion and any updates during these processes. However, following scenarios are not fully supported:

When an NDT producer receives an NDT service or NDT job request from a MnS consumer, the NDT producer needs to check first whether existing NDT instances can already fulfil this request. If a suitable instance is found, the NDT producer reuses the existing resources or invokes the corresponding NDT instance. If no appropriate existing instance is available, a new NDT instance is created to handle the request and guarantee service quality. An NDT job may have dependencies on multiple NDTs or DTs, which can be created or reused as part or all of a NDT instance to support new NDT jobs or NDT services.

This approach prevents the overhead of creating duplicate instances, maximizes resource reuse, and ensures the efficient delivery of NDT capabilities. Through this dynamic and flexible NDT instance management and orchestration, the NDT MnS enables capability sharing and instance reuse among multiple MnS consumers. This not only optimizes overall resource utilization but also ensures that each request is served by the most suitable NDT capabilities.

5.X.2	Potential requirements

REQ-NDT-instance -1: The 3GPP management system should support NDT MnS producer to check and reuse the existing NDT instance.

REQ-NDT-instance -2: The 3GPP management system should support a capability that allows the MnS consumer to express the relationships among multiple NDTs and DTs to support the querying, reuse, and creation of NDT instances by the NDT MnS Producer.

5.X.3	Potential solutions

5.X.4	Evaluation of potential solutions

* * * End of Changes * * * *