3GPP TSG-SA5 Meeting #163	S5-254635

Wuhan, China, 13 - 17 October 2025

Source:	Nokia, China Mobile, Ericsson, Samsung

Title:	Pseudo-CR on Rel-19 pCR TR 28.869 Add Evaluation to the use of VNF generic OAM functions

Document for:	Approval

Agenda item:	          6.19.6

Spec:	3GPP TR 28.869

Version:	1.6.0

Work Item:	FS_Cloud_OAM

Comments

The pCR proposes to add evaluation of solutions for the use cases in clause 5.1 of TR 28.869

Proposed Changes

* * * First Change * * * *

#### 5.1.1.4	Evaluation of solutions

The solutions presented in clauses 5.1.1.3.1, 5.1.1.3.2 and 5.1.1.3.3 describe how the VNF generic OAM functions can be used to convey/apply non-application configuration of cloud-native VNFs.  The VNF generic OAM functions and corresponding interfaces are specified in ETSI GS NFV-IFA 049 [2]. Their specification is out of scope of 3GPP.

The solutions presented in clause 5.1.1.3.4 and 5.1.1.3.6 relies on the use of existing SA5 specifications to support configuration of cloud-native VNFs. These solutions do not require any further enhancements to support the use case requirement REQ-CVNF_CM-1.

* * * Next Change * * * *

#### 5.1.2.4	Evaluation of solutions

The solution presented in clause 5.1.2.3.1 describes how the VNF generic OAM functions can be used to convey/apply non-application policies related to the cloud-native VNF. The VNF generic OAM functions and corresponding interfaces are  specified in ETSI GS NFV-IFA 049 [2]. Their specification is out of scope of 3GPP.

The solution presented in clause 5.1.2.3.2 relies on the use  of existing 3GPP SA5 specifications to support policy management for the cloud-native VNFs. The solution does not require any further enhancements in the 3GPP management system to satisfy the use case requirement REQ-policy-1.

* * * Next Change * * * *

#### 5.1.3.4	Evaluation of solutions

The role of the 3GPP management system in supporting traffic management (as per the use case requirement REQ-CVNF_TM-1) is ambiguous. The use case has not defined what “Cloud-native VNF traffic” means and the relation if any with 3GPP signalling traffic.

Subsequently, if cloud-native VNF traffic is not the 3GPP signalling traffic, then the traffic management use case is out of scope for 3GPP SA5 in this release.

* * * Next Change * * * *

#### 5.1.4.4	Evaluation of solutions

The solution presented in clauses 5.1.4.3.1 describes how the VNF generic OAM functions can be used to support the upgrade of the non-application parameters of the cloud-native VNFs.  The VNF generic OAM functions and corresponding interfaces is  specified in ETSI GS NFV-IFA 049 [2]. Their specification is out of scope of 3GPP.

The solution presented in clause 5.1.4.3.2 relies on the use of existing SA5 specifications to support the upgrade of cloud-native VNFs. The solution does not require any further enhancements in the 3GPP management system to satisfy the use case requirement REQ-CVNF_UP-1.

* * * End of Changes * * * *