3GPP TSG-SA5 Meeting #163	S5-254688

Wuhan, China, 13. - 17. October 2025

Source:	Huawei, China Mobile, Verizon

Title:	Pseudo-CR on TR 28.886 Add requirements and solution for failure resolution

Document for:	Approval

Agenda item:	6.20.6

Spec:	3GPP TR 28.886

Version:	0.1.0

Work Item:	FS_MDA_Ph4

Comments

It is proposed to add requirements and solution for the use case “MDA assisted failure resolution”.

Proposed Changes

* * * First Change * * * *

#### 5.3.2.2	Potential requirements

REQ-FAILURE_RESOLUTION_MDA-01: MDA capability for network failure resolution should be able to provide the analytics output including the probable impact of the network failure.

REQ-FAILURE_RESOLUTION_MDA-02: MDA capability for network failure resolution should be able to provide the analytics output including recommended actions to resolve the network failure.

#### 5.3.2.3	Potential solutions

Update TS 28.104 [2] to add a new MDA type MDAAssistedFaultManagement.FailureAnalysis to allow an MnS consumer to request analytics related to a network failure.

Update TS 28.104 [2] to add a new analytics output for network failure analysis, including (but not limited to) information related to

- related alarm(s) which are likely to be correlated to the network failure

- managed objects (e.g. NFs, cells) that are impacted by the network failure

- metrics or KPIs that are impacted by the network failure

- probable impacted domain and cause of the network failure

- recommended actions to resolve the network failure

- estimated time to resolve the network failure

* * * End of Changes * * * *