3GPP TSG-SA5 Meeting #163	S5-254699

Wuhan, China, 13th Oct 2025 - 17th Oct 2025	(revision of S5-254522)

Source:	Nokia

Title:	Pseudo-CR on Rel-20 TR 28.885 Add new use case related to service adjustments to adapt to energy-related characteristics

Document for:	Approval

Agenda item:	6.20.5

Spec:	3GPP TR 28.885

Version:	0.0.0

Work Item:	FS_Energy_Ph4_OAM

Comments

To introduce a new use case to study for "Enhancements to support energy efficiency as a service criteria", related to WT-1.

This pCR introduces use case and potential requirements related to WT1.2 of the SI FS_Energy_Ph4_OAM.

"

WT-1: Study 3GPP management system enhancements to support energy efficiency as a service criteria.

NOTE 1: This WT is to study the impacts on 3GPP management system to support the requirements specified by SA1 (in clause 6.15a TS 22.261) as part of 5GA Rel-20 WI EnergyServ_Ph2-REQ.

…

WT-1.2: Study management mechanisms on how energy consumption can be controlled (e.g. energy rationing) and how 5G network can adapt to energy-related characteristics. This includes:

Study management mechanisms on how 5G network can enable the operator to provide means to adapt service performance to meet energy rationing control.

NOTE 3: WT-1.2 may be updated based on inputs from other WGs.

…

"

Proposed Changes

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[A]	3GPP TS 22.261: "Service requirements for the 5G system".

[B]	3GPP TS 28.310: "Management and orchestration; Energy efficiency of 5G".

[H]	ETSI GS OEU 020 (v1.1.1): "Operational energy Efficiency for Users (OEU); Carbon equivalent Intensity measurement; Operational infrastructures; Global KPIs; Global KPIs for ICT Sites".

[I]	ETSI ES 202 706-1 V1.7.1 (2022-08): "Environmental Engineering (EE); Metrics and measurement method for energy efficiency of wireless access network equipment; Part 1: Power consumption - static measurement method".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

carbon emission: quantity of equivalent carbon dioxide emitted (e.g. kg of CO2 equivalent).

NOTE A:	This definition is taken from 3GPP TS 22.261 [A].

carbon intensity: quantity of CO2 equivalent emission per unit of final energy consumption for an operational period of use. See ETSI GS OEU 020 (v1.1.1) [H].

NOTE B:	This definition is taken from 3GPP TS 22.261 [A].

energy availability: the remaining amount of energy (e.g. in kWh) locally available for consumption. For devices, network elements and functions, energy availability may be limited and/or intermittent, in particular when relying on batteries and/or renewable energy sources (e.g. off-grid base stations, satellites etc) or during power grid heavy load or disruptions.

NOTE C:	This definition is taken from 3GPP TS 22.261 [A].

energy capacity: the maximum amount of energy (e.g. in kWh) that can be locally available for consumption (either locally produced and/or stored) by a device or a network element or function.

NOTE D:	This definition is taken from 3GPP TS 22.261 [A].

Energy Consumption (EC): integral of power consumption over time.

NOTE X: This definition is taken from TS 28.310 [B]

energy rationing: A situation in which the availability of energy either across the network or at a particular network element or function is limited or reduced.

NOTE Y:	This definition is taken from 3GPP TS 22.261 [A].

energy-related characteristics: information which characterize the energy to power the operator’s network in terms of energy consumption, energy supply mix, carbon footprint, energy capacity and availability conditions.

NOTE F:	Which energy-related characteristics are relevant depends on the scenario.

renewable energy: energy from renewable sources as energy from renewable non-fossil sources, namely wind, solar, aerothermal, geothermal, hydrothermal and ocean energy, hydropower, biomass, landfill gas, sewage treatment plant gas and biogases

NOTE H:	This definition is taken from TS 22.261 [A].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

<ABBREVIATION>	<Expansion>

* * * Next Change * * * *

### 5.1.B	Use case #<B>: Management mechanisms to support service adjustments to adapt to energy-related characteristics and energy rationing

#### 5.1.B.1	Description

Under energy rationing constraints (i.e., when energy availability is limited or restricted), there is a potential risk for the entire network to shut-down causing service disruption. During such unavoidable hard constraints, the operators would like the network to provide services to their consumers to extend the time duration for which the services are available, and this could be by degrading the network service performance. For example, the operators might want to prioritize the voice and emergency services (e.g. eMBB) over some data services (e.g., URLLC or Gaming service or Video service). The 3GPP management system can utilize the information related to energy rationing and other energy-related characteristics (such as energy capacity and energy availability), and provide mechanisms to the operator to prevent their network from going out of service and extend the duration for which the services are available.

Energy rationing and energy-related characteristics are defined in TS 22.261 [A].

The requirement specified in clause 6.15a.2.2 of TS 22.261 [A] includes providing means to enable the operator to degrade service performance to meet energy rationing constraints.

Another requirement specified in clause 6.15a.2.2 of TS 22.261 [A] include providing mechanisms to adjust communication service considering change of energy supply mix of the network as one of the factors. For example, adjustments based on the carbon emissions of different network functions.

This use case is to study

-	management mechanisms to enable the operator to provide means to adapt the services provided by the network (e.g., by modifying the QoS requirements) to meet energy rationing control.

-	management mechanisms to enable the network to adjust communication service (e.g., activate or deactivate communication services) adapting to energy-related characteristics.

#### 5.1.B.2	Potential requirements

PREQ-Energy_Related_Service_Adjustment-1: The 3GPP management system should enable the operator to adapt the network performance of existing communication services to meet energy rationing control utilizing the available energy rationing information.

PREQ-Energy_Related_Service_Adjustment-2: The 3GPP management system should enable the network to adjust communication services adapting to energy-related characteristics.

#### 5.1.B.3	Potential solutions

#### 5.1.B.3.i	Potential solution #<i>: <Potential Solution i Title>

##### 5.1.B.3.i.1	Introduction

Editor's Note:	This clause describes briefly the potential solution at a high-level.

##### 5.1.B.3.i.2	Description

Editor's Note:	This clause further details the potential solution, and any assumptions made.

#### 5.1.B.4	Evaluation of potential solutions

Editor's Note:	This clause provides the evaluation of potential solutions.

* * * End of Changes * * * *