3GPP TSG-SA5 Meeting #163	S5-254745

Wuhan, China, 13. - 17. October 2025

Source:	China Mobile, Nokia

Title:	Pseudo-CR on TR 28.882 Add concepts and new use cases Management of AI/ML monitoring

Document for:	Approval

Agenda item:	6.20.2

Spec:	3GPP TR 28.882

Version:	V0.0.1

Work Item:	FS_AIML_MGT_Ph3

Comments

This contribution proposes to address issues with respect to AIML lifecycle management features for TR 28.882.

Proposed Changes

* * * First Change * * * *

5	Management capabilities for AI/ML lifecycle

5.A	Management of ML E2E LCM capability

5.A.1	Description

Single-dimension monitoring may not reflect the actual root cause of performance degradation for an ML model or a group of ML models. For example, an MnS Consumer may monitor network performance to decide whether and when to activate or deactivate an AI/ML inference related to AI/ML-based NG-RAN issues. However, when network performance degradation occurs, the consumer may simply deactivate the ML model and not be able to identify or resolve the underlying problem. There are multiple-dimensional potential causes include rapidly changing network conditions, an ongoing update of the ML model version, or an incorrect configuration of the ML testing function (e.g., using inappropriate performance indicators).

Therefore, multi-dimensional enables the MnS Consumer to obtain a global view of the LCM status and to identify potential or ongoing issues in a comprehensive manner.

5.A.1.1	Management of AI/ML monitoring

During the ML model lifecycle, the MnS Consumer may wish to monitor targets including, but not limited to:

- Network conditions,

- Network performance (e.g., PM/KPI, service performance, etc.),

- ML model performance (e.g., ML model validation/testing performance, AI/ML inference performance per TS 28.105), and ML model I/O,

- ML LCM components (e.g., ML repository, ML Training Function, ML Testing Function, AI/ML inference emulation function, ML update function, AI/ML inference function, etc.).

The MnS Consumer may aim to optimize the ML model by considering overall multi-dimensional monitored data/information and to receive suggested actions when the monitoring reflects a global or joint event. In such case, the MnS consumer may wish to request AI/ML monitoring that contains multiple monitoring target, based on which the producer would perform AI/ML monitoring. The consumer can also receive the analysis result which may contain the suggested actions by the producer according to the monitored data/information from a specific monitoring instance.

In addition, the AIML monitoring consumer may pay more attention to their concerned monitoring targets. Therefore, the producer should enable the consumer to have the capability to indicate priority of the monitoring target.

5.A.1.3	Potential requirements

REQ-AIML_MON-1: The AI/ML monitoring MnS producer should have a capability to enable an authorized MnS consumer to request AI/ML monitoring on multiple monitoring targets.

REQ-AIML_MON-2: The AI/ML monitoring MnS producer may have a capability to analysis the monitored information.

REQ-AIML_MON-3: The AI/ML monitoring MnS producer may have a capability to identify a global or joint event according to AI/ML monitoring analysis result.

REQ-AIML_MON-4: The AI/ML monitoring MnS producer may have a capability to report to an authorized consumer with the analysis result.

REQ-AIML_MON-5: The AI/ML monitoring may have a capability to enable an authorized MnS consumer to configure the priority of the monitoring targets.

5.A.1.4	Possible solutions

TBD

* * * End of Changes * * * *