3GPP TSG-SA5 Meeting #163	S5-254775

Wuhan, China, 13. - 17. October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.541 | CR | 1625 | rev | 1 | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.541 Correct the issues for MWAB NRM fragment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile, Huawei,Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AdNRM_Ph3 |  |  |  |  |  | Date: |  |  | 2025-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Following issues observed for MWAB NRM fragment: 1. MWAB is an optional feature supported by NR, however, in the Figure 4.2.1.1-24: NRM fragment for MWAB, NRCellDU must associated to one MWAB. This is wrong for the NR which doesn’t support MWAB feature. 2. In clause 4.3.2.2, the isWritable property for attribute “mWABRef” is T, however, in TS28541_NrNrm.yaml, it defined as DNRo which present readOnly DN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the Figure 4.2.1.1-24: NRM fragment for MWAB. - Update the attribute definition for “mWABRef” in TS28541_NrNrm.yaml. - Rephrase the wording in 4.3.91.1 - Add platuml code for Figure 4.2.1.1-24: NRM fragment for MWAB |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incosistence exist in the MWAB NRM fragment. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.1.1, 4.3.91.1, Q.5 Following YAML files normatively defined in the forge are updated: - OpenAPI/TS28541_NrNrm.yaml |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1919 at commit 8bd74f775bb2d3f89a23b4e865479d0e6b494bd6 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

4.2.1	Class diagram for gNB and en-gNB

4.2.1.1	Relationships

This clause depicts the set of classes (e.g. IOCs) that encapsulates the information relevant for this gNB and en-gNB. For the UML semantics, see 3GPP TS 32.156 [43]. Subsequent clauses provide more detailed specification of various aspects of these classes.

The model fragments are for management representation of gNB and en-gNB for all NG-RAN deployment scenario as listed below.

-	Non-split NG-RAN deployment scenario, represents the gNB defined in TS 38.401[4]. In this scenario, a gNB is represented by a combination of a GNBCUCPFunction, one or more GNBCUUPFunctions and one or more GNBDUFunctions.

-	2-split NG-RAN deployment scenario, represents the gNB consist of gNB-CU and gNB-DU defined in TS 38.401[4] clause 6.1.1. In this scenario, a gNB-CU is represented by a combination of a GNBCUCPFunction and one or more GNBCUUPFunctions, whereas a gNB-DU is represented by a GNBDUFunction.

-	3-split NG-RAN deployment scenario, represents the gNB consist of gNB-CU-CP, gNB-CU-UP and gNB-DU defined in TS 38.401[4] clause 6.1.2. In this scenario, a gNB-CU-CP is represented by a GNBCUCPFunction, a gNB-CU-UP is represented by a GNBCUUPFunction, and a gNB-DU is represented by a GNBDUFunction.

Figure 4.2.1.1-1: NRM for all deployment scenarios

Figure 4.2.1.1-2: NRM for EPs for all deployment scenarios

Figure 4.2.1.1-3: NRM for <<IOC>>NRSectorCarrier,<<IOC>>BWP, <<IOC>>SectorEquipmentFunction and <<IOC>> BWPSet for all deployment scenarios

Figure 4.2.1.1-4: Cell Relation view for all deployment scenarios

NOTE 1:	The above NRM fragment uses SubNetwork to hold both NR and LTE external entities and frequencies.

Figure 4.2.1.1-5: Cell Relation view for all deployment scenarios

NOTE 2:	The above NRM fragment uses ExternalNRNetwork to hold NR external entities and frequency and using ExternalEUtraNetwork to hold LTE external entities and frequency.

Figure 4.2.1.1-6: NRM fragment for abstract RRM Policies

Figure 4.2.1.1-6a: NRM fragment for RRMPolicyRatio

Figure 4.2.1.1-7: NRM fragment to support RIM

Figure 4.2.1.1-8: NRM fragment for pre-configured 5QIs in NG-RAN

NOTE 3:	Void.

Figure 4.2.1.1-9: NRM fragment for DANR Management

Figure 4.2.1.1-10: NRM fragment for DES Management

Figure 4.2.1.1-11: NRM fragment for DRACH Management

Figure 4.2.1.1-12: NRM fragment for DMRO Management

Figure 4.2.1.1-13: NRM fragment for DPCI Management

Figure 4.2.1.1-14: NRM fragment for CES Management

Figure 4.2.1.1-15: NRM fragment for CPCI Management

Figure 4.2.1.1-16: NRM fragment for dynamically assigned 5QIs in NG-RAN

Figure 4.2.1.1-17: NRM fragment for NG-RAN MOCN network sharing with multiple cell identity broadcast feature

Figure 4.2.1.1-18: NRM fragment for F1 related EPs to support individual F1 interface for NG-RAN MOCN network sharing with multiple cell identity broadcast feature

Figure 4.2.1.1-19: NRM fragment for DLBO Management

Figure 4.2.1.1-20: NRM fragment for CCO Management

Figure 4.2.1.1-20A: NRM fragment for distribute CCO Management

Figure 4.2.1.1-21: NRM fragment to support NTN management

Figure 4.2.1.1-22: NRM fragment for ExternalGNBCUCPFunction, ExternalGNBCUUPFunction and ExternalGNBDUFunction

Figure 4.2.1.1-23: NRM fragment for energy cost mapping rule

Figure 4.2.1.1-24: NRM fragment for MWAB

Note:	The above figure is subject to change as appropriate.

Figure 4.2.1.1-25: NRM relationship fragment for AIOT

Figure 4.2.1.1-26: NRM fragment for RedCap Access

Figure 4.2.1.1-27: NRM fragment for IAB

Next Change

4.3.91	MWAB

4.3.91.1	Definition

MWAB provides an NR access link to UEs in proximity and connects to the 5GC serving the UE through an IP connectivity provided by a Backhaul PDU session(s). The MWAB consists of a gNB component (MWAB-gNB) and a UE component (MWAB-UE), see [2]. This IOC defines the configuration information for the MWAB-gNB.

The attribute currentLocation defines the the current location of the MWAB-gNB. This shall be set by the provisioning MnS producer. Provisioning MnS Consumer needs to subscribe to get change notifications regarding location of the MWAB-gNB (i.e the attribute currentLocation). Provisioning MnS Producer calculates the current geographical location of the MWAB-gNB. Provisioning MnS Producer would notify Provisioning MnS consumer about the current location of the MWAB-gNB (the notifications will be sent each time the location of the MWAB changes). Provisioning MnS consumer will then provide location specific MWAB configuration for the MWAB-gNB.

The functionality of activating/deactivating the MWAB-gNB operation can be controlled by updating the administrativeState as appropriate.

4.3.91.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| operationalState | M | T | F | F | T |
| administrativeState | M | T | T | F | T |
| allowedArea | O | T | T | F | T |
| allowedTime | O | T | T | F | T |
| currentLocation | M | T | F | F | T |

NOTE:	The attribute definition of the MWAB IOC is subject to change as appropriate.

4.3.91.3	Attribute constraints

None

4.3.91.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

Next Change

Q.5	NR-NRM diagrams

Note: To avoid diagrams that are in a different order than in the main body of the document (clause 4.2) and to avoid later reordering the diagrams, here all diagrams for clause "4.2		Class diagram" for the NR-NRM are kept in a single clause (Q.x).

'Figure 4.2.1.1-2: Figure 4.2.1.1-2: NRM for EPs for all deployment scenarios

@startuml Figure 4.2.1.1-2: Figure 4.2.1.1-2: NRM for EPs for all deployment scenarios

hide empty members

hide circle

skinparam class {

BackgroundColor White

ArrowColor Black

BorderColor Black

}

skinparam ClassStereotypeFontStyle normal

skinparam linetype ortho

skinparam nodesep 2

skinparam ranksep 80

'left to right direction

class GNBDUFunction <<InformationObjectClass>>

class EP_F1C <<InformationObjectClass>>

class EP_F1U <<InformationObjectClass>>

class ManagedEntity <<ProxyClass>>

note left of ManagedEntity

Represents

GNBCUCPFunction or ExternalGNBCUCPFunction

end note

class "ManagedEntity " <<ProxyClass>>

note right of "ManagedEntity "

Represents

GNBCUUPFunction or ExternalGNBCUUPFunction

end note

GNBDUFunction "1" *-- "0..1" EP_F1C: <<names>>

GNBDUFunction "1" *-- "*" EP_F1U: <<names>>

EP_F1C  "*" --> "1" ManagedEntity

EP_F1U  "*" --> "1" "ManagedEntity "

@enduml

@startuml

'Figure 4.2.1.1-3: UML diagram from 3GPP TS 28.541 clause 4 NRSectorCarrier, BWP and BWPSet for all deployment scenarios

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide members

hide circle

class GNBDUFunction <<InformationObjectClass>>

class BWPSet  <<InformationObjectClass>>

class BWP  <<InformationObjectClass>>

class NRCellDU  <<InformationObjectClass>>

class NRSectorCarrier  <<InformationObjectClass>>

class SectorEquipmentFunction  <<InformationObjectClass>>

class CommonBeamformingFunction  <<InformationObjectClass>>

class Beam  <<InformationObjectClass>>

GNBDUFunction "1 " *-d- "*" BWPSet: <<names>>

BWPSet "0..12" --> "*" BWP

BWPSet "*\n+bWPSetRef" <-- "*" NRCellDU

GNBDUFunction "1 " *-d- "*" BWP: <<names>>

GNBDUFunction "1 " *-d- "*" NRCellDU: <<names>>

GNBDUFunction "1 " *-d- "*" NRSectorCarrier: <<names>>

NRCellDU "*" --> "+bWPRef\n*" BWP

NRCellDU "0..1" --> "*" NRSectorCarrier

SectorEquipmentFunction "0..1" <-- "*" NRSectorCarrier

NRSectorCarrier "1" *--> "0..1" CommonBeamformingFunction: <<names>>

CommonBeamformingFunction "1" *-- "*" Beam: <<names>>

@enduml

@startuml

'Figure 4.2.1.1-19: UML diagram from 3GPP TS 28.541 clause 4 DLBO Management

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide members

hide circle

class ManagedEntity <<ProxyClass>>

class DLBOFunction  <<InformationObjectClass>>

ManagedEntity "1 " *-d- "0..1" DLBOFunction: <<names>>

note left of ManagedEntity

This represents SubNetwork, ManagedElement

or NRCellCU

end note

@enduml

'Figure 4.2.1.1-18: NRM fragment for F1 related EPs to support individual F1 interface for NG-RAN MOCN network sharing with multiple cell identity broadcast feature

@startuml Figure 4.2.1.1-18: NRM fragment for F1 related EPs to support individual F1 interface for NG-RAN MOCN network sharing with multiple cell identity broadcast feature

hide empty members

hide circle

skinparam class {

BackgroundColor White

ArrowColor Black

BorderColor Black

}

skinparam ClassStereotypeFontStyle normal

skinparam linetype ortho

skinparam nodesep 2

skinparam ranksep 80

'left to right direction

class OperatorDU <<InformationObjectClass>>

class EP_F1C <<InformationObjectClass>>

class EP_F1U <<InformationObjectClass>>

class ManagedEntity <<ProxyClass>>

note left of ManagedEntity

Represents

GNBCUCPFunction or ExternalGNBCUCPFunction

end note

class "ManagedEntity " <<ProxyClass>>

note right of "ManagedEntity "

Represents

GNBCUUPFunction or ExternalGNBCUUPFunction

end note

OperatorDU "1" *-- "0..1" EP_F1C: <<names>>

OperatorDU "1" *-- "*" EP_F1U: <<names>>

EP_F1C  "*" --> "1" ManagedEntity

EP_F1U  "*" --> "1" "ManagedEntity "

@enduml

@startuml Figure 4.2.1-21: Time based configuration NRM fragment

skinparam monochrome true

skinparam ClassStereotypeFontStyle normal

class ManagedEntity <<ProxyClass>>

class NTNTimeBasedConfig <<InformationObjectClass>>

class NTNFunction <<InformationObjectClass>>

class EphemerisInfoSet <<InformationObjectClass>>

class GNBCUCPFunction <<InformationObjectClass>>

hide empty members

hide circle

ManagedEntity "1" *-d- "0..1" NTNFunction : <<names>>

NTNFunction "1" *-d- "*" NTNTimeBasedConfig : <<names>>

NTNFunction "1" *-d- "*" EphemerisInfoSet  : <<names>>

GNBCUCPFunction "*" -r-> "0..1" EphemerisInfoSet

note left of ManagedEntity

Represents the following IOCs:

SubNetwork or

ManagedElement

End note

@enduml

@startuml Figure 4.2.2-11: Time based configuration NRM fragment

hide empty members

skinparam ClassStereotypeFontStyle normal

hide circle

skinparam class {

BackgroundColor White

ArrowColor Black

BorderColor Black

}

skinparam linetype ortho

'skinparam BoxPadding 40

skinparam nodesep 2

class Top <<InformationObjectClass>>

class NTNTimeBasedConfig <<InformationObjectClass>>

class NTNFunction <<InformationObjectClass>>

class EphemerisInfoSet <<InformationObjectClass>>

Top <|-- NTNFunction

Top <|-- EphemerisInfoSet

Top <|-- NTNTimeBasedConfig

@enduml

@startuml

hide empty members

skinparam ClassStereotypeFontStyle normal

hide circle

skinparam class {

BackgroundColor White

ArrowColor Black

BorderColor Black

}

skinparam   Shadowing false

skinparam  Monochrome true

skinparam  ClassBackgroundColor White

skinparam  NoteBackgroundColor White

class GNBDUFunction <<InformationObjectClass>>

class NRSectorCarrier <<InformationObjectClass>>

class AIOTReader <<InformationObjectClass>>

GNBDUFunction "1" *-- "*" AIOTReader: <<names>>

AIOTReader -right-> NRSectorCarrier: 0..1      *

@enduml

@startuml

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide members

hide circle

class Top << InformationObjectClass >>

class AIOTReader <<InformationObjectClass>>

Top <|-- AIOTReader

@enduml

@startuml Figure 4.2.1.1-27: NRM fragment for IAB

skinparam monochrome true

skinparam ClassStereotypeFontStyle normal

class ManagedEntity <<ProxyClass>>

class IAB <<InformationObjectClass>>

class GNBDUFunction <<InformationObjectClass>>

hide empty members

hide circle

ManagedEntity "1" *-d- "*" IAB : <<names>>

GNBDUFunction  "1" -up-> "0..1" IAB

note left of ManagedEntity

Represents the following IOCs:

SubNetwork or

ManagedElement

End note

@enduml

@startuml Figure 4.2.1.2-16: NRM Inheritance fragment for IAB

hide empty members

skinparam ClassStereotypeFontStyle normal

hide circle

skinparam class {

BackgroundColor White

ArrowColor Black

BorderColor Black

}

skinparam linetype ortho

'skinparam BoxPadding 40

skinparam nodesep 2

class Top <<InformationObjectClass>>

class IAB <<InformationObjectClass>>

Top <|-- IAB

@enduml

@startuml Figure 4.2.1.1-24: NRM fragment for MWAB

skinparam monochrome true

skinparam ClassBackgroundColor White

skinparam ClassStereotypeFontStyle normal

class ManagedEntity <<ProxyClass>>

class MWAB <<InformationObjectClass>>

class GNBCUCPFunction <<InformationObjectClass>>

hide empty members

hide circle

ManagedEntity "1" *-d- "*" MWAB : <<names>>

GNBCUCPFunction  "1" -right-> "   0..1" MWAB

note left of ManagedEntity

Represents the following IOCs:

SubNetwork or

ManagedElement

End note

@enduml

Next Change

Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1919 at commit 8bd74f775bb2d3f89a23b4e865479d0e6b494bd6

*** START OF CHANGE 1 ***

*** OpenAPI/TS28541_NrNrm.yaml ***

<CODE BEGINS>

openapi: 3.0.1

info:

title: NR NRM

version: 19.5.0

description: >-

OAS 3.0.1 specification of the NR NRM

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.541; 5G NRM, NR NRM

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.541/

paths: {}

components:

schemas:

#-------- Definition of types-----------------------------------------------------

GnbId:

type: integer

minimum: 0

maximum: 4294967295

GnbIdLength:

type: integer

minimum: 22

maximum: 32

GnbName:

type: string

maxLength: 150

GnbDuId:

type: integer

minimum: 0

maximum: 68719476735

GnbCuUpId:

type: integer

minimum: 0

maximum: 68719476735

readOnly: true

Sst:

type: integer

minimum: 0

maximum: 255

Snssai:

type: object

properties:

sst:

$ref: '#/components/schemas/Sst'

sd:

type: string

pattern: '^[A-Fa-f0-9]{6}$'

SatelliteId:

type: string

pattern: '^[0-9]{5}$'

PlmnIdList:

type: array

uniqueItems: true

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/PlmnId'

PlmnInfo:

type: object

properties:

plmnId:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/PlmnId'

snssai:

$ref: '#/components/schemas/Snssai'

sliceExpiryTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

PlmnInfoList:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/PlmnInfo'

minItems: 1

NPNIdentityList:

type: array

uniqueItems: true

items:

$ref: 'TS28623_GenericNrm.yaml#/components/schemas/NpnId-Type'

minItems: 1

GgNBId:

type: object

properties:

plmnId:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/PlmnId'

gnbIdLength:

$ref: '#/components/schemas/GnbIdLength'

gnbId:

$ref: '#/components/schemas/GnbId'

GeNBId:

type: object

properties:

plmnId:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/PlmnId'

enbId:

type: integer

minimum: 0

maximum: 4194303

GgNBIdList:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/GgNBId'

GeNBIdList:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/GeNBId'

NrPci:

type: integer

maximum: 503

NRTAC:

$ref: 'TS28623_GenericNrm.yaml#/components/schemas/Tac'

NRTACList:

type: array

uniqueItems: true

items:

$ref: 'TS28623_GenericNrm.yaml#/components/schemas/Tac'

TaiList:

type: array

uniqueItems: true

items:

$ref: 'TS28623_GenericNrm.yaml#/components/schemas/Tai'

BackhaulAddress:

type: object

properties:

gnbId:

$ref: '#/components/schemas/GnbId'

tai:

$ref: "TS28623_GenericNrm.yaml#/components/schemas/Tai"

MappingSetIDBackhaulAddress:

type: object

properties:

setId:

type: integer

backhaulAddress:

$ref: '#/components/schemas/BackhaulAddress'

LoadTimeThreshold:

type: object

properties:

loadThreshold:

type: integer

timeDuration:

type: integer

IntraRatEsActivationOriginalCellLoadParameters:

$ref: '#/components/schemas/LoadTimeThreshold'

IntraRatEsActivationCandidateCellsLoadParameters:

$ref: '#/components/schemas/LoadTimeThreshold'

IntraRatEsDeactivationCandidateCellsLoadParameters:

$ref: '#/components/schemas/LoadTimeThreshold'

EsNotAllowedTimePeriod:

type: object

properties:

startTime:

type: string

description: >-

Time of day is in HH:MM or H:MM 24-hour format per UTC time zone.

Examples, 20:15, 20:15-08:00 (for 8 hours behind UTC).

endTime:

type: string

description: >-

Time of day is in HH:MM or H:MM 24-hour format per UTC time zone.

Examples, 20:15, 20:15-08:00 (for 8 hours behind UTC).

daysOfWeek:

type: string

enum:

- MONDAY

- TUESDAY

- WEDNESDAY

- THURSDAY

- FRIDAY

- SATURDAY

- SUNDAY

InterRatEsActivationOriginalCellParameters:

$ref: '#/components/schemas/LoadTimeThreshold'

InterRatEsActivationCandidateCellParameters:

$ref: '#/components/schemas/LoadTimeThreshold'

InterRatEsDeactivationCandidateCellParameters:

$ref: '#/components/schemas/LoadTimeThreshold'

UeAccProbabilityDist:

type: array

items:

$ref: '#/components/schemas/UeAccProbability'

UeAccProbability:

type: object

properties:

targetProbability:

type: integer

minimum: 0

maximum: 100

NumberOfPreamblesSent:

type: integer

minimum: 0

maximum: 200

UeAccDelayProbabilityDist:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/UeAccDelayProbability'

UeAccDelayProbability:

type: object

properties:

targetProbability:

type: integer

minimum: 0

maximum: 100

accessDelay:

type: integer

minimum: 10

maximum: 560

NRPciList:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/NrPci'

minItems: 0

maxItems: 1007

CSonPciList:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/NrPci'

minItems: 1

maxItems: 100

MaximumDeviationHoTrigger:

type: integer

minimum: -20

maximum: 20

MaximumDeviationHoTriggerLow:

type: integer

minimum: -20

maximum: 20

MaximumDeviationHoTriggerHigh:

type: integer

minimum: -20

maximum: 20

MinimumTimeBetweenHoTriggerChange:

type: integer

minimum: 0

maximum: 604800

TstoreUEcntxt:

type: integer

minimum: 0

maximum: 1023

CellState:

type: string

enum:

- IDLE

- INACTIVE

- ACTIVE

readOnly: true

CyclicPrefix:

type: string

enum:

- NORMAL

- EXTENDED

TxDirection:

type: string

enum:

- DL

- UL

- DL_AND_UL

BwpContext:

type: string

enum:

- DL

- UL

- SUL

IsInitialBwp:

type: string

enum:

- INITIAL

- INITIAL_REDCAP

- OTHER

IsESCoveredBy:

type: string

enum:

- NO

- PARTIAL

- FULL

RRMPolicyMember:

type: object

properties:

plmnId:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/PlmnId'

snssai:

$ref: '#/components/schemas/Snssai'

RRMPolicyMemberList:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/RRMPolicyMember'

minItems: 1

AddressWithVlan:

type: object

properties:

iPAddress:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/IpAddr'

vlanId:

type: integer

minimum: 0

maximum: 4096

LocalAddress:

type: object

properties:

addressWithVlan:

$ref: '#/components/schemas/AddressWithVlan'

port:

type: integer

minimum: 0

maximum: 65535

RemoteAddress:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/IpAddr'

QOffsetRange:

type: integer

default: 0

enum:

- -24

- -22

- -20

- -18

- -16

- -14

- -12

- -10

- -8

- -6

- -5

- -4

- -3

- -2

- -1

- 0

- 24

- 22

- 20

- 18

- 16

- 14

- 12

- 10

- 8

- 6

- 5

- 4

- 3

- 2

- 1

QOffsetFreq:

type: number

default: 0

TReselectionNRSf:

type: integer

enum:

- 25

- 50

- 75

- 100

SsbPeriodicity:

type: integer

enum:

- 5

- 10

- 20

- 40

- 80

- 160

SsbDuration:

type: integer

enum:

- 1

- 2

- 3

- 4

- 5

SsbSubCarrierSpacing:

type: integer

enum:

- 15

- 30

- 120

- 240

CoverageShape:

type: integer

maximum: 65535

DigitalTilt:

type: integer

minimum: -900

maximum: 900

DigitalAzimuth:

type: integer

minimum: -1800

maximum: 1800

RSSetId:

type: integer

maximum: 4194303

RSSetType:

type: string

enum:

- RS1

- RS2

FrequencyDomainPara:

type: object

properties:

rimRSSubcarrierSpacing:

type: integer

rIMRSBandwidth:

type: integer

nrofGlobalRIMRSFrequencyCandidates:

type: integer

rimRSCommonCarrierReferencePoint:

type: integer

minimum: 0

maximum: 3279165

rimRSStartingFrequencyOffsetIdList:

type: array

uniqueItems: true

items:

type: integer

minimum: 0

maximum: 550

minItems: 1

maxItems: 4

description: >

It is a list of configured frequency offsets in units of resource blocks.

Only 1,2 or 4 number of elements allowed in the array.

SequenceDomainPara:

type: object

properties:

nrofRIMRSSequenceCandidatesofRS1:

type: integer

rimRSScrambleIdListofRS1:

type: array

uniqueItems: true

items:

type: integer

minimum: 0

maximum: 1023

minItems: 1

maxItems: 8

nrofRIMRSSequenceCandidatesofRS2:

type: integer

rimRSScrambleIdListofRS2:

type: array

uniqueItems: true

items:

type: integer

minimum: 0

maximum: 1023

minItems: 1

maxItems: 8

enableEnoughNotEnoughIndication:

type: string

enum:

- ENABLE

- DISABLE

default: DISABLE

rIMRSScrambleTimerMultiplier:

type: integer

rIMRSScrambleTimerOffset:

type: integer

TimeDomainPara:

type: object

properties:

dlULSwitchingPeriod1:

type: string

enum:

- MS0P5

- MS0P625

- MS1

- MS1P25

- MS2

- MS2P5

- MS3

- MS4

- MS5

- MS10

- MS20

symbolOffsetOfReferencePoint1:

type: integer

dlULSwitchingPeriod2:

type: string

enum:

- MS0P5

- MS0P625

- MS1

- MS1P25

- MS2

- MS2P5

- MS3

- MS4

- MS5

- MS10

- MS20

symbolOffsetOfReferencePoint2:

type: integer

totalnrofSetIdofRS1:

type: integer

totalnrofSetIdofRS2:

type: integer

nrofConsecutiveRIMRS1:

type: integer

nrofConsecutiveRIMRS2:

type: integer

consecutiveRIMRS1List:

type: array

uniqueItems: true

items:

type: integer

consecutiveRIMRS2List:

type: array

uniqueItems: true

items:

type: integer

enablenearfarIndicationRS1:

type: string

enum:

- ENABLE

- DISABLE

default: DISABLE

enablenearfarIndicationRS2:

type: string

enum:

- ENABLE

- DISABLE

default: DISABLE

RimRSReportInfo:

type: object

properties:

detectedSetID:

type: integer

propagationDelay:

type: integer

functionalityOfRIMRS:

type: string

enum:

- RS1

- RS2

- RS1_FOR_ENOUGH_MITIGATION

- RS1_FOR_NOT_ENOUGH_MITIGATION

RimRSReportConf:

type: object

properties:

reportIndicator:

type: string

enum:

- ENABLE

- DISABLE

default: DISABLE

reportInterval:

type: integer

nrofRIMRSReportInfo:

type: integer

maxPropagationDelay:

type: integer

rimRSReportInfoList:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/RimRSReportInfo'

TceIDMappingInfo:

type: object

properties:

tceIPAddress:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/IpAddr'

tceID:

type: integer

pLMNTarget:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/PlmnId'

TceIDMappingInfoList:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/TceIDMappingInfo'

minItems: 1

ResourceType:

type: string

enum:

- PRB

- PRB_UL

- PRB_DL

- RRC_CONNECTED_USERS

- DRB

ParameterRange:

type: object

properties:

maxValue:

type: integer

minValue:

type: integer

NTNTAClist:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/NRTAC'

Ephemeris:

type: object

oneOf:

- required: [ positionVelocity ]

- required: [ orbital ]

required:

- satelliteId

- epochTime

properties:

satelliteId:

$ref: '#/components/schemas/SatelliteId'

epochTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

positionVelocity:

$ref: '#/components/schemas/PositionVelocity'

orbital:

$ref: '#/components/schemas/Orbital'

EphemerisInfos:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/Ephemeris'

minItems: 1

PositionVelocity:

type: object

properties:

positionX:

type: integer

default: 0

minimum: 0

maximum: 604800

positionY:

type: integer

default: 0

minimum: 0

maximum: 604800

positionZ:

type: integer

default: 0

minimum: 0

maximum: 604800

velocityVX:

type: integer

default: 0

minimum: -131072

maximum: 131071

velocityVY:

type: integer

default: 0

minimum: -131072

maximum: 131071

velocityVZ:

type: integer

default: 0

minimum: -131072

maximum: 131071

Orbital:

type: object

properties:

semiMajorAxis:

type: integer

default: 0

minimum: 0

maximum: 8589934591

eccentricity:

type: integer

default: 0

minimum: -524288

maximum: 524287

periapsis:

type: integer

default: 0

minimum: 0

maximum: 16777215

longitude:

type: integer

default: 0

minimum: 0

maximum: 2097151

inclination:

type: integer

default: 0

minimum: -524288

maximum: 524287

meanAnomaly:

type: integer

default: 0

minimum: 0

maximum: 16777215

MappedCellIdInfo:

type: object

properties:

ntnGeoArea:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/GeoArea'

mappedCellId:

$ref: 'TS28541_5GcNrm.yaml#/components/schemas/Ncgi'

MappedCellIdInfoList:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/MappedCellIdInfo'

QceIdMappingInfo:

type: object

properties:

qoECollectionEntityAddress:

oneOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/Ipv4Addr'

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/Ipv6Addr'

qoECollectionEntityIdentity:

type: string

pLMNTarget:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/PlmnId'

QceIdMappingInfoList:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/QceIdMappingInfo'

minItems: 1

MdtUserConsentReqList:

type: array

uniqueItems: true

items:

type: string

enum:

- M1

- M2

- M3

- M4

- M5

- M6

- M7

- M8

- M9

- MDT_UE_LOCATION

NTNEntityConf:

type: object

properties:

nTNConfEntity:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

nTNConfList:

type: array

uniqueItems: true

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeNameValuePairSet'

LocationInfo:

type: object

properties:

gNBId:

type: integer

pLMNId:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/PlmnId'

cellLocalId:

type: integer

nRTAC:

type: string

tAI:

$ref: 'TS28623_GenericNrm.yaml#/components/schemas/Tai'

geoArea:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/GeoArea'

ServedAIOTAreaID:

type: object

properties:

pLMNId:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/PlmnId'

nID:

$ref: 'TS28541_5GcNrm.yaml#/components/schemas/Nid'

aIotAreaCode:

type: string

#-------- Definition of types for name-containments ------

SubNetwork-ncO-NrNrm:

type: object

properties:

NRFrequency:

$ref: '#/components/schemas/NRFrequency-Multiple'

ExternalGNBCUCPFunction:

$ref: '#/components/schemas/GNBCUCPFunction-Multiple'

ExternalGNBCUUPFunction:

$ref: '#/components/schemas/ExternalGNBCUUPFunction-Multiple'

ExternalGNBDUFunction:

$ref: '#/components/schemas/ExternalGNBDUFunction-Multiple'

ExternalENBFunction:

$ref: '#/components/schemas/ExternalENBFunction-Multiple'

EUtranFrequency:

$ref: '#/components/schemas/EUtranFrequency-Multiple'

DESManagementFunction:

$ref: '#/components/schemas/DESManagementFunction-Single'

DRACHOptimizationFunction:

$ref: '#/components/schemas/DRACHOptimizationFunction-Single'

DMROFunction:

$ref: '#/components/schemas/DMROFunction-Single'

DLBOFunction:

$ref: '#/components/schemas/DLBOFunction-Single'

DPCIConfigurationFunction:

$ref: '#/components/schemas/DPCIConfigurationFunction-Single'

CPCIConfigurationFunction:

$ref: '#/components/schemas/CPCIConfigurationFunction-Single'

CESManagementFunction:

$ref: '#/components/schemas/CESManagementFunction-Single'

RedCapAccessCriteria:

$ref: '#/components/schemas/RedCapAccessCriteria-Single'

Configurable5QISet:

$ref: 'TS28541_5GcNrm.yaml#/components/schemas/Configurable5QISet-Multiple'

RimRSGlobal:

$ref: '#/components/schemas/RimRSGlobal-Single'

Dynamic5QISet:

$ref: 'TS28541_5GcNrm.yaml#/components/schemas/Dynamic5QISet-Multiple'

CCOFunction:

$ref: '#/components/schemas/CCOFunction-Single'

NTNFunction:

$ref: '#/components/schemas/NTNFunction-Single'

NRECMappingRule:

$ref: '#/components/schemas/NRECMappingRule-Multiple'

MWAB:

$ref: '#/components/schemas/MWAB-Multiple'

ManagedElement-ncO-NrNrm:

type: object

properties:

GNBDUFunction:

$ref: '#/components/schemas/GNBDUFunction-Multiple'

GNBCUUPFunction:

$ref: '#/components/schemas/GNBCUUPFunction-Multiple'

GNBCUCPFunction:

$ref: '#/components/schemas/GNBCUCPFunction-Multiple'

DESManagementFunction:

$ref: '#/components/schemas/DESManagementFunction-Single'

DRACHOptimizationFunction:

$ref: '#/components/schemas/DRACHOptimizationFunction-Single'

DMROFunction:

$ref: '#/components/schemas/DMROFunction-Single'

DLBOFunction:

$ref: '#/components/schemas/DLBOFunction-Single'

DPCIConfigurationFunction:

$ref: '#/components/schemas/DPCIConfigurationFunction-Single'

CPCIConfigurationFunction:

$ref: '#/components/schemas/CPCIConfigurationFunction-Single'

CESManagementFunction:

$ref: '#/components/schemas/CESManagementFunction-Single'

Configurable5QISet:

$ref: 'TS28541_5GcNrm.yaml#/components/schemas/Configurable5QISet-Multiple'

Dynamic5QISet:

$ref: 'TS28541_5GcNrm.yaml#/components/schemas/Dynamic5QISet-Multiple'

NTNFunction:

$ref: '#/components/schemas/NTNFunction-Single'

NRECMappingRule:

$ref: '#/components/schemas/NRECMappingRule-Multiple'

MWAB:

$ref: '#/components/schemas/MWAB-Multiple'

#-------- Definition of abstract IOCs --------------------------------------------

RRMPolicy_-Attr:

type: object

properties:

resourceType:

$ref: '#/components/schemas/ResourceType'

RRMPolicyMemberList:

$ref: '#/components/schemas/RRMPolicyMemberList'

#-------- Definition of concrete IOCs --------------------------------------------

GNBDUFunction-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

gnbDuId:

$ref: '#/components/schemas/GnbDuId'

gnbDuName:

$ref: '#/components/schemas/GnbName'

gnbId:

$ref: '#/components/schemas/GnbId'

gnbIdLength:

$ref: '#/components/schemas/GnbIdLength'

isOnboardSatellite:

type: boolean

onboardSatelliteId:

$ref: '#/components/schemas/SatelliteId'

rimRSReportConf:

$ref: '#/components/schemas/RimRSReportConf'

configurable5QISetRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

dynamic5QISetRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnRo'

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-ncO'

- type: object

properties:

RRMPolicyRatio:

$ref: '#/components/schemas/RRMPolicyRatio-Multiple'

NRCellDU:

$ref: '#/components/schemas/NRCellDU-Multiple'

BWP-Multiple:

$ref: '#/components/schemas/BWP-Multiple'

NRSectorCarrier-Multiple:

$ref: '#/components/schemas/NRSectorCarrier-Multiple'

EP_F1C:

$ref: '#/components/schemas/EP_F1C-Single'

EP_F1U:

$ref: '#/components/schemas/EP_F1U-Multiple'

DRACHOptimizationFunction:

$ref: '#/components/schemas/DRACHOptimizationFunction-Single'

OperatorDU:

$ref: '#/components/schemas/OperatorDU-Multiple'

BWPSet:

$ref: '#/components/schemas/BWPSet-Multiple'

Configurable5QISet:

$ref: 'TS28541_5GcNrm.yaml#/components/schemas/Configurable5QISet-Multiple'

Dynamic5QISet:

$ref: 'TS28541_5GcNrm.yaml#/components/schemas/Dynamic5QISet-Multiple'

AIOTReader:

$ref: '#/components/schemas/AIOTReader-Multiple'

OperatorDU-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

gnbId:

$ref: '#/components/schemas/GnbId'

gnbIdLength:

$ref: '#/components/schemas/GnbIdLength'

- type: object

properties:

EP_F1C:

$ref: '#/components/schemas/EP_F1C-Single'

EP_F1U:

$ref: '#/components/schemas/EP_F1U-Multiple'

configurable5QISetRef:

description: This attribute is condition optional. The condition is NG-RAN Multi-Operator Core Network (NG-RAN MOCN) network sharing with operator specific 5QI is supported.

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

dynamic5QISetRef:

description: This attribute is condition optional. The condition is NG-RAN Multi-Operator Core Network (NG-RAN MOCN) network sharing with operator specific 5QI is supported.

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnRo'

NROperatorCellDU:

$ref: '#/components/schemas/NROperatorCellDU-Multiple'

GNBCUUPFunction-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

gnbId:

$ref: '#/components/schemas/GnbId'

gnbIdLength:

$ref: '#/components/schemas/GnbIdLength'

gnbCuUpId:

$ref: '#/components/schemas/GnbCuUpId'

isOnboardSatellite:

type: boolean

onboardSatelliteId:

$ref: '#/components/schemas/SatelliteId'

PlmnInfoList:

$ref: '#/components/schemas/PlmnInfoList'

configurable5QISetRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

dynamic5QISetRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnRo'

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-ncO'

- type: object

properties:

RRMPolicyRatio:

$ref: '#/components/schemas/RRMPolicyRatio-Multiple'

EP_E1:

$ref: '#/components/schemas/EP_E1-Single'

EP_XnU:

$ref: '#/components/schemas/EP_XnU-Multiple'

EP_F1U:

$ref: '#/components/schemas/EP_F1U-Multiple'

EP_NgU:

$ref: '#/components/schemas/EP_NgU-Multiple'

EP_X2U:

$ref: '#/components/schemas/EP_X2U-Multiple'

EP_S1U:

$ref: '#/components/schemas/EP_S1U-Multiple'

Configurable5QISet:

$ref: 'TS28541_5GcNrm.yaml#/components/schemas/Configurable5QISet-Multiple'

Dynamic5QISet:

$ref: 'TS28541_5GcNrm.yaml#/components/schemas/Dynamic5QISet-Multiple'

GNBCUCPFunction-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

gnbId:

$ref: '#/components/schemas/GnbId'

gnbIdLength:

$ref: '#/components/schemas/GnbIdLength'

gnbCuName:

$ref: '#/components/schemas/GnbName'

plmnId:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/PlmnId'

x2BlockList:

$ref: '#/components/schemas/GgNBIdList'

xnBlockList:

$ref: '#/components/schemas/GgNBIdList'

x2AllowList:

$ref: '#/components/schemas/GgNBIdList'

xnAllowList:

$ref: '#/components/schemas/GgNBIdList'

x2HOBlockList:

$ref: '#/components/schemas/GeNBIdList'

xnHOBlockList:

$ref: '#/components/schemas/GgNBIdList'

mappingSetIDBackhaulAddressList:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/MappingSetIDBackhaulAddress'

minItems: 1

isOnboardSatellite:

type: boolean

onboardSatelliteId:

$ref: '#/components/schemas/SatelliteId'

tceIDMappingInfoList:

$ref: '#/components/schemas/TceIDMappingInfoList'

configurable5QISetRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

dynamic5QISetRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnRo'

ephemerisInfoSetRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnRo'

dCHOControl:

type: boolean

dDAPSHOControl:

type: boolean

mappedCellIdInfoList:

$ref: '#/components/schemas/MappedCellIdInfoList'

qceIdMappingInfoList:

$ref: '#/components/schemas/QceIdMappingInfoList'

mdtUserConsentReqList:

$ref: '#/components/schemas/MdtUserConsentReqList'

mWABRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

nRECMappingRuleRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-ncO'

- type: object

properties:

RRMPolicyRatio:

$ref: '#/components/schemas/RRMPolicyRatio-Multiple'

NRCellCU:

$ref: '#/components/schemas/NRCellCU-Multiple'

EP_XnC:

$ref: '#/components/schemas/EP_XnC-Multiple'

EP_E1:

$ref: '#/components/schemas/EP_E1-Multiple'

EP_F1C:

$ref: '#/components/schemas/EP_F1C-Multiple'

EP_NgC:

$ref: '#/components/schemas/EP_NgC-Multiple'

EP_X2C:

$ref: '#/components/schemas/EP_X2C-Multiple'

DANRManagementFunction:

$ref: '#/components/schemas/DANRManagementFunction-Single'

DESManagementFunction:

$ref: '#/components/schemas/DESManagementFunction-Single'

DMROFunction:

$ref: '#/components/schemas/DMROFunction-Single'

DLBOFunction:

$ref: '#/components/schemas/DLBOFunction-Single'

Configurable5QISet:

$ref: 'TS28541_5GcNrm.yaml#/components/schemas/Configurable5QISet-Multiple'

Dynamic5QISet:

$ref: 'TS28541_5GcNrm.yaml#/components/schemas/Dynamic5QISet-Multiple'

NRNetwork:

$ref: '#/components/schemas/NRNetwork-Single'

EUtranNetwork:

$ref: '#/components/schemas/EUtraNetwork-Single'

NRCellCU-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

cellLocalId:

type: integer

plmnInfoList:

$ref: '#/components/schemas/PlmnInfoList'

nRFrequencyRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnRo'

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-ncO'

- type: object

properties:

RRMPolicyRatio:

$ref: '#/components/schemas/RRMPolicyRatio-Multiple'

NRCellRelation:

$ref: '#/components/schemas/NRCellRelation-Multiple'

EUtranCellRelation:

$ref: '#/components/schemas/EUtranCellRelation-Multiple'

NRFreqRelation:

$ref: '#/components/schemas/NRFreqRelation-Multiple'

EUtranFreqRelation:

$ref: '#/components/schemas/EUtranFreqRelation-Multiple'

DESManagementFunction:

$ref: '#/components/schemas/DESManagementFunction-Single'

DMROFunction:

$ref: '#/components/schemas/DMROFunction-Single'

DLBOFunction:

$ref: '#/components/schemas/DLBOFunction-Single'

CESManagementFunction:

$ref: '#/components/schemas/CESManagementFunction-Single'

DPCIConfigurationFunction:

$ref: '#/components/schemas/DPCIConfigurationFunction-Single'

NRCellDU-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

administrativeState:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AdministrativeState'

operationalState:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/OperationalState'

cellLocalId:

type: integer

cellState:

$ref: '#/components/schemas/CellState'

plmnInfoInfoList:

$ref: '#/components/schemas/PlmnInfoList'

nPNIdentityList:

$ref: '#/components/schemas/NPNIdentityList'

nrPci:

$ref: '#/components/schemas/NrPci'

nRTAC:

$ref: 'TS28623_GenericNrm.yaml#/components/schemas/Tac'

nTNTAClist:

type: array

uniqueItems: true

items:

$ref: 'TS28623_GenericNrm.yaml#/components/schemas/Tac'

minItems: 1

maxItems: 12

arfcnDL:

type: integer

arfcnUL:

type: integer

arfcnSUL:

type: integer

bSChannelBwDL:

type: integer

bSChannelBwUL:

type: integer

bSChannelBwSUL:

type: integer

ssbFrequency:

type: integer

minimum: 0

maximum: 3279165

ssbPeriodicity:

$ref: '#/components/schemas/SsbPeriodicity'

ssbSubCarrierSpacing:

$ref: '#/components/schemas/SsbSubCarrierSpacing'

ssbOffset:

type: integer

minimum: 0

maximum: 159

ssbDuration:

$ref: '#/components/schemas/SsbDuration'

uECellBarredAccess:

type: array

uniqueItems: true

items:

type: string

enum:

- REDCAP_1RX

- REDCAP_2RX

nRSectorCarrierRef:

type: array

uniqueItems: true

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

bWPRef:

description: "Condition is BWP sets are not supported"

type: array

uniqueItems: true

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

bWPSetRef:

description: "Condition is BWP sets are supported"

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

rimRSMonitoringStartTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

redCapAccessCriteriaRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

rimRSMonitoringStopTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

rimRSMonitoringWindowDuration:

type: integer

rimRSMonitoringWindowStartingOffset:

type: integer

rimRSMonitoringWindowPeriodicity:

type: integer

rimRSMonitoringOccasionInterval:

type: integer

rimRSMonitoringOccasionStartingOffset:

type: integer

nRFrequencyRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

victimSetRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

aggressorSetRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-ncO'

- type: object

properties:

RRMPolicyRatio:

$ref: '#/components/schemas/RRMPolicyRatio-Multiple'

CPCIConfigurationFunction:

$ref: '#/components/schemas/CPCIConfigurationFunction-Single'

DRACHOptimizationFunction:

$ref: '#/components/schemas/DRACHOptimizationFunction-Single'

BWPSet-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

bWPList:

type: array

uniqueItems: true

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

maxItems: 12

NROperatorCellDU-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

cellLocalId:

type: integer

administrativeState:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AdministrativeState'

plmnInfoList:

$ref: '#/components/schemas/PlmnInfoList'

nRTAC:

$ref: 'TS28623_GenericNrm.yaml#/components/schemas/Tac'

NRFrequency-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

absoluteFrequencySSB:

type: integer

minimum: 0

maximum: 3279165

ssbSubCarrierSpacing:

$ref: '#/components/schemas/SsbSubCarrierSpacing'

multiFrequencyBandListNR:

type: integer

minimum: 1

maximum: 256

readOnly: true

EUtranFrequency-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

earfcnDL:

type: integer

minimum: 0

maximum: 262143

multiBandInfoListEutra:

type: integer

minimum: 1

maximum: 256

NRSectorCarrier-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

txDirection:

$ref: '#/components/schemas/TxDirection'

configuredMaxTxPower:

type: integer

arfcnDL:

type: integer

arfcnUL:

type: integer

bSChannelBwDL:

type: integer

bSChannelBwUL:

type: integer

sectorEquipmentFunctionRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-ncO'

- type: object

properties:

CommonBeamformingFunction:

$ref: '#/components/schemas/CommonBeamformingFunction-Single'

BWP-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

bwpContext:

$ref: '#/components/schemas/BwpContext'

isInitialBwp:

$ref: '#/components/schemas/IsInitialBwp'

subCarrierSpacing:

type: integer

cyclicPrefix:

$ref: '#/components/schemas/CyclicPrefix'

startRB:

type: integer

numberOfRBs:

type: integer

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-ncO'

CommonBeamformingFunction-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- type: object

properties:

coverageShape:

$ref: '#/components/schemas/CoverageShape'

digitalAzimuth:

$ref: '#/components/schemas/DigitalAzimuth'

digitalTilt:

$ref: '#/components/schemas/DigitalTilt'

- type: object

properties:

Beam:

$ref: '#/components/schemas/Beam-Multiple'

CCOWeakCoverageParameters:

$ref: '#/components/schemas/CCOWeakCoverageParameters-Single'

CCOPilotPollutionParameters:

$ref: '#/components/schemas/CCOWeakCoverageParameters-Single'

CCOOvershootCoverageParameters:

$ref: '#/components/schemas/CCOOvershootCoverageParameters-Single'

Beam-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- type: object

properties:

beamIndex:

type: integer

readOnly: true

beamType:

type: string

readOnly: true

enum:

- SSB_BEAM

beamAzimuth:

type: integer

readOnly: true

minimum: -1800

maximum: 1800

beamTilt:

type: integer

readOnly: true

minimum: -900

maximum: 900

beamHorizWidth:

type: integer

readOnly: true

minimum: 0

maximum: 3599

beamVertWidth:

type: integer

readOnly: true

minimum: 0

maximum: 1800

RRMPolicyRatio-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- $ref: '#/components/schemas/RRMPolicy_-Attr'

- type: object

properties:

rRMPolicyMaxRatio:

type: integer

default: 100

minimum: 0

maximum: 100

rRMPolicyMinRatio:

type: integer

default: 0

minimum: 0

maximum: 100

rRMPolicyDedicatedRatio:

type: integer

default: 0

minimum: 0

maximum: 100

NRCellRelation-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

nRTCI:

type: integer

cellIndividualOffset:

type: array

items:

$ref: '#/components/schemas/QOffsetRange'

minItems: 6

maxItems: 6

adjacentNRCellRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

nRFreqRelationRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

isRemoveAllowed:

type: boolean

isHOAllowed:

type: boolean

isESCoveredBy:

$ref: '#/components/schemas/IsESCoveredBy'

isENDCAllowed:

type: boolean

isMLBAllowed:

type: boolean

EUtranCellRelation-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

adjacentEUtranCellRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-ncO'

NRFreqRelation-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

offsetMO:

type: array

items:

$ref: '#/components/schemas/QOffsetRange'

minItems: 6

maxItems: 6

blockListEntry:

type: array

uniqueItems: true

items:

type: integer

minimum: 0

maximum: 503

maxItems: 16

blockListEntryIdleMode:

type: array

uniqueItems: true

items:

type: integer

minimum: 0

maximum: 1007

maxItems: 16

cellReselectionPriority:

type: integer

cellReselectionSubPriority:

type: number

minimum: 0.2

maximum: 0.8

multipleOf: 0.2

CellReselectionRedcap:

type: object

properties:

sSearchDeltaPStationary:

type: integer

enum:

- 2

- 3

- 6

- 9

- 12

- 5

tSearchDeltaPStationary:

type: integer

enum:

- 5

- 10

- 20

- 30

- 60

- 120

- 180

- 240

- 300

sSearchThresholdP2:

type: integer

minimum: 0

maximum: 31

sSearchThresholdQ2:

type: integer

minimum: 0

maximum: 31

pMax:

type: integer

minimum: -30

maximum: 33

qOffsetFreq:

$ref: '#/components/schemas/QOffsetFreq'

qQualMin:

type: number

qRxLevMin:

type: integer

minimum: -140

maximum: -44

threshXHighP:

type: integer

minimum: 0

maximum: 62

threshXHighQ:

type: integer

minimum: 0

maximum: 31

threshXLowP:

type: integer

minimum: 0

maximum: 62

threshXLowQ:

type: integer

minimum: 0

maximum: 31

tReselectionNr:

type: integer

minimum: 0

maximum: 7

tReselectionNRSfHigh:

$ref: '#/components/schemas/TReselectionNRSf'

tReselectionNRSfMedium:

$ref: '#/components/schemas/TReselectionNRSf'

nRFrequencyRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

EUtranFreqRelation-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

cellIndividualOffset:

type: array

items:

$ref: '#/components/schemas/QOffsetRange'

minItems: 6

maxItems: 6

blockListEntry:

type: array

uniqueItems: true

items:

type: integer

minimum: 0

maximum: 503

maxItems: 16

blockListEntryIdleMode:

type: array

uniqueItems: true

items:

type: integer

minimum: 0

maximum: 1007

maxItems: 16

cellReselectionPriority:

type: integer

default: 0

cellReselectionSubPriority:

type: number

minimum: 0.2

maximum: 0.8

multipleOf: 0.2

pMax:

type: integer

minimum: -30

maximum: 33

qOffsetFreq:

$ref: '#/components/schemas/QOffsetFreq'

qQualMin:

type: number

qRxLevMin:

type: integer

minimum: -140

maximum: -44

threshXHighP:

type: integer

minimum: 0

maximum: 62

threshXHighQ:

type: integer

minimum: 0

maximum: 31

threshXLowP:

type: integer

minimum: 0

maximum: 62

threshXLowQ:

type: integer

minimum: 0

maximum: 31

tReselectionEutran:

type: integer

minimum: 0

maximum: 7

tReselectionNRSfHigh:

$ref: '#/components/schemas/TReselectionNRSf'

tReselectionNRSfMedium:

$ref: '#/components/schemas/TReselectionNRSf'

eUTranFrequencyRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

DANRManagementFunction-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

intrasystemANRManagementSwitch:

type: boolean

intersystemANRManagementSwitch:

type: boolean

DESManagementFunction-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

desSwitch:

type: boolean

intraRatEsActivationOriginalCellLoadParameters:

$ref: "#/components/schemas/IntraRatEsActivationOriginalCellLoadParameters"

intraRatEsActivationCandidateCellsLoadParameters:

$ref: "#/components/schemas/IntraRatEsActivationCandidateCellsLoadParameters"

intraRatEsDeactivationCandidateCellsLoadParameters:

$ref: "#/components/schemas/IntraRatEsDeactivationCandidateCellsLoadParameters"

esNotAllowedTimePeriod:

$ref: "#/components/schemas/EsNotAllowedTimePeriod"

interRatEsActivationOriginalCellParameters:

$ref: "#/components/schemas/InterRatEsActivationOriginalCellParameters"

interRatEsActivationCandidateCellParameters:

$ref: "#/components/schemas/InterRatEsActivationCandidateCellParameters"

interRatEsDeactivationCandidateCellParameters:

$ref: "#/components/schemas/InterRatEsDeactivationCandidateCellParameters"

isProbingCapable:

type: string

readOnly: true

enum:

- YES

- NO

energySavingState:

type: string

readOnly: true

enum:

- IS_NOT_ENERGY_SAVING

- IS_ENERGY_SAVING

mLModelRefList:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnListRo'

aIMLInferenceFunctionRefList:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnListRo'

DRACHOptimizationFunction-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

drachOptimizationControl:

type: boolean

ueAccProbabilityDist:

$ref: "#/components/schemas/UeAccProbabilityDist"

ueAccDelayProbabilityDist:

$ref: "#/components/schemas/UeAccDelayProbabilityDist"

DMROFunction-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

dmroControl:

type: boolean

maximumDeviationHoTriggerLow:

$ref: '#/components/schemas/MaximumDeviationHoTriggerLow'

maximumDeviationHoTriggerHigh:

$ref: '#/components/schemas/MaximumDeviationHoTriggerHigh'

minimumTimeBetweenHoTriggerChange:

$ref: '#/components/schemas/MinimumTimeBetweenHoTriggerChange'

tstoreUEcntxt:

$ref: '#/components/schemas/TstoreUEcntxt'

mLModelRefList:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnListRo'

aIMLInferenceFunctionRefList:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnListRo'

DLBOFunction-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

dlboControl:

type: boolean

maximumDeviationHoTrigger:

$ref: '#/components/schemas/MaximumDeviationHoTrigger'

minimumTimeBetweenHoTriggerChange:

$ref: '#/components/schemas/MinimumTimeBetweenHoTriggerChange'

mLModelRefList:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnListRo'

aIMLInferenceFunctionRefList:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnListRo'

DPCIConfigurationFunction-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

dPciConfigurationControl:

type: boolean

nRPciList:

$ref: "#/components/schemas/NRPciList"

CPCIConfigurationFunction-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

cPciConfigurationControl:

type: boolean

cSonPciList:

$ref: "#/components/schemas/CSonPciList"

CESManagementFunction-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

cesSwitch:

type: boolean

intraRatEsActivationOriginalCellLoadParameters:

$ref: "#/components/schemas/IntraRatEsActivationOriginalCellLoadParameters"

intraRatEsActivationCandidateCellsLoadParameters:

$ref: "#/components/schemas/IntraRatEsActivationCandidateCellsLoadParameters"

intraRatEsDeactivationCandidateCellsLoadParameters:

$ref: "#/components/schemas/IntraRatEsDeactivationCandidateCellsLoadParameters"

esNotAllowedTimePeriod:

$ref: "#/components/schemas/EsNotAllowedTimePeriod"

interRatEsActivationOriginalCellParameters:

$ref: "#/components/schemas/IntraRatEsActivationOriginalCellLoadParameters"

interRatEsActivationCandidateCellParameters:

$ref: "#/components/schemas/IntraRatEsActivationOriginalCellLoadParameters"

interRatEsDeactivationCandidateCellParameters:

$ref: "#/components/schemas/IntraRatEsActivationOriginalCellLoadParameters"

energySavingControl:

type: string

enum:

- TO_BE_ENERGY_SAVING

- TO_BE_NOT_ENERGY_SAVING

energySavingState:

type: string

enum:

- IS_NOT_ENERGY_SAVING

- IS_ENERGY_SAVING

RimRSGlobal-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

frequencyDomainPara:

$ref: '#/components/schemas/FrequencyDomainPara'

sequenceDomainPara:

$ref: '#/components/schemas/SequenceDomainPara'

timeDomainPara:

$ref: '#/components/schemas/TimeDomainPara'

RimRSSet:

$ref: '#/components/schemas/RimRSSet-Multiple'

RedCapAccessCriteria-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

nRCellDURef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

criteriaConditonRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

RimRSSet-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

setId:

$ref: '#/components/schemas/RSSetId'

setType:

$ref: '#/components/schemas/RSSetType'

nRCellDURefs:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnListRo'

ExternalGNBDUFunction-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

gnbId:

$ref: '#/components/schemas/GnbId'

gnbIdLength:

$ref: '#/components/schemas/GnbIdLength'

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-ncO'

- type: object

properties:

EP_F1C:

$ref: '#/components/schemas/EP_F1C-Multiple'

EP_F1U:

$ref: '#/components/schemas/EP_F1U-Multiple'

NRNetwork-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

NRFrequency:

$ref: '#/components/schemas/NRFrequency-Multiple'

ExternalGNBCUCPFunction:

$ref: '#/components/schemas/ExternalGNBCUCPFunction-Multiple'

ExternalGNBCUUPFunction:

$ref: '#/components/schemas/ExternalGNBCUUPFunction-Multiple'

ExternalGNBDUFunction:

$ref: '#/components/schemas/ExternalGNBDUFunction-Multiple'

ExternalGNBCUUPFunction-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

gnbId:

$ref: '#/components/schemas/GnbId'

gnbIdLength:

$ref: '#/components/schemas/GnbIdLength'

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-ncO'

- type: object

properties:

EP_E1:

$ref: '#/components/schemas/EP_E1-Multiple'

EP_F1U:

$ref: '#/components/schemas/EP_F1U-Multiple'

EP_XnU:

$ref: '#/components/schemas/EP_XnU-Multiple'

ExternalGNBCUCPFunction-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- $ref: >-

TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-Attr

- type: object

properties:

gnbId:

$ref: '#/components/schemas/GnbId'

gnbIdLength:

$ref: '#/components/schemas/GnbIdLength'

plmnId:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/PlmnId'

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-ncO'

- type: object

properties:

ExternalNRCellCU:

$ref: '#/components/schemas/ExternalNRCellCU-Multiple'

EP_XnC:

$ref: '#/components/schemas/EP_XnC-Multiple'

EP_E1:

$ref: '#/components/schemas/EP_E1-Multiple'

EP_F1C:

$ref: '#/components/schemas/EP_F1C-Multiple'

ExternalNRCellCU-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

cellLocalId:

type: integer

nrPci:

$ref: '#/components/schemas/NrPci'

plMNIdList:

type: array

uniqueItems: true

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/PlmnId'

minItems: 1

maxItems: 12

nRFrequencyRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-ncO'

EUtraNetwork-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

EUtranFrequency:

$ref: '#/components/schemas/EUtranFrequency-Multiple'

ExternalENBFunction:

$ref: '#/components/schemas/ExternalENBFunction-Multiple'

ExternalENBFunction-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

eNBId:

type: integer

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-ncO'

- type: object

properties:

ExternalEUTranCell:

$ref: '#/components/schemas/ExternalEUTranCell-Multiple'

ExternalEUTranCell-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

EUtranFrequencyRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-ncO'

EP_XnC-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: '#/components/schemas/LocalAddress'

remoteAddress:

$ref: '#/components/schemas/RemoteAddress'

EP_E1-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: '#/components/schemas/LocalAddress'

remoteAddress:

$ref: '#/components/schemas/RemoteAddress'

EP_F1C-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: '#/components/schemas/LocalAddress'

remoteAddress:

$ref: '#/components/schemas/RemoteAddress'

EP_NgC-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: '#/components/schemas/LocalAddress'

remoteAddress:

$ref: '#/components/schemas/RemoteAddress'

EP_X2C-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: '#/components/schemas/LocalAddress'

remoteAddress:

$ref: '#/components/schemas/RemoteAddress'

EP_XnU-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: '#/components/schemas/LocalAddress'

remoteAddress:

$ref: '#/components/schemas/RemoteAddress'

EP_F1U-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: '#/components/schemas/LocalAddress'

remoteAddress:

$ref: '#/components/schemas/RemoteAddress'

epTransportRefs:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnListRo'

EP_NgU-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: '#/components/schemas/LocalAddress'

remoteAddress:

$ref: '#/components/schemas/RemoteAddress'

epTransportRefs:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnListRo'

EP_X2U-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: '#/components/schemas/LocalAddress'

remoteAddress:

$ref: '#/components/schemas/RemoteAddress'

EP_S1U-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: '#/components/schemas/LocalAddress'

remoteAddress:

$ref: '#/components/schemas/RemoteAddress'

CCOFunction-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

cCOControl:

type: boolean

CCOWeakCoverageParameters:

$ref: '#/components/schemas/CCOWeakCoverageParameters-Single'

CCOPilotPollutionParameters:

$ref: '#/components/schemas/CCOPilotPollutionParameters-Single'

CCOOvershootCoverageParameters-Single:

$ref: '#/components/schemas/CCOOvershootCoverageParameters-Single'

CCOParameters-Attr:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

coverageShapeList:

type: array

items:

$ref: '#/components/schemas/CoverageShape'

downlinkTransmitPowerRange:

$ref: '#/components/schemas/ParameterRange'

antennaTiltRange:

$ref: '#/components/schemas/ParameterRange'

antennaAzimuthRange:

$ref: '#/components/schemas/ParameterRange'

digitalTiltRange:

$ref: '#/components/schemas/ParameterRange'

digitalAzimuthRange:

$ref: '#/components/schemas/ParameterRange'

CCOWeakCoverageParameters-Single:

$ref: '#/components/schemas/CCOParameters-Attr'

CCOPilotPollutionParameters-Single:

$ref: '#/components/schemas/CCOParameters-Attr'

CCOOvershootCoverageParameters-Single:

$ref: '#/components/schemas/CCOParameters-Attr'

NTNFunction-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

nTNpLMNInfoList:

$ref: '#/components/schemas/PlmnInfoList'

nTNTAClist:

$ref: '#/components/schemas/NRTACList'

EphemerisInfoSet:

$ref: '#/components/schemas/EphemerisInfoSet-Multiple'

nTNTimeBasedConfig:

$ref: '#/components/schemas/NTNTimeBasedConfig-Multiple'

EphemerisInfoSet-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- type: object

properties:

ephemerisInfos:

$ref: '#/components/schemas/EphemerisInfos'

MWAB-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

administrativeState:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AdministrativeState'

operationalState:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/OperationalState'

allowedArea:

type: array

uniqueItems: true

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/GeoArea'

allowedTime:

type: array

uniqueItems: true

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/TimeWindow'

currentLocation:

$ref: '#/components/schemas/LocationInfo'

NRECMappingRule-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- type: object

properties:

ecMRInputMinimumValue:

type: integer

ecMRInputMaximumValue:

type: integer

ecTimeInterval:

type: integer

NTNTimeBasedConfig-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- type: object

properties:

timeWindow:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/TimeWindow'

nTNEntityConfigList:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/NTNEntityConf'

minItems: 1

AIOTReader-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

readerId:

type: integer

administrativeState:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AdministrativeState'

supportedAIOTServices:

type: array

uniqueItems: true

items:

type: string

enum:

- INVENTORY

- COMMAND

plmnId:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/PlmnId'

servedAIOTAreas:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/ServedAIOTAreaID'

readerLocation:

type: string

nRSectorCarrierRef:

type: array

uniqueItems: true

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

#-------- Definition of JSON arrays for name-contained IOCs ----------------------

GNBDUFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/GNBDUFunction-Single'

OperatorDU-Multiple:

type: array

items:

$ref: '#/components/schemas/OperatorDU-Single'

GNBCUUPFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/GNBCUUPFunction-Single'

GNBCUCPFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/GNBCUCPFunction-Single'

BWPSet-Multiple:

type: array

items:

$ref: '#/components/schemas/BWPSet-Single'

NRCellDU-Multiple:

type: array

items:

$ref: '#/components/schemas/NRCellDU-Single'

NROperatorCellDU-Multiple:

type: array

items:

$ref: '#/components/schemas/NROperatorCellDU-Single'

NRCellCU-Multiple:

type: array

items:

$ref: '#/components/schemas/NRCellCU-Single'

NRFrequency-Multiple:

type: array

minItems: 1

items:

$ref: '#/components/schemas/NRFrequency-Single'

EUtranFrequency-Multiple:

type: array

minItems: 1

items:

$ref: '#/components/schemas/EUtranFrequency-Single'

NRSectorCarrier-Multiple:

type: array

items:

$ref: '#/components/schemas/NRSectorCarrier-Single'

BWP-Multiple:

type: array

items:

$ref: '#/components/schemas/BWP-Single'

Beam-Multiple:

type: array

items:

$ref: '#/components/schemas/Beam-Single'

RRMPolicyRatio-Multiple:

type: array

items:

$ref: '#/components/schemas/RRMPolicyRatio-Single'

NRCellRelation-Multiple:

type: array

items:

$ref: '#/components/schemas/NRCellRelation-Single'

EUtranCellRelation-Multiple:

type: array

items:

$ref: '#/components/schemas/EUtranCellRelation-Single'

NRFreqRelation-Multiple:

type: array

items:

$ref: '#/components/schemas/NRFreqRelation-Single'

EUtranFreqRelation-Multiple:

type: array

items:

$ref: '#/components/schemas/EUtranFreqRelation-Single'

RimRSSet-Multiple:

type: array

items:

$ref: '#/components/schemas/RimRSSet-Single'

ExternalGNBDUFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/ExternalGNBDUFunction-Single'

ExternalGNBCUUPFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/ExternalGNBCUUPFunction-Single'

ExternalGNBCUCPFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/ExternalGNBCUCPFunction-Single'

ExternalNRCellCU-Multiple:

type: array

items:

$ref: '#/components/schemas/ExternalNRCellCU-Single'

ExternalENBFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/ExternalENBFunction-Single'

ExternalEUTranCell-Multiple:

type: array

items:

$ref: '#/components/schemas/ExternalEUTranCell-Single'

EP_E1-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_E1-Single'

EP_XnC-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_XnC-Single'

EP_F1C-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_F1C-Single'

RedCapAccessCriteria-Multiple:

type: array

items:

$ref: '#/components/schemas/RedCapAccessCriteria-Single'

EP_NgC-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_NgC-Single'

EP_X2C-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_X2C-Single'

EP_XnU-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_XnU-Single'

EP_F1U-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_F1U-Single'

EP_NgU-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_NgU-Single'

EP_X2U-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_X2U-Single'

EP_S1U-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_S1U-Single'

EphemerisInfoSet-Multiple:

type: array

items:

$ref: '#/components/schemas/EphemerisInfoSet-Single'

NRECMappingRule-Multiple:

type: array

items:

$ref: '#/components/schemas/NRECMappingRule-Single'

NTNTimeBasedConfig-Multiple:

type: array

items:

$ref: '#/components/schemas/NTNTimeBasedConfig-Single'

MWAB-Multiple:

type: array

items:

$ref: '#/components/schemas/MWAB-Single'

AIOTReader-Multiple:

type: array

items:

$ref: '#/components/schemas/AIOTReader-Single'

#-------- Definitions in TS 28.541 for TS 28.532 ---------------------------------

resources-nrNrm:

oneOf:

- $ref: '#/components/schemas/GNBDUFunction-Single'

- $ref: '#/components/schemas/GNBCUUPFunction-Single'

- $ref: '#/components/schemas/GNBCUCPFunction-Single'

- $ref: '#/components/schemas/OperatorDU-Single'

- $ref: '#/components/schemas/NRCellCU-Single'

- $ref: '#/components/schemas/NRCellDU-Single'

- $ref: '#/components/schemas/NROperatorCellDU-Single'

- $ref: '#/components/schemas/NRNetwork-Single'

- $ref: '#/components/schemas/EUtraNetwork-Single'

- $ref: '#/components/schemas/NRFrequency-Single'

- $ref: '#/components/schemas/EUtranFrequency-Single'

- $ref: '#/components/schemas/NRSectorCarrier-Single'

- $ref: '#/components/schemas/BWP-Single'

- $ref: '#/components/schemas/BWPSet-Single'

- $ref: '#/components/schemas/CommonBeamformingFunction-Single'

- $ref: '#/components/schemas/Beam-Single'

- $ref: '#/components/schemas/RRMPolicyRatio-Single'

- $ref: '#/components/schemas/NRCellRelation-Single'

- $ref: '#/components/schemas/EUtranCellRelation-Single'

- $ref: '#/components/schemas/NRFreqRelation-Single'

- $ref: '#/components/schemas/EUtranFreqRelation-Single'

- $ref: '#/components/schemas/DANRManagementFunction-Single'

- $ref: '#/components/schemas/DESManagementFunction-Single'

- $ref: '#/components/schemas/DRACHOptimizationFunction-Single'

- $ref: '#/components/schemas/DMROFunction-Single'

- $ref: '#/components/schemas/DLBOFunction-Single'

- $ref: '#/components/schemas/DPCIConfigurationFunction-Single'

- $ref: '#/components/schemas/CPCIConfigurationFunction-Single'

- $ref: '#/components/schemas/CESManagementFunction-Single'

- $ref: '#/components/schemas/RimRSGlobal-Single'

- $ref: '#/components/schemas/RimRSSet-Single'

- $ref: '#/components/schemas/ExternalGNBDUFunction-Single'

- $ref: '#/components/schemas/ExternalGNBCUUPFunction-Single'

- $ref: '#/components/schemas/ExternalGNBCUCPFunction-Single'

- $ref: '#/components/schemas/ExternalNRCellCU-Single'

- $ref: '#/components/schemas/ExternalENBFunction-Single'

- $ref: '#/components/schemas/ExternalEUTranCell-Single'

- $ref: '#/components/schemas/EP_XnC-Single'

- $ref: '#/components/schemas/EP_E1-Single'

- $ref: '#/components/schemas/EP_F1C-Single'

- $ref: '#/components/schemas/EP_NgC-Single'

- $ref: '#/components/schemas/EP_X2C-Single'

- $ref: '#/components/schemas/EP_XnU-Single'

- $ref: '#/components/schemas/EP_F1U-Single'

- $ref: '#/components/schemas/EP_NgU-Single'

- $ref: '#/components/schemas/EP_X2U-Single'

- $ref: '#/components/schemas/EP_S1U-Single'

- $ref: '#/components/schemas/CCOFunction-Single'

- $ref: '#/components/schemas/CCOWeakCoverageParameters-Single'

- $ref: '#/components/schemas/CCOPilotPollutionParameters-Single'

- $ref: '#/components/schemas/CCOOvershootCoverageParameters-Single'

- $ref: '#/components/schemas/NTNFunction-Single'

- $ref: '#/components/schemas/EphemerisInfoSet-Single'

- $ref: '#/components/schemas/MWAB-Single'

- $ref: '#/components/schemas/NRECMappingRule-Single'

- $ref: '#/components/schemas/NTNTimeBasedConfig-Single'

- $ref: '#/components/schemas/RedCapAccessCriteria-Single'

- $ref: '#/components/schemas/AIOTReader-Single'

<CODE ENDS>

*** END OF CHANGE 1 ***

End of Changes