3GPP TSG-SA5 Meeting #163	S5-254824

Wu Han, China, 13 - 17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.291 | CR | 0648 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 32.291 Correction on Cardinality and Presence |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-SBI_CH, TEI19 |  |  |  |  |  | Date: |  |  | 2025-10-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to the clause 5.2.4.2 Structured data types of TS 29.501, a cardinality of "M..N", is only allowed for data types "array(<type>)" and "map(<type>)". In order to keep alignment with the principle of data type definition specified in TS 29.501, the charging attributes should be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The charging data types are not aligned with the principles and guidelines for service definition. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Correct the Cardinality and Presence condition |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.6.2.2.8,6.1.6.2.4.3, 6.1.6.2.9.3,6.1.6.2.10.3, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of S5-254371. |  |  |  |  |  |  |  |  |

First Change

6.1.6.2.2.8	Type PDUSessionInformation

Table 6.1.6.2.2.8-1: Definition of type PDUSessionInformation

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| networkSlicingInfo | NetworkSlicingInfo | OM | 0..1 | information of network slice serving the PDU session |  |
| pduSessionID | PduSessionId | M | 1 | identifier of the PDU session |  |
| pduType | PduSessionType | OM | 0..1 | type of the PDU session, the PDN type non-IP is mapped to unstructured |  |
| sscMode | SscMode | OC | 0..1 | information of SSC Mode type. |  |
| hPlmnId | PlmnId | OC | 0..1 | PLMN identifier of the home network |  |
| servingNetworkFunctionID | ServingNetworkFunctionID | OC | 0..1 | This field holds serving Network Function identifier. |  |
| servingCNPlmnId | PlmnId | OC | 0..1 | Serving Core Network Operator PLMN ID selected by the UE in shared networks. |  |
| ratType | RatType | OC | 0..1 | the RAT Type of the PDU session |  |
| mAPDUNon3GPPRATType | RatType | OC | 0..1 | the RAT Type of non-3GPP access for the MA PDU session | ATSSS |
| dnnId | Dnn | M | 1 | a Data Network Name |  |
| dnnSelectionMode | DnnSelectionMode | OC | 0..1 | This field indicates how the DNN was selected. |  |
| chargingCharacteristics | string | OC | 0..1 | the Charging Characteristics for this PDU session. It carries the value in hexadecimal representation Pattern: '^[0-9a-fA-F]{1,4}$' |  |
| chargingCharacteristicsSelectionMode | ChargingCharacteristicsSelectionMode | OC | 0..1 | information about how the "Charging Characteristics" was selected. |  |
| startTime | DateTime | OC | 0..1 | the UTC time which represents the start of a PDU session at the SMF |  |
| stopTime | DateTime | OC | 0..1 | the UTC time which represents the stop of a PDU session at the SMF |  |
| 3gppPSDataOffStatus | 3GPPPSDataOffStatus | OC | 0..1 | This field holds the 3GPP Data off Status when UE’s 3GPP Data Off status is Activated or Deactivated. |  |
| sessionStopIndicator | boolean | OC | 0..1 | This field indicates to the CHF that the PDU session has been terminated. |  |
| pduAddress | PDUAddress | OC | 0..1 | Group of user ip address/prefix |  |
| diagnostics | Diagnostics | OC | 0..1 | provides a detailed cause value from SMF. |  |
| enhancedDiagnostics | EnhancedDiagnostics5G | OC | 0..1 | provides a more detailed cause value from SMF. | EnhancedDiagnostics |
| authorizedQoSInformation | AuthorizedDefaultQos | OC | 0..1 | This field holds the authorized QoS applied to PDU session. |  |
| subscribedQoSInformation | SubscribedDefaultQos | OC | 0..1 | This field holds the subscribed Default QoS |  |
| authorizedSessionAMBR | Ambr | OC | 0..1 | This field holds the authorized session-AMBR. |  |
| subscribedSessionAMBR | Ambr | OC | 0..1 | This field holds the subscribed session-AMBR. |  |
| mAPDUSessionInformation | MAPDUSessionInformation | OC | 0..1 | This field holds the MA PDU session information. | ATSSS |
| redundantTransmissionType | RedundantTransmissionType | OC | 0..1 | Indicates the redundant transmission type. If this field isn’t present, it should be seen as a non-redundant transmission. | URLLC |
| pDUSessionPairID | Uint32 | OC | 0..1 | This field identifies the two redundant PDU Sessions that belong together for dual connectivity based end to end redundant user plane paths type. | URLLC |
| cpCIoTOptimisationIndicator | boolean | OC | 0..1 | This field holds the indicator whether control plane optimization CIoT for 5GS is used during the PDU session, if this feature is enabled. The default value is false. | 5GSCIoT |
| 5GSControl PlaneOnlyIndicator | boolean | OC | 0..1 | This field holds the indicator whether the control plane only is used, i.e., the PDU data only transfers to control plane in case of control plane CIoT optimization. The default value is false. | 5GSCIoT |
| smallDataRateControlIndicator | boolean | OC | 0..1 | This field holds the indicator whether the small data rate control for 5GS CIoT is used during the PDU session. The default value is false. | 5GSCIoT |
| 5GLANTypeService | 5GLANTypeService | OC | 0..1 | 5G LAN Type service information, if present, the 5G LAN Type service is used. | 5GLAN |
| sNPNInformation | SNPNInformation | OC | 0..1 | This field holds information associated to SNPN. | SNPN |
| 5GMulticastService | 5GMulticastService | OC | 0..1 | 5G Multicast service information, if present, the 5G MBS service is used. | 5MBS_CH |
| 5GSBridgeInformation | 5GSBridgeInformation | OC | 0..1 | This field holds the bridge information of the 5GS TSN, including bridge ID and port numbers. | TSN |
| satelliteAccessIndicator | boolean | OC | 0..1 | This field holds the indicator whether the Satellite Access is used during the PDU session. The default value is false. | SatelliteAccess |
| satelliteBackhaulInformation | SatelliteBackhaulInformation | OC | 0..1 | Satellite backhaul Information, if present, the Satellite backhaul is used. | 5GSATB |

Next Change

6.1.6.2.4.3	Type RegistrationChargingInformation

Table 6.1.6.2.4.3-1: Definition of type RegistrationChargingInformation

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| registrationMessagetype | RegistrationMessageType | M | 1 | Message type received by the AMF: registration (initial, initial, mobility, periodic, emergency), deregistration. |  |
| userInformation | UserInformation | OM | 0..1 | Includes information of user and user equipment |  |
| userLocationinfo | UserLocation | OC | 0..1 | Information on the location and location time |  |
| pSCellInformation | PSCellInformation | OC | 0..1 | Primary SCG (Secondary Cell Group) Cell |  |
| uetimeZone | TimeZone | OC | 0..1 | UE Timezone the UE is currently located |  |
| rATType | RatType | OC | 0..1 | RAT Type of the registration |  |
| 5gMMCapability | Bytes | OC | 0..1 | 5GMM capability IE as specified in clause 9.11.3.1 of 3GPP TS 24.501 [303] |  |
| mICOModeIndication | MICOModeIndication | OC | 0..1 | Indicates whether the requested use of MICO mode is accepted or not by the network |  |
| smsIndication | SmsIndication | OC | 0..1 | Indicates whether the SMS delivery over NAS is supported |  |
| taiList | array(Tai) | OC | 0..N | An array of TAIs representing the set of tracking areas composing the Registration Area. |  |
| serviceAreaRestriction | ServiceAreaRestriction | OC | 0..1 | Service Area Restriction for the UE. |  |
| requestedNSSAI | array(Snssai) | OC | 0..N | Requested NSSAI. |  |
| allowedNssai | array(Snssai) | OC | 0..N | Allowed NSSAI. |  |
| rejectedNSSAI | array(Snssai) | OC | 0..N | Rejected NSSAI. |  |
| nSSAIMapList | array(NSSAIMap) | OC | 0..N | Mapping of VPLMN S-NSSAIs to HPLMN S-NSSAIs. |  |
| alternativeNSSAIMap | array(AlternativeNSSAIMap) | OC | 0..N | Mapping of S-NSSAIs to be replaced and alternative S-NSSAIs. | NSREP |
| amfUeNgapId | integer | OM | 0..1 | UE association over the N2 interface within the AMF. |  |
| ranUeNgapId | integer | OM | 0..1 | RAN UE NGAP ID over N2 interface |  |
| ranNodeId | GlobalRanNodeId | OC | 0..1 | Identity of the RAN node. |  |
| sNPNID | PlmnIdNid | OC | 0..1 | This field holds PLMN ID and the NID which identifies the SNPN. | SNPN |
| cAGIDList | array(CagId) | OC | 0..N | This field holds the Closed Access Group Identifier List. |  |
| satelliteAccessIndicator | boolean | OC | 0..1 | This field holds the indicator whether the Satellite Access is used. | SatelliteAccess |

Next Change

6.1.6.2.9.3	Type VariablePart

Table 6.1.6.2.9.3-1: Definition of type VariablePart

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| variablePartType | VariablePartType | M | 1 | the type of the variable part i.e., how the value is to be interpreted. |  |
| variablePartValue | array(string) | M | 1..N | the variable part to be played. |  |
| variablePartOrder | Uint32 | OC | 0..1 | The order in which the variable part shall be played, where zero is the first. If the field is not present or several have the same priority, the order is receiver dependent. |  |

Next Change

6.1.6.2.10.3	Type MMTelChargingInformation

Table 6.1.6.2.10.3-1: Definition of type MMTelChargingInformation

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| supplementaryServices | array(SupplementaryService) | OM | 1..N | This field holds the associated supplementary services. It can be present multiple times as necessary to present the parallel activity of the different supplementary services. |  |

End of Change