3GPP TSG-SA5 Meeting #164	S5-255265

Dallas, USA, 17 - 21 November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0755 | rev | - | Current version: | 20.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-20 CR TS 28.552 Add N6 Measurements for XRM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2-OAM |  |  |  |  |  | Date: |  |  | 2025-11-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As specified in the clause 5.37.8 in TS 23.501, traffic assistance information may be provided by the CN to NG-RAN in order to configure UE power saving management scheme for connected mode DRX, which which may include UL and/or DL Periodicity and N6 Jitter Information associated with the DL Periodicity. Therefore, to support the feature above, new measurements should be specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add XRM service related measurements： - N6 DL Periodicity in PSA UPF - Average N6 Jitter in PSA UPF |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Cannot monitor the N6 Traffic Parameters for XRM measurement |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.X(new), 5.4.X.1(new), 5.4.X.2(new), A.X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of First change

5.4.2.X	N6 Traffic Parameters

5.4.2.X.1	N6 DL Periodicity in PSA UPF

a)	This measurement provides the N6 DL Periodicity in PSA UPF.

b)	DER(N=1)

c)	This measurement is obtained according to the following method:

For each data burst (End of Data Burst, see TS 26.522), the UPF records the following information:

- T is the time stamp when the PSA UPF receives the last PDU of a Data Burst at N6 interface.

The UPF counts the number (N) of Data Burst and takes the following calculation:

i=1N(Ti+1-Ti)N-1

d)	Each measurement is an integer value representing the data rate in microseconds.

e)	The measurement name has the form RTP.N6DlPerioPsaUpf

f)	UPFFunction

g)	Valid for packet switched traffic.

h)	5GS.

5.4.X.2	Average N6 Jitter in PSA UPF

a)	This measurement provides the N6 Jitter in PSA UPF.

b)	DER(N=1)

c)	This measurement is obtained according to the following method:

For each data burst (End of Data Burst, see TS 26.522), the UPF records the following information:

- T is the time stamp when the PSA UPF receives the last PDU of a Data Burst at N6 interface.

- TPerio is the N6 DL Periodicity in PSA UPF, which may be measured in PSA UPF (see clause 5.4.2.X.1) or provided by SMF (see clause 5.37.8.2 in TS 23.501)

The UPF counts the number (N) of Data Burst and takes the following calculation:

i=1N(Ti-TPerio)N-1

d)	Each measurement is an integer value representing the Jitter in microseconds.

e)	The measurement name has the form RTP.N6JitterPsaUpfMean

f)	UPFFunction

g)	Valid for packet switched traffic.

h)	5GS.

Start of Second change

A.X	Use case for monitoring of Periodicity and N6 Jitter Information associated with Periodicity for XR and Media Service

eXtended Reality (XR) and Media service is considered as one of the key candidate services in 5GA and 6G networks. As specified in the clause 5.37.8 in TS 23.501, traffic assistance information may be provided by the CN to NG-RAN in order to configure UE power saving management scheme for connected mode DRX, which may include UL and/or DL Periodicity and N6 Jitter Information associated with the DL Periodicity.

If the measurement of N6 Jitter Information associated with the DL Periodicity is required and the DL Periodicity is available at the SMF, the SMF sends the DL Periodicity to the UPF. The UPF reports the measured N6 Traffic Parameters to SMF via N4 interface. To support the functionality above, the management system should support the measurements of N6 Traffic Periodicity and N6 Jitter at PSA UPF.

Start of Second change